3GPP TSG-RAN WG4 Meeting # 116bis	R4- 2513499

# Prague, Czech, 13th – 17th October, 2025

Source:	LG Electronics

Title:	Discussion on inter-vendor training collaboration and interoperability for two-sided models

Agenda item:	7.9.3

Document for:	Discussion

Introduction

In WID [1], objectives for CSI compression using two-sided model are captured as follows:

Inter-vendor training collaboration for two-sided AI/ML models - Fully defined/specified reference model (“Direction C”) with RAN1 scalability study outcome taken into account [RAN4/RAN1] – check-point in RAN#110 upon RAN4 feedback - Specification of standardized encoder model structure plus parameter exchange (“Direction A, sub-option 3a-1” without target CSI sharing) leveraging defined/reference model of “Direction C” and taking RAN1 scalability study outcome into account [RAN4/RAN1/RAN2/RAN3] – check-point in RAN#110 upon SA WG feedback - Specification of standardized dataset format/content plus dataset exchange (“Direction A, sub-option 4-1”) [RAN1/RAN2/RAN3/RAN4] – check-point in RAN#110 upon SA WG feedback Interoperability and RRM requirement [RAN4]: - To define the requirement for the encoder, specify the standardized test decoder to guarantee interoperability - The fully specified test decoder, data set, and corresponding reference encoder in RAN4 to be used for inter-vendor collaboration - Necessary RRM requirement for specified LCM procedures. - NOTE: offline training is assumed for the purpose of this project. Strive to leverage on RAN1’s study on scalable model structure in FS_NR_AIML_Air.

In this contribution, we provide our views on inter-vendor training collaboration and interoperability for two-sided models.

Discussion

According to WF [2], following issues are supposed to be discussed in RAN4#116bis.

- Guidance for the WI discussion for 2-sided CSI The following were noted as issues needing discussion during the WI. They are not agreements, but companies are encouraged to contribute on these topics: - Discuss and align on the dataset assumptions and dataset - Discuss and align on assumptions for the test decoder and reference encoder structures - Also may need to consider how many test decoders and encoders - Discuss and align on the scalability alternatives - Discuss and agree on expectations on whether the decoder/encoder for RAN4 are the same as for option C - Discuss whether to decide this before or after agreeing the model - Develop a means to capture model structures and model parameters in 3GPP - Discuss and agree the test metric for the performance requirements. - Discuss and agree channel model for the requirements. - For the dataset, discuss the following: - Eigenvectors or raw channel (or both) ? - What it is important to capture within the dataset - metrics for checking alignment - For example,. PSE - Consider examples from SI datasets that did not align

  - Simulation results

In order to verify the feasibility of common decoder which is compatible with encoders with different backbones, our simulation results of low-complexity encoders with different backbones, i.e., transformer, CNN and MLP corresponding to Encoder-2, Encoder-3 and Encoder-4, respectively, are provided in Table 1. Detailed evaluation assumptions are provided in [3].

Table 1. Simulation results for AI based CSI compression with low-complexity encoders having different backbones

| Option 3 low-complexity encoders with different backbones |  |  |  |
| --- | --- | --- | --- |
|  | （1-to-1）Joint training | （N-to-1）Joint training SGCS-4 Step-1 | New encoder training with frozen decoder from Step-1 SGCS-4 Step-2 |
| Encoder-1 (Agreed encoder, CNN with 47M FLOPs) | SGCS-0 | SGCS-4 (step-1) | SGCS-4 (step-2) |
|  | 0.7044 | 0.6847 | 0.6929 |
| Encoder-2 (Transformer with 3.63M FLOPs) | SGCS-3 | SGCS-4 (step-1) | SGCS-4 (step-2) |
|  | - | 0.6905 | 0.6920 |
| Encoder-3 (CNN with 4.14M FLOPs) | SGCS-3 | SGCS-4 (step-1) | SGCS-4 (step-2) |
|  | - | 0.6790 | 0.6815 |
| Encoder-4 (MLP with 0.22M FLOPs) | SGCS-3 | SGCS-4 (step-1) | SGCS-4 (step-2) |
|  | - | 0.6680 | 0.6674 |

Based on above results, following can be observed.

Observation 1: Compared to 1-to-1 joint training, N-to-1 joint training provides marginal performance loss of around 1.7%.

Observation 2: Low complexity encoder with Transformer backbone provides almost the same performance to that of Encoder 1 which has 13 times higher computational complexity.

  - Assumptions for the test decoder and reference encoder structures

In [2], following agreement regarding criteria for selecting the reference encoder/decoder models were made as captured below.

Criteria for selecting the encoder/decoder model: Agreements: - Achievable performance, complexity and robustness in different conditions need to be taken into account in selecting the model. - Whether a single or multiple test decoders is needed will need more elaboration during the WI phase - For option 3 the reference encoder should have been joint trained with the test decoder.

Regarding on the number of decoders, in our view, a single decoder is preferred. The generalization performance of a single decoder can be achieved by training the test decoder with mixed dataset obtained from various deployment scenarios, frequency bands, antenna configurations, etc. However. according to the Rel-19 study, there is no evaluation whether a single decoder can be generalized across all considered scenarios. If the generalization cannot be achieved with a single decoder, multiple decoder can be considered, and this may lead to a burden on the storage in 3GPP. Therefore, if multiple decoder is indeed needed, the max number of test decoders should be limited, e.g., 2.

For the reference encoder, if the low-complexity encoder (e.g., transformer based) can achieve similar performance compared to high complexity encoder as shown in the section 2.1, there is no need to consider high complexity encoder.

Proposal 1: RAN4 to consider a single test decoder trained from sufficient amount of mixed dataset. If multiple decoder is considered, the max number should be limited, e.g., 2.

Summary

In this contribution, we provide our views on inter-vendor training collaboration and interoperability for two-sided models. Based on above, observations and proposal are proposed as below.

Observation 1: Compared to 1-to-1 joint training, N-to-1 joint training provides marginal performance loss of around 1.7%.

Observation 2: Low complexity encoder with Transformer backbone provides almost the same performance to that of Encoder 1 which has 13 times higher computational complexity.

Proposal 1: RAN4 to consider a single test decoder trained from sufficient amount of mixed dataset. If multiple decoder is considered, the max number should be limited, e.g., 2.

Reference

[1] RP-251870, “New WI: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements”, Qualcomm

[2] R4-2511849, “WF on [116][132] NR_AIML_air_part2”, Ericsson

[3] R4-2508084, Simulation assumptions on AI/ML CSI compression, Huawei, Hisilicon, OPPO, CATT, Vivo, NTU, Nokia, Ericsson, Qualcomm, Xiaomi, Mediatek, APPLE, Rohde & Schwarz, Samsung, Intel, ZTE Corporation, Sanechips, CAICT, RAN4#115.