3GPP TSG-RAN WG4 Meeting #116bis	R4-2513538

Prague, Cezh, 13th – 17th Oct, 2025

Agenda Item:	7.9.3

Source:	CAICT

Title:	Discussion on 	Inter-vendor training collaboration and interoperability for two-sided AI/ML models

Document for:	Discussion

1	Introduction

In last meeting, moderator provided a comprehensive list of topics to initiate WI discussion. In this contribution, we provide our views on selective topics.

# Guidance for the WI discussion for 2-sided CSI The following were noted as issues needing discussion during the WI. They are not agreements, but companies are encouraged to contribute on these topics: - Discuss and align on the dataset assumptions and dataset - Discuss and align on assumptions for the test decoder and reference encoder structures - Also may need to consider how many test decoders and encoders - Discuss and align on the scalability alternatives - Discuss and agree on expectations on whether the decoder/encoder for RAN4 are the same as for option C - Discuss whether to decide this before or after agreeing the model - Develop a means to capture model structures and model parameters in 3GPP - Discuss and agree the test metric for the performance requirements. - Discuss and agree channel model for the requirements. - For the dataset, discuss the following: - Eigenvectors or raw channel (or both) ? - What it is important to capture within the dataset - metrics for checking alignment - For example,. PSE - Consider examples from SI datasets that did not align

2	Discussion

Topic 1: Discuss and align on the dataset assumptions and dataset

Discussion on dataset for inter-vendor collaboration is carrying on in RAN1. RAN4 could postpone the discussion until sufficient progress is achieved in RAN1.

Proposal 1: Postpone the discussion of dataset in RAN4 and wait for more progress in RAN1.

Topic 2: Discuss and align on assumptions for the test decoders and reference encoder structures

For the test decoder, we propose to consider two structures as starting points: the CNN structure used in RAN4 evaluations and the Transformer structure used in RAN1 evaluations. For the pre-defined CNN reference model, specific optimizations can be introduced—with careful consideration of the trade-off between computational complexity and overall performance.

For the reference encoder, we consider that, as the UE vendors need to consider issues such as power consumption and latency caused by high model complexity, evaluation research on low-complexity encoders should continue in the initial stage of WI. However, it is necessary to first select one or two model structures and then determine the low-complexity model to reduce the workload of evaluation.

Proposal 2: CNN structure used in RAN4 evaluations with specific optimization (if any) and the Transformer structure used in RAN1 evaluations could be considered as starting point for test decoder.

Proposal 3: Evaluation of low-complexity reference encoders could be considered in the initial stage of WI. Suggest to choose one or two model structures to reduce the workload.

Topic 3: Discuss and align on the scalability alternatives

According to the discussion in RAN1, the scalability of CSI compression (case 0) are mainly considered over numbers of Tx ports, CSI feedback payload sizes, and bandwidths (including both different number of sub-bands and different sub-band sizes). RAN4 could start the discussion from the three aspects and select the value range for each aspect that needs to be considered for the scalability.

Proposal 4: Suggest to start scalability discussion from the three aspects, numbers of Tx ports, CSI feedback payload sizes, and bandwidths.

Topic 4: Discuss and agree on expectations on whether the decoder/encoder for RAN4 are the same as for option C

- Discuss whether to decide this before or after agreeing the model

In our understanding, this hypothesis has already been confirmed in both RAN1’s agreements and WID. Following highlighted part are relevant content.

Agreement (RAN1 #121) For addressing inter-vendor collaboration complexity, RAN1 identifies the following specification impacts for supporting sub-option 3a-1 (including with target CSI and without target CSI), sub-option 4-1, and Direction C - Reference model / structure specification (for sub-option 3a-1 and Direction C) - Note: The so called fully specified reference model discussed in RAN1 for Direction C is assumed to be equivalent to the to-be-specified model, if specified, for performance requirements in RAN4. (Other part is omitted)

RP-252445 Revised WID for AI/ML for NR air interface Phase 2 (Other part is omitted) Inter-vendor training collaboration for two-sided AI/ML models - Fully defined/specified reference model (“Direction C”) with RAN1 scalability study outcome taken into account [RAN4/RAN1] – check-point in RAN#110 upon RAN4 feedback - Specification of standardized encoder model structure plus parameter exchange (“Direction A, sub-option 3a-1” without target CSI sharing) leveraging defined/reference model of “Direction C” and taking RAN1 scalability study outcome into account [RAN4/RAN1/RAN2/RAN3] – check-point in RAN#110 upon SA WG feedback - Specification of standardized dataset format/content plus dataset exchange (“Direction A, sub-option 4-1”) [RAN1/RAN2/RAN3/RAN4] – check-point in RAN#110 upon SA WG feedback (Omitted) Interoperability and RRM requirement [RAN4]: - To define the requirement for the encoder, specify the standardized test decoder to guarantee interoperability - The fully specified test decoder, data set, and corresponding reference encoder in RAN4 to be used for inter-vendor collaboration - Necessary RRM requirement for specified LCM procedures. - NOTE: offline training is assumed for the purpose of this project. Strive to leverage on RAN1’s study on scalable model structure in FS_NR_AIML_Air.

Therefore, RAN4 could also confirm that the fully specified test decoders/reference encoders in RAN4 is same as that of direction C.

Proposal 5: RAN4 to confirm that the fully specified test decoders/reference encoders in RAN4 is same as that of direction C.

Topic 5: Discuss and agree the test metric for the performance requirements

In our understanding, the metric of current PMI reporting requirement in 38.101-4 shall at least be supported for CSI compression case. The test metric agreed for CSI prediction could be taken as starting point for discussion.

Besides metric of PMI reporting, RAN1 is also carrying on the discussion on CQI/RI determination, but without further agreements. So discussion on test metric for CQI/RI reporting could be postponed until more progress is achieved in RAN1.

Proposal 6: Support the metric of current PMI reporting requirement in 38.101-4 for CSI compression case. The test metric agreed for CSI prediction could be taken as starting point.

Proposal 7: Postpone the discussion on test metric of CQI/RI reporting until more progress is achieved in RAN1.

Conclusion

In this contribution, inter-vendor training collaboration and interoperability for two-sided AI/ML models are discussed with following proposals.

Proposal 1: Postpone the discussion of dataset in RAN4 and wait for more progress in RAN1.

Proposal 2: CNN structure used in RAN4 evaluations with specific optimization (if any) and the Transformer structure used in RAN1 evaluations could be considered as starting point for test decoder.

Proposal 3: Evaluation of low-complexity reference encoders could be considered in the initial stage of WI. Suggest to choose one or two model structures to reduce the workload.

Proposal 4: Suggest to start scalability discussion from the three aspects, numbers of Tx ports, CSI feedback payload sizes, and bandwidths.

Proposal 5: RAN4 to confirm that the fully specified test decoders/reference encoders in RAN4 is same as that of direction C.

Proposal 6: Support the metric of current PMI reporting requirement in 38.101-4 for CSI compression case. The test metric agreed for CSI prediction could be taken as starting point.

Proposal 7: Postpone the discussion on test metric of CQI/RI reporting until more progress is achieved in RAN1.