3GPP TSG-RAN WG4 #116bis	R4-2513553

Prague, Czech Republic, October 13-17, 2025

Agenda item:	6.12.3		Demodulation performance and CSI reporting requirements

Source:	Qualcomm Incorporated

Title:	Views on demodulation performance requirements for Rel-19 MIMO Phase 5

Document for:	Discussion

# Introduction

In RAN4 #115, the work on Rel-19 MIMO Phase 5 demod requirements has been started. In RAN4 #116 several additional agreements have been made [1].

In this paper we provide our views for the open issues identified in the way forward in [1].

# Discussion

In the following sections we share our view on demodulation and CSI reporting requirements for MIMO Phase 5.

## UE Reporting Enhancements for CJT

In RAN4 #116 it was discussed whether demod requirements in 38.101-4 should be introduced but no conclusion could be reached.

The difficulty in introducing demod requirements is that it is not straight forward how the reported time, frequency and phase offsets will be applied by the gNB. This requires detailed further discussion and specification of an algorithm that must be implemented in the test equipment. In addition, also more discussions are required how to model time, frequency and phase offset. In particular for frequency and phase offsets we believe this is not straightforward since it depends on the behavior of the oscillators in case of frequency offset reporting and the Tx/Rx frontends in case of phase offset reporting.

In the other side, we see an increasing number of algorithms that require specification of algorithms running in the test equipment. Another example is SRS port grouping, also being discussed in this paper. Here specification of an beamforming algorithm for the TE is needed that aims to orthogonalize the two codewords sent to those antennas that belong to a SRS port group.

Generally, we see value in a discussion whether RAN4 should start introducing simple algorithms for the TE to allow better testing of features that involve device and gNB. In some form we already do this in case of two-sided AI/ML model for CSF compression. However, we believe that this discussion requires more time and should not be done anymore for 5G. Instead, we think that such discussions should take place in the 6G study item.

Observation 1: Demod and CSF requirements for CJT enhancements require detailed discussion and specification of an algorithm that must be implemented in the test equipment. In addition, also more discussions are required how to model time, frequency and phase offset.

Observation 2: An increasing number of features require specification of algorithms running in the test equipment. Another example is SRS port grouping, also being discussed in MIMO Phase 5.

Observation 3: Generally, RAN should discuss whether RAN4 should start introducing simple algorithms for the TE to allow better testing of features that involve device and gNB. In some form RAN4 already does this in case of two-sided AI/ML model for CSF compression. However, we believe that this discussion requires more time and should not be done anymore for 5G. Instead, we think that such discussions should take place in the 6G study item.

Proposal 1: RAN4 should not introduce demod and/or CSF requirements for reporting enhancements of CJT in Rel-19.

## 128 CSI-RS Ports

In RAN4#116, PMI requirements with sixty-four ports for both TDD and FDD were agreed. For 128 ports it was also agreed that PMI requirements if introduced shall be limited to only TDD. In this section, we provide few observations based on initial simulation results for settings agreed in [1].

Test settings

- TDD, 64 X 4, XP -Medium, Single Panel Type I, TDLA30-5, Rank 2

| TDD MCS22 | XP – Medium | XP- High |
| --- | --- | --- |
| SNR (dB) at 90% maximum throughput | 1 | 0 |
| Gamma | Infinity | Infinity |

Observation 4: With XP medium correlation, 90% of maximum throughput is achieved at 1 dB SNR. At this SNR, the throughput achieved with random precoding is zero which implies an infinite Gamma.

Observation 5: To have a finite and stable Gamma, legacy approach to choose SNR point is not useful.

Observation 6: With High correlation too, legacy approach to choose SNR point results infinite Gamma.

Proposal 2: RAN4 to pick high SNR point (not 90% of maximum throughput point) for stable Gamma for PMI requirements.

If choosing different SNR points is not convenient or simple, then another alternative is to pick SNR at 90% of maximum throughput and a large Gamma for e.g., 100. Large Gamma is chosen since the throughput with random precoder is zero.

Issue 1-1-1: Clarify if PMI reporting requirements should be specified for eTypeII-r19

Way forward:

-	FFS on RAN4 should specify PMI reporting requirements for eTypeII-r19

-	Interested companies are welcomed to provide simulation results for next meeting

Proposal 3: RAN4 to choose a large Gamma at 90% of maximum throughput SNR point for PMI requirements.

Transmit beamforming gains with higher number of ports can be measured with Single Panel Type I precoder. Moreover, other Type II based precoders are optional UE capability with no widespread deployments seen so far. So, we do not see the need to have PMI requirements with eTypeII-r19 based codebook. Nevertheless, we are open to discussion for need to have PMI requirements with eTypeII-r19 codebook.

Issue 1-1-2: Clarify if PMI reporting requirements should be specified for typeII-Doppler-r19

Way forward:

-	FFS on RAN4 should specify PMI reporting requirements for eTypeII-Doppler-r19

Doppler codebook primarily enables the UE to predict the channel and allows gNB to match precoders to UE channel estimates in future time slots with limited CSF overhead. So, PMI requirements for 64/128 ports with Rel-19 Doppler codebook are not essential in our view. Please note that Rel-18 Doppler codebook PMI requirements are met by Rel-16 eTypeII codebook-based transmissions too. So, we do not support specifying PMI requirements for eTypeII-Doppler-r19.

Proposal 4: RAN4 to not consider PMI requirements with eTypeII Doppler-r19 codebook.

## SRS Port Grouping

In RAN4#116, there was tepid interest in specifying demodulation requirements with SRS based transmissions.

Issue 1-1-3: Clarify if UE demodulation/CSI reporting requirements should be specified for SRS port grouping

Proposals:

-	Option 1: RAN4 to discuss to reuse an existing3-layer, 4Rx demodulation requirement with a simple set up for SRS port grouping feature.

-	Option 2: RAN4 should not introduce UE demodulation/CSI reporting requirements for SRS port grouping

-	Option 3: Discuss feasibility of test setup for introducing requirements with SRS port grouping in Rel-19

Zero forcing-based precoding at gNB requires knowledge of channel information based on SRS in TDD systems. To come up with a requirement for such a case imposes lot of assumption on TE too. We also note that there is no precedence on demodulation requirements with gNB precoding based on SRS transmissions. We are open to discuss if there are any simple proposals to specify demodulation requirements.

Observation 7: 6/8Rx device supporting SRS port grouping is a new hybrid 6/8Rx receiver. No demodulation requirements exist for such receivers.

Proposal 5: RAN4 to discuss any simple test set up to specify demodulation requirements for SRS port grouping feature.

# Conclusions

In this paper we provided our views on several of the open issues identified in RAN4 #116. Our observations and proposals are summarized as follows:

UE Reporting Enhancements for CJT

Observation 1: Demod and CSF requirements for CJT enhancements require detailed discussion and specification of an algorithm that must be implemented in the test equipment. In addition, also more discussions are required how to model time, frequency and phase offset.

Observation 2: An increasing number of features require specification of algorithms running in the test equipment. Another example is SRS port grouping, also being discussed in MIMO Phase 5.

Observation 3: Generally, RAN should discuss whether RAN4 should start introducing simple algorithms for the TE to allow better testing of features that involve device and gNB. In some form RAN4 already does this in case of two-sided AI/ML model for CSF compression. However, we believe that this discussion requires more time and should not be done anymore for 5G. Instead, we think that such discussions should take place in the 6G study item.

Proposal 1: RAN4 should not introduce demod and/or CSF requirements for reporting enhancements of CJT in Rel-19.

128 CSI-RS Ports

Observation 4: With XP medium correlation, 90% of maximum throughput is achieved at 1 dB SNR. At this SNR, the throughput achieved with random precoding is zero which implies an infinite Gamma.

Observation 5: To have a finite and stable Gamma, legacy approach to choose SNR point is not useful.

Observation 6: With High correlation too, legacy approach to choose SNR point results infinite Gamma.

Proposal 2: RAN4 to pick high SNR point (not 90% of maximum throughput point) for stable Gamma for PMI requirements.

Proposal 3: RAN4 to choose a large Gamma at 90% of maximum throughput SNR point for PMI requirements.

Proposal 4: RAN4 to not consider PMI requirements with eTypeII Doppler-r19 codebook.

SRS Port Grouping

Observation 7: 6/8Rx device supporting SRS port grouping is a new hybrid 6/8Rx receiver. No demodulation requirements exist for such receivers.

Proposal 5: RAN4 to discuss any simple test set up to specify demodulation requirements for SRS port grouping feature.

# References

- R4-2512527, Way Forward for [116][327] NR_MIMO_demod, August 2025