3GPP TSG-RAN WG4 Meeting #116bis	R4-2513555

# Prague, Czech, 13th – 17th Oct., 2025

Source:	LG Electronics

Title:	Discussion on RRM performance requirements of OD-SSB for Rel-19 NES

Agenda item:	6.15.2.1

Document for:	Discussion

Introduction

In this contribution, we provide our views on RRM performance requirements of OD-SSB for Rel-19 network energy saving.

Discussion

Test scope

General

Issue 3-1-1: Scenario - General - RAN4 to introduce test to verify FMW for deactivated Scell measurement and corresponding interruption requirement. - No new test case for A.7.7 Measurement Performance requirements for NES OD-SSB. - RAN4 should not define fast SCell activation test cases for OD-SSB. - RAN4 only choose one test case for case 2 to justify the effective periodicity of OD-SSB and AO-SSB. - RAN4 to further discuss the test case scope for scenario #2, scenario #2A and scenario #3B as follow. - Option 1: - In Case 1, RAN4 should define the test cases for scenario #2, scenario #2A. - In Case 2, RAN4 should define the test cases for scenario #2, scenario #2A and scenario #3B. - The test cases should include deactivation SCell measurement, SCell activation procedure and the L1/L3 measurement for activated SCell. - Option 2: RAN4 not to define test cases for scenario #3B. - Option 3: RAN4 to define the OD-SSB based test cases for - Case #1 and Scenario #2 - Case #2 and Scenario #2A - RAN4 to further discuss the test case scope for Case 2 as follow. - FFS: RAN4 to define the test cases to verify the UE behaviour in Alt. Time C1, Alt. Time C2 and scenario 3 if RAN4 agrees to define the RRM requirements for all these scenarios.

In the core requirement for OD-SSB, RAN4 introduced FMW for deactivated SCell measurement, SCell activation and interruption for OD-SSB for Case 1 and Case 2.

For the test case scope for scenario #2, scenario #2A and scenario #3B, the core requirements defined scenario #2 (deactivated SCell measurement and SCell activation procedure), scenario #2A (SCell activation procedure) and scenario #3B(updating OD-SSB periodicity and SMTC periodicity). Hence, it is reasonable to verify all the cases.

Proposal 1: RAN4 to define the test case scope for scenario #2, scenario #2A and scenario #3B as follow

- In Case 1, RAN4 should define the test cases for scenario #2, scenario #2A.

- In Case 2, RAN4 should define the test cases for scenario #2, scenario #2A and scenario #3B.

- The test cases should include deactivation SCell measurement, SCell activation procedure and the L1/L3 measurement for activated SCell.

For the test case scope for Case 2, RRM core requirements defined Time C1,and Alt Time C2. RAN4 need to define the test cases for Time C1 and Time C2

Proposal 2: RAN4 to define the test case scope for Case 2 Alt Time C1, and Alt Time C2

SCell activation scope

Issue 3-1-2: Scenario – Scell activation scope - Proposals - Proposal 1: RAN4 should focus on SCell activation for case 1 and case 2 Alt Time-C1 and multiple SCells activation. The other SCell activation scenarios should have lower priority. - Proposal 2: RAN4 only define the test cases for single SCell activation. - Recommended WF - Further discussion

For the test case scope for Scell activation scope, RRM core requirements defined Single Scell activation as well as multiple SCell activation. It is reasonable to define both cases.

Proposal 3: RAN4 to define SCell activation for case 1 and case 2 Alt Time-C1 and multiple SCells activation

Known/unknown Scell activation

Issue 3-1-3: Scenario – known/unknown Scell activation - Proposal 1: RAN4 to define the OD-SSB based SCell activation test case for both known SCell and unknown SCell. - Proposal 2: RAN4 to define the OD-SSB based SCell activation test case for known SCell only. - Proposal 3: RAN4 to define the OD-SSB based test case for both Case 1 and Case 2. One in known scenario and the other in unknown scenario such as: - Case 1 in unknown scenario - Case 2 in known scenario

For the test case scope for Scell activation scope, RRM core requirements defined Single Scell activation as well as multiple SCell activation. It is reasonable to define both cases.

Proposal 4: RAN4 to define the OD-SSB based SCell activation test case for both known SCell and unknown

Test case list

Issue 3-1-4: Test case list Agreement RAN4 to at least define the following OD-SSB test cases. - SCell activation in Case 1 - Intra-frequency measurement(deactivated SCell) in Case 1 - L1-RSRP measurement with OD-SSB parameter update(Scenario 3B) RAN4 to further discuss whether the following OD-SSB test cases can be agreed. - SCell activation in Case 2 Alt Time C-1 - OD-SSB based multiple SCell activation - OD-SSB based direct SCell Activation at SCell addition in Case 1 - OD-SSB based direct SCell Activation at SCell addition in Case 2 - OD-SSB based PUCCH SCell Activation in Case 1 - OD-SSB based PUCCH SCell Activation in Case 2 - Fast SCell Activation with OD-SSB; - OD-SSB based SCell Activation with the L3 reporting during activation - Intra-frequency measurement(deactivated SCell) in Case 2 - Intra-frequency measurement in Case 1 - Intra-frequency measurement in Case 2 - Interruption at OD-SSB activation/deactivation - Interruptions during measurements on deactivated NR SCC

In the core requirement for OD-SSB, RAN4 introduced FMW for deactivated SCell measurement, SCell activation and interruption for OD-SSB for Case 1 and Case 2. Regarding SCell activation test case for Case 2, in order to reduce the dupliacated test cases, we think only SCell activation in Case 2 Alt time C-1 is needed. Thus, for OD-SSB test case, following can be considered:

Proposal 5: RAN4 to define following test cases for OD-SSB

- SCell activation in Case 2 Alt Time C-1

- OD-SSB based multiple SCell activation

- OD-SSB based direct SCell Activation at SCell addition in Case 1

- OD-SSB based PUCCH SCell Activation in Case 1

- Fast SCell Activation with OD-SSB;

- OD-SSB based SCell Activation with the L3 reporting during activation

- Intra-frequency measurement(deactivated SCell) in Case 2

- Intra-frequency measurement in Case 1

- Intra-frequency measurement in Case 2

- Interruption at OD-SSB activation/deactivation

- Interruptions during measurements on deactivated NR SCC

Test configuration

Issue 3-1-5: General principle RAN4 to discuss whether the following principles to design the OD-SSB test case can be agreed. - Both FR1 and FR2 - Test OD-SSB periodicities with 5ms, 20ms. 10ms - Only test non-DRX mode - The test without SSB time index detection - Follow the legacy test methodology to use L3 event trigger reporting and L1-RSRP reporting - SMTC and measCycleSCell as 40ms/160ms - Only NCD-SSB for OD-SSB test case. Issue 3-1-6: OD-SSB configuration - Proposals - In Case 1, RAN4 to define the OD-SSB based SCell activation test case with 5ms periodicity. - In Case 2, RAN4 to define the OD-SSB based SCell activation test case as follow. - OD-SSB(5ms) is overlapping with AO-SSB(20ms) - The union of OD-SSB(20ms) and AO-SSB(20ms) meets the legacy SSB periodicity(10ms) - RAN4 to introduce new NCD-SSB RMC configuration for 5ms, 10ms and 20ms periodicity.

For OD-SSB principle, following can be considered:

  - Both FR1 and FR2

  - At least test OD-SSB periodicities with 10ms, 20ms

  - Only test non-DRX mode

  - The test without SSB time index detection

  - Follow the legacy test methodology to use L3 event trigger reporting and L1-RSRP reporting

  - SMTC and measCycleSCell as 40ms/160ms

For OD-SSB configuration, following can be considered:

  - In Case 1, RAN4 to define the OD-SSB based SCell activation test case with 10ms periodicity.

  - In Case 2, RAN4 to define the OD-SSB based SCell activation test case as follow.

    - OD-SSB(10ms) is overlapping with AO-SSB(20ms)

    - The union of OD-SSB(20ms) and AO-SSB(20ms) meets the legacy SSB periodicity(10ms)

  - RAN4 to introduce new NCD-SSB RMC configuration for 10ms and 20ms periodicity.

Proposal 6: For OD-SSB principle, following can be considered:

  - Both FR1 and FR2

  - At least test OD-SSB periodicities with 10ms, 20ms

  - Only test non-DRX mode

  - The test without SSB time index detection

  - Follow the legacy test methodology to use L3 event trigger reporting and L1-RSRP reporting

  - SMTC and measCycleSCell as 40ms/160ms

Proposal 7: For OD-SSB configuration, following can be considered:

  - In Case 1, RAN4 to define the OD-SSB based SCell activation test case with 10ms periodicity.

  - In Case 2, RAN4 to define the OD-SSB based SCell activation test case as follow.

    - OD-SSB(10ms) is overlapping with AO-SSB(20ms)

    - The union of OD-SSB(20ms) and AO-SSB(20ms) meets the legacy SSB periodicity(10ms)

  - RAN4 to introduce new NCD-SSB RMC configuration for 10ms and 20ms periodicity.

Conclusion

In this contribution, we provide views on RRM performance requirement for Rel-19 NES

Proposal 1: RAN4 to define the test case scope for scenario #2, scenario #2A and scenario #3B as follow

- In Case 1, RAN4 should define the test cases for scenario #2, scenario #2A.

- In Case 2, RAN4 should define the test cases for scenario #2, scenario #2A and scenario #3B.

- The test cases should include deactivation SCell measurement, SCell activation procedure and the L1/L3 measurement for activated SCell.

Proposal 2: RAN4 to define the test case scope for Case 2 Alt Time C1, and Alt Time C2

Proposal 3: RAN4 to define SCell activation for case 1 and case 2 Alt Time-C1 and multiple SCells activation

Proposal 4: RAN4 to define the OD-SSB based SCell activation test case for both known SCell and unknown

Proposal 5: RAN4 to define following test cases for OD-SSB

- SCell activation in Case 2 Alt Time C-1

- OD-SSB based multiple SCell activation

- OD-SSB based direct SCell Activation at SCell addition in Case 1

- OD-SSB based PUCCH SCell Activation in Case 1

- Fast SCell Activation with OD-SSB;

- OD-SSB based SCell Activation with the L3 reporting during activation

- Intra-frequency measurement(deactivated SCell) in Case 2

- Intra-frequency measurement in Case 1

- Intra-frequency measurement in Case 2

- Interruption at OD-SSB activation/deactivation

- Interruptions during measurements on deactivated NR SCC

Proposal 6: For OD-SSB principle, following can be considered:

  - Both FR1 and FR2

  - At least test OD-SSB periodicities with 10ms, 20ms

  - Only test non-DRX mode

  - The test without SSB time index detection

  - Follow the legacy test methodology to use L3 event trigger reporting and L1-RSRP reporting

  - SMTC and measCycleSCell as 40ms/160ms

Proposal 7: For OD-SSB configuration, following can be considered:

  - In Case 1, RAN4 to define the OD-SSB based SCell activation test case with 10ms periodicity.

  - In Case 2, RAN4 to define the OD-SSB based SCell activation test case as follow.

    - OD-SSB(10ms) is overlapping with AO-SSB(20ms)

    - The union of OD-SSB(20ms) and AO-SSB(20ms) meets the legacy SSB periodicity(10ms)

  - RAN4 to introduce new NCD-SSB RMC configuration for 10ms and 20ms periodicity.

Reference

[1]