3GPP TSG-RAN WG4 Meeting #116bis											R4-2513561

Prague, Czech Republic, Oct. 13-17, 2025

Source: 	vivo

Title: 	Discussion on the co-existence assumption for outdoor active device

Agenda Item:	7.11.3

Document for:	Approval

- Introduction

RAN4-led

- Study coexistence between Device 2b/Device C and NR/LTE in the above outdoor scenarios.

- Use band n8 as an example band.

In [1], the SID for device outdoor was agreed for R20, and the RAN4 scope is listed below:

In this contribution, we provide our views on co-existence simulation assumption.

- Discussion

## Co-existence scenario

The possible co-existence scenario is listed in table below:

| Deployment scenario and topology | spectrum | Operation mode | aggressor | victim | Note |
| --- | --- | --- | --- | --- | --- |
|  | DL:R2D UL:D2R | Standalone | Device D2R | NR UL | Normal evaluation cases |
|  |  | Standalone | NR UL | Reader D2R |  |
|  |  | Standalone | Reader R2D | NR DL |  |
|  |  | Standalone | NR DL | Device R2D |  |
|  |  | In-band | Device D2R | NR UL | Same as standalone, since BS in-band selectivity is assumed to be same as out of band ACS in R19 |
|  |  | In-band | NR UL | Reader D2R | UE in-band emission is worse than out of band emission, this case could be worse than standalone |
|  |  | In-band | Reader R2D | NR DL | The R2D and NR signal should be orthogonal, evaluation is not needed |
|  |  | In-band | NR DL | Device R2D | Device could not use OFDM receiver, this case cannot be ignored. Could be worse than standalone depending on in-band emission assumption |

The standalone is the typical case what we evaluated in the co-existence, but for the in-band operation, we try to figure out whether we can minimize the evaluation cases. The comments for each case are captured in the table above.

Proposal 1: The following cases are proposed to be evaluated for outdoor device:

| Operation mode | aggressor | victim |
| --- | --- | --- |
| Stand alone | Device D2R | NR UL |
| Stand alone | NR UL | Reader D2R |
| Stand alone | Reader R2D | NR DL |
| Stand alone | NR DL | Device R2D |
| In-band | NR UL | Reader D2R |
| In-band | NR DL | Device R2D |

## Simulation assumption

2.2.1 Deployment

In [2], the LS from RAN1 clarify the R2D channel bandwidth:

Agreement For Rel-20 study, R2D transmission bandwidth assumption in link budget and LLS for evaluation purpose is from the followings: - 180kHz as starting point - Other values e.g. 360kHz, 540kHz or other multiples of 180kHz are not precluded and up to company report

The deployment assumption in R19 can be the baseline. Based on the description in SID, the detailed assumption can be found in table below:

| Parameter | Value |
| --- | --- |
| Carrier frequency | 900 MHz (Band n8) |
| BW for NR | 10MHz with 15KHz SCS |
| BW for A-IOT system | 180kHz, for 15kHz SCS |
| Waveform (R2D) | OOK waveform generated by OFDM modulator |
| A-IoT DL power control | No |
| A-IoT UL power control | Depending on RAN1, can take NO as starting point |
| Traffic model | Full buffer |
| Frequency reuse | 1 |
| NR BS/Reader deployment | Hexagonal grid, 19 macro sites, 3 sectors per site with wrap around Inter-NR BS distance: 750m |
| Device distribution | Device Height= 1.5 m A-IoT devices drop uniformly distributed over the horizontal area Number of A-IoTs = Total area × activated density (1.5 A-IOT devices/m²) 1 active A-IoT device under one reader at one drop FFS on FDM operation, which is depending on RAN1 Minimum distance between reader and device is 35m |
| NR UE dropping | NR UE 100% outdoor, uniformly distributed outdoor. Minimum distance between BS and UE is 35m UE number: - DL active UE: 1 UE per cell - UL active UE: 3 UE per cell |
| Propagation model | Refer to TR 38.901 |
| Multi operators layout | coordinated operation (0% Grid Shift) |

2.2.2 Reader/NR BS characteristic

In [2], the LS from RAN1 clarify the BS power value:

Agreement For Rel-20 study, BS Tx power assumption for coverage purpose is from the followings: - For standalone: 43 dBm - For in-band: - 38 or 33 dBm over R2D transmission BW - Note1: any necessary PSD limitation can be applied by RAN4 - Note2: companies should report any PSD limitation they assume in their RAN1 evaluations

In our understanding, for in-band operation, the worst case for co-existence of in-band operation should be the NR BS only schedule the resource for the AIoT and the outpower of R2D would be same as the NR BS, e.g., 46 dBm. The RAN1 assumption may consider the power scaling when both NR and R2D are transmitted but in RAN4 co-existence, we should consider the worst case. The assumption for NR BS/in-band Reader should be similar for outdoor scenario and the assumption in TR38.769 also can be the starting point.

| Parameter | Values for evaluation |  |  |
| --- | --- | --- | --- |
| Tx power (dBm) | NR BS: 46 | Standalone reader: 43 | In-band reader: 46 |
| Antenna gain (dBi) and antenna pattern for reader and NR BS | Antenna Array Geometry：BS point at fixed beam direction: vertical: θtilt + 90°, horizontal: 0, 120, 240 ° Antenna pattern (horizontal) (For 3-sector cell sites with fixed antenna patterns): AHϕ=-min12ϕϕ3dB2,Am φ3dB = 65 degrees, Am = 25 dB Antenna pattern (vertical) (For 3-sector cell sites with fixed antenna patterns): AVθ=-min12θ-90°-θetiltθ3dB2,SLAv θ3dB = 10 degrees, SLAv = 25 dB, θetilt= 9 degrees Combining method in 3D antenna pattern: Aφ,θ=-min⁡{-AHφ+AVθ, Am} BS antenna gain (including feeder loss):15 dBi |  |  |
| Height (m) | 25 |  |  |
| Noise Figure(dB) | 5 |  |  |
| Network location | outdoor |  |  |
| NR BS ACLR | Standalone: 45 dBc In-band: 30 dBc |  |  |

2.2.3 NR UE/active device characteristic

For the NR UE, the assumption is same as R19 AIoT co-existence study.

| NR UE Parameter | Values for evaluation |
| --- | --- |
| UE TX power in dBm | -40 to 23 |
| NR UE Antenna gain (dBi) | 0 |
| NR UE ACLR（dB） | 30 |
| NR UE Noise Figure（dB） | 9 |
| Antenna configuration | Omni direction antenna |

However, the assumption for active device 2b/c need more discussion. In SID, the maximum output power for device 2b is -20~-10 dBm and for device c is -3~+5 dBm. The specific value may be related to RAN1 coverage study but from co-existence perspective, we can choose the maximum value as starting point.

Another parameter is noise figure. In TR38.769, the noise figure of device 2 with RF-ED architecture is 20 dB and 15 dB for IF/ZIF architecture. In SID, the descriptions of device 2b/c are listed below:

- Device 2b: ≤ a few hundred µW peak power consumption, has energy storage, IF envelope detector receiver or ZIF receiver, initial sampling frequency offset (SFO) up to 10Y ppm, R2D and/or D2R amplification in the device. The device’s D2R transmission is generated internally by the device.

- Device C: ≤ 1 mW to ≤ 10 mW peak power consumption, has energy storage, IF envelope detector receiver or ZIF receiver, initial sampling frequency offset (SFO) up to 10Y ppm, R2D and/or D2R amplification in the device. The device’s D2R transmission is generated internally by the device.

Both device 2b and device c are assumed to use IF/ZIF architecture, so 15 dB could be used as the starting point for noise figure.

| A-IoT device parameters | Device 2b | Device c |
| --- | --- | --- |
| A-IoT device effective antenna gain per Tx or Rx branch (dBi) | 0 |  |
| Tx power in dBm | [-10] dBm | [+5] dBm |
| A-IoT device noise figure (dB) | [15] |  |
| Guard band | 0PRB |  |
| Antenna configuration | Omni direction antenna |  |

## Evaluation methodology

The methodology should be same as the TR 38.769, as listed below

1) Aggressor and victim network are generated. NR UEs and A-IoT devices are distributed as described by parameter assumptions.

2)	UEs are associated to BS based on coupling loss, and A-IoT devices are associated to A-IoT reader based on coupling loss.

a)	The device access to the reader is randomly selected from candidate devices whose R2D receiving power/coupling from one reader is higher than the other.

b)	The device can access the reader with best receiving power among all readers.

3)	Once association is done, round robin scheduling is used.

4)	For inter-system interference (between A-IoT and NR):

-	When AIoT as victim, if SINR degradation is smaller than and equal to 1 dB, it can be considered that inter-system interference is negligible.

- For SINR degradation, SINR refers to the 5% and 50% CDF SINR

-	When NR as victim, if throughput loss is smaller than and equal to 5%, it can be considered that inter-system interference is negligible.

Proposal 2：Take the simulation assumptions listed in this contribution as the starting point for R20 outdoor active device co-existence evaluation.

- Conclusion

In this contribution, we provide our views on the co-existence study for outdoor AIoT device:

Proposal 1: The following cases are proposed to be evaluated for outdoor device:

| Operation mode | aggressor | victim |
| --- | --- | --- |
| Stand alone | Device D2R | NR UL |
| Stand alone | NR UL | Reader D2R |
| Stand alone | Reader R2D | NR DL |
| Stand alone | NR DL | Device R2D |
| In-band | NR UL | Reader D2R |
| In-band | NR DL | Device R2D |

Proposal 2：Take the simulation assumptions listed in this contribution as the starting point for R20 outdoor active device co-existence evaluation.

- References

- RP-251884 New SID on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices

- R1-2506575, LS on BS Tx power assumption for coverage evaluation in A-IoT