# 3GPP TSG-RAN WG4 Meeting #116bis                              R4-2513645

# Prague, Czech Republic, Oct. 13-17, 2025

Agenda Item:	6.17.2.3

Source: 	CMCC

Title: 			 Discussion on RRM performance requirements for conditional Intra-CU LTM

Document for:	Discussion

- Introduction

In RAN #102 meeting, the new WID on NR mobility enhancements Phase 4 was approved [1]. In RAN #107 meeting, the revised WID was approved [2]. The detailed objectives are duplicated as following.

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support - Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure. - Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Specify RRM requirements related to the above objectives as necessary [RAN4]

In last meeting, there is discussion for RRM performance requirements for NR mobility enhancements, and a WF was approved [3]. This contribution provides discussion on RRM performance requirements for conditional Intra-CU LTM.

- Discussion

Both CLTM and subsequent CLTM are considered. CLTM cell switch delay requirements and subsequent CLTM  cell switch delay requirements will be defined. It is necessary to define test cases for the two cases.

Proposal 1: it is proposed to define test cases for CLTM cell switch delay and subsequent CLTM cell switch delay.

In previous meeting, it was agreed that Rel-19 CLTM requirements are applicable for NR SA scenarios. In detail, following cases are agreed.

- PCell switch to a neighboring LTM candidate cell

- FR1 cell to FR1 cell

- FR1 cell to FR2 cell

- FR2 cell to FR2 cell

- FR2 cell to FR1 cell

- PCell switch to an LTM candidate cell that is a serving SCell in MCG

- FR1 cell to FR1 cell

- FR2 cell to FR2 cell

From test coverage point of view, it is necessary to define test cases to cover these cases.

Proposal 2: for CLTM and subsequent CLTM, it is proposed to define the test cases for following scenarios:

- PCell switch to a neighboring LTM candidate cell

- FR1 cell to FR1 cell

- FR1 cell to FR2 cell

- FR2 cell to FR2 cell

- FR2 cell to FR1 cell

- PCell switch to an LTM candidate cell that is a serving SCell in MCG

- FR1 cell to FR1 cell

- FR2 cell to FR2 cell

- Issue 4-3-2: CLTM test scope - intra/inter-frequency Candidate options: - Option 1: cover both intra and inter-frequency CLTM. (Nokia, Apple, CTC, vivo, ZTE, HW) Recommended WF: - Continue discussion. - Issue 4-3-3: CLTM test scope – L1/L3 based CLTM Candidate options: - Option 1: cover both L1 and L3 based CLTM. (Nokia, CATT, CTC, ZTE, E///) - Option 1a: RAN4 to discuss how to verify L1 and L3 measurement based candidate evaluation for CLTM such as whether to reuse partial LTM test procedure. (E///) - Option 2: only test L1 based CLTM. (OPPO, MTK) Recommended WF: - Discuss the options - Issue 4-3-4: CLTM test scope – RACH-less and RACH-based CLTM Candidate options: - Option 1: cover both RACH-less and RACH based CLTM. (Nokia, CATT, CTC, OPPO, vivo, ZTE, HW) - Option 2: RAN4 to discuss how to verify both RACH based and RACH-less procedure for CLTM such as re-use partial LTM test procedure. (E///) Recommended WF: - Discuss the options

In R18 LTM, the test cases are defined for both intra-frequency and inter-frequency. For CLTM, it is necessary to cover both cases. Similar situation exits for RACH-based and RACH-less cases, both need to be considered.

Proposal 2: it is proposed to define test cases for both intra-frequency LTM and inter-frequency LTM.

Proposal 3: it is proposed to define test cases to cover both RACH-less and RACH based CLTM.

Normally, conditional LTM is based on L1 measurement. But RAN4 also agreed to define the requirements for L3 measurement based conditional LTM. This agreements only apply to FR1 CLTM and not apply to FR2 CLTM. It is necessary to define test cases to cover the case with L1 measurement based and the case with L3 measurement based.

Proposal 4: it is proposed to define test cases to cover both L1 and L3 based CLTM

- Conclusion

This contribution provides discussion on RRM performance requirements for conditional Intra-CU LTM. The proposals are:

Proposal 1: it is proposed to define test cases for CLTM cell switch delay and subsequent CLTM cell switch delay.

Proposal 2: for CLTM and subsequent CLTM, it is proposed to define the test cases for following scenarios:

- PCell switch to a neighboring LTM candidate cell

- FR1 cell to FR1 cell

- FR1 cell to FR2 cell

- FR2 cell to FR2 cell

- FR2 cell to FR1 cell

- PCell switch to an LTM candidate cell that is a serving SCell in MCG

- FR1 cell to FR1 cell

- FR2 cell to FR2 cell

Proposal 2: it is proposed to define test cases for both intra-frequency LTM and inter-frequency LTM.

Proposal 3: it is proposed to define test cases to cover both RACH-less and RACH based CLTM.

Proposal 4: it is proposed to define test cases to cover both L1 and L3 based CLTM

- Reference

- RP-234036, New WID: NR mobility enhancements Phase 4

- RP-250339, Revised Work Item: NR mobility enhancements Phase 4

- R4-2512141, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple

- R4-2508282, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple