# 3GPP TSG-RAN WG4 Meeting #116bis		  R4-2513691

Prague, Czech Republic, Oct. 13-17, 2025

Agenda item:	6.11.4

Source:		Qualcomm Incorporated

Title:			Discussion on AI/ML beam management testability

Document for:	Approval

# 1  Introduction

In RAN4#116, the following agreements regarding test setup are captured in [1].

Issue 2-6: Test system setup Agreement: Consider multi AoA based RRM testing setup as baseline(enhanced IFF system with 4 probes used for FR2 RRM conformance testing) Single AoA tests can also be performed in the multi AoA based RRM test setup Further analyze what spatial channel models can be emulated with this test setup - Companies are invited to bring analysis on what simplified spatial channel (multiple clusters coming from multiple directions) model can be emulated in this test setup Single AoA tests are not precluded Issue 2-7: Test system channel model Agreement: Companies to bring analysis into what simplifications to CDL channels are needed to be able to emulate the channels in the enhanced IFF chamber. Take CDL-C Umi as one example Other channel models can be further discussed

In this paper, we provide further views on the testability issues for AI/ML beam management.

# 2  Discussion

## 2.1 General

In general, the high-level test procedure for AI/ML BM-case 1 can be described below:

- Progress A: UE BM performance with AI/ML for BM-case 1

  - Step 1: Model training step - Channel parameters set #1

    - Contuct by offline

  - Step 2: Measurement step - Channel parameter set #2

    - TE transmits Set B beams

  - Step 3: Prediction step

    - UE reports top K predicted beams of set A based on measurement of set B beams.

- Progress B: Ground truth of UE BM performance without AI/ML for BM-case 1

  - Step 1: Measurement step – Channel parameters set #2

    - TE transmits set A beams.

    - UE reports measured RSRP of all beams of set A.

  - Setp 2: TE compares the measured RSRP of the strongest beam and the maximum RSRP of top K predicted beam.

Figure 1: High-level test procedure for AI/ML BM-case 1

In addition, it is essential to randomize the transmission of setA beams during step1 of progress B of Figure 1 to properly test UE’s AI-ML model.

Additionally, if the transmission of setA beams is not randomized during measurement step of progressB, UE can simply inform TE that its top-1 strongest beam is same as its top-1 predicted beam of progressA. UE does not even need to measure the beams to predict it. However, UE won’t be able to cheat the test like this if the transmission of setA beams is randomized during measurement step of progressB.

Based on the illustration of the test steps, we have the following observation.

Observation 1: The key requirements for AI/ML BM test setup are listed following:

- Requirement#1: Channel model in the test

  - The test should reflect performance in the field. Hence, CDL or simplified CDL to properly verify beam management performance.

- Requirement#2: Avoid the cheating

  - Progress A: Model training for Set A’s is generated with channel parameters #1. Set B in UE measurement should have enough randomness compared to Set A’.

  - Progress B: Set A is generated to make sure to use the same parameters (initial phase, angular spread, delay spread) as the Set B in the Progress A measurement step. NW antenna/beam configurations for Set A and Set B should be deterministic. For some prediction metrics (e.g., option 2 and 3), UE can pass the test without actual measurement if TE does not randomize the transmission of its beams.

- Requirement#3: Support the beam number of Set A

  - TE needs to support Set A for ground truth verification.

## 2.2 Multiple AoA test setup with enhanced IFF (eIFF) system

FR2 RRM test setup eIFF was agreed as the baseline to verify the AI/ML beam management [1]. As shown in Figure 2, there are 4 physical probes in the current FR2 RRM test setup. These 4 probes can transmit power simultaneously with different amplitude and phase. Therefore, the combination of 4 probes can be considered to emulate a multiple-AoAs test environment. Hence the eIFF multiple AoAs/Probes setup can be used with simplified CDL model since there may be no need to consider implementing multiple clusters as CDL channel model for AI/ML BM testing.

Figure 2: An example test setup for multiple probes on top of legacy RRM test setup

Observation 2: To generate Set A and Set B, multiple AoAs/probes test setup is necessary.

Observation 3: Using Multiple AoA test setup with FR2 RRM test can reuse the existing FR2 test setup to save the test cost and a simplified CDL channel model needs to be specified.

To generate a simplified UMi CDL-C channel model under eIFF test setup, since there are limited number of probes, we can consider reducing the number of clusters, i, e., removing the weak clusters in which the power is less than a threshold.

Table 1 gave an exmaple to show how to reduce the cluster number (highlighted by yellow in Table 1) for UMi CDL-C channel model.

Table 1: Channel model parameters for UMi CDL-C at 28 GH

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -4.4215 | -30.4353 | -134.4434 | 98.9242 | 83.3318 |
| 2 | 12.594 | -1.25 | -20.9269 | 129.1633 | 99.1915 | 72.5229 |
| 3 | 13.314 | -3.4684 | -20.9269 | 129.1633 | 99.1915 | 72.5229 |
| 4 | 13.974 | -5.2294 | -20.9269 | 129.1633 | 99.1915 | 72.5229 |
| 5 | 13.056 | -2.5215 | -28.0782 | -152.8206 | 99.5732 | 71.1282 |
| 6 | 38.196 | 0 | -11.6982 | 164.1145 | 99.306 | 74.7544 |
| 7 | 38.688 | -2.2185 | -11.6982 | 164.1145 | 99.306 | 74.7544 |
| 8 | 39.36 | -3.9794 | -11.6982 | 164.1145 | 99.306 | 74.7544 |
| 9 | 39.504 | -7.4215 | 17.3861 | 84.3647 | 100.4513 | 69.2454 |
| 10 | 47.61 | -7.1215 | -37.5865 | 92.0623 | 98.5616 | 66.7349 |
| 11 | 49.278 | -10.7215 | 20.2226 | -97.7585 | 100.6231 | 72.0348 |
| 12 | 56.016 | -11.1215 | -50.6106 | 78.4702 | 98.218 | 64.4337 |
| 13 | 73.71 | -5.1215 | -33.911 | 93.1719 | 100.165 | 85.4238 |
| 14 | 78.498 | -6.8215 | -37.5066 | -112.0441 | 100.2604 | 64.1548 |
| 15 | 130.224 | -8.7215 | -43.1797 | 102.4645 | 98.1225 | 64.7824 |
| 16 | 162.63 | -13.2215 | 29.2116 | 67.2359 | 100.2604 | 92.467 |
| 17 | 255.534 | -13.9215 | 27.8133 | 34.5731 | 98.4852 | 65.6889 |
| 18 | 276.018 | -13.9215 | 23.6584 | 48.5813 | 98.1416 | 68.7572 |
| 19 | 329.412 | -15.8215 | -52.5282 | 36.4455 | 97.9698 | 59.1339 |
| 20 | 336.462 | -17.1215 | 25.0168 | 52.6729 | 100.7376 | 65.3402 |
| 21 | 378.39 | -16.0215 | 25.4562 | 49.8296 | 98.1225 | 58.4365 |
| 22 | 398.244 | -15.7215 | 30.7697 | 46.4316 | 98.1034 | 65.2705 |
| 23 | 422.562 | -21.6215 | 35.9234 | 30.759 | 100.4513 | 62.6903 |
| 24 | 519.138 | -22.8215 | -61.2775 | 69.2469 | 100.9476 | 61.993 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 0.799 | 10.4021 | 0.5726 | 4.8814 | 7 |  |

The principle of removing weak clusters is two-fold.

- First, the weak clusters have less impact on the UE beam management performance due to the narrow UE beam width, especially if the clusters fall outside UE’s spherical coverage. For example, an over 10dB gap between peak and 50%-tile spherical coverage is defined for FR2 PC3 UE. And the clusters highlighted in yellow, which have power less than [-10dB], could be removed as they might have a limited impact on beam management.

- Second, the weak clusters are unlikely to contribute to the strongest gNB Tx beam selection. TE will create its own beams based on the predefined multi-beam codebook of setA and setB. The beams created by the TE may not be exactly aligned with the AoDs/ZoDs of different clusters of a CDL model. Hence, the strongest beam may not be adjacent to the AoD and ZoD of the strongest cluster. However, the AoDs/ZoDs of very weak clusters are unlikely to influence the selection of the strongest beam in any reasonably designed codebook of setA and setB.

Observation 4: The UE’s antenna gain gap between peak and 50%-tile spherical coverage is over 10dB for PC3. The very weak clusters would not influence the UE beam management even if they reach the UE within the UE’s spherical coverage.

Observation 5: The AoDs/ZoDs of very weak clusters are unlikely to influence the selection of the strongest beam in any reasonably designed codebook of Set A and Set B.

Observation 6: The weak clusters that have a limited impact on the beam management could be removed to reduce the number of clusters in the CDL channel model.

In addition, the clusters that have the same/similar AoAs could be merged (e.g., highlighted by green in Table 1). And the angle spread intra cluster, e.g., CASD, CASA, CZSD, CZSA can be reduced to simplify the test setup.

Observation 7: The clusters that have the same/similar AoAs could be merged. In addition, the intra-cluster angle spread, e.g., CASD, CASA, CZSD, CZSA can be reduced to simplify the test setup. In this case, a new simplified channel model could be emulated by test setup.

With above observatios, we have the following propsoals:

Propsoal 1: RAN4 to consider the following options to define the simplfied CDL channel model with eIFF test setup:

- Revmoe the weak clusters that would not influence the UE beam management

- Merge the clusters that have the same/similer AoAs

- Reduce values of per-cluster parameters such as CASD, CASA, CZSD, CZSA

Propsoal 2: RAN4 to evlaute whether the new simiplied CDL channel model generated from eIFF system can be used to properly verify AI/ML BM performance through AIML BM simulations.

## 2.3 Impact of measurment uncertainty

RAN4 has agreed to perform the study to assess the impact of UE measurement error on the prediction accuracy. In the core requirements discussion, the impact of baseband error and RF error were studied. In addtion to UE measurmenet error, the measurement uncertainty introduced by test system in the OTA chamber may also have inpact on the prediction accuracy.

In general, the measurement uncertainty source inlcudes the following two stages:

- Stage 1: the calibration of the absolute level of the DUT measurement results is performed by means of using a calibration antenna whose absolute gain is known at the frequencies of measurement

- Stage 2: the actual measurement with the DUT as either the transmitter or receiver is performed.

An example of measurement uncertainty for multiple probes test setup is provided in Table B.2.5-1, TS 38.151[2]. The totally expanded uncertainty for DL MIMO sensitivity is 5.05dB. The measurement uncertainty for EIS with single probe test setup in IFF is also larger than 5dB [3]. Therefore, RAN4 needs to study whether to consider the impact of the measurement uncertainty when specifying AI/ML BM core requirements.

Proposal 3: RAN4 to investigate whether measurement uncertainty should be considered when defining AI/ML BM core requirements.

# 3. Conclusion

In summary, we have following observations and proposals.

Observation 1: The key requirements for AI/ML BM test setup are listed following:

- Requirement#1: Channel model in the test

  - The test should reflect performance in the field. Hence, CDL or simplified CDL to properly verify beam management performance.

- Requirement#2: Avoid the cheating

  - Progress A: Model training for Set A’s is generated with channel parameters #1. Set B in UE measurement should have enough randomness compared to Set A’.

  - Progress B: Set A is generated to make sure to use the same parameters (initial phase, angular spread, delay spread) as the Set B in the Progress A measurement step. NW antenna/beam configurations for Set A and Set B should be deterministic. For some prediction metrics (e.g., option 2 and 3), UE can pass the test without actual measurement if TE does not randomize the transmission of its beams.

- Requirement#3: Support the beam number of Set A

  - TE needs to support Set A for ground truth verification.

Observation 2: To generate Set A and Set B, multiple AoAs/probes test setup is necessary.

Observation 3: Using Multiple AoA test setup with FR2 RRM test can reuse the existing FR2 test setup to save the test cost and a simplified CDL channel model needs to be specified.

Observation 4: The UE’s antenna gain gap between peak and 50%-tile spherical coverage is over 10dB for PC3. The very weak clusters would not influence the UE beam management even if they reach the UE within the UE’s spherical coverage.

Observation 5: The AoDs/ZoDs of very weak clusters are unlikely to influence the selection of the strongest beam in any reasonably designed codebook of Set A and Set B.

Observation 6: The weak clusters that have a limited impact on the beam management could be removed to reduce the number of clusters in the CDL channel model.

Propsoal 1: RAN4 to consider the following options to define the simplfied CDL channel model with eIFF test setup:

- Revmoe the weak clusters that would not influence the UE beam management

- Merge the clusters that have the same/similer AoAs

- Reduce values of per-cluster parameters such as CASD, CASA, CZSD, CZSA

Propsoal 2: RAN4 to evlaute whether the new simiplied CDL channel model generated from eIFF system can be used to properly verify AI/ML BM performance through AIML BM simulations.

Proposal 3: RAN4 to investigate whether measurement uncertainty should be considered when defining AI/ML BM core requirements.

# 4  References

- R4-2502856, ‘WF on AI/ML for air interface [116][131]’, Qualcomm

- TS 38.151

- TR 38.508-1