3GPP TSG-RAN WG4 Meeting #116bis	R4-2513702

Prague, Czech, Oct. 13-17, 2025

Source: 	vivo

Title: 	Discussion on RRM performance for Rel-19 MIMO UEIEBM

Agenda Item:	6.12.2.1

Document for:	Discussion

- Introduction

The core part of Rel-19 MIMO was completed in last meeting with agreements captured in [1]. In this contribution, we provide our analysis on performance part for UEIEBM.

- Discussion

The performance part has been initially discussed in last meeting with status captured in following table.

Issue 2-1-1: Test scope

<Way forward >

|  | QC | Apple | Samsung | CMCC | Huawei | ZTE | Nokia | Ericsson | MediaTek |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| High level: Define TCs to verify UEIBM | Y | Y | Y | Y | Y | Y | Y | Y | Y |
| Whether to cover Event-2/Event-1/Event-7? | Event-2: Y Event-1/Event-7: FFS | Event-2: Y Event-1/Event-7: No | Event-2: Y Event-1: limited TC Event-7: No | All | Event-2 FFS on others | All | Event-2 | Event-2&7 | Event-2 |
| Whether to cover Mode A and Mode B? |  | Either A or B | Both | Both |  | Both | Mode A&B as Test configurations. Can be skip depends on UE capability | Mode A&B as Test configurations. Can be skip depends on UE capability |  |
| Event-trigger for Intra-cell BM for SSB | Y | N | Y | Y | Y | Y | Y | Y | Y |
| Event-trigger for Intra-cell BM for CSI-RS | Y | Y | Y | Y | Y | Y | Y | Y | Y |
| Event-trigger for Inter-cell BM for SSB | Y | Y | Y | Y | FFS | Y | Y | ? | Y |
| Event-trigger for Inter-cell BM for CSI-RS | N | N | N | Y |  | Y | N |  | N |
| Whether to cover no time window and eventInstanceCount-r19 > 1? | N | N | Y | Y |  | Y | Y | Y |  |
| Frequency Range – FR2? |  | Y. No FR1 | Y. FR2 |  |  | Y. FR2 |  |  |  |
| Beam switching delay for TCI states switching | Y Both known and unknown TCI | N |  |  |  | FFS on core req. | FFS on core req. | No MAC CE-based TCI state switching FFS on DCI-based TCI state switching |  |
| L1 measurement reporting delay |  |  |  | Y |  |  |  |  |  |
| CMI |  |  |  |  |  |  | N |  |  |

Regarding whether to define test cases for different Event (i.e., Event 1/2/7), from our understanding, only define test case for event 2 is sufficient to verify the performance of the feature, since event 2 is triggered by the comparation between the new beam and current beam.

Observation 1: Only define test cases for Event 2 is sufficient to verify the measurement performance of the feature.

Proposal 1: Only define test cases for Event 2.

Regarding the type of RS for current beam and new beam, both SSB and CSI-RS shall be considered. For CSI-RS based cases, though RAN4 clarify that applicability rule of inter-cell scenario, there is no need to introduce dedicated inter-cell test cases since the measurement behaviour is the same. For SSB-based case, both intra-cell and inter-cell shall be verified, but necessary applicability rules shall be defined to limit the number of test cases.

Proposal 2: Define test cases for CSI-RS based Event 2 for intra-cell.

Proposal 3: Define test case for SSB-based Event 2 for both intra-cell and inter-cell. If UE can the test case for inter-cell, the test case for intra-cell can be skipped.

Regarding whether to define test case for window-based Event 2, the conclusion for core requirements is summarize as follows:

#### Issue 1-1-a: How to specify requirements for “with time window based with eventInstanceCount-r19” if option 2 in Issue 1-1-1 is supported? Agreement: For the window-based solution: if eventDetectionTimeWindowLength-r19 is provided, the event triggered measurement reporting delay shall be no larger than (eventInstanceCount-r19 + L) * TL1-meas_basic + Tfirst UL channel - eventInstanceCount-r19 is configured by NW - L is the number of measurement instances (TL1-meas_basic) that do not satisfy the event condition, counted after the event counter is started and before the required number of events (eventInstanceCount-r19) are detected within the specified time window (eventDetectionTimeWindowLength-r19). - TL1-meas_basic is the L1 measurement period which agreed in without window-based soltuion, in details, it is the maximum of L1-RSRP measurement periods on the beams corresponding to the event The above requriement is applicable when eventDetectionTimeWindowLength-r19 is not less than (eventInstanceCount-r19 + L-1) * TL1-meas_basic , and no requirement when eventDetectionTimeWindowLength-r19 is less than (eventInstanceCount-r19 + L-1) * TL1-meas_basic.

It could be observed that the window-based event 2 triggering is actually relaxed than non-window-based case as UE is not forbidden to trigger the event earlier. Thus, there is no need to introduced test case for window-based Event 2 reporting.

Proposal 4: Do not introduced test case for window-based Event 2 reporting.

Based on the analysis above, there is no need to introduce other test cases apart from UE initiated/event triggered beam reporting.

- Conclusion

Observation 1: Only define test cases for Event 2 is sufficient to verify the measurement performance of the feature.

Proposal 1: Only define test cases for Event 2.

Proposal 2: Define test cases for CSI-RS based Event 2 for intra-cell.

Proposal 3: Define test case for SSB-based Event 2 for both intra-cell and inter-cell. If UE can the test case for inter-cell, the test case for intra-cell can be skipped.

Proposal 4: Do not introduced test case for window-based Event 2 reporting.

References

[1] R4-2512130	WF on RRM requirements for NR_MIMO_Ph5_Part1