3GPP TSG-RAN WG4 Meeting #116bis	R4-2513721

Prague, CZ, October 13 – 17, 2025

Source:	Nokia, Nokia Shanghai Bell

Title:	Core maintenance for network energy saving

Agenda item:	4.21.1

Document for:	Discussion

- Introduction

At RAN4#116 meeting, the core part of R19 WI on network energy saving was declared to be completed. The agreements were captured in the WFs [1] [2] and the BigCR was approved in [3]. In this contribution, we further discuss some issues identified from review of the CRs and the performance part discussion, respectively.

- OD-SSB SCell operation

  - Terminology for OD-SSB and AO-SSB

In existing approved BigCR [3], first SSB and second SSB are used to indicate OD-SSB and AO-SSB respectively. While it is intended to keep alignment with the RAN1 specification, it is very unclear what they mean in RAN4 spec. We understood the discussion of removing “first/second” is still going in RAN1. In addition, RAN2 does not use “first/second SSB” in their spec, instead they refer to specific IEs which is clearly associated with OD-SSB or AO-SSB. From RAN4, to facilitate reading of the spec, we would prefer using OD-SSB/AO-SSB instead of “first/second SSB” and provide clear definitions for the OD-SSB/AO-SSB referring to the first/second SSB specified in RAN1.

An example of definitions can be as follows:

Always-on SSB: first SS/PBCH blocks with frequency location provided by absoluteFrequencySSB on an SCell as defined in clause 4.4 of TS 38.213 [3]. On-demand SSB: second SS/PBCH blocks indicated by od-ssb-config [12, TS 38.331] or by a MAC CE as defined in clause 4.4 of TS 38.213 [3].

- We prefer using “OD-SSB”/”AO-SSB” instead of “first/second SSB”, and provide clear definitions for the OD-SSB/AO-SSB referring to the first/second SSB specified in RAN1.

- SSB adaptation

Core part issues have been generally finalized at RAN4 #116. However, one issue was raised during the discussion for performance part, which has also impact to RRM core part for SSB adaptation, namely the support of NCD-SSB configurations for SSB adaptation. This is considered in this section.

During the discussion of the performance part, proposals and the recommended WF on NCD-SSB configuration from RAN4#116 [1] were captured by Moderator which are shown below.

Issue 3-3-4: NCD-SSB configuration - Proposals - Proposal 1(Ericsson): - RAN4 to introduce new NCD-SSB configuration with 20ms and 80ms. - Recommended WF - Further discussion.

The issue has been raised at RAN4 #115 and has not been further discussed at RAN4 #116.

It is noted, that this issue is related to both RRM core requirements and RRM performance requirements. The NCD-SSB configuration is specified in TS 38.331, an extract being depicted below.

#### – NonCellDefiningSSB The IE NonCellDefiningSSB is used to configure a NCD-SSB to be used while the UE operates in an RedCap-specific initial BWP that does not contain the CD-SSB or a dedicated BWP that does not contain the CD-SSB. NonCellDefiningSSB information element -- ASN1START -- TAG-NONCELLDEFININGSSB-START NonCellDefiningSSB-r17 ::= SEQUENCE { absoluteFrequencySSB-r17 ARFCN-ValueNR, ssb-Periodicity-r17 ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1 } OPTIONAL, -- Need S ssb-TimeOffset-r17 ENUMERATED { ms5, ms10, ms15, ms20, ms40, ms80, spare2, spare1 } OPTIONAL, -- Need S ... } -- TAG-NONCELLDEFININGSSB-STOP -- ASN1STOP NonCellDefiningSSB field descriptions. absoluteFrequencySSB Frequency of the NCD-SSB. The network configures this field so that the SSB is within the bandwidth of the BWP configured in BWP-DownlinkCommon.. ssb-Periodicity The periodicity of this NCD-SSB. The network configures only periodicities that are larger than the periodicity of serving cell's CD-SSB. If the field is absent, the UE applies the SSB periodicity of the CD-SSB (ssb-periodicityServingCell configured in ServingCellConfigCommon or ServingCellConfigCommonSIB).. ssb-TimeOffset The time offset between CD-SSB of the serving cell and this NCD-SSB. Value ms5 means the first burst of NCD-SSB is transmitted 5ms later than the first burst of CD-SSB transmitted after the first symbol of SFN=0 of the serving cell, value ms10 means the first burst of NCD-SSB is transmitted 10ms later than the first burst of CD-SSB transmitted after the first symbol in SFN=0 of the serving cell, and so on. If the field is absent, UE considers that the time offset between the first burst of CD-SSB transmitted in the serving cell and the first burst of this NCD-SSB transmitted is zero. For UEs in TDD cells, the network configures this time offset to be an integer multiple of the periodicity of the serving cell's CD-SSB..

When NCD-SSB is used for SSB adaptation, then in our view the NCD-SSB configuration has to be updated to include the combination of SSB periodicities which are used before and after SSB adaptation via DCI command 2_9. There are different options:

Option 1: 	Extend the Rel-17 IE by adding a Rel-19 IE with the parameter ssb-PeriodicityAdditional-r19 to define the additional SSB periodicity value.  If the ssb-Periodicity-r17 specifies the default (denser) value, then ssb-PeriodicityAdditional-r19 specifies the sparser value, e.g. ranging from 20 ms to 160 ms with some spare values. The ssb-TimeOffset parameter is then shared between both SSB periodicities (no different time offset allowed according RAN1 design). This option is flexible allowing to configure different SSB periodicities as part of the NCD-SSB IE and is future-proof in extending the range of the additional SSB periodicity. The extension of the NCD-SSB IE is shown below.

NonCellDefiningSSB information element

-- ASN1START

-- TAG-NONCELLDEFININGSSB-START

NonCellDefiningSSB-r17 ::=      SEQUENCE {

absoluteFrequencySSB-r17        ARFCN-ValueNR,

ssb-Periodicity-r17             ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1 }       OPTIONAL,   -- Need S

ssb-TimeOffset-r17              ENUMERATED { ms5, ms10, ms15, ms20, ms40, ms80, spare2, spare1 }      OPTIONAL,   -- Need S

ssb-PeriodicityAdditional-r19   ENUMERATED { ms20, ms40, ms80, ms160, spare4, spare3, spare2, spare1 }   OPTIONAL,   -- Need S

...

}

-- TAG-NONCELLDEFININGSSB-STOP

-- ASN1STOP

Option 2: 	The spare bits (2) for ssb-Periodicity-r17 are used to accommodate two combinations of SSB periodicities, i.e. spare 1 := SSB periodicity combination with 20ms and 80ms and spare 2 := SSB periodicity combination with 20ms and 40ms.

In fact, option 2 only allows a limited set (=2) of SSB periodicity combinations and hence is not future proof. Thus, we propose to proceed along option 1 and ask RAN2 to do such modifications to IE NonCellDefiningSSB.

Proposal #2: 	RAN4 to ask RAN2 if NCD-SSB is considered for SSB adaptation in TS 38.331 and if not, to extend the NCD-SSB IE in TS 38.331 by adding the Rel-19 field for the additional SSB periodicity based on the depicted change below, or similarly:

NonCellDefiningSSB information element

-- ASN1START

-- TAG-NONCELLDEFININGSSB-START

NonCellDefiningSSB-r17 ::=      SEQUENCE {

absoluteFrequencySSB-r17        ARFCN-ValueNR,

ssb-Periodicity-r17             ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1 }       OPTIONAL,   -- Need S

ssb-TimeOffset-r17              ENUMERATED { ms5, ms10, ms15, ms20, ms40, ms80, spare2, spare1 }      OPTIONAL,   -- Need S

ssb-PeriodicityAdditional-r19   ENUMERATED { ms20, ms40, ms80, ms160, spare4, spare3, spare2, spare1 }   OPTIONAL,   -- Need S

...

}

-- TAG-NONCELLDEFININGSSB-STOP

-- ASN1STOP

- 4. Conclusion

In this contribution, we discussed open issues on RRM core requirements both for OD-SSB SCell operation and SSB adaptation based on RAN1/2 progress and presented our views on the potential RAN4 impact.

The following proposals are made:

- We prefer using “OD-SSB”/”AO-SSB” instead of “first/second SSB”, and provide clear definitions for the OD-SSB/AO-SSB referring to the first/second SSB specified in RAN1.

- RAN4 to ask RAN2 if NCD-SSB is considered for SSB adaptation in TS 38.331, and if not, to extend the NCD-SSB IE in TS 38.331 by adding the Rel-19 field for the additional SSB periodicity based on the depicted change below, or similarly:

NonCellDefiningSSB information element

-- ASN1START

-- TAG-NONCELLDEFININGSSB-START

NonCellDefiningSSB-r17 ::=      SEQUENCE {

absoluteFrequencySSB-r17        ARFCN-ValueNR,

ssb-Periodicity-r17             ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1 }       OPTIONAL,   -- Need S

ssb-TimeOffset-r17              ENUMERATED { ms5, ms10, ms15, ms20, ms40, ms80, spare2, spare1 }      OPTIONAL,   -- Need S

ssb-PeriodicityAdditional-r19   ENUMERATED { ms20, ms40, ms80, ms160, spare4, spare3, spare2, spare1 }   OPTIONAL,   -- Need S

...

}

-- TAG-NONCELLDEFININGSSB-STOP

-- ASN1STOP

The draft LS reply is found in the Annex of this contribution.

- References

[1]	R4-2512327, WF on RRM requirements for Netw_Energy_NR_enh_Part1, Ericsson

[2]	R4-2512137, WF on RRM requirements for Netw_Energy_NR_enh_Part2, Huawei

[3]	R4-2510919, Big CR for RRM requirements of Enhancements of network energy savings, Ericsson, Huawei

- Annex: Draft LS to RAN2

3GPP TSG-RAN WG4 Meeting #116bis	R4-251xxxx

Prague, Czech Republic, October 13th – 17th, 2025

Title:	LS on NCD-SSB configuration for SSB adaptation

Response to:

Release:	Rel-19

Work Item:	Netw_Energy_NR_enh-Core

Source:	RAN WG4

To:	RAN WG2

Cc: 	RAN WG1

Contact Person:

Name:	Juergen Hofmann

Email:	juergen.hofmann@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN4 would like to inform RAN2 about the agreement to apply SSB adaptation to NCD-SSB configuration for SCell aligned with previous RAN1 agreement. Thus, RAN4 would like to check with RAN2, if NCD-SSB is supported for SSB adaptation in SCell in TS 38.331. If not, RAN4 would like to ask RAN2 to provide support for it. One potential way is to add the additional NCD-SSB configuration to the NonCellDefiningSSB information element as depicted in the bold marked change below.

NonCellDefiningSSB information element

-- ASN1START

-- TAG-NONCELLDEFININGSSB-START

NonCellDefiningSSB-r17 ::=      SEQUENCE {

absoluteFrequencySSB-r17        ARFCN-ValueNR,

ssb-Periodicity-r17             ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1 }       OPTIONAL,   -- Need S

ssb-TimeOffset-r17              ENUMERATED { ms5, ms10, ms15, ms20, ms40, ms80, spare2, spare1 }      OPTIONAL,   -- Need S

ssb-PeriodicityAdditional-r19   ENUMERATED { ms20, ms40, ms80, ms160, spare4, spare3, spare2, spare1 }   OPTIONAL,   -- Need S

...

}

-- TAG-NONCELLDEFININGSSB-STOP

-- ASN1STOP

RAN4 respectfully asks RAN2 to take above into account and evaluate whether any update is needed to their TS 38.331.

2. Actions:

To RAN2:

ACTION: RAN4 respectfully asks RAN2 to take above into account and evaluate whether any update is needed to their TS 38.331.

3. Date of Next RAN4 Meetings:

RAN4#117	17th – 21st November 2025			Dallas, US

RAN4#118	9th – 13th February 2026			Gothenburg, Sweden