3GPP TSG-RAN WG4 Meeting #116bis											R4-2513724

Prague, Czech Republic, Oct. 13-17, 2025

Agenda item:	12

Source:	KDDI, NTT DOCOMO, SoftBank

Title:	Introduction of a new band for 1.5GHz

Document for:	Information

Background

In Rel-15, band n74(UL: 1427-1470 MHz, DL: 1475-1518 MHz) was standardized to aim for achieving an eco-system with wider frequency range, in order to accommodate demands of some MNOs from different regions/countries. Actually, the above frequency range is fully aligned to the result of WRC-15. Therefore, band n11 and n21 having same frequency ranges as Band 11 and Band 21 were not standardized in 3GPP.

However, after that, based on other kind of different demands, other bands focusing with narrower frequency ranges were also standardized eventually, for example n32 and n75/n76 [1].

On the other hand, a technical complexity of n74 is found from UE implementation point of view. This is mainly because the 5MHz gap between UL/DL of n74 is too narrow as known well. Nowadays, no any n74 UE products are still provided eventually.

Discussion

The co-signing Japanese MNOs of this contribution operating following frequency as LTE Band 11 and Band 21 [2]. However, when they refarm the legacy frequency to NR, in case of adapting n74, there are issues as shown in the previous section. Therefore, considering more realistic UE implementation, we propose to standardize a new band on following frequency ranges. This proposal can expand a gap between DL/UL from 5 to 13 MHz. Noting typical timelines to commercialisation, an expedited path to standardisation is required.

- UL: 1427.9-1462.9 MHz

  - SoftBank: 1427.9-1437.9 MHz, KDDI: 1437.9-1447.9 MHz, Docomo: 1447.9-1462.9 MHz

- DL: 1475.9-1510.9 MHz

  - SoftBank: 1475.9-1485.9 MHz, KDDI: 1485.9-1495.9 MHz, Docomo: 1495.9-1510.9 MHz

It is our view that the scope of required standardisation work will be based on a single duplexer solution considering recent progress of advanced technologies.

We request that 3GPP RAN recognizes the commercial driver for this work to proceed. The co-signing companies will proactively work towards a proposal at RAN#110.

References

[1] 		TS 38.101-1 NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone

[2]		https://www.tele.soumu.go.jp/resource/e/search/myuse/use0303/1400m.pdf