# 3GPP TSG-RAN WG4 Meeting #116bis	R4-2513734

# Prague, Czech Republic, Oct. 13-17, 2025

Agenda Item:	7.1.4

Source: 	LG Electronics

Title: 	Discussion on UE RF enhancements for NR FR1, Phase 5

Document for:	Discussion

Introduction

In the RAN#109, the WID on “UE RF enhancements for NR FR1, Phase 5” was approved. In this discussion, we share our point of view for the UE RF enhancements for NR FR1 on the approved WID [1].

Table 1 WID on UE RF enhancements for NR FR1, Phase 5 [1].

High power UE (HPUE) for NR single carrier operation - Specify the UE RF requirements for single carrier TDD to support 4Tx for CPE/FWA UE with 4x26dBm PA configurations - Co-existence study for ACLR, specify ACLR requirements if needed - Specify the UE RF requirements including maximum output power (MOP), MPR requirements, etc. - Supporting 4x4 UL MIMO and TxD - Specify the band specific requirements for exemplary bands - Example bands: n41, n77/n78, n79, n104 - Maximum output power is up to 32 dBm - RAN4 to determine whether to reuse PC1 or introduce new power class - PC1.5 with 2Tx for FDD bands for handheld and FWA UE - Specify the UE RF requirements including maximum output power (MOP), MPR requirements, etc. - Supporting 2x2 UL MIMO and TxD - Baseline PA configuration: 2 x 26 dBm - Specify the band specific requirements for exemplary bands - Example bands: n1, n25 - PC1.5 1Tx for TDD bands for FWA UE - Study the possibility of applying PC1.5 with 2Tx MPR/A-MPR requirements to PC1.5 with 1Tx, if feasible, specify the corresponding UE RF requirements, including maximum output power (MOP), MPR/A-MPR requirements, etc. - Baseline PA configuration: 1 x 29 dBm - Example bands: n41, n77/n78, n79 - This work is to start in 2026 Q4 - 2Tx for TDD bands for FWA UE - Study the possibility of applying PC1 with 4Tx A-MPR/MPR requirements to PC1 with 2Tx, if feasible, specify the corresponding UE RF requirements, including maximum output power (MOP), MPR/A-MPR requirements, etc. - Baseline PA configuration: 2 x 29 dBm Example bands: n41, n77/n78, n79, n104 - Maximum output power is up to 32 dBm - RAN4 to determine whether to reuse PC1 or introduce new power class - This work is to start in 2026 Q4

Discussion

Single carrier TDD to support 4Tx for CPE/FWA UE with 4x26dBm PA configurations

The current PC1 requirement specifies a maximum output power (MOP) of 31 dBm, whereas a 4Tx configuration with 4 × 26 dBm PAs can achieve up to 32 dBm. If RAN4 decides to adopt 32 dBm as the MOP, this would exceed the existing PC1 limit, and therefore a new power class may need to be introduced. Alternatively, if PC1 is reused, the MOP would need to remain at 31 dBm.

For PC1.5 with 4Tx, the same ACLR requirements were reused without introducing additional complexity. A similar approach could be considered for PC1 to maintain consistency and minimize additional workload. Once the MOP issue is resolved, the related requirements such as MPR and A-MPR can be defined accordingly.

Observation 1: A 4Tx configuration with 4 × 26 dBm PAs can achieve up to 32 dBm, which exceeds the current PC1 limit of 31 dBm.

Observation 2: For PC1.5 with 4Tx, the same ACLR requirements were reused without introducing additional complexity. A similar approach could be considered for PC1 to maintain consistency and minimize additional workload.

Proposal 1: Consider applying approach used in PC1.5 with 4Tx to PC1 4Tx, including existing ACLR reuse, to minimize workload.

PC1.5 with 2Tx for FDD bands for handheld and FWA UE

The baseline PA configuration is 2 × 26 dBm, supporting 2x2 UL MIMO and TxD. Example bands include n1 and n25. It is expected that Tx requirements for FDD 2Tx will be similar to those for TDD 2Tx, while Rx requirements may require additional consideration due to simultaneous Tx/Rx operation in FDD mode.

Observation 3: Tx requirements for FDD 2Tx are expected to be largely aligned with those for TDD 2Tx, including parameters such as MOP, MPR/A-MPR, and ACLR. Therefore, reusing the TDD 2Tx approach for Tx requirements could minimize complexity.

Observation 4: Unlike TDD, FDD operates with simultaneous Tx and Rx on different frequencies, which introduces the potential for Receiver Desensitization (RSD) due to Tx leakage into the Rx path. This effect may be more pronounced for high-power UEs (PC1.5), and therefore RSD requirements may need to be defined.

Proposal 2: Align Tx requirements for FDD 2Tx with those for TDD 2Tx to maintain consistency and reduce workload.

Proposal 3: Specify the RSD requirement for example bands n1 and n25.

Conclusion

Observation 1: A 4Tx configuration with 4 × 26 dBm PAs can achieve up to 32 dBm, which exceeds the current PC1 limit of 31 dBm.

Observation 2: For PC1.5 with 4Tx, the same ACLR requirements were reused without introducing additional complexity. A similar approach could be considered for PC1 to maintain consistency and minimize additional workload.

Observation 3: Tx requirements for FDD 2Tx are expected to be largely aligned with those for TDD 2Tx, including parameters such as MOP, MPR/A-MPR, and ACLR. Therefore, reusing the TDD 2Tx approach for Tx requirements could minimize complexity.

Observation 4: Unlike TDD, FDD operates with simultaneous Tx and Rx on different frequencies, which introduces the potential for Receiver Desensitization (RSD) due to Tx leakage into the Rx path. This effect may be more pronounced for high-power UEs (PC1.5), and therefore RSD requirements may need to be defined.

Proposal 1: Consider PC1.5 with 4Tx approach for PC1 4Tx, including reusing existing ACLR requirements, to minimize additional workload.

Proposal 2: Align Tx requirements for FDD 2Tx with those for TDD 2Tx to maintain consistency and reduce workload.

Proposal 3: Specify the RSD requirement for example bands n1 and n25.

- References

# [1] R4-252942, New WID: UE RF enhancements for NR FR1, Phase 5 (vivo)