3GPP TSG-RAN WG4 Meeting #116bis                                                               R4-2513785

Prague, CZ, 13th-17th, Oct, 2025

Source:	Samsung

Title:	On inter-vendor training collaboration and interoperability for two-sided models

Agenda item:			7.9.3

Document for:	Discussion

# 1. Introduction

Based on Rel-18 study on AI/ML Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface and the follow-up study on the CSI compression with two-sided model has been conducted in Rel-19. Accordingly, Rel-20 new WID was originally approved in RAN#108, which has been further updated in RAN#109 [1]. Among objectives of this work item, the following are RAN4 relevant work scope on inter-vendor training collaboration for two-sided AI/ML models, and RAN4 requirements of interoperability and RRM:

Inter-vendor training collaboration for two-sided AI/ML models - Fully defined/specified reference model (“Direction C”) with RAN1 scalability study outcome taken into account [RAN4/RAN1] – check-point in RAN#110 upon RAN4 feedback - Specification of standardized encoder model structure plus parameter exchange (“Direction A, sub-option 3a-1” without target CSI sharing) leveraging defined/reference model of “Direction C” and taking RAN1 scalability study outcome into account [RAN4/RAN1/RAN2/RAN3] – check-point in RAN#110 upon SA WG feedback - Specification of standardized dataset format/content plus dataset exchange (“Direction A, sub-option 4-1”) [RAN1/RAN2/RAN3/RAN4] – check-point in RAN#110 upon SA WG feedback Interoperability and RRM requirement [RAN4]: - To define the requirement for the encoder, specify the standardized test decoder to guarantee interoperability - The fully specified test decoder, data set, and corresponding reference encoder in RAN4 to be used for inter-vendor collaboration - Necessary RRM requirement for specified LCM procedures. - NOTE: offline training is assumed for the purpose of this project. Strive to leverage on RAN1’s study on scalable model structure in FS_NR_AIML_Air.

As per the WID, the RAN1 study outcome on model scalability should be taken into account when RAN4 specifies the test decoder and reference encoder. Model scalability is essential for model specification because it’s impractical to design one specific model for each pair of input size and output size. This paper discusses RAN4 work on the model scalability in Rel-20.

# 2. Discussion

## 2.1 Model structure scalability in Rel-19 RAN1

In Rel-19, RAN4 studied the feasibility of model interoperability without considering scalable input size and output size. RAN1 has study the model scalability on three scalable aspects based on the agreed transformer-based model.

According to [2], agreements on model structure scalability are listed below.

RAN1 #120 agreements Agreement For model structure scalability study for temporal domain Case 0, - For the choice of token dimension and feature dimension, - Alt 1: Use subband as the token dimension and Tx port as a feature dimension - The number of tokens varies with the number of subbands. - Alt 2: Use Tx port as the token dimension and subband as a feature dimension - The number of tokens varies with the number of Tx ports. - Alt 3: Use a fixed-size sub-block of Tx ports and subbands matrix (e.g., n_Tx_ports*m_Subbands) as a token and represent the input as a sequence of tokens. - The number of tokens varies with the number of Tx ports and the number of subbands. - For scalability over the feature dimension, - Alt1: specific embedding layer for each feature size - Alt2: a common embedding layer with padding (e.g., zero-padding or other techniques for padding values) - For scalability over the token dimension, - Alt1: positional embedding specific to each token index - Ntoken,active tokens out of Ntoken token positions are used as input. - Alt2: Padding at the input - For scalability over payload configurations, - Alt1: specific output linear layer for each payload configuration - Alt2: truncation/masking of the output linear layer output - Alt3: by varying quantization parameters - Notes - Other Alternatives are not precluded. - Different Alternatives may be used in combination. - Same/similar approach is applied at the decoder side. - Evaluations to consider: - Case 1 (scalable structure): Scalable model structure described above - Using model structure as indicated in above diagram with fixed hyperparameters, companies may train a single parameter set or different parameter sets across different {number of Tx ports, CSI feedback payload size, bandwidth} assuming a common model structure. - To report whether a single parameter set or different parameter sets were used across different {number of Tx ports, CSI feedback payload size, bandwidth}. (e.g., single parameter set across different payload sizes and bandwidths, different parameter set across different number of Tx ports) - Case 2 (dedicated structure): Using model structure as indicated in above diagram with different hyperparameters, where the input and the output related hyperparameters are chosen optimally corresponding to each specific {number of Tx ports, CSI feedback payload size, bandwidth} without scalability considerations. - Different parameter sets are trained across different number of Tx ports, CSI feedback payload sizes, and bandwidths. - For each scalable model structure choice, to evaluate the SGCS performance of the non-AI/ML benchmark (e.g., Rel-16 eType2), Case 1, and Case 2, for each of {number of Tx ports, CSI feedback payload size, bandwidth}, and report the average gain (%) in SGCS of Case 1 and Case 2 over the non-AI/ML benchmark, as well as the loss (%) in the average gain of Case 1 w.r.t. Case 2. The average is performed by first calculating the SGCS gain (%) for each {number of Tx ports, CSI feedback payload size, bandwidth} and then averaging the SGCS gain (%) values over {number of Tx ports, CSI feedback payload size, bandwidth}. Agreement For model structure scalability study for temporal domain Case 0, for the choice of {number of Tx ports, CSI feedback payload size, bandwidth}, take the following as baseline values. - Number of Tx ports - [N1, N2, P] = [2, 8, 2] (32 CSI-RS ports) - [N1, N2, P] = [4,4,2] (32 CSI-RS ports) - [N1, N2, P] = [2, 4, 2] (16 CSI-RS ports) - CSI feedback payload size - Multiple values, encouraged to pick at least one value from each of small, medium, and large payload regions X, Y, and Z. - Bandwidth & subband size - All subbands - A subset of all the subbands.

According to [3], following evaluation observations on model scalability are summarized.

Observations For choice of token/feature dimension in the scalable model design, companies observe that there is small loss in SGCS for Case1 over Case2 under tokenization (Alt1, Alt3) For layer1, under tokenization (Alt1), - 8 sources [Qualcomm, CATT, Xiaomi, vivo, ZTE, Fujitsu, Samsung, Huawei] observe that average gain in SGCS of Case 1 over Case 2 is in the range of -3.34% to 0.33% with median value of -1.10% - 2 sources [CATT, Xiaomi] observe average gain in SGCS of Case 1 (scalable structure) over benchmark [G1] is in the range of 2.04% to 7.84% with median value of 6.61% - 3 sources [CATT, Xiaomi, Ericsson] observe average gain (%) in SGCS of Case 2 (scalable structure) over benchmark [G2] is in the range of 5.17% to 10.2% with median value of 10.1% For layer1, under tokenization (Alt3), - 1 source [vivo] observes average gain (%) in SGCS of Case 1 over Case 2 is in the range of -0.7% to 0.4% with median value of -0.3% - 1 source [Ericsson] observes average gain (%) in SGCS of Case 2 (scalable structure) over benchmark [G2] is in the range of 5.4% to 6% For scalability over the feature dimension, companies observe that there is small loss in SGCS for Case1 over Case2 assuming (Alt1, Alt2, Alt1+Alt2) For layer1, under (Alt1), - 4 sources [Qualcomm, CATT, Xiaomi, Fujitsu ] observes average gain (%) in SGCS of Case 1 over Case 2 is in the range of -3.34% to -0.4% with median value of -2.08% - 2 sources [CATT, Xiaomi] observe average gain (%) in SGCS of Case 1 (scalable structure) over benchmark [G1] is in the range of 3% to 7.84% with median value of 6.71% - 2 sources [CATT, Xiaomi] observe average gain (%) in SGCS of Case 2 (scalable structure) over benchmark [G2] is in the range of 5.17% to 10.2% with median value of 10.1% For layer1, under (Alt2), - 7 sources [Qualcomm, Xiaomi, vivo, ZTE, Fujitsu, Samsung, Huawei] observe average gain in SGCS of Case 1 over Case 2 is in the range of -3.2% to 0.1% with median value of -0.85% - 1 source [Xiaomi] observes gain in SGCS of Case 1 (scalable structure) over benchmark [G1] is 2.04% - 1 source [Xiaomi] observes gain (%) in SGCS of Case 2 (scalable structure) over benchmark [G2] is 5.17% For layer1, under (Alt1+Alt2), - 1 source [Qualcomm] observes average gain (%) in SGCS of Case 1 over Case 2 is in the range of -3.3% to -2.9% with median value of -3.1%. For scalability over the token dimension, companies observe that there is small loss in SGCS for Case1 over Case2 assuming (Alt1, Alt2) For layer1, under (Alt1), - 2 sources [Qualcomm, vivo] observe average gain in SGCS of Case 1 over Case 2 is in the range of -3.2% to 0.9% with median value of -1.5%. For layer1, under (Alt2), - 3 sources [CATT, vivo, ZTE ] observe average gain in SGCS of Case 1 over Case 2 is in the range of -3.34% to 0.4% with median value of -0.75%. - 1 source [CATT] observes average gain (%) in SGCS of Case 1 (scalable structure) over benchmark [G1] is in the range of 6.51% to 7.84% with median value of 6.87% - 1 source [CATT] observes average gain (%) in SGCS of Case 2 (scalable structure) over benchmark [G2] is in the range of 10.12% to 10.2% with median value of 10.1% For scalability over the payload configurations, companies observe that there is small loss in SGCS for Case1 over Case2 assuming (Alt1, Alt2) For layer1, under (Alt1), - 4 sources [Qualcomm, CATT, Xiaomi, Huawei] observe average gain (%) in SGCS of Case 1 over Case 2 is in the range of -0.73% to -3.34% with median value of -2.08% - 2 sources [CATT, Xiaomi] observe average gain (%) in SGCS of Case 1 (scalable structure) over benchmark [G1] is in the range of 3% to 7.84% with median value of 6.71% - 2 sources [CATT, Xiaomi] observe average gain (%) in SGCS of Case 2 (scalable structure) over benchmark [G2] is in the range of 5.17% to 10.2% with median value of 10.1% For layer1, under (Alt2), - 3 sources [Xiaomi, vivo, ZTE] observe average gain in SGCS of Case 1 over Case 2 is in the range of -3.2% to 0.4% with median value of -0.85% - 1 source [Xiaomi] observes average gain (%) in SGCS of Case 1 (scalable structure) over benchmark [G1] is 2.04% - 1 source [Xiaomi] observes average gain (%) in SGCS of Case 2 (scalable structure) over benchmark [G2] is 5.17% For scalability over feature dimension, token dimension and payload configuration jointly, For tokenization Alt1, feature scalability Alt1, token scalability Alt2 and payload scalability Alt1, - 1 source [CATT] observes average gain in SGCS of Case 1 over case 2 is in the range of -3.34%~2.03% with median value of -2.93% - 1 source [CATT] observes average gain (%) in SGCS of Case 1 (scalable structure) over benchmark [G1] is in the range of 6.51%~7.84% - 1 source [CATT] observes average gain (%) in SGCS of Case 2 (scalable structure) over benchmark [G2] is in the range of 10.12%~10.23% For tokenization Alt1, feature scalability Alt2, token scalability Alt2, payload configuration Alt2, - 2 sources [ZTE, vivo] observe average gain in SGCS of Case 1 over case 2 is in the range of -2.8%~1.2% with median value of -1.2%, under same parameter set. - 1 source [vivo] observes average gain in SGCS of Case 1 over Case 2 is in the range of -0.1% ~ 0.8% with median value of 0.1%, under different parameter set For tokenization Alt1, feature scalability Alt2, token scalability Alt1, payload configuration Alt2, - 1 source [vivo] observes average gain in SGCS of Case 1 over Case 2 is in the range of -1.7% ~ 5.4% with median value of -2.6%, under same parameter set - 1 source [vivo] observes average gain in SGCS of Case 1 over Case 2 is in the range of -2.6% ~ 0% with median value of -0.9%, under different parameter set For tokenization Alt1, feature scalability Alt1, payload configuration Alt1, - 1 source [Xiaomi] observes average loss in SGCS of Case 1 over case 2 is in the range of -4.76%~-0.82% with median value of -2.08%. - 1 source [Xiaomi] observes average gain (%) in SGCS of Case 1 (scalable structure) over benchmark [G1] is 3% - 1 source [Xiaomi] observes average gain (%) in SGCS of Case 2 (scalable structure) over benchmark [G2] is 5.17% For tokenization Alt1, feature scalability Alt2, payload configuration Alt2, - 1 source [Xiaomi] observes average loss in SGCS of Case 1 over case 2 is in the range of -5.52%~-1.59% with median value of -2.96%. - 1 source [Xiaomi] observes average gain (%) in SGCS of Case 1 (scalable structure) over benchmark [G1] is 2.04% - 1 source [Xiaomi] observes average gain (%) in SGCS of Case 2 (scalable structure) over benchmark [G2] is 5.17% For tokenization Alt1, feature scalability Alt2, token scalability Alt2, - 1 source [ZTE] observes average gain in SGCS of Case 1 over case 2 is in the range of -0.4%~0.9% with median value of -0.23%. For tokenization Alt3, token scalability Alt1, payload configuration Alt2, - 1 source [vivo] observes average gain in SGCS of Case 1 over Case 2 is in the range of -2.1% ~ 1.6% with median value of -0.5%, under same parameter set - 1 source [vivo] observes average gain in SGCS of Case 1 over Case 2 is in the range of -0.8% ~ 0% with median value of -0.4%, under different parameter set

Observation 1: Three scalability aspects, the number of Tx ports, CSI feedback payload size and subband number are considered variable to verify the model scalability in RAN1.

There are too many potential combinations of the variable three aspects. To reduce the workload of simulation, RAN4 need to select a subset of all the alternatives, including typical Tx port number and antenna layout, typical payload sizes and typical subband number.

Proposal 1: RAN4 should determine a set of combinations with typical values of Tx port number and antenna layout, CSI feedback payload size and subband number to align the scalability solutions.

The preferred model scalability methods with acceptable losses are highlighted above and summarized into Table 1. It could be used as the starting point for RAN4 aligning the scalable models.

Proposal 2: The following model scalability methods could be the starting point for aligning the scalable models in RAN4.

Table 1. basic methods for model scalability of Transformer backbone.

| choice of token dimension and feature dimension | Use subband as the token dimension and Tx port as a feature dimension |
| --- | --- |
| scalability over the feature dimension | a common embedding layer with padding (e.g., zero-padding or other techniques for padding values) |
| scalability over the token dimension | Padding at the input |
| scalability over payload configurations | truncation/masking of the output linear layer output |

## 2.2 Model structure scalability in Rel-20 RAN4

To evaluate the performance of model scalability, RAN4 should generate one mixed dataset that takes the set of typical value combinations on the scalable dimensions into account.

Proposal 3: One mixed dataset should be generated that contains the typical values on the three scalable dimensions.

For different model backbones, different scalability methods are used. For Transformer backbone, as provided in the above Proposal 2, the preferred methods for model scalability are provided based on RAN1 conclusion.

For CNN backbone, there is no need to discriminate token dimension and feature dimension. Subband dimension could be appointed as the height dimension and port dimension could be appointed as the width dimension. The methods for model scalability of CNN backbone are proposed as Table 2.

Table 2. methods for model scalability of CNN backbone.

| scalability over the subband dimension | Padding at the input to the maximum size |
| --- | --- |
| scalability over the port dimension | Padding at the input to the maximum size |
| scalability over payload configurations | truncation/masking of the output linear layer output |

Proposal 4: If CNN backbone is used, the methods for model scalability are proposed as below table.

Table 2. methods for model scalability of CNN backbone.

| scalability over the subband dimension | Padding at the input to the maximum size |
| --- | --- |
| scalability over the port dimension | Padding at the input to the maximum size |
| scalability over payload configurations | truncation/masking of the output linear layer output |

To evaluate the scalability performance, two possible designs are considered.

- Alt 1 (scalable structure with joint training on scalable scenarios): use the scalable model structure and scalability methods, train the model with single parameter set across different {port number, payload size, subband number} data.

- Alt 2 (dedicated model structure trained on dedicated data): train one dedicated model with the input size and output size exactly matches the dedicated {port number, payload size, subband number} data.

The performance losses between case 1 and case 2 are used to evaluated the scalability methods.

Observation 2: Both Alt 1 (scalable model structure with joint training on scalable scenarios) and Alt 2 (dedicated model structure trained on dedicated data), are possible designs for scalability evaluation.

# 3. Conclusion

In this contribution, we provided our viewpoints on the model structure scalability, accordingly the following observations and proposals are obtained:

Observation 1: Three scalability aspects, the number of Tx ports, CSI feedback payload size and subband number are considered variable to verify the model scalability in RAN1.

Proposal 1: RAN4 should determine a set of combinations with typical values of Tx port number and antenna layout, CSI feedback payload size and subband number to align the scalability solutions.

Proposal 2: following model scalability methods could be the starting point for aligning the scalable models in RAN4.

Table 1. basic methods for model scalability of Transformer backbone.

| choice of token dimension and feature dimension | Use subband as the token dimension and Tx port as a feature dimension |
| --- | --- |
| scalability over the feature dimension | a common embedding layer with padding (e.g., zero-padding or other techniques for padding values) |
| scalability over the token dimension | Padding at the input |
| scalability over payload configurations | truncation/masking of the output linear layer output |

Proposal 3: One mixed dataset should be generated that contains the typical values on the three scalable dimensions.

Proposal 4: If CNN backbone is used, the methods for model scalability are proposed as below table.

Table 2. methods for model scalability of CNN backbone.

| scalability over the subband dimension | Padding at the input to the maximum size |
| --- | --- |
| scalability over the port dimension | Padding at the input to the maximum size |
| scalability over payload configurations | truncation/masking of the output linear layer output |

Observation 2: Both Alt 1 (scalable model structure with joint training on scalable scenarios) and Alt 2 (dedicated model structure trained on dedicated data), are possible designs for scalability evaluation.

# 4. Reference

[1] RP-252445 (revised from RP-251870), “Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2”, Qualcomm, RAN#109.

[2] R1-2501682, “Report of 3GPP TSG RAN WG1 #120”

[3] R1-2503201, “Report of 3GPP TSG RAN WG1 #120bis”