3GPP TSG-RAN WG4 Meeting #116b							         	       	R4-2513810

Prague, Czech Republic, Oct. 13-17, 2025

Title: 	Feasibility of LB switching FDD+SDL example combinations

Source: 	Skyworks Solutions, Inc.

Agenda Item:	7.8.3	UE RF requirements [NR_LBCA_Sw_enh-Core]

Document for:	Approval

# 1	Background

In RAN#109 a new Release 20 WI on enhanced LBLB switching was agreed. In this contribution we discuss the feasibility of the RF front-end for the proposed example band combinations.

# Discussion

## 2.1 HPUE objectives in R20 new WI: UE RF enhancements for NR FR1

The new Release 20 WI on LB switching enhancements for NR FR1 [1] introduces FDD+SDL bands combinations switching between FDD UL/DL and FD DL+SDL DL, the corresponding RAN4 UERF objectives are copied below in italics.

- Specify the necessary UE RF and RRM core requirements to allow switching between {case 1, case 3} to enable low band carrier aggregation via switching for the dual downlink scenario between FDD band and SDL band [RAN4].

  - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2.

  - Case 3: Rx on both FDD carrier 1 and SDL carrier 2 and no Tx on FDD carrier 1.

  - Example band combinations: CA_n12A-n29A and CA_n28A-n67A, additional FDD+SDL band combinations can be added in the specific basket WI after this WI is completed.

  - Reuse the time mask requirements defined in Rel-19 for low-band CA via switching as much as possible

  - Note: Case 2 is specified in Rel-19 WID of NR_LBCA_Sw

- Reuse the semi-static switching mechanism based on RRC configuration defined in Rel-19 for low-band CA via switching

- Consider the following deployment constraints:

  - The carrier frequency for all cases is <1GHz.

  - Co-located and synchronized network deployment for both carriers.

  - Both carriers are in a single TAG.

  - SCS 15Khz on both carriers.

  - No in-gap interference between DL carriers is expected

Note 1: Reuse Rel-19 LB-CA RAN4 requirements as much as possible.

In this contribution, we provide our preliminary input on RF front-end feasibility for the example band combinations CA_n12A-n29A and CA_n28A-n67A.

## 2.2 RF front end architecture based on two LB antennas

To get a sense for the RF front-end architecture it is important to see where the bands sit in the low band frequency range which can be visualized in Figure 1.

Figure 1: Band n12, n28, n29 and n67

Observations:

- n67 SDL band overlaps with n28UL (n28B side) and is adjacent to Band n28 DL

- n29 SDL band has only 1MHz gap to Band n12 (and n85) UL and 1MHz gap to Band n12 DL (and adjacent to n85 DL)

- In most cases:

  - Band n12 uses a n85 implementation

  - Band n28 uses a full band filter.

Based on these observations, there are a few consequences:

- For the SDL DL + FDD DL operation, the two DL filters should be combined on the main and diversity path as there is only a 1MHz or no gap. For separate filters, 4 antennas would be needed.

- Since the SDL+FDD DL bands are combined in the filter, they need to share the LNA and the two bands can only be separated after the LNA with separate down conversions

- Combined filter cannot be reused for the FDD UL/DL operation as:

  - n29 is only 1MHz away from n12(n85) UL band

  - n67 overlaps with n28F and only has 5MHz gap to n28A

- Combined filter may be reusable for the SDL band CA operation with other bands due to blocking requirements.

- Combined DL filter bandwidth is:

  - 18+11=29MHz for n12DL+n29DL (4% BW)

  - 45+20=65MHz for n28F+n67 (8.4% BW)

  - 30+20=50MHz for n28A+n67 (6.6%BW).

The needed combined filter bandwidths are compared in Table 1 with the current low-band worst case bandwidth filters (n28 and n105 duplexers)

Table 1: comparison of combined SDL+FDD DL filter with worst-case LB filters

| Band/Band combination | Flow | Fhigh | BW |  | Observations |
| --- | --- | --- | --- | --- | --- |
|  | MHz | MHz | MHz | % |  |
| n29+n12DL | 717 | 746 | 29 | 4.0% | Lower BW than current LB worst case |
| n67+n28FDL | 738 | 803 | 65 | 8.4% | New largest BW in LB |
| n67+n28ADL | 738 | 788 | 50 | 6.6% | Similar to current largest LB BW |
| n105DL | 663 | 703 | 40 | 5.9% | Current worst case filter BW in LB (duplexers) |
| n105UL | 612 | 652 | 40 | 6.3% |  |
| n28UL | 703 | 748 | 45 | 6.2% |  |
| n28DL | 758 | 803 | 45 | 5.8% |  |

Observations:

- n29+n12DL bandwidth is smaller than current worst-case filters that are duplexer which have more demanding OOB rejections  => n29+n12DL filter is feasible

- n67+n28ADL bandwidth is similar than current worst-case filters that are duplexer which have more demanding OOB rejections => n67+n28ADL filter is feasible

- n67+n28FDL bandwidth is 25% larger than current worst-case filters that are duplexer which have more demanding OOB rejections => although the OOB rejection is less demanding than for a duplexer the feasibility of n67+n28FDL needs to be confirmed.

Based on the above, the architecture for the support of switched LB CA_n12+n29 and CA_n12+n29 is illustrated in Figure 2.

Figure 2: RF front-end architecture to support switching CA between FDD DL+UL and adjacent FDD DL + SDL bands

It should be noted that this architecture can only be used if the SDL and FDD DL band are adjacent or in very close proximity. If the SDL band and the FDD DL band have a frequency gap, like for example for Band n5 and Band n29, the two DL band will require two separate filters that will need to be duplexed. This other case is illustrated in Figure 3.

Figure 3: RF front-end architecture to support switching CA between FDD DL+UL and separate FDD DL + SDL bands

As can be observed there are significant differences between the two cases. The case in Figure 3 could also be supported with a FDD UL + FDD DL + SDL triplexer but there would then be no reason to use LBLB switching.

Proposal:

- Requirements are developed based on a two-antenna architecture and a combined FDD DL+SDL filter based on example bands where the FDD DL and SDL bands are adjacent.

  - The post LNA FDD DL and SDL band separation is not the not the default implementation and may also need to be considered in the requirements.

- For the feature to be applicable to other LBLB case, there may be a need to be another example case where the SDL band is separate from the FDD DL band and the SDL/FDD DL filters are duplexed

- The n12 DL+n29 filter and n28A DL+n67 combined filters should be feasible. However, the n28F DL+n67 combined filter feasibility needs to be studied.

# Conclusion

In this contribution, we provide our preliminary views on the new Release 20 LBLB switching architecture and filters feasibility.

Proposal:

- Requirements are developed based on a two-antenna architecture and a combined FDD DL+SDL filter based on example bands where the FDD DL and SDL bands are adjacent.

  - The post LNA FDD DL and SDL band separation is not the not the default implementation and may also need to be considered in the requirements.

- For the feature to be applicable to other LBLB case, there may be a need to be another example case where the SDL band is separate from the FDD DL band and the SDL/FDD DL filters are duplexed

- The n12 DL+n29 filter and n28A DL+n67 combined filters should be feasible. However, the n28F DL+n67 combined filter feasibility needs to be studied.

# References

[1] RP-252953 New WID on enhancement of low NR band carrier aggregation via switching, Huawei, RAN#109