3GPP TSG-RAN WG4 Meeting #116-bis	R4-2513824

Prague, Czech Republic, October 13th – October 17th, 2025

Source:	Iridium, Cambridge Consultants (CCL), Thales

Title:	Maintenance of RRM core requirements for IoT NTN TDD

Agenda item:	4.27.4

Document for:	Discussion

Introduction

In RAN#106 revised WID RP-243293 was approved [1] to introduce IoT NTN TDD mode.

The Objectives of the WID, as per [1]:

The work item aims to specify enhancements for NB-IoT NTN to enable NTN operation with a NB-IoT TDD mode leveraging commonalities with half-duplex NB-IoT FDD NTN, by defining a new NB-IoT TDD mode for NTN based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures for the NB-IoT operation in the targeted unpaired MSS allocated band (TN deployment is not expected in this band). The feature is not intended to be applicable to existing 3GPP bands. Note3: Reuse the existing RAN4 requirements as much as possible (no intention to have any relaxation of the RF emissions requirements)

At RAN#116 in Bangalore, we reached consensus on most of the RRM core requirements for IoT NTN TDD mode. However, a few topics remain open for further discussion during the maintenance phase. This document outlines our perspectives on those remaining aspects.

Discussion

RRC Connection Redirection to Non-Anchor Carrier (6.9A)

Section 6.9A of TS 36.133 outlines the delay requirements for RRC Connection Redirection to a non-anchor carrier in NB-IoT NTN. The total delay in this procedure is influenced by several contributing time components. In RAN#116 there was a proposal to change values for some of these time components but in absence of consensus, decision was deferred to RAN4#116-bis.

The remaining topics related to RRC Connection Redirection to non-anchor carriers are:

To be further discussed and concluded in RAN4#116-bis: - Change ‘110 ms’ in the following bullets to an integer number of 90ms + DL-to-UL gap within the 90 ms TDD pattern, where the exact value is TBD upon completion of RAN1 spec. - TRRC_procedure_delay: It is the RRC procedure for processing the received message “CarrierConfigDedicated-NB”. It shall be less than 110 ms. - When the NPUSCH ACK transmission for the received RRC message takes longer than 110ms, the overall RRC connection redirection delay may be extended. - TBD for the values of 10 ms, 40 ms, and 8 ms in the following bullets, and whether to retain the DL-Bitmap-NB IE upon completion of RAN1 and RAN2 spec: - TDL-UL switch: It is the time between the end of the last subframe in the repetition period of NPDCCH received on the non-anchor carrier and the start of the first subframe in the repetition period of the corresponding NPUSCH transmitted on the non-anchor carrier. - Option 1: TDL-UL switch is 8 ms. - Option 2: TDL-UL switch is 50 ms (THALES, Iridium)

The total time delay is specified from the end of NPDSCH carrying RRC redirection signalling message received on an anchor carrier to the end of the first NPUSCH transmission on the non-anchor carrier.

TRRC_procedure_delay refers to the RRC procedure delay incurred after receiving the redirection signalling message. In IoT NTN TDD mode, the time required for RRC procedure processing should remain consistent once the message has been received. However, this field not only captures the processing delay associated with the RRC procedure but also includes the delay in transmitting the HARQ ACK of the redirection signalling message on the anchor carrier.

The currently specified value of 110ms remains unchanged from terrestrial requirements. In NTN operations, HARQ ACK transmission can be further delayed by the Koffset parameter, which can take values from 0 to 1023ms. It appears that while specifying NTN RRM requirements, a Koffset = 0 assumption was made. Additionally, a general note addressing this is present in both Section 6.9.2 and 6.9A.2, and reads like the sentence below, in both sections:

“When the NPUSCH ACK transmission for the received RRC message takes longer than 110ms, the overall RRCconnection redirection delay may be extended.”

In IoT NTN TDD mode, there is a fixed 50ms time gap between the downlink (RRC signalling message) and the subsequent uplink (HARQ ACK) opportunity. This gap is not part of the TDL-UL switch, which only accounts for delay from receiving uplink grant to NPUSCH transmission over the non-anchor carrier. For clarity, the TRRC_procedure_delay should also include this additional 50ms time gap.

Increase the value TRRC_procedure_delay from 110ms to 160ms to accommodate fixed time gap of 50ms between active downlink and active uplink time.

Another time component is TDL-UL switch which specifies the time gap from the NPDCCH to the scheduled start of the NPUSCH subframe. In IoT NTN TDD mode, there is a fixed time gap of 50ms between the active downlink opportunity and the next active uplink opportunity. Thus, existing value of 8ms is not sufficient to cover the delay for switching from downlink reception to uplink transmission. This value should be updated to be at least equal to 50ms.

Existing value of 8ms for TDL-UL switch is not sufficient for IoT NTN TDD mode and should be updated to 50ms.

Tperiod_DL_bitmap refers to the periodicity of the downlink subframe configuration for downlink transmission on the non-anchor carrier. It is configured via IE DL-Bitmap-NB and can be 10ms or 40ms.

As per RAN1 agreement, the IE DL-Bitmap-NB is not applicable in IoT NTN TDD mode which means all non-sync subframes are considered valid NB-IoT downlink subframes. This field defines the periodicity of valid downlink subframes, but not the number of valid downlinks subframes within each period.

In IoT NTN TDD mode, the periodicity of downlink subframes is fixed at 90ms. Setting the value of Tperiod_DL_bitmap  = 90ms would be misleading, as the DL bitmap is not applicable in IoT NTN TDD mode.

We propose three options to account for the time delay associated with the periodicity of active downlink subframes, with a preference for Option 1.

To take into account the time delay associated with the concept of Tperiod_DL_bitmap , RAN4 could consider (at least) three options:

- Option-1:  Increase the value of TRRC_procedure_delay from 110ms to 250ms to accommodate the fixed time gap of 50ms between active downlink and active uplink periods, as well as the 90ms periodicity of active downlink subframes. In this case Tperiod_DL_bitmap = 0.

- Option-2: Introduce a new time component Tperiod_DL_periodcity = 90ms in the place of Tperiod_DL_bitmap.

- Option-3: Use the value Tperiod_DL_bitmap = 90ms.

- Proposal 4: Increase the value of TRRC_procedure_delay from 110ms to 250ms to accommodate the fixed time gap of 50ms between active downlink and active uplink periods, as well as the 90ms periodicity of active downlink subframes. In this case Tperiod_DL_bitmap = 0.

RRC re-establishment (6.5A)

Following agreements are made in RAN4#116 related to RRM requirements for RRC re-establishment procedure. The interpretation of the field Ksatellite,i  is left for further study.

## Issue 3: RRC re-establishment (6.5A) Agreement: - Tsearch_NB1-NC,i in RRC re-establishment requirement is - If the target cell is known, then Tsearch_NB1-NC,i = 0 ms. If the target cell is unknown, Tsearch_NB1-NC,i = Ksatellite,i *2.52 sec. - FFS the definition of Ksatellite,i

The current definition of Ksatellite,i  is the number NGSO satellites to be measured on i-th frequency for RRC re-establishment which should also apply for IoT NTN TDD mode.

- Proposal 5: For RRC re-establishment delay requirement, re-use existing definition of Ksatellite,i.

Conclusion

In this contribution, we have evaluated the last remaining RRM core requirement topics from RAN4#116 and provided our views. Our conclusions and proposals are listed below:

- Increase the value TRRC_procedure_delay from 110ms to 160ms to accommodate fixed time gap of 50ms between active downlink and active uplink time.

- Existing value of 8ms for TDL-UL switch is not sufficient for IoT NTN TDD mode and should be updated to 50ms.

To take into account the time delay associated with the concept of Tperiod_DL_bitmap , RAN4 could consider (at least) three options:

- Option-1:  Increase the value of TRRC_procedure_delay from 110ms to 250ms to accommodate the fixed time gap of 50ms between active downlink and active uplink periods, as well as the 90ms periodicity of active downlink subframes. In this case Tperiod_DL_bitmap = 0.

- Option-2: Introduce a new time component Tperiod_DL_periodcity = 90ms in the place of Tperiod_DL_bitmap.

- Option-3: Use the value Tperiod_DL_bitmap = 90ms.

- Proposal 4: Increase the value of TRRC_procedure_delay from 110ms to 250ms to accommodate the fixed time gap of 50ms between active downlink and active uplink periods, as well as the 90ms periodicity of active downlink subframes. In this case Tperiod_DL_bitmap = 0.

- Proposal 5: For RRC re-establishment delay requirement, re-use existing definition of Ksatellite,i .

- References

- RP-243293, “Revised WID on introduction of IoT-NTN TDD mode”, 3GPP TSG RAN Meeting #106, Madrid, Spain, December 9-12, 2024

- R4-2512149, WF on RRM requirements for IoT_NTN_TDD, Moderator (Qualcomm Incorporated), RAN4#116, Bangalore, India, 25th – 29th Aug 2025