# 3GPP TSG-RAN WG4 Meeting #116bis	R4-2513831

# Prague, Czech Republic, Oct. 13-17, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.863 | CR | - | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TR 38.863 – Regulatory aspects |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_Ku_bands-Core |  |  |  |  |  | Date: |  |  | 2025-10-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the scope of the NTN KU bands specification, the regulatory context which as been used by RAN4 shall eb captured in the TR 38.863. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Capture ITU-R, FCC and ECA frequency allocation + ECC Decisions and ETSI Harmonized Standards |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The regulatory context used to specify the NTN Ku-bands will not be captured in the TR 38.863 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.5 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of the change>

### 5.2.5 NTN Satellite band: UL13.75-14.5 GHz / DL 10.7-12.75 GHz

#### 5.2.5.1	ITU-R

The ITU-R Radio Regulations [3] specify the following service allocation for those frequency ranges, shown in Table 5.2.5.1-1 (10.7-12.75 GHz) and Table : 5.2.5.1-2 (13.75-14.5 GHz).

Table 5.2.5-1: Allocation of 10.7 – 12.75 GHz in the ITU-R Radio Regulations

| Allocation to services |  |  |  |  |
| --- | --- | --- | --- | --- |
| Region 1 |  | Region 2 |  | Region 3 |  |
| 10.7-10.95 FIXED FIXED-SATELLITE (space-to-Earth) 5.441 (Earth-to-space) 5.484 MOBILE except aeronautical mobile |  | 10.7-10.95 FIXED FIXED-SATELLITE (space-to-Earth) 5.441 MOBILE except aeronautical mobile |  |  |  |
| 10.95-11.2 FIXED FIXED-SATELLITE (space-to-Earth) 5.484A 5.484B (Earth-to-space) 5.484 MOBILE except aeronautical mobile |  | 10.95-11.2 FIXED FIXED-SATELLITE (space-to-Earth) 5.484A 5.484B MOBILE except aeronautical mobile |  |  |  |
| 11.2-11.45 FIXED FIXED-SATELLITE (space-to-Earth) 5.441 (Earth-to-space) 5.484 MOBILE except aeronautical mobile |  | 11.2-11.45 FIXED FIXED-SATELLITE (space-to-Earth) 5.441 MOBILE except aeronautical mobile |  |  |  |
| 11.45-11.7 FIXED FIXED-SATELLITE (space-to-Earth) 5.484A 5.484B (Earth-to-space) 5.484 MOBILE except aeronautical mobile |  | 11.45-11.7 FIXED FIXED-SATELLITE (space-to-Earth) 5.484A 5.484B MOBILE except aeronautical mobile |  |  |  |
| 11.7-12.5 FIXED MOBILE except aeronautical mobile BROADCASTING BROADCASTING-SATELLITE 5.492 | 11.7-12.1 FIXED 5.486 FIXED-SATELLITE (space-to-Earth) 5.484A 5.484B 5.488 Mobile except aeronautical mobile 5.485 |  | 11.7-12.2 FIXED MOBILE except aeronautical mobile BROADCASTING BROADCASTING-SATELLITE 5.492 |  |  |
|  | 12.1-12.2 FIXED-SATELLITE (space-to-Earth) 5.484A 5.484B 5.488 |  |  |  |  |
|  | 5.485 5.489 |  | 5.487 5.487A |  |  |
|  | 12.2-12.7 FIXED MOBILE except aeronautical mobile BROADCASTING BROADCASTING-SATELLITE 5.492 |  | 12.2-12.5 FIXED FIXED-SATELLITE (space-to-Earth) 5.484A 5.484B MOBILE except aeronautical mobile BROADCASTING |  |  |
| 5.487 5.487A |  |  | 5.487 |  |  |
| 12.5-12.75 FIXED-SATELLITE (space-to-Earth) 5.484A 5.484B (Earth-to-space) 5.494 5.495 5.496 | 5.487A 5.488 5.490 |  | 12.5-12.75 FIXED FIXED-SATELLITE (space-to-Earth) 5.484A 5.484B MOBILE except aeronautical mobile BROADCASTING- SATELLITE 5.493 |  |  |
|  | 12.7-12.75 FIXED FIXED-SATELLITE (Earth-to-space) MOBILE except aeronautical mobile |  |  |  |  |

Table 5.2.5-2: Allocation of 13.75 – 14.5 GHz in the ITU-R Radio Regulations

| Allocation to services |  |  |  |  |
| --- | --- | --- | --- | --- |
| Region 1 | Region 2 |  | Region 3 |  |
| 13.75-14 FIXED-SATELLITE (Earth-to-space) 5.484A RADIOLOCATION Earth exploration-satellite Standard frequency and time signal-satellite (Earth-to-space) Space research 5.499 5.500 5.501 5.502 5.503 |  |  |  |  |
| 14-14.25 FIXED-SATELLITE (Earth-to-space) 5.457A 5.457B 5.484A 5.484B 5.506 5.506B RADIONAVIGATION 5.504 Mobile-satellite (Earth-to-space) 5.504B 5.504C 5.506A Space research 5.504A 5.505 |  |  |  |  |
| 14.25-14.3 FIXED-SATELLITE (Earth-to-space) 5.457A 5.457B 5.484A 5.484B 5.506 5.506B RADIONAVIGATION 5.504 Mobile-satellite (Earth-to-space) 5.504B 5.506A 5.508A Space research 5.504A 5.505 5.508 |  |  |  |  |
| 14.3-14.4 FIXED FIXED-SATELLITE (Earth-to-space) 5.457A 5.457B 5.484A 5.484B 5.506 5.506B MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) 5.504B 5.506A 5.509A Radionavigation-satellite 5.504A |  | 14.3-14.4 FIXED-SATELLITE (Earth-to-space) 5.457A 5.484A 5.484B 5.506 5.506B Mobile-satellite (Earth-to-space) 5.506A Radionavigation-satellite 5.504A |  | 14.3-14.4 FIXED FIXED-SATELLITE (Earth-to-space) 5.457A 5.484A 5.484B 5.506 5.506B MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) 5.504B 5.506A 5.509A Radionavigation-satellite 5.504A |
| 14.4-14.47 FIXED FIXED-SATELLITE (Earth-to-space) 5.457A 5.457B 5.484A 5.484B 5.506 5.506B MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) 5.504B 5.506A 5.509A Space research (space-to-Earth) 5.504A |  |  |  |  |
| 14.47-14.5 FIXED FIXED-SATELLITE (Earth-to-space) 5.457A 5.457B 5.484A 5.506 5.506B MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) 5.504B 5.506A 5.509A Radio astronomy 5.149 5.504A |  |  |  |  |

#### 5.2.5.2	FCC

The FCC specifies the following service allocation for the 10.7-12.75 GHz and 13.75-14.5 GHz frequency ranges, as shown in Table 5.2.5.2-1 (10.7-12.75 GHz) and Table 5.2.5.2-2 (13.75-14.5 GHz).

Table 5.2.5.2-1 	FCC allocation of 10.7 – 12.75 GHz

| United States Table |  | FCC Rule Part(s) |
| --- | --- | --- |
| Federal Table | Non-Federal Table |  |
| 10.7-11.7 US131 US211 | 10.7-11.7 FIXED FIXED-SATELLITE (space-to-Earth) 5.441 US131 US211 NG52 | Satellite Communications (25) Fixed Microwave (101) |
| 11.7-12.2 | 11.7-12.2 FIXED-SATELLITE (space-to- Earth) 5.485 5.488 NG143 NG527A | Satellite Communications (25) |
| 12.2-12.75 | 12.2-12.7 FIXED BROADCASTING-SATELLITE 5.487A 5.488 5.490 | Satellite Communications (25) Fixed Microwave (101) |
|  | 12.7-12.75 FIXED NG118 FIXED-SATELLITE (Earth-to-space) MOBILE | TV Broadcast Auxiliary (74F) Cable TV Relay (78) Fixed Microwave (101) |

Table 5.2.5.2-2 	FCC allocation of 13.75 – 14.5 GHz

| United States Table |  | FCC Rule Part(s) |
| --- | --- | --- |
| Federal Table | Non-Federal Table |  |
| 13.75-14 RADIOLOCATION G59 Standard frequency and time signal-satellite (Earth-to-space) Space research US337 US356 US357 | 13.75-14 FIXED-SATELLITE (Earth-to-space) US337 Standard frequency and time signal-satellite (Earth-to-space) Space research Radiolocation US356 US357 | Satellite Communications (25) Private Land Mobile (90) |
| 14-14.2 Space research US133 | 14-14.2 FIXED-SATELLITE (Earth-to-space) NG527A Mobile-satellite (Earth-to-space) Space research US133 | Satellite Communications (25) |
| 14.2-14.4 | 14.2-14.47 FIXED-SATELLITE (Earth-to-space) NG527A Mobile-satellite (Earth-to-space) | Satellite Communications (25) |
| 14.4-14.47 Fixed Mobile |  |  |
| 14.47-14.5 Fixed Mobile US113 US133 US342 | 14.47-14.5 FIXED-SATELLITE (Earth-to-space) NG527A Mobile-satellite (Earth-to-space) US113 US133 US342 |  |

Also, the 13.75-14 GHz frequency range is submitted to additional restrictions on minimum antenna diameter, minimum and maximum EIRP.

(356)  US356  In the band 13.75-14 GHz, an earth station in the fixed-satellite service shall have a minimum antenna diameter of 4.5 m and the e.i.r.p. of any emission should be at least 68 dBW and should not exceed 85 dBW.  In addition the e.i.r.p., averaged over one second, radiated by a station in the radiolocation service shall not exceed 59 dBW.  Receiving space stations in the fixed-satellite service shall not claim protection from radiolocation transmitting stations operating in accordance with the United States Table of Frequency Allocations in this section.  ITU Radio Regulation No. 5.43A does not apply.

ESIM operations are allowed in the following frequency ranges (CFR 47 25.202):

- 10.7-11.7 GHz (s-to-E) for both GSO and NGSO.

- 11.7-12.2 GHz (s-to-E) for both GSO and NGSO.

- 14.0-14.5 GHz (E-to-s) for both GSO and NGSO.

But under the condition of footnote NG527A, which stipulates that ESIM operation in 10.7-11.7 GHz frequency range may be authorized under the conditions that those earth stations don’t claim protection from fixed service.

(527)  NG527A  Earth Stations in Motion (ESIMs), as regulated under part 25 of this chapter, are an application of the fixed-satellite service (FSS) and the following provisions shall apply:

(i) In the bands 10.7-11.7 GHz, 19.3-19.4 GHz, and 19.6-19.7 GHz, ESIMs may be authorized for the reception of FSS emissions from geostationary and non-geostationary satellites, subject to the conditions that these earth stations may not claim protection from transmissions of non-Federal stations in the fixed service and that non-geostationary-satellite systems not cause unacceptable interference to, or claim protection from, geostationary-satellite networks.

(ii) In the bands 11.7-12.2 GHz (space-to-Earth), 14.0-14.5 GHz (Earth-to-space), 18.3-18.8 GHz (space-to-Earth), 19.7-20.2 GHz (space-to-Earth), 28.35-28.6 GHz (Earth-to-space), and 29.25-30.0 GHz (Earth-to-space), ESIMs may be authorized to communicate with geostationary satellites on a primary basis.

(iii) In the bands 11.7-12.2 GHz (space-to-Earth), 14.0-14.5 GHz (Earth-to-space), 18.3-18.6 GHz (space-to-Earth), 19.7-20.2 GHz (space-to-Earth), 28.4-28.6 GHz (Earth-to-space), and 29.5-30.0 GHz (Earth-to-space), ESIMs may be authorized to communicate with non-geostationary satellites, subject to the condition that non-geostationary-satellite systems may not cause unacceptable interference to, or claim protection from, geostationary-satellite networks.

#### 5.2.5.3	CEPT

In the considered frequency ranges, the ECC Decisions captured in Table 5.2.5.3-1 are relevant for in the scope of the NTN specification work.

Table 5.2.5.3-1 	Relevant ECC Decisions

| ECC Decision | Frequency Range | Title |
| --- | --- | --- |
| ECC Decision(18)04 | 10.7 - 12.75 GHz (S-to-E) 14.0 – 14.5 GHz (E-to-S) | The harmonised use, exemption from individual licensing and free circulation and use of land based Earth Stations In-Motion (ESIM) operating with GSO FSS satellite systems in the frequency bands 10.7-12.75 GHz and 14.0- 14.5 GHz |
| ECC Decision(18)05 | 10.7 - 12.75 GHz (S-to-E) 14.0 – 14.5 GHz (E-to-S) | The harmonised use, exemption from individual licensing and free circulation and use of Earth Stations In-Motion (ESIM) operating with NGSO FSS satellite systems in the frequency bands 10.7-12.75 GHz and 14.0-14.5 GHz |
| ECC Decision(19)04 | 10.7 - 12.75 GHz (S-to-E) 12.75 – 13.25 GHz (E-to-S) | The harmonised use of spectrum, free circulation and use of earth stations on-board aircraft operating with GSO FSS networks and NGSO FSS systems in the frequency bands 12.75-13.25 GHz (Earth-to-space) and 10.7-12.75 GHz (space-to-Earth) |
| ECC Decision(05)10 | 10.7 – 11.7 GHz and 12.5 – 12.75 GHz (S-to-E) 14.0 - 14.5 GHz (E-to-S) | The free circulation and use of Earth Stations on board Vessels operating in fixed satellite service networks in the frequency bands 14-14.5 GHz |
| ECC Decision(17)04 | 10.7 - 12.75 GHz (S-to-E) 14.0 – 14.5 GHz (E-to-S) | The harmonised use and exemption from individual licensing of fixed earth stations operating with NGSO FSS satellite systems in the frequency bands 10.7-12.75 GHz and 14.0-14.5 GHz |
| ECC Decision(03)04 | 10.7 – 11.7 GHz (S-to-E) 14.25 – 14.5 GHz (E-to-S) | Exemption from Individual Licensing of Very Small Aperture Terminals (VSAT) operating in the frequency bands 14.25-14.50 GHz Earth-to-space and 10.70-11.70 GHz space-to-Earth |
| ECC Decision(06)03 | 10.7 – 12.75 GHz (S-to-E) 14.0 – 14.25 GHz (E-to-S) | Exemption from Individual Licensing of high e.i.r.p. satellite terminals (HEST) operating with geostationary satellites and in the frequency bands 10.70-12.75 GHz or 19.70-20.20 GHz space-to-Earth and 14.00-14.25 GHz or 29.50-30.00 GHz Earth-to-space |
| ERC Decision(00)08 | 10.7 – 12.5 GHz (S-to-E) | ERC Decision of 19 October 2000 on the use of the band 10.7 – 12.5 GHz by the fixed service and Earth stations of the broadcasting-satellite and fixed-satellite service (space-to-Earth) |

In the considered frequency ranges, Figure 5.2.5.3-1 captures the relevant ETSI Harmonized Standards in the scope of the NTN specification work.

Figure 5.2.5.3-1 	Relevant ETSI Harmonized Standards

<End of the change>