3GPP RAN WG4 Meeting #116bis		R4-2513848

Prague, Czech Republic, October 13th – 17th, 2025

Agenda item:	7.1.5.2

Source:	Apple

Title:	On the introduction of 6MHz CBW

WI/SI:	NR_UE_RF_Ph5-Core

Release:	Rel-20

Document for:	Discussion

# 1	Introduction

The WI [1] has the goal to introduce 6MHz channel bandwidth for n5, n8, n12, n85 and n29. For the initial discussion this contribution focuses on selected objectives from the WID. Before any further analysis can be made it is important to specify the spectrum utilization. Another important objective is to assess the work on additional spectrum emission so that the A-MPR evaluation can be started.

# 2  Discussion

##### 2.1 Spectrum utilisation of 6MHz CBW

Maximising spectrum utilisation is an important aspect for 5G as it allows to optimise throughput. Through the efforts made on spectrum utilisation the NR channels feature more RBs compared to LTE at 15kHz SCS. When introducing the new 6MHz NR channel it is paramount to optimise spectrum utilisation. It is critical to select the right amount of total RBs which leave enough guard band. The guard band should be higher than the 5MHz guard but lower than the 7MHz guard. Setting NRB to 30 sets the spectrum utilization to around 0.96% and ensures a minimum guard band of 292.5 kHz.

Proposal 1: For 6MHz CBW set the NRB to 30 for 15kHz SCS.

##### 2.2 Spectral emission mask modification

The SEM from clause 6.5.2.2 of TS 38.101-1 needs to be updated. It is proposed to add the channel to the row below 10MHz. The modification to Table 6.5.2.2-1 from TS 38.101-1 is provided on Table 1.

Table 1: Modification of SEM to introduce 6MHz channel

| ΔfOOB (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) |  |  |  | Measurement bandwidth |
| --- | --- | --- | --- | --- | --- |
|  | 3 | 5 | 6, 7, 10, 15, 20, 25, 30, 35, 40, 45 | 50, 60, 70, 80, 90, 100 |  |
| ± 0-1 | -13 | -13 | -13 |  | 1 % of channel BW |
| ± 0-1 |  |  |  | -24 | 30 kHz |
| ± 1-5 | -10 | -10 | -10 |  | 1 MHz |
| ± 5-6 | -25 | -13 |  |  |  |
| ± 6-10 |  | -25 |  |  |  |
| ± 5-BWChannel |  |  | -13 |  |  |
| ± BWChannel-(BWChannel+5) |  |  | -25 |  |  |

Proposal 2: Introduce 6MHz channel to SEM requirements as proposed in Table 1.

##### 2.2 Network signalling and A-MPR

Whenever adding a new channel to bands featuring network signalling labels an update to A-MPR is typically required. A shortened version of Table 6.2.3.1-1A from TS 38.101-1 is provided below. It has been reduced to only contain the target bands n5, n8, n12 and n85. The band n29 does not have any additional spectrum emission but is missing in table 6.2.3.1-1A. RAN4 should introduce the band to the table with a maintenance CR from Rel-16.

Table 2: Mapping of network signalling label

| NR band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n5 | NS_01 | NS_100 | NS_14 (NOTE 3) | NS_15 (NOTE 3) |  |  |  | Reserved |
| n8 | NS_01 | NS_100 | NS_43 | NS_43U |  |  |  | Reserved |
| n12 | NS_01 | NS_06 |  |  |  |  |  | Reserved |
| n85 | NS_01 | NS_06 |  |  |  |  |  | Reserved |

It can be observed that four bands feature additional spectrum emission which need to be handled. The flags NS_15 and  NS_43 potentially require introducing new A-MPR for 6MHz CBW. NS_06 only specifies A-MPR for power class 1 and does not require A-MPR for power class 3. Regarding NS_14 it has been found in the 7MHz WI that no A-MPR is needed. The same is expected for 6MHz as well. The network signalling label NS_100 is dedicated to the protection of UTRA systems. It provides A-MPR allowance for outer allocations and is bandwidth independent. Since 5MHz channel and 7MHz channel are covered by NS_100 it is safe to assume that NS_100 will be suited for 6MHz channel as well. Therefore, we do not expect any requirements work for NS_100.

Observation 1: The flags NS_15 and  NS_43 potentially require introducing new A-MPR for 6MHz CBW. NS_06 only specifies A-MPR for power class 1 and does not require A-MPR for power class 3. Regarding NS_14 it has been found in the 7MHz WI that no A-MPR is needed. The same is expected for 6MHz as well. The network signalling label NS_100 is dedicated to the protection of UTRA systems. It provides A-MPR allowance for outer allocations and is bandwidth independent. Since 5MHz channel and 7MHz channel are covered by NS_100 it is safe to assume that NS_100 will be suited for 6MHz channel as well. Therefore, we do not expect any requirements work for NS_100.

Observation 2: Band n29 is currently missing in Table 6.2.3.1-1A but should be listed with NS_01.

Proposal 3: Discuss which network signalling labels require the introduction of 6MHz channel to enable the work on A-MPR.

##### 2.3 REFSENS Requirements for n5, n8, n12, n85 and n29 with 6MHz CBW

Assuming NRB of 30, the REFSENS with 6MHz for these bands can be derived using the following scaling formula:  REFSENS6Mz = REFSENS5MHz + 10*log(30/25), assuming NRB = 30.

The calculated values of REFSENS using the formula above are shown in Table 3 below:

Table 3: REFSENS with 6MHz CBW

| CBW | n5 | n8 | n12 | n85 | n29 |
| --- | --- | --- | --- | --- | --- |
| 6MHz | -97.2dBm | -96.2dBm | -96.2dBm | -96.2dBm | -96.2dBm |

Proposal 4: Adopt the REFSENS for 6MHz CBW as shown in the table below:

| CBW | n5 | n8 | n12 | n85 | n29 |
| --- | --- | --- | --- | --- | --- |
| 6MHz | -97.2dBm | -96.2dBm | -96.2dBm | -96.2dBm | -96.2dBm |

# 4	Conclusions

This contribution provides initial insight and discussion points for the introduction of 7MHz channel. The following observations and proposals are made:

Proposal 1: For 6MHz CBW set the NRB to 30 for 15kHz SCS.

Proposal 2: Introduce 6MHz channel to SEM requirements as proposed in Table 1.

Observation 1: The flags NS_15 and  NS_43 potentially require introducing new A-MPR for 6MHz CBW. NS_06 only specifies A-MPR for power class 1 and does not require A-MPR for power class 3. Regarding NS_14 it has been found in the 7MHz WI that no A-MPR is needed. The same is expected for 6MHz as well. The network signalling label NS_100 is dedicated to the protection of UTRA systems. It provides A-MPR allowance for outer allocations and is bandwidth independent. Since 5MHz channel and 10MHz channel are covered by NS_100 it is safe to assume that NS_100 will be suited for 6MHz channel as well. Therefore, we do not expect any requirements work for NS_100.

Observation 2: Band n29 is currently missing in Table 6.2.3.1-1A but should be listed with NS_01.

Proposal 3: Discuss which network signalling labels require the introduction of 6MHz channel to enable the work on A-MPR.

Proposal 4: Adopt the REFSENS for 6MHz CBW as shown in the table below:

| CBW | n5 | n8 | n12 | n85 | n29 |
| --- | --- | --- | --- | --- | --- |
| 6MHz | -97.2dBm | -96.2dBm | -96.2dBm | -96.2dBm | -96.2dBm |

# 4	References

- RP-252942, “New WID: UE RF enhancements for NR FR1, Phase 5”, vivo, 3GPP TSG RAN Meeting #109