3GPP TSG-RAN4 Meeting #116bis	R4- 2513852

# Prague, Czech Republic, October 13th – 17th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR |  | rev |  | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for 38.101-1 on enabling HPUE support for band combinations without HPUE applicability notes in clause 5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_RF_Ph4-Core |  |  |  |  |  | Date: |  |  | 2025-10-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | One of the main reasons the PC2 and PC1.5 applicability notes in band combination configuration tables of the RAN4 UE RF specifications are necessitated is to ensure the corresponding MSD requirements are specified, in particular, for the impacted cross DL bands without UL configured. With the introduction of HPUE MSD look-up tables (LUTs), the HPUE support for band combinations without HPUE applicability notes in band combination configuration tables can be enabled provided the said HPUE power class for the UL configurations are specified in clause 6 and the ΔMSD are specified in the HPUE MSD LUTs. This can be done by adding respective general notes for PC2 and PC1.5 in 2-band inter-band CA table for single carrier UL configuration and in 3-band inter-band CA table for 2-band UL configuration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add NOTE 17 below Table 5.5A.3.1-1o (last CA configuration table for inter-band CA with 2 bands) as a general note to enable PC2 support for single carrier UL without NOTE 8. - Add NOTE 18 below Table 5.5A.3.1-1o (last CA configuration table for inter-band CA with 2 bands) as a general note to enable PC1.5 support for single carrier UL without NOTE 9. - Add NOTE 15 below Table 5.5A.3.2-1c (last CA configuration table for inter-band CA with 3 bands) as a general note to enable PC2 support for 2-band UL configurations without NOTE 7 or NOTE 13. - Add NOTE 16 below Table 5.5A.3.2-1c (last CA configuration table for inter-band CA with 3 bands) as a general note to enable PC1.5 support for 2-band UL configurations without NOTE 14. - Remove the texts in clause 7.3A.1 introduced via RAN task for CA combinations with single uplink as they are no longer needed after the HPUE MSD LUT introduction. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | HPUE are not supported for many band combinations due to the lack of HPUE applicability notes in CA configuration tables in clause 5 despite the MSD requirements are already specified via the introduction of HPUE MSD LUTs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5A.3, 7.3A.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< Start of changed sections >>>

Table 5.5A.3.1-1o: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n100A-n101A | CA_n100A-n101A | n100 | 3, 5 | 0 |
|  |  | n101 | 5, 10 |  |

The following notes are applied to the above tables:

NOTE 1:	This UE channel bandwidth is applicable only to downlink.

NOTE 2:	The minimum requirements for intra-band contiguous or non-contiguous CA apply.

NOTE 3:	For each channel bandwidth of each component carrier, refer to Table 5.3.5-1 for the applicable SCSs. For a given band, not all UE channel bandwidths support the same SCSs.

NOTE 4:	This UE channel bandwidth is optional in this release of the specification.

NOTE 5:	For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an SCell part of DC or CA configuration.

NOTE 6:	For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an downlink SCell part of CA configuration

NOTE 7:	Limited to operation at 3450-3550 MHz and 3700–3980 MHz.

NOTE 8:	Minimum requirements for Power Class 2 are applicable for this uplink combination with 1Tx antenna connector in each band or single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination

NOTE 9:	Minimum requirements for Power Class 1.5 are applicable for this single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination

NOTE 10: 	Only single uplink carriers with power class other than PC3 are listed.

NOTE 11: The CA configurations are given in Table 5.5A.1-1 or Table 5.5A.2-1 in this specification

NOTE 12: Void.

NOTE 13: Minimum requirements for Power Class 2 are applicable for this uplink configuration with 1Tx antenna connector in one band and 2Tx antenna connectors in the other band.

NOTE 14 Minimum requirements for Power Class 1.5 are applicable for this uplink configuration with 1Tx antenna connector in one band and 2Tx antenna connectors in the other band.

NOTE 15: Uplink is only in n5 for CA_n5-n8.

NOTE 16: For UEs only supporting DL CA_n26-n28, uplink support in band n26 is optional, if the UE supports CA_n26-n28 UL configuration, it should also support UL in band n26 and n28.

NOTE 17: For single uplink carrier with up to 2Tx antenna connectors without NOTE 8, minimum requirements for Power Class 2 are applicable provided the said power class has been specified in Table 6.2.1-1 or Table 6.2D.1-1 respectively and the corresponding PC2 ΔMSD is specified in Table 7.3A.2.3.1-1 and Table 7.3A.2.3.1-2 for this downlink/uplink combination.

NOTE 18: For single uplink carrier with up to 2Tx antenna connectors without NOTE 9, minimum requirements for Power Class 1.5 are applicable provided the said power class has been specified in Table 6.2.1-1 and the corresponding PC1.5 ΔMSD is specified in Table 7.3A.2.3.1-1 and Table 7.3A.2.3.1-2 for this downlink/uplink combination.

<<< Unchanged sections omitted >>>

##### Table 5.5A.3.2-1c

<<< Table 5.5A.3.2-1c omitted >>>

The following notes are applied to the above tables.

NOTE 1:	This UE channel bandwidth is applicable only to downlink

NOTE 2:	For the 20 MHz bandwidth, the minimum requirements are specified for NR UL carrier frequencies confined to either 713-723 MHz or 728-738 MHz.

NOTE 3: 	For each channel bandwidth of each component carrier, refer to Table 5.3.5-1 for the applicable SCSs. For a given band, not all UE channel bandwidths support the same SCSs.

NOTE 4:	The minimum requirements only apply for non-simultaneous Rx/Tx between all carriers for TDD combinations.

NOTE 5:	Simultaneous Rx/Tx capability for TDD combinations does not apply for UEs supporting band n78 with an n77 implementation.

NOTE 6:	Only single uplink carriers with power class other than PC3 are listed.

NOTE 7:	Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination

NOTE 8:	For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an SCell part of DC or CA configuration.

NOTE 9:	Minimum requirements for Power Class 1.5 are applicable for single uplink carrier in this downlink/uplink combination

NOTE 10:	For a band combination which include band n7 and n38 simultaneously, carriers in band n7 and n38 can only be configured as downlink carriers. Power imbalance between downlink carriers on Band n7 and Band n38 is assumed to be within 6dB.

NOTE 11: UL carrier shall be supported in Band n28 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within 6dB.

NOTE 12:	For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as a downlink SCell part of CA configuration.

NOTE 13: Minimum requirements for Power Class 2 are applicable for this uplink configuration with 1Tx antenna connector in one band and 2Tx antenna connectors in the other band.

NOTE 14 Minimum requirements for Power Class 1.5 are applicable for this uplink configuration with 1Tx antenna connector in one band and 2Tx antenna connectors in the other band.

NOTE 15: For a 2-band UL configuration without NOTE 7 or NOTE 13, minimum requirements for Power Class 2 are applicable provided the said power class has been specified in Table 6.2A.1.3-1 or Table 6.2H.3.1-1 respectively and the corresponding PC2 ΔMSD is specified in Table 7.3A.2.3.2-1 and Table 7.3A.2.3.2-2 for this downlink/uplink combination.

NOTE 16: For a 2-band UL configuration without NOTE 14, minimum requirements for Power Class 1.5 are applicable provided the said power class has been specified in Table 6.2H.3.1-1 and the corresponding PC1.5 ΔMSD is specified in Table 7.3A.2.3.2-1 and Table 7.3A.2.3.2-2 for this downlink/uplink combination.

<<< Unchanged sections omitted >>>

### 7.3A.1	General

The reference sensitivity power level REFSENS is the minimum mean power applied to each one of the UE antenna ports for all UE categories, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel. For operations with 4 Rx or 8 Rx antenna ports, the MSD in the applicable bands shall be increased by the absolute value of ΔRIB,4R in Table 7.3.2-2 or ΔRIB,8R in Table 7.3.2-2a when MSD > 0.

For reference sensitivity exception test points where the specified carrier frequency does not correspond to a valid NR-ARFCN, the closest NR-ARFCN as specified in clause 5.4.2 applies.

For reference sensitivity level tests or reference sensitivity exception tests specified in clause 7.3A, SCS=15kHz based UL test configuration can be replaced by SCS=30kHz based UL test configuration. The equivalent substitution relationship between different SCS UL test configuration is shown in Table 7.3A.1-1 for the operating bands above 2.2GHz.

Table 7.3A.1-1: Equivalent substitution relationship between different SCS UL test configuration

| SCS (kHz) | (BW[MHz], Lcrb) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | (5, 25) | (10, 50) | (15, 75) | (20, 100) | (25, 128) | (30, 160) | (35, 180) | (40, 216) | (45, 240) | (50, 270) |
| 30 | (5,12) | (10, 24) | (15, 36) | (20, 50) | (25, 64) | (30, 75) | (35, 90) | (40, 100) | (45, 108) | (50, 128) |

<<< End of changed sections >>>