3GPP TSG-RAN WG4 Meeting #116bis	R4-2513870

Prague, CZ, Oct. 13 – 17, 2025

Agenda item:	6.1.2

Source:	Samsung

Title:	Simulation results on UE demodulation performance requirements for 6Rx

Document for:	Information

- Introduction

In the RAN#103 meeting, the WID [1] for “UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4” has been approved, in which “6Rx for handheld and FWA” is an important objective for both core and performance parts. This is an enhancement for receiver in addition to Rel-15 4Rx and Rel-18 8Rx improvements (which are specified for fixed wireless access (FWA) types of devices). The WID for performance part has been updated as below [2] according latest RAN4#114 meeting agreements [3].

The objectives of performance part for UE RF enhancements for NR FR1 and FR2 Phase 4 include: 6Rx for handheld and FWA UE - Specify the performance requirements to enable 6Rx on higher frequency bands (>2.5GHz) targeting at support of smartphone for NR FR1 single carrier scenario - Investigate and if necessary, specify RLM test cases to support 6Rx - Specify UE PDSCH demodulation performance and CSI requirements to support 6 Rx - 6 MIMO layers performance requirements will be only defined for FWA. - 4 MIMO layers performance requirements will be defined and will apply to FWA and handheld UE devices. - Specify the SDR requirements to support 6Rx General for all the areas - Specify release independence requirements for performance part in TS 38.307/36.307, if needed and feasible.

In RAN4#114bis, RAN4#115 and RAN4#116 meetings, work plan, general scopes and some test assumptions were agreed [4][5][6]. In this contribution, we provide our further simulation results on 6Rx PDSCH demodulation performance and CSI requirements, and 6Rx SDR requirements.

- Simulation for PDSCH demodulation performance

  - Simulation assumptions

According agreements in RAN4#114bis, RAN4#115 and RAN4#116 meetings [4][5][6], the simulation assumptions for PDSCH perforamnce test as Table 2-1.

Table 2-1. Simulation Assumptions for PDSCH perforamnce test

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Test case type |  |  | Only single carrier test |
| TDD pattern |  |  | 7DS2U, S: 6D+4G+4U |
| Channel bandwidth / SCS |  |  | 40 MHz / 30 kHz |
| Antenna configuration |  |  | 4 Layers: 4T6R 6 Layers: 8T6R |
| MCS |  |  | 4 Layers - ULA Low: MCS17(Table 1) - ULA Medium: MCS13 (Table 1) 6 Layers: MCS17(Table 1) |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
| TRS configuration |  |  | Symbol#{5, 9} |
| PT-RS |  |  | Not configured |
| DM-RS configuration |  |  | 4 Layer: Rel-15 Type 1 DM-RS with 1+1 6 Layers: FFS |
| Overhead |  |  | 0 |
| Coodebook for PDCCH for PDSCH tests with 4Tx and 8Tx |  |  | Keep same number of Tx for PDSCH and PDCCH during PDSCH test. Set “codebookMode” to 1 |
| (N1, N2), (O1, O2) |  |  | 8Tx case: (N1,N2) = (4,1), (O1, O2) = (4,1) 4Tx case: (N1,N2)=(2,1), (O1, O2) = (4,1) |
| Special slot (S slot) scheduling |  |  | Not schedule PDSCH in special slot for 6 layers cases |
| Number of HARQ Processes |  |  | 8 |
| HARQ ACK/NACK bundling |  |  | Not configured |
| Maximum HARQ transmissions |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,2,3,1} |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity |
| Receiver type assumption |  |  | MMSE-IRC with joint 6Rx MIMO detector |
| Tx EVM assumption |  |  | - 6% for QPSK/16QAM/64QAM - 3% for 256QAM |
| Test metric |  |  | SNR@70% max TP |

  - Simulation results

6Rx PDSCH Demodulation Performance with 4 MIMO layers

For 4 MIMO layer case with TDLA30-10 ULA, there are two options for the MIMO correlations. The first one is α = 0 and β = 0 suitable for FWA UE, the second one is α = 0 and β = 0.07179 targeting for handheld UE. Therefore, we evaluate the performance with different correlation values as Figure 2-1, and the ideal SNR working points are summarized in Table 2-2.

Figure 2-1. Performance comparison with different MCS values for 4T6R4Layer ULA Low

Table 2-2. Ideal SNR working points for 4T6R4Layer

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | MCS value | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | 40 / 30 | MCS17 (Table 1) | TDLA30-10 | 4x6, ULA Low | 70 | 11.58 |
| 1-2 | 40 / 30 | MCS13 (Table 1) | TDLA30-10 | 4x6, ULA Med alpha=0, beta=0.07179 | 70 | 18.0 |

From the simulation results, obvious SNR difference could be observed for different correlation matrixes.

6Rx PDSCH Demodulation Performance with 6 MIMO Layers

According agreements for 6 layers, 8T6R6Layer with TDLA30-10 ULA Low and MCS17 (Table 1) should be used for PDSCH demodulation performance test for 6 Layers. Figure 2-2 is the simulation results for 6 Layers and the ideal SNR working points are summarized in Table 2-3.

Figure 2-2. PDSCH Demodulation Performance for 8T6R6Layer ULA Low

Table 2-3. Ideal SNR working points for 8T6R6Layer ULA Low

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | MCS value | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | 40 / 30 | MCS17 (Table 1) | TDLA30-10 | 8x6, ULA Low | 70 | 18.0 |

- Simulation for SDR

  - Simulation assumptions

According agreements in RAN4#114bis, RAN4#115 and RAN4#116 meetings [4][5][6], the simulation assumptions for SDR requirements test as Table 3-1.

Table 3-1. Simulation Assumptions for SDR requirements test

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | wideband |
|  | Resource allocation type |  | Type 0 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Length |  | Rank 2/4: 1 Rank 6/8: 2 |
|  | Antenna ports indexes |  | {1000, 1001} for 2 Layers CCs |
|  | Number of PDSCH DMRS CDM group(s) without data |  | 1 for 1 layer and 2 layers CCs 2 for 4 Layers, 6 Layers and 8 Layers CCs |
| PTRS configuration |  |  | PTRS is not configured |
| Maximum number of code block groups for ACK/NACK feedback |  |  | 1 |
| Maximum number of HARQ transmission |  |  | 4 |
| PUCCH HARQ ACK spaitial bundling |  |  | Not configured |
| Redundancy version coding sequence |  |  | {0,2,3,1} |
| Propagation condition for 2T6R |  |  | Static propagation condition No external noise sources are applied H=111j-jj111-jj-j |
| Propagation condition for 4T6R |  |  | - H=111111111-1-1-122j-2j22j3010j305j-33010j-22j2j-22j-3010j305j-3010j |
| Propagation condition for 8T6R (Fixed precoding with i1,1=i1,2=i2=0) |  |  | - H=11111-j1111-1j1-j1-11-1-1j1-11-111-1-11-j11-1-1-1j-1j1-1-111-j1-1-11 |
| Antenna configuration | 1 layer CCs |  | 1x1 or 1x2 or 1x4 |
|  | 2 layers CCs |  | 2x2 or 2x4 or 2x6 or 2x8 |
|  | 4 layers CCs |  | 4x4 or 4x6 or 4x8 |
|  | 6 layers CCs |  | 8x6 |
|  | 8 layers CCs |  | 8x8 |
| Physical signals, channels mapping and precoding |  |  | As specified in Annex B.4.1 |
| Es |  | dBm/Hz | -112 for MCS indexes in Table 5.5A-5: -110.5 For 1024QAM MCS indexes in Table 5.5A-6: |
| Note 1: UE assumes that the TCI state for the PDSCH is identical to the TCI state applied for the PDCCH transmission Note 2: Point A coincides with minimum guard band as specified in Table 5.3.3-1 from TS 38.101-1 [6] for tested channel bandwidth and subcarrier spacing |  |  |  |

  - Simulation results

For 64QAM with 6 MIMO layers, the simulation results could be summarized as Figure 3-1.

Figure 3-1. SDR Performance for 64QAM with 6 MIMO layers

For 256QAM with 6 MIMO layers, the simulation results could be summarized as Figure 3-2.

Figure 3-2. SDR Performance for 256QAM with 6 MIMO layers

For 1024QAM with 2 MIMO layers, the simulation results could be summarized as Figure 3-3.

Figure 3-3. SDR Performance for 1024QAM with 2 MIMO layers

For 1024QAM with 4 MIMO layers, the simulation results could be summarized as Figure 3-4.

Figure 3-4. SDR Performance for 1024QAM with 4 MIMO layers

From the performance in Figure 3-1, it is obvious that the MCS value for 64QAM with 6 MIMO layers should be MCS28. Considering the assumption of TxEVM for 256QAM is 3%, the maximum achievable SNR should be 30.4576dB. From the performance in Figure 3-2, we could know the MCS value for 256QAM with 6 MIMO layers should be MCS27.  Similarly, the assumption of TxEVM for 1024QAM is 2.5%, the maximum achievable SNR should be 32.0412dB. The ideal SNR working points for different MCS values with 1024QAM 2 MIMO layers as Figure 3-3, in which for MCS25, only when SNR is about 32dB (seems too high), maximum throughput could be achieved. Therefore, the MCS value for 1024QAM with 2 MIMO layers should be MCS24. The ideal SNR working points for different MCS values with 1024QAM 4 MIMO layers as Figure 3-4, in which for MCS24, only when SNR is about 32dB (seems too high), maximum throughput could be achieved. Therefore, the MCS value for 1024QAM with 4 MIMO layers should be MCS23. Accordingly, the MCS values for 6 MIMO layers could be summarized as Table 3-2, and the MCS values for 1024QAM with 2 MIMO layers could be summarized as Table 3-3.

Table 3-2. MCS values for QPSK/16QAM/64QAM/256QAM with 6 MIMO layers

| Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- |
| 6 | 8 | 1 | 27 |
| 6 | 8 | 0.8 | 23 |
| 6 | 8 | 0.75 | 22 |
| 6 | 8 | 0.4 | 12 |
| 6 | 6 | 1 | 28 |
| 6 | 6 | 0.8 | 24 |
| 6 | 6 | 0.75 | 23 |
| 6 | 6 | 0.4 | 14 |
| 6 | 4 | 1 | 16 |
| 6 | 4 | 0.8 | 16 |
| 6 | 4 | 0.75 | 16 |
| 6 | 4 | 0.4 | 11 |
| 6 | 2 | 1 | 9 |
| 6 | 2 | 0.8 | 9 |
| 6 | 2 | 0.75 | 9 |
| 6 | 2 | 0.4 | 5 |
| Note 1: MCS Index for maximum modulation format 2,4 and 6 is based on MCS index Table 1 defined in clause 5.1.3.1 of TS 38.214 [12] Note 2: MCS Index for maximum modulation format 8 is based on MCS index Table 2 defined in clause 5.1.3.1 of TS 38.214 [12] |  |  |  |

Table 3-3. MCS values for 1024QAM with 2 and 4 MIMO layers

| Supported RX antenna ports | Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- | --- |
| 6RX | 2 | 10 | 1 | 24 |
|  | 2 | 10 | 0.8 | 21 |
|  | 2 | 10 | 0.75 | 19 |
|  | 2 | 10 | 0.4 | 9 |

- Conclusion

In this contribution, we provide our initial simulation results for PDSCH demodulation performance, SDR requirements on “6Rx for handheld and FWA” topic.

The ideal working point for PDSCH demodulation performance could be summarized as following.

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | MCS value | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | 40 / 30 | MCS17 (Table 1) | TDLA30-10 | 4x6, ULA Low | 70 | 11.58 |
| 1-2 | 40 / 30 | MCS13 (Table 1) | TDLA30-10 | 4x6, ULA Med alpha=0, beta=0.07179 | 70 | 18.0 |
| 2-1 | 40 / 30 | MCS17 (Table 1) | TDLA30-10 | 8x6, ULA Low | 70 | 18.0 |

The SDR requirements for QPSK/16QAM/64QAM/256QAM with 6 MIMO layers could be summarized as below.

| Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- |
| 6 | 8 | 1 | 27 |
| 6 | 8 | 0.8 | 23 |
| 6 | 8 | 0.75 | 22 |
| 6 | 8 | 0.4 | 12 |
| 6 | 6 | 1 | 28 |
| 6 | 6 | 0.8 | 24 |
| 6 | 6 | 0.75 | 23 |
| 6 | 6 | 0.4 | 14 |
| 6 | 4 | 1 | 16 |
| 6 | 4 | 0.8 | 16 |
| 6 | 4 | 0.75 | 16 |
| 6 | 4 | 0.4 | 11 |
| 6 | 2 | 1 | 9 |
| 6 | 2 | 0.8 | 9 |
| 6 | 2 | 0.75 | 9 |
| 6 | 2 | 0.4 | 5 |
| Note 1: MCS Index for maximum modulation format 2,4 and 6 is based on MCS index Table 1 defined in clause 5.1.3.1 of TS 38.214 [12] Note 2: MCS Index for maximum modulation format 8 is based on MCS index Table 2 defined in clause 5.1.3.1 of TS 38.214 [12] |  |  |  |

The SDR requirements for 1024QAM with 2 MIMO layers could be summarized as following.

| Supported RX antenna ports | Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- | --- |
| 6RX | 2 | 10 | 1 | 24 |
|  | 2 | 10 | 0.8 | 21 |
|  | 2 | 10 | 0.75 | 19 |
|  | 2 | 10 | 0.4 | 9 |

- Reference

[1] RP-240828, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, RAN#103 meeting, Huawei

[2] RP-250780, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, RAN#107 meeting, Huawei

[3] R4-2503014, WF on 6Rx UE requirements, RAN4#114 meeting, AT&T

[4] R4-2504750, Way Forward for [114bis][325] NR_ENDC_RF_Ph4_Demod_6Rx, RAN4#114bis meeting, Huawei, HiSilicon

[5] R4-2508727, Way Forward for Topic [115][326] NR_ENDC_RF_Ph4_Demod_6Rx, RAN4#115 meeting, Huawei, HiSilicon

[6] R4-2512617, Way Forward for Topic for [116][325] NR_ENDC_RF_Ph4_Demod_6Rx, RAN4#116 meeting, Huawei, HiSilicon