3GPP TSG-RAN WG4 Meeting #116bis		                		       R4-2513899

Prague, Czech Republic, Oct. 13-17, 2025

Agenda item:	4.15.1

Source: 	ZTE Corporation, Saneships

Title: 	Discussion on the fast SCell activation for UE supporting Rel-18 EMR

Document for:	Approval

Introduction

The RRM enhancement on fast SCell activation for UE supporting Rel-18 EMR was completed in previous meetings. In last RAN4 meeting, UE capability were discussed with agreements captured in [1]. In this contribution, we further provide our views on the UE capability.

Discussion

According the work item, valid early measurement report would be considered in the existing known/unknown SCell condition during SCell configuration/activation in Release 19, which could reduce delay requirement for the target SCell activation. It’s a common understanding that is feasible and beneficial to consider fast SCell activation for UE supporting Rel-18 EMR.

In previous meetings, we have discussed and reached consensus on how to use R18 EMR on SCell configuration/activation in Release 19 to reduce delay requirement for the target SCell activation.  And defined the new requirements in the RRM specification. In the last meeting, there is a remaining issue on UE capability should be further discussed [1].

Issue 2-1-1: UE capability for EMR-based fast SCell activation FFS: - Option 1: Introduce a UE capability for R19 SCell activation based on EMR - Prerequisite feature groups: 39-8 or 39-9 in R18 RAN4 feature list - Need for the gNB to know if the feature is supported: No - Granularity: Per-UE - FDD/TDD differentiation: No - FR1/FR2 differentiation: Yes - Option 2: Not to introduce additional UE capability Issue 2-1-1: UE capability for EMR-based fast SCell activation Agreement in Friday online: - Introduce one single UE capability for R19 SCell activation based on EMR, further discuss the detail.

In R18 eEMR, we discussed how to reuse the IDLE/INACTIVE mode measurement results which to be reported during RRC connection setup/resume in order to improve SCell/SCG setup delay. As the result, EMR and cell reselection measurement are considered, and two UE capabilities measValidationReportEMR-r18 and measValidationReportReselectionMeasurements-r18 are introduced. Besides, R16 EMR is supported by UE capability idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16.

According to the discussion about applicability, fast SCell activation delay requirements apply to following cases:

-	UE supports measValidationReportEMR and measIdleValidityDuration-r18 is configured or,

-	UE supports measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured or,

-	UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 hasn’t expired at the moment of initiation of RRC state transition to Connected mode

Comparing the UE capability introduced in Release 18 with the capabilities required by Release 19, we find that the existing UE features (including R16 and R18) can meet all the requirements defined in Release 19 for fast SCell activation for UE supporting Rel-18 EMR. In our understanding, there is no need to discuss UE capability for fast SCell activation with Rel-18 eEMR report because existing UE capabilities are enough. However, if the UE capability of R19 is not introduced, the UE may not be able to distinguish between the requirements of R18 and R19, and there may be situations where the R18 UE cannot complete the R19 requirements or the R19 UE only completes the R18 requirements.

Based on this, UE capability can be introduced and it is only to indicate that the UE is an R18 or R19 UE to avoid mismatches. As we mentioned above, three configurations have been discussed in core requirements, and all of them should be considered as the pre-requisite of R19 UE capability, including measValidationReportEMR, measValidationReportReselectionMeasurements and idleInactiveNR-MeasReport-r16.

Proposal 1: Introduce additional UE capability for fast SCell activation with Rel-18 eEMR report, and existing capabilities from Rel-16/Rel-18 should be considered as pre-requisite.

Conclusions

In this paper, we have some discussion on fast SCell activation for UE supporting Rel-18 EMR. And we have the following proposal:

Proposal 1: Introduce additional UE capability for fast SCell activation with Rel-18 eEMR report, and existing capabilities from Rel-16/Rel-18 should be considered as pre-requisite.

References

- R4-2512125, WF on RRM requirements for NR_RRM_Ph5_Part2, CATT