# 3GPP TSG-RAN WG4 Meeting #116bis                            R4-2513929

# Prague, CZ, 13th-17th Oct., 2025

# Source:        ZTE Corporation, Sanechips

# Title:           Discussion on 6MHz CBW UE RF

# Agenda Item:   7.1.5.2

# Document for:  Approval

- Introduction

In last RAN plenary meeting, a new WI to introduce 6MHz CBW for bands n5, n8, n12, n85, n29 was approved [1]. The objective of this WI is:

6MHz channel bandwidth - Specify the generic core requirements to support 6 MHz NR FR 1 channel bandwidth [RAN 4] - Only include 15 kHz SCS - Specify the spectrum utilization - Specify UE RF requirements for UL and DL - Specify BS RF requirements (OTA and conducted) for UL and DL - Specify the band specific requirements to introduce 6 MHz for NR band n5, n8, n12, n85, n29 [RAN 4] - BS and UE Channel bandwidth per operating band - UE reference sensitivity - When needed - Additional Maximum Power Reduction (A-MPR) - Once RAN 4 has introduced the new channel bandwidth, RAN 2 will introduce the necessary signalling for this channel bandwidth [RAN 2] - Note This should be similar to the signalling for 7 MHz channel bandwidth - Once RAN 4 has introduced the new channel bandwidth, RAN 3 will introduce the necessary signalling for this channel bandwidth [RAN 3] - Note This should be similar to the signalling for 7 MHz channel bandwidth NOTE: Once this WI is completed, the 6 MHz bandwidth can be introduced also for other bands (not addressed in this WI) via the appropriate basket WI. The support of 6MHz bandwidth is release independent.

First of all, the system parameters should be defined then we can come to requirements. In this contribution, we provide our views on the system parameters for 6MHz CBW and the impacts on UE RF requirements.

- Discussion

## 2.1 System parameters

For maximum transmission bandwidth configuration, the spectrum utilization is the most important issue at the very beginning. Fortunately that we have introduced 7MHz CBW and this can be a good reference. Since the spectrum utilization of 3MHz, 5MHz and 7MHz with 15kHz SCS are all 90%, it is feasible that 90% spectrum utilization also applies to 6MHz CBW. Thus, NRB=30 with 90% spectrum utilization are most suitable for 6MHz CBW, 15kHz SCS as shown in Table 1.

Table 1  Maximum transmission bandwidth configuration for 6MHz CBW with NRB=30

| CBW(MHz) | 3MHz | 5MHz | 6MHz | 7MHz | 10MHz | 20MHz |
| --- | --- | --- | --- | --- | --- | --- |
| SCS(kHz) | 15 | 15 | 15 | 15 | 15 | 15 |
| NRB | 15 | 25 | 30 | 35 | 52 | 106 |
| Spectrum Utilization | 90% | 90% | 90% | 90% | 93.6% | 95.76% |

Then, the minimum guardband (kHz) can be calculated as listed in Table 2.

Table 2  Minimum guardband (kHz) for 6MHz CBW with NRB=30

| SCS(kHz) | 3MHz | 5MHz | 6MHz | 7MHz | 10MHz | 20MHz |
| --- | --- | --- | --- | --- | --- | --- |
| 15 | 142.5 | 242.5 | 292.5 | 342.5 | 312.5 | 382.5 |
| 30 | N/A | 505 | N/A | N/A | 665 | 645 |
| 60 | N/A | N/A | N/A | N/A | 1010 | 990 |

Proposal 1: Maximum transmission bandwidth configuration for 6MHz CBW could be 30 RBs.

Proposal 2: Minimum guardband for 6MHz CBW could be 292.5kHz.

## 2.2 UE RF requirements

Then after system parameters are defined, the RF requirements should be defined for 6MHz CBW. The spec impact on TS38.101-1 and our views on the UE RF requirements are listed in Table 3.

Table 3  Spec impact on UE RF to introduce 6MHz CBW

| Clause | Requirements | ZTE’s views on the UE RF requirements |
| --- | --- | --- |
| 6.2.3 | UE additional maximum output power reduction | Evaluate A-MPR for 6MHz CBW for: - n5 under NS_15 (NS_14 can be precluded based on the simulation for 7MHz CBW) - n8 under NS_43 and NS_43U if needed, add 6MHz CBW A-MPR requirements and introduce A-MPR regions and values to these NSs, related to 6.5.3.3. |
| 6.3.1 | Minimum output power | Add 6MHz CBW to Table 6.3.1-1, reuse legacy minimum output power for 3~20MHz CBW. |
| 6.3.2 | Transmit OFF power | Add 6MHz CBW to Table 6.3.2-1, reuse legacy transmit OFF power for 3~50MHz CBW. |
| 6.5.1 | Occupied bandwidth | Add 6MHz CBW to Table 6.5.1-1, reuse legacy occupies channel bandwidth for existing CBW. |
| 6.5.2.2 | Spectrum emission mask | Introduce general SEM for 6MHz CBW, there are two options: - Same as 5MHz; 2) Same as 7MHz. - The only difference is -25dBm applies to 1) ΔfOOB = ± 6-10 or 2) ΔfOOB = ± 6-11. - We prefer option 2. |
| 6.5.2.3 | Additional spectrum emission mask | - 6MHz should be added for NS_06 because of bands n12 and n85. |
| 6.5.2.4 | Adjacent channel leakage ratio | Add 6MHz CBW to Table 6.5.2.4.1-1 for NR ACLR, reuse legacy ACLR measurement bandwidth calculation method for 3~50MHz CBW. |
| 6.5.3.1 | General spurious emissions | Add 6MHz CBW to Table 6.5.3.1-1, reuse legacy OOB boundary FOOB for 5~100MHz CBW. |
| 6.5.3.3 | Additional spurious emissions | Add 6MHz CBW to spurious emissions requirements for corresponding NSs if needed, related to 6.2.3. |
| 7.3.2 | ### Reference sensitivity power level | Evaluate reference sensitivity for 6MHz CBW for TDD bands n5, n8, n12, n85 and SDL band n29. |
| 7.4 | Maximum input level | Add 6MHz CBW to Table 7.4-1, reuse legacy maximum input level for 3~20MHz CBW. |
| 7.5 | Adjacent channel selectivity | Add 6MHz CBW to Table 7.5-1, the ACS requirement is same as legacy 3, 5, 7, 10MHz CBW and the test parameters are same as 5, 7, 10MHz CBW. |
| 7.6.2 | In-band blocking | Add 6MHz CBW to Table 7.6.2-1, reuse legacy IBB parameters for 5, 7, 10MHz CBW with FDL_high < 2700 MHz and FUL_high < 2700 MHz. |
| 7.6.3 | Out-of-band blocking | Add 6MHz CBW to Table 7.6.3-1, reuse Power in transmission bandwidth configuration as REFSENS + 6.0 dB. |
| 7.6.4 | Narrow band blocking | Add 6MHz to Table 7.6.4-1 and evaluate Pw for 6MHz CBW. 15dB is preferred. |
| 7.7 | Spurious response | Add 6MHz CBW to Table 7.7-1, reuse legacy spurious response parameters for 3, 5, 7, 10MHz CBW with FDL_high < 2700 MHz and FUL_high < 2700 MHz. |
| 7.8.2 | Wide band Intermodulation | Add 6MHz CBW to Table 7.8.2-1, reuse legacy wide band intermodulation parameters for 5, 7, 10MHz CBW with FDL_high < 2700 MHz and FUL_high < 2700 MHz. |

Proposal 3: Agree with the UE RF requirements for 6MHz CBW as listed in Table 3.

- Conclusion

Proposal 1: Maximum transmission bandwidth configuration for 6MHz CBW could be 30 RBs.

Proposal 2: Minimum guardband for 6MHz CBW could be 292.5kHz.

Proposal 3: Agree with the UE RF requirements for 6MHz CBW as listed in Table 3.

- Reference

- RP-252952 New WID: UE RF enhancements for NR FR1, Phase 5