TSG-RAN Working Group 4 (Radio) meeting #116-bis	R4-2513942

Prague, EU, 13th to 17th October 2025

Source:	Ericsson

Title:	On enhancement of BS co-location requirements

Agenda item:	7.2.3

Document for:	Approval

# Introduction

At the last RAN4 meeting (RAN4#116 in Bangalore) all findings from the work to investigate possible ways to improve co-location requirements as part of Rel-19 BS RF evolution Work Item (WI) were captured in a CR to TR 37.941. The CR [1] was formally approved at RAN#109 in Beijing.

At RAN#109 in Beijing a Work Item Description (WID) for a second phase of BS RF evolution was approved for Rel-20 in [2]. The new WI (NR_BS_RF_req_evo_Ph2-Core) have two separate objectives:

- Enhancement of co-location requirements

- SBFD BS to SBFD BS adjacent channel co-existence

In this contribution we present an overview of the work needed to improve the co-location requirements in BS RF specifications.

# Discussion

In following sections, we summarise the background and work to be done to be able to improve co-location requirements and test method in BS RF specifications (TS 38.104, TS 37.105, TS 38.141-2 and TS 37.145-2).

  - Background

In the discussion during Rel-19 technical issues related to the current concept used for BS type 1-O co-location requirements was identified. Detailed technical background related to technical challenges and open issues can be found in [3, 4, 5, 6, 7, 8, 10]. The issues can be summarized as:

- The description of Co-Location Reference Antenna (CLRA) in TS 38.104 and TS 37.105 and corresponding Co-Location Test Antenna (CLTA) in TS 38.141-2 and TS 37.145-2 is not complete and for many bands in the upper region of FR1 conflicting. To allow a BS vendor to design a customized CLTA a complete description of the antenna is required in the specification to guarantee robust co-location requirements.

- The intention with the CLTA concept was that commercially available passive BS antennas to be used. For bands in the upper region of FR1 the availability of such antennas is very limited or not existing.

- With current concept many CLTA will be needed for conformance testing to be able to declare co-location with many bands used is different regions. This will add logistical challenges during the conformance test phase. This would result in a situation where the test lab with access to antennas can do the test, but they cannot be re-produced at any other laboratory without access to the same pool of CLTAs.

- The CLTA test procedure is not linked to a regular OTA parameter and also utilizes measurement in non-boresight directions of the test antenna (the CLTA). A calibration stage of the test setup would be needed to guarantee the purpose of the requirements. Calibration of a test antenna that utilizes non-boresight measurements is however cumbersome. These short comings need to be solved to obtain a test method with repeatable test results and controlled measurement uncertainty. Note, this does not preclude the use of a CLTA as a test antenna.

To improve the co-location requirement concept and corresponding BS RF requirements RAN4 was tasked to study technical background and testability aspects in Rel-19.

Since the introduction of co-location requirements in RAN4 specifications 30 dB coupling loss between aggressor BS RF port and victim BS RF port have been assumed for all bands. This assumption was derived based on a link budget calculation between to neighbouring antenna towers in TR 25.942, clause 10.1 for 2 GHz and have later been confirmed by measurements for side-by-side deployments for bands below 3 GHz. Moving up in frequency the port-to-port coupling loss tend to increase, which now have been confirmed in Rel-19 outcome, as shown for intra-band coupling loss in Table 2.1-1 and for inter-band coupling loss in Table 2.1-2.

Table 2.1-1: Intra-band coupling loss summary

| Co-location scenario | Coupling loss type | Coupling loss (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 2.6 GHz | 3.5 GHz | 4.9 GHz | 7 GHz |
| Vertical separation | SA-to-SA | M: 54 (R4-2504358) | M: 53 (R4-2504358) | M: 56 (R4-2504358) | FFS |
|  | A-to-SA | FFS | FFS | FFS | FFS |
|  | A-to-A | FFS | FFS | FFS | FFS |
| Horizontal separation | SA-to-SA | M: 31 (R4-2504358) | M: 34 (R4-2504358) | M: 46 (R4-2504358) | FFS |
|  | A-to-SA | FFS | M: 35 (R4-2507633) S: 55±15 (R4-2506307) | M: 51 (R4-2507445) M: 69 (R4-2507511) | M: 55 (R4-2507633) S: 70±15 (R4-2506307) |
|  | A-to-A | FFS | FFS | FFS | S: 77±15 (R4-2506307) |

Table 2.1-2: Inter-band coupling loss summary

| Co-location scenario | Coupling loss type | Coupling loss (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 2.6 GHz to 3.5 GHz | 3.5 GHz to 4.9 GHz | 4.9 GHz to 7 GHz | 3.5 GHz to 7 GHz |
| Vertical separation | SA-to-SA | FFS | FFS | FFS | FFS |
|  | A-to-SA | FFS | FFS | FFS | FFS |
|  | A-to-A | FFS | FFS | FFS | FFS |
| Horizontal separation | SA-to-SA | FFS | FFS | FFS | FFS |
|  | A-to-SA | FFS | FFS | FFS | S: 57±10 (R4-2506307) |
|  | A-to-A | FFS | FFS | FFS | S: 54±10 (R4-2506307) |

To get better understanding on coupling loss variations, different co-location scenarios, coupling loss types, frequencies was studied based on network simulation results (S) and measurement results (M).

Two typical co-location scenarios are defined as:

- Vertical separation: Where two BS is mounted on top of each other in the same tower.

- Horizontal separation: Where two BS is mounted on a wall side-by-side separated 0.1 m, according to description in TS 38.104, clause 4.9.

Since the individual requirements for an AAS BS would need different type of coupling loss assumption for requirement derivation different coupling types was studied:

- Sub-Array-to-Sub-Array (SA-to-SA) coupling loss

- Array-to-Sub-Array (A-to-SA) coupling loss

- Array-to-Array (A-to-A) coupling loss

For the table entries left blank (marked with FFS) no information have been provided to RAN4, which would indicate that the work is not complete. Even though the coupling loss evaluation is not complete for all relevant cases, it’s clear that the outcome indicates that the assumption of a fixed BS-to-BS port coupling loss assumed to 30 dB for FR1 is not relevant anymore, especially not for AAS BS operating in the upper region of FR1. From the combined result from measurements and simulations it can be concluded that the coupling loss varies considering all scenarios, frequencies and coupling types. The general trend indicates that coupling loss increases for bands in the upper region of FR1. The outcome from this work is captured in TR 37.941, clause 6.4.4.2.

Observation 1: It can be noticed that the coupling loss evaluation from Rel-19 is not complete for all relevant situations.

As expected, vertical separation results in higher coupling loss, hence horizontal separation is seen as the worst-case co-location scenario.

Regarding coupling loss type of interest for co-location requirement level derivation, it can be noticed that SA-to-SA is easy to measure, but not really relevant for requirement derivation. The cases to focus on is marked in bold green text in tables above.

RAN4 need to further evaluate BS-to-BS coupling loss for different coupling situations relevant for considered requirements. Related to the technical background, RAN4 observed that the BS-to-BS coupling loss typically is larger than 30 dB for bands in the upper region of FR1.

This will have significantly impact on the requirement derivation for each specific co-location requirement. As a summary, the requirement background for how the requirement levels is derived is captured in TR 37.941, clause 6.4.4.3. It can be concluded that with larger coupling loss test aspects, levels that can be detected in OTA test ranges using regular OTA parameters needs further considerations.

Observation 2: If the requirement background is changed using a coupling loss assumption larger than before also the testability aspects needs to be considered. Eventually, the requirement can be defined in such a way that regular OTA parameters can be used and tested in regular OTA test ranges avoiding any specific CLTA to be used.

In TR 37.941, clause 6.4.4.4 findings related to using alternatives to CLTA to improve conformance testing. Instead of using a CLTA according to description in TS 38.141-2, clause 4.12, a wide band horn antenna was considered. It can be concluded that it is not straight forward to use an alternative antenna and expect to get the same results. If the requirement levels were defined based on OTA parameters defined in the near-field region alternative antennas could be used.

Observation 3: In Rel-19 it was concluded that there is no straightforward one-to-one translation to use wideband horn antenna as CLTA with the current CLTA arrangement.

  - Enhancement of co-location requirements

For Rel-20 WI the technical justification in the WID can be summarized as:

The upper bands in NR FR1 are becoming more important. A number of issues for BS type 1-O requirements were discussed in the previous releases. To make BS type 1-O more relevant for AAS BS operating in the upper bands of FR1, RAN4 need to investigate and enhance the co-location requirements.

The objective in the WID captured two options on how to improve co-location requirements:

Enhancement of co-location requirements:

Enhancement of co-location requirements for BS type 1-O for upper frequencies of FR1 targeting above 3.3 GHz

- Evaluate impact of Rel-19 BS-to-BS isolation findings captured in TR 37.941 (clause 6.4.4) on OTA co-location requirements.

- Consider impact on the following OTA co location requirements:

  - Co-location transmitter spurious emission requirement

  - Co-location receiver out-of-band blocking requirement

  - Transmitter OFF power requirement

  - Transmitter intermodulation requirement

- Enhance OTA co-location requirements by Improving the core requirements with reasonable expected BS-to-BS isolation for the upper FR1 frequencies. Two options will be considered

  - Option 1: Investigate and improve CLRA concept (check robustness)

  - Option 2: Investigate possibility to define requirements using the existing OTA metrics (such as TRP, EIRP, EIS, field strength, …), if any

It can be noticed that Option 1 and Option 2 is not mutual exclusive. Both options can be considered. RAN4 can later decide if enhancements related to both options can solve identified technical issues related to bands in the upper region of FR1.

Observation 4: For Rel-20, Option 1 and Option 2 are not mutually exclusive. For both Option 1 and Option 2, simulation results will be needed.

Regarding Option 2, it is worth to point out that for existing parameters (such as TRP, EIRP and EIS) are related to other OTA parameters such as power density and field strength that can be more suitable for defining requirements to captured near-field effects due to BS co-location close to each other.

  - Work plan

In Rel-20 a work package with the objective to improve and enhance the co-location requirement for BS type 1-O for frequencies above 3.3 GHz was included in BS RF requirement evolution phase 2 WI. The work package is supposed to run over complete Rel-20, starting at RAN4#116-bis October 2025 and expected to end at RAN4#124 August 2027 including both BS RF core aspects and conformance test aspects. The time allocation is 0.25 to 0.5 TU considering the split between other work packages in the same WI.

Based on Rel-19, it can be concluded that coupling loss evaluation for relevant situation is essential to improve co-location requirements. Therefore, we need to continue to evaluate coupling loss for inter-band and intra band situations as indicated in Table 2.3-1.

Table 2.3-1: Requirement derivation assumptions

| Requirement | Coupling type | Frequency aspect | Description |
| --- | --- | --- | --- |
| Transmitter spurious emission | A-to-A | Inter-band | The requirement is defined to protect another band, hence coupling between bands are relevant. The approach used to derive the requirement level is defined to consider A-to-A coupling. |
| Receiver out-of-band blocking | A-to-SA | Inter-band | The requirement is defined to protect another band, hence coupling between bands are relevant. Since the receiver blocking is related to induvial receiver branches, A-to-SA is considered when the requirement level is derived. |
| Transmitter OFF power | A-to-A | Intra-band | The requirement is defined for TDD operation to protect the neighbouring BS in the same network. The approach used to derive the requirement level is defined to consider A-to-A coupling within the same band. The A-to-A coupling for this requirement should be based on a neighbouring BS in the same network and not side-by-side at the same site. For this requirement the coupling loss between two neighbouring BS is different cells should be considered. The coupling loss for this requirement can be evaluated via network simulations using far-field assumptions. |
| Transmitter intermodulation | A-to-SA | Intra-band | This requirement is defined to guarantee that unwanted emission requirements is met in the case of an interferer signal. The interferer signal is injected within the band. Since the requirement is related to individual transmitter branches, A-to-SA is considered when the requirement level is derived. |

For tower sharing, transmitter spurious emission and receiver out-of-band blocking are the two essential co-location requirements. Work to improve those should be prioritized, hence studies to evaluate inter-band coupling loss cases should get priority before intra-band cases.

Modelling assumptions of inter-band emissions needs to be agreed. These could be based on TR 38.922, clause 7.3.2 in [9] that suggests a way to model the average directivity pattern based on a correlation parameter. A model to generate statistically distributed sample patterns is also needed.

Observation 5: For tower sharing, transmitter spurious emission and receiver out-of-band blocking requirements are essential and should be prioritised.

Observation 6: Transmitter OFF power requirement is not related to the side-by-side co-location scenario. To derive proper requirement levels relevant parameters considering two neighbouring BS in the same network should be considered.

Observation 7: For transmitter intermodulation RAN4 should study impact due to intra-system coupling and inter-system coupling for relevant AAS BS architectures and conclude.

To progress the work a work plan including a simulation campaign followed with requirement derivation and conformance test evaluation is needed. A work plan following RAN4 way-of-working including an initial simulation campaign adopted for near-field simulations required for co-location requirements can be summarized as:

- For each co-location requirement define co-location scenarios and define relevant simulation assumptions (ISD, Number of UEs, BS height, BS array configuration, band combinations, etc.).

- Define scope in terms of frequency ranges (e.g., 3, 4.9 and 7 GHz) for relevant inter-band and intra-band cases.

- Agree on models (e.g. out-of-band emission model based on correlation factor roll-off).

- Based on EM-modelling (for defined co-location scenario) evaluate coupling matrix (including victim BS and aggressor BS elements) as described in [8]. For the EM simulation RAN4 need to consider an approach using a simplified antenna model that can be shared between companies.

- Using coupling matrix and apply circuit theory to map elements towards define array structure including sub-arrays as described in [8]. Depending on requirement considered the coupling loss type is different.

- Conduct a network simulation to capture BS beam-forming aspects due to UE locations in victim network and aggressor network as described in [8].

- Evaluate the coupling loss statistically and decide on proper value on the CDF to be used for requirement derivation.

- For each requirement derive relevant requirement levels and introduce them in TS 37.105 and TS 38.104.

- Define conformance test requirement and corresponding test methods in TS 37.145-2 and TS 38.141-2.

In Annex in section 5 of this contribution an example of a near-field simulation methodology from [10] is attached as reference. In addition to normal RAN4 simulations additional steps to cover near-field aspects is included.

Observation 8: To be able to determine relevant coupling loss values for requirement derivations a simulation campaign including near-field modelling of victim BS and aggressor BS is necessary.

The coupling loss between co-located AAS BSs depends on the frequency bands used. To arrive at a reasonable number of combinations the FR1 range would need to be subdivided in sub-ranges. For each combination of sub-ranges, a coupling loss value should be agreed.

Observation 9: The coupling loss assumptions should be captured for all band combinations picked from a division in sub-ranges.

# Conclusion

In this contribution we summarise the background, outcome from Rel-19 discussion and outlines what to do in Rel-20.

Based on the information in this contribution we observe following:

Observation 1: It can be noticed that the coupling loss evaluation from Rel-19 is not complete for all relevant situations.

Observation 2: If the requirement background is changed using a coupling loss assumption larger than before also the testability aspects needs to be considered. Eventually, the requirement can be defined in such a way that regular OTA parameters can be used and tested in regular OTA test ranges avoiding any specific CLTA to be used.

Observation 3: In Rel-19 it was concluded that there is no straightforward one-to-one translation to use wideband horn antenna as CLTA with the current CLTA arrangement.

Observation 4: For Rel-20, Option 1 and Option 2 are not mutually exclusive. For both Option 1 and Option 2, simulation results will be needed.

Observation 5: For tower sharing, transmitter spurious emission and receiver out-of-band blocking requirements are essential and should be prioritised.

Observation 6: Transmitter OFF power requirement is not related to the side-by-side co-location scenario. To derive proper requirement levels relevant parameters considering two neighbouring BS in the same network should be considered.

Observation 7: For transmitter intermodulation RAN4 should study impact due to intra-system coupling and inter-system coupling for relevant AAS BS architectures and conclude.

Observation 8: To be able to determine relevant coupling loss values for requirement derivations a simulation campaign including near-field modelling of victim BS and aggressor BS is necessary.

Observation 9: The coupling loss assumptions should be captured for all band combinations picked from a division in sub-ranges.

Based on the observation we propose the following:

Proposal 1: Establish simulation methodology required for side-by-side co-location scenario considering near-field effects.

Proposal 2: As outcome from RAN4#116-bis agree on a way-forward including a work-plan for co-location requirements enhancements part of Rel-20.

# References

[1]	RP-252586, “CR to TR 37.941: Rel-19 CLTA-related agreements”, Huawei, HiSilicon, ZTE, Nokia, Ericsson

[2]	RP-252905, “New WID on NR base station (BS) RF requirement evolution for FR1 and testing phase 2”, Moderator (Huawei)

[3]	R4-2415971, “On the topic of aspects related to BS-to-BS co-location”, Ericsson

[4]	R4-2408086, “Evolution of BS co-location requirements”, Ericsson

[5]	R4-2412912, “On the topic of BS RF evolution related to co-location requirements”, Ericsson

[6]	R4-2510233, “On alternatives to current CLTA concept”, Ericsson

[7]	R4-2510229, “Discussion on BS RF evolution general aspects”, Ericsson

[8] 	R4-2506307, “Overview of BS-to-BS coupling loss simulation results”, Ericsson

[9] 	TR 38.922, “Study on International Mobile Telecommunications (IMT) parameters for 4400 - 4800 MHz, 7125 - 8400 MHz and 14800 - 15350 MHz”, 3GPP

[10]	R4-2510227, “On improvements of co-location requirements”, Ericsson

# Annex

  - Near-Field simulation methodology

Performing a co-location coupling analysis of two side-by-side large arrays is a formidable task that easily leads to memory overflow and long simulation times, especially if a complete coupling model is used in an Electromagnetic (EM) simulation tool. To overcome this, simplifications have been used such as using a simplified EM model of the antenna element and analytical circuit theory models of subarray networks. The overall idea is to choose these simplifications in a conservative way, i.e., to minimize the coupling loss. For example, there are no isolating structures on the sides of the arrays and no losses in the subarray models. Hence, in a real case the coupling loss should be larger compared to these simulations.

Subarray beamforming networks are represented by S-matrices modelled analytically using circuit theory, see Section 5.1.4. Finally, array ports are obtained by applying beamforming weights to the subarray ports, see Section 5.1.6.

For the Near-Field simulation methodology presented in this contribution, we have split the simulations in two parts:

- In the first stage, the Element-to-Element coupling is represented by a scattering matrix simulated in an EM simulator, see Section 5.1.1. The victim and aggressor array antennas are placed according to decided co-location scenario side-by-side separated 0.1 m, as illustrated in Figure 5.1.2-1. The scattering matrix (S-matrix) includes all elements (both polarizations) of both arrays and contains element-to-element coupling loss and associated phase information. If needed, the S-matrix is calculated for several frequencies within the frequency range that is considered.

- In the second stage sub-arrays and array ports are created. Subarray beamforming networks are represented by S-matrices modelled analytically using circuit theory, see Section 5.1.4, and array ports are obtained by applying beamforming weights to the subarray ports (and positions), see Section 5.1.6. For this stage a general-purpose linear algebra computation tool, e.g., MathWorks MATLAB or Python can be used.

A block diagram of the simulation methodology is shown in Figure 5.1-1.

Figure 5.1-1: Overview of the simulation methodology

    - S-matrix simulation

The antenna element scattering matrix is simulated using a commercial 3D EM field simulation software, in this case CST Studio Suite. The S-matrix contains data for all elements and polarizations in both arrays.

As an example, in the case of 3.5 GHz arrays, each of the arrays has 192 ports (12 rows x 8 columns x 2 polarization). This results in a 384x384 scattering matrix for each frequency point in the defined frequency range. For more details on geometry and element model, see below.

    - Geometry

For simulating the coupling between two arrays, two antenna arrays are placed next to each other as shown in Figure 5.1.2-1.

Figure 5.1.2-1: Two antenna arrays in an edge-to-edge co-location scenario

Each array comprises 12x8 dual-polarized radiating elements and a metallic reflector. The vertical element separation is 0.7λ and the horizontal element separation 0.5λ at specific design frequency (e.g., 3.5 GHz).

The distance between the two antennas (edge-to-edge) has been increased from 10 cm to 16 cm to take account of the mechanical support structures, which are present in a real product, and of space for the enclosure, which is also not included in the simulation.

    - Antenna element model

A simplified antenna element was used to get reasonable simulation time for the Element-to-Element S-matrix. Mechanical details, not relevant for the EM simulation was removed to speed up the simulation stage. These removed details include, e.g., as corners, mechanical support structures, logotypes, etc. As example of such the antenna element model used for simulation is shown in Figure 5.1.3-1. It consists of a thin crossed half-wavelength dipole which is fed in the centre by a coaxial port for each polarization (+/- 45°) and a metallic reflector at quarter-wave distance below the dipoles. There is no connection between the reflector and the dipole.

Figure 5.1.3-1: Perspective, top and side view of a simplified antenna element model

    - Sub-array modelling

The antenna elements are grouped in 3-element single column sub-arrays, as shown in Figure 5.1.4-1. A sub-array RDN, having 3 element ports and one feed port, is modelled with lossless ideal transmission lines and T-splitters, see Figure 5.1.5-1. To achieve equal power on the three elements, one of the T-splitters, S1, has an uneven power split ratio of 2:1, and the other, S2, has an even power split. Impedances of the quarter wave transformers on the output ports are chosen so that the desired power split ratio and impedance matching is achieved. The lengths of the lines L12, L13, L22 and L23 are chosen to achieve the desired fixed subarray tilt and long enough for the physical element spacing. The propagation constant of the transmission lines is chosen to equal that of free space for simplicity.

# Figure 5.1.4-1: Array and subarray layout

    - Sub-array ports S-matrix (cascading)

The S-matrices of the components in the antenna RF model are combined to an S-matrix describing the entire antenna including RDNs using well known circuit simulation theory, as visualised in Figure 5.1.5-1.

Z0, L13

Z0, L12

Z0, L23

Z0, L22

S1

S2

Figure 5.1.5-1: A 1:3 RDN is simulated by standard RF circuit techniques using S-matrix cascading

    - Deployment and beamforming weights

In the simulation it is assumed that two AAS base stations are co-located at the same site according to the enclosure edge-to-edge scenario defined in Rel-15 for BS type 1-O co-location requirements in TS 38.104, clause 4.9. In this scenario it is assumed that both the aggressor AAS base station and the victim AAS base station are mounted side-by-side separated 0.1 m. The aggressor network and the victim network are configured in a hexagonal layout with 0% grid shift as visualised in Figure 5.1.6-1.

# Figure 5.1.6-1: Network layout

The victim and aggressor networks are co-sited. Narrow beams (similar to layer 1 codebook generated beams) are formed from the aggressor AAS to the UEs of the aggressor network and the corresponding power is received on the subarray ports of the co-sited AAS of the victim network. Only one cell is considered in the analysis.

Details related to deployment and BS assumptions is listed in Table 5.1.6-1, and Table 5.1.6-2.

Table 5.1.6-1: Network assumptions

| Parameter | Assumption |
| --- | --- |
| Network layout | Hexagon |
| ISD | 500 m |
| Network grid shift | 0 % |
| BS height | 25 m |
| UE height | 1.5 m |
| Number of UEs per sector | 1500 |
| Minimum 2D distance between BS and UE | 35 m |

Table 5.1.6-2: BS assumptions

| Parameter | 3500 MHz | 7000 MHz |
| --- | --- | --- |
| Sub-array configuration | 3x1 | 3x1 |
| Array configuration | 4x8 | 8x16 |
| Pre-set sub-array down-tilt | 3 degrees | 3 degrees |
| Mechanical down-tilt | 3 degrees | 3 degrees |
| Vertical element separation | 0.7l | 0.7l |
| Horizontal element separation | 0.5l | 0.5l |

For the evaluation in this contribution the physical size of the AAS BS antennas considered for 3500 MHz and 7000 MHz is equal. Considering other band combinations, such as lower bands, it is expected that the physical size will differ between aggressor and victim.

In the case of using randomly positioned UEs and A-to-A coupling loss, 1500 randomly distributed UEs per system are used. Then, at each time instance a pair of users are chosen, which are used to generate one sample of coupling loss, i.e., there will be a total of 1500 coupling loss samples. In Figure 5.1.6-2, an example of UE locations is visualised for the aggressor network and victim network.

In the case of A-to-SA coupling, only one system uses the above beamforming, and coupling is calculated between the array and the sub-array ports.

Figure 5.1.6-2: UE distribution

For an AAS BS array antenna operating outside the design frequency it is expected that the directivity will drop due to decorrelation effects due to out-of-band RF filter characteristics, transmission effects in the feeder network part of RDN and unwanted emission signal correlation.

For out-of-band beam weights a correlation model is used, corresponding to adding white noise to the antenna element weights. Here, 100% correlation (r equal to 1) corresponds to clean beam weights with no noise and 0% correlation (r equal to 0) corresponds to fully noisy beam weights. The effect of the beam pattern is that the directivity will decrease when the correlation decreases. In turn this leads to relatively more power being radiated sideways, which seems to minimize the coupling loss. Hence, 0% correlation could be used to derive conservative co-location requirements. The actual value of the correlation is unknown and is dependent on the specific implementation. Specifically, the digital predistortion will influence the out-of-band beam weights significantly.

Assuming a polynomial model, the beam weights for even harmonic frequencies equals the square of the beam weights of the carrier frequency, wc, while for odd harmonics the weights are equal to the beam weights of the carrier.

wnom=wcfor odd harmonicswc2/wc for even harmonics

where wnom, are the nominal beam weights.

All out-of-band beam weights, including the harmonics, are modelled as partly correlated with the correlation as a parameter. Partly correlated beam weights are modelled as:

w=ρ wnom+1-ρ wuncorr

where wuncorr are random weights:

wuncorr=x+jy2N

Here, x and y are normally distributed random variables with expected value 0, and N is the number of subarray ports.