3GPP TSG-RAN WG4 Meeting #116bis	R4-2513946

Prague, Czech Republic, October 13th-17th, 2025

Source:	Nokia

Title:	Discussion on Conditional Intra-CU LTM Test Cases

Agenda item:	6.17.2.3

Document for:	Discussion

Introduction

In this contribution, we discuss the test case scope for CLTM.

Discussion

CLTM test case scope in terms of FR1/FR2, intra/inter-frequency, and L1/L3

RAN4 has defined cell switch delay requirements for CLTM cell switch. The delay requirement is to cover the case of L1- and L3-based conditional cell switch. Furthermore, the evaluation delay Tmeasure may depend on whether gap-based or gap-less intra- or inter-frequency measurements are configured, and whether the cell switch happens towards an FR1 or FR2 target cell. Even further, the CLTM cell switch delay may depend on whether the cell switch is RACH-less or RACH-based, whether early ASN.1 decoding is supported, and whether early DL sync is performed. RAN4 has also defined requirements for subsequent CLTM cell switch.

In the last meeting, RAN4 made the following agreement about the frequency ranges for the test cases under Issue 4-3-1.

Issue 4-3-1: CLTM test scope – frequency range Agreement: - CLTM test frequency range: - FR1 to FR1 - FR2 to FR2 - Other options are not precluded.

As the agreed scope is the same as for Rel-18 LTM cell switch delay test cases, we do not see a need to add any further scenarios in the scope.

The FR scope for CLTM is the same as the FR scope for Rel-18 LTM.

Regarding intra- and inter-frequency test cases, all contributing companies proposed to include both scenarios in the test cases in the last meeting (Option 1).

Issue 4-3-2: CLTM test scope - intra/inter-frequency Candidate options: - Option 1: cover both intra and inter-frequency CLTM. (Nokia, Apple, CTC, vivo, ZTE, HW) Recommended WF: - Continue discussion.

We propose to follow Option 1, i.e., define CLTM test cases for both intra- and inter-frequency cell switch for both FR1 to FR1 and FR2 to FR2 cell switch.

Define test cases for CLTM cell switch to both intra-frequency and inter-frequency target cell.

Regarding L1 and L3 triggered CLTM cell switch, the discussed options were listed under Issue 4-3-3 in last meeting’s way forward.

Issue 4-3-3: CLTM test scope – L1/L3 based CLTM Candidate options: - Option 1: cover both L1 and L3 based CLTM. (Nokia, CATT, CTC, ZTE, E///) - Option 1a: RAN4 to discuss how to verify L1 and L3 measurement based candidate evaluation for CLTM such as whether to reuse partial LTM test procedure. (E///) - Option 2: only test L1 based CLTM. (OPPO, MTK) Recommended WF: - Discuss the options

Some companies commented that there is no need to test L3 triggered CLTM as this is similar to CHO. However, we would like to highlight that CHO is also an optional feature, and a UE supporting CLTM does not need to support CHO. Therefore, we think it is necessary to define test cases for both L1 and L3 triggered CLTM. L3 triggered CLTM applies only for FR1, so there is no need to define L3 triggered CLTM test cases for FR2.

Define test cases for both L1 and L3 triggered CLTM cell switch in FR1. Define test cases only for L1 triggered CLTM cell switch in FR2.

Early UL and DL synchronization in CLTM test cases

Regarding early UL synchronization, Issue 4-3-4 covered the following options.

Issue 4-3-4: CLTM test scope – RACH-less and RACH-based CLTM Candidate options: - Option 1: cover both RACH-less and RACH based CLTM. (Nokia, CATT, CTC, OPPO, vivo, ZTE, HW) - Option 2: RAN4 to discuss how to verify both RACH based and RACH-less procedure for CLTM such as re-use partial LTM test procedure. (E///) Recommended WF: - Discuss the options

We think testing of both RACH-based and RACH-less CLTM cell switch should be covered. The test cases should be defined so that the UE is ensured to have a valid TA at the time of triggering the cell switch. This means that the TA timer needs to be running at the time of cell switch.

Define test cases for both RACH-less and RACH-based CLTM cell switch.

In RACH-less CLTM cell switch test case, ensure that the TA timer is running at the time of triggering the cell switch.

Regarding early DL synchronization, Issue 4-3-5 covered the following options.

Issue 4-3-5: CLTM test scope – early DL sync Candidate options: - Option 1: cover both CLTM with and without early DL sync. (Nokia) - Option 2: only test CLTM with early DL sync. Do not test CLTM without early DL sync. (OPPO, MTK) Recommended WF: - Discuss the options

Similar to early UL synchronization, RAN4 should test cell switch delay requirements both with and without early DL synchronization. In Rel-18 cell switch test cases, early TCI state activation was either included or not included in the same test case depending on UE support of the feature. We think we can follow the same approach for CLTM test cases. It should be ensured that the UE is expected to have early DL synchronization valid at the time of cell switch, so TCI state activation command has to be sent close to the cell switch triggering.

Cover both with and without early DL sync in the CLTM cell switch test cases. These can be defined so that, depending on UE capability, early TCI state activation is included or not.

Ensure that UE has DL synchronization at the time of cell switch, if UE supports early TCI state activation capability in the test case.

Subsequent CLTM test cases

Options for subsequent CLTM cell switch test cases were discussed under Issue 4-3-7.

Issue 4-3-7: subsequent CLTM test scope Candidate options: - Option 1: Apple - Intra-frequency subsequent CLTM from FR1 to FR1 - Inter-frequency subsequent CLTM from FR2 to FR2 - Option 1a: MTK - subsequent CLTM from FR1 to FR1 - subsequent CLTM from FR2 to FR2 - Option 2: OPPO - (SA) RACH based intra-frequency subsequent CLTM for PCell switch from FR1 to FR1 - (SA) RACH less inter-frequency subsequent CLTM for PCell switch from FR2 to FR2 - (NR-DC) RACH based intra-frequency subsequent CLTM for PSCell switch from FR1 to FR2 - (NR-DC) RACH less inter-frequency subsequent CLTM for PSCell switch from FR2 to FR1 - Option 3: CMCC - PCell switch to a neighboring LTM candidate cell - FR1 cell to FR1 cell - FR1 cell to FR2 cell - FR2 cell to FR2 cell - FR2 cell to FR1 cell - PCell switch to an LTM candidate cell that is a serving SCell in MCG - FR1 cell to FR1 cell - FR2 cell to FR2 cell - Option 4: HW - RACH based Intra-frequency subsequent conditional PCell switch - RACH based Inter-frequency subsequent conditional PCell switch - RACH less Intra-frequency subsequent conditional PCell switch - RACH less Inter-frequency subsequent conditional PCell switch Recommended WF: - Discuss the options

We think that we can follow the same test coverage for subsequent CLTM cell switch as for initial CLTM cell switch.

Apply same test coverage for initial and subsequent CLTM cell switch delay test cases.

In Issue 4-3-9, the following proposal was captured regarding the initial and subsequent test case applicability.

Issue 4-3-9: test applicability Candidate options: - Proposal 1: for UE which can pass subsequent CLTM, test case for CLTM can be skipped, since subsequent CLTM naturally includes a complete CLTM procedure. (Apple, MTK) Recommended WF: - Discuss the options

Proposal 1 under Issue 4-3-9 is acceptable to us.

For UE which can pass subsequent CLTM test case, test case for CLTM can be skipped, since subsequent CLTM naturally includes a complete CLTM procedure.

Other options for CLTM test cases

PCell vs. PSCell cell switch was discussed under Issue 4-3-6.

Issue 4-3-6: CLTM test scope – PCell/PSCell Candidate options: - Option 1: cover both PCell swith and PSCell switch. (ZTE) Recommended WF: - Discuss the options

RAN2 agreed that CLTM is only supported for PCell cell switch and there are no requirements for PSCell cell switch, so there is no need to cover PSCell cell switch in the test cases.

PSCell conditional cell switch is not supported, so define test cases for PCell conditional cell switch only.

Conclusion

The following observations and proposals were made:

Observation 1: The FR scope for CLTM is the same as the FR scope for Rel-18 LTM.

Proposal 1: Define test cases for CLTM cell switch to both intra-frequency and inter-frequency target cell.

Proposal 2: Define test cases for both L1 and L3 triggered CLTM cell switch in FR1. Define test cases only for L1 triggered CLTM cell switch in FR2.

Proposal 3: Define test cases for both RACH-less and RACH-based CLTM cell switch.

Proposal 4: In RACH-less CLTM cell switch test case, ensure that the TA timer is running at the time of triggering the cell switch.

Proposal 5: Cover both with and without early DL sync in the CLTM cell switch test cases. These can be defined so that, depending on UE capability, early TCI state activation is included or not.

Proposal 6: Ensure that UE has DL synchronization at the time of cell switch, if UE supports early TCI state activation capability in the test case.

Proposal 7: Apply same test coverage for initial and subsequent CLTM cell switch delay test cases.

Proposal 8: For UE which can pass subsequent CLTM test case, test case for CLTM can be skipped, since subsequent CLTM naturally includes a complete CLTM procedure.

Proposal 9: PSCell conditional cell switch is not supported, so define test cases for PCell conditional cell switch only.