3GPP TSG-RAN WG4 Meeting #116bis	R4-2513958

Prague, Czech, 13th – 17th October 2025

Agenda Item:	6.8.4

Source:	Ericsson

Title:	On UE demodulation requirements for ATG enhancement

Document for:	Discussion

# 1	Introduction

In 3GPP release 18, RAN4 has completed the standardization work for the air-to-ground (ATG) network WI, the main work focused on the FR1 single carrier scenario. To enhance the throughput to meet growing ATG capacity demand and fully utilize operators’ spectrum, the RAN plenary meeting #103 has approved a new WI on enhancements for the air-to-ground network for NR [1].

### 4.2 Objective of Performance part WI NOTE: Leave empty if the WI proposal does not contain a RAN performance part. The performance part of the work item includes: - Specify corresponding RRM performance and demodulation requirements for intra-band co-located and inter-band co-located DL CA. - Specify corresponding demodulation requirements for support of UL MIMO with 2TX for single CC for UE with dual-polarization antenna. - Specify demodulation requirements for support of DL MIMO for UE with limited test cases. - Specify conformance testing and conformance requirements for BS.

For the past three meetings, companies have agreed on the scope to introduce new PDSCH requirements for DL CA and DL MIMO features. For DL CA, all parameter assumptions have been agreed thus companies are encouraged to provide simulation results in this meeting. For DL MIMO, there are still some open issue left for further discussion.

In this paper, we provided our views towards left open issues.

# 2	Discussion

## 2.1 Downlink CA

We have provided our simulation results for all CA test cases in our companion simulation result paper.

## 2.2 Downlink MIMO

Another objective is to specify demodulation requirements for the support of DL MIMO for UE with limited test cases.

RAN4 only considered 1 layer for ATG PDSCH demodulation requirements in the previous release since companies had concerns regarding potential inter-layer interference brought by multiple layers transmission causing by the agreed high correlation channel. Therefore, we propose to evaluate the performance of 2 layers first, configuring with both 2Rx and 4Rx, to see the feasibility of introducing requirements for 2 MIMO layers.

Currently, there are several candidates [2] for the MCS value, together with the consideration of XP-low antenna correlation.

Issue 2-1-4: MCS Agreement: - For initial simulation, consider 16QAM Table 1 MCS13, 64QAM Table 1 MCS 22 and 256QAM Table 2 MCS 24 Issue 2-1-1: Antenna configuration Agreement: - 2T2R and 2T4R FFS the antenna correlation: - Proposal 1: XP-low

To facilitate the parameter assumption, we provided our initial simulation results based on the legacy propagation condition but different MCS and antenna configurations.

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Frequency offset | antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1 | 10 / 15 | 16QAM, 0.48 | AWGN (Note 1) | 220Hz | 2x2 | 70 | 6.4 |
| 2 | 10 / 15 | 64QAM, 0.65 | AWGN (Note 1) | 220Hz | 2x2 | 70 | 13.5 |
| 3 | 10 / 15 | 256QAM, 0.82 | AWGN (Note 1) | 220Hz | 2x2 | 70 | 22.46 |
| 4 | 10 / 15 | 16QAM, 0.48 | AWGN (Note 1) | 220Hz | 2x4 | 70 | 3.4 |
| 5 | 10 / 15 | 64QAM, 0.65 | AWGN (Note 1) | 220Hz | 2x4 | 70 | 10.5 |
| 6 | 10 / 15 | 256QAM, 0.82 | AWGN (Note 1) | 220Hz | 2x4 | 70 | 19.45 |

Note 1: Apply the static propagation condition specified in TS 38.101-4 B.1.

From the simulation results we can observe that the performance of configuring three different MCS are all good enough for the requirement. We also observed expected MIMO gain on the maximum throughput over the result of rank 1, see figures below.

Figure 2.2-1 Performance with 16QAM

Figure 2.2-2 Performance with 64QAM

Figure 2.2-3 Performance with 256QAM

Observation 1: From the performance point of view, all three modulation orders could be selected for defining requirements

During the discussion in RAN4 116 meeting in Bangalore, a key question is whether the test case should use 64QAM (CR≈0.65) or 256QAM (CR≈0.82) as the modulation order and coding rate combination.
In live networks, higher-order modulation is typically used only at high SINR and for short bursts. For long-duration PDSCH loading as required in SDR testing, 64QAM is more representative and stable in real deployments. Considering this is a requirement for 2 MIMO layer, we think it’s more typical for a 64QAM scheduling comparing with 256QAM.

For 256QAM, though it might be a little challenging for testing concerning the current SNR point plus some additional margin, RF limitation and implementation constrains, there could be some possibilities for scheduling it because the ATG channel is dominated by LoS propagation, large-scale fading, and high Doppler shifts, which can be considered stable with high SNR condition. Given this analysis above, we think the test case with 256QAM plus 2-layer can be also considered.

In addition, we see the meaning of introducing the test case of 16QAM + 2-layer as well, since lower modulation orders are more likely to be scheduled frequently when considering higher layer transmission. It is expected that 16QAM can better sustain a robust 2-layer transmission.

To summarize, we propose the following:

Proposal 1: Define 2-layer requirements with 16AM, 64QAM and 256QAM

# 3	Summary

In this contribution, we shared our views for the demodulation requirement discussion for ATG enhancement on DL CA and MIMO. Proposals are summarized as follows:

Proposal 1: Define 2-layer requirements with 16AM, 64QAM and 256QAM

# References

[1]	RP-243285: Revised WID on Enhancements for Air-to-ground network for NR, CMCC, ZTE Corporation, CATT, Huawei, HiSilicon, Verizon, China Telecom, China Unicom, RAN #106, Madrid, Spain

[2]	R4-2512602 WF on NR_ATG_enh_demodulation requirements, ZTE Cooperation, CMCC, RAN4 #116, Bengaluru, India

[3]	TS38.101-4