3GPP TSG-RAN4 Meeting # 116bis	R4-2514043

# Prague, Czech Republic, 13 Oct - 17 Oct , 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev | – | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on Test case of Inter-RAT measurement without MG of R19 CSSF enh |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_Ph5-Perf |  |  |  |  |  | Date: |  |  | 2025-10-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The CSSF enhancement solution 1 has been agreed as followings. Test cases need to be defined to verify the measurement requirements to consider Aspect 1, Aspect 2 and Aspect 3. Issue 1-3-1: Tentative solution 1: measure one serving CC per band Agreement: - If network indicates to enable this CSSF enhancement, UE only needs to measure one serving CC per band if multiple serving CCs are configured for measurement in the same band and UE is allowed to determine the CC in the band to be measured based on followings, - Network indication of enable/disable this CSSF enhancement is per-UE. - Follow the following principle for serving and neighboring cell measurement - Measure PCC if PCC in band - Measure PSCC if PSCC in band - Measure SCC on which UE is configured to report SSB based measurements, if - there is only one SCC on which UE is configured to report SSB based measurements on same FR2 band and no PCC/PSCC in band - Follow the network indication to measure a SCC, if - neither PCC nor PSCC is in band, and - if there is no SCCs on which UE is configured to report SSB based measurements in band, or - if there are multiple SCCs on which UE is configured to report SSB based measurements in band. - Network indication of a specific SCC per-band for measurement. - if network indicates which of the SCC to be measured in a band, UE will perform both serving and neighbour cell measurements on the indicated SCC in a band. - Otherwise, UE will perform serving and neighbor cell measurements on one SCC per band, which SCC to measure is up to UE implementation. - Otherwise, UE will follow the R15 spec to perform serving and neighbor cell measurements. - Note: Following the WID, the above agreement applies for the measurement without gap. Agreement: - The following aspects in CA/DC to use L3 measurement delay reduction by optimizing CSSF shall be discussed, and further prioritization among the 3 aspects can be discussed in future meeting: - Aspect 1: SSB based Intra-frequency measurement without MG, including: - TPSS/SSS_sync_intra and TSSB_measurement_period_intra - CSSFintra for intra-frequency measurement without gap which is defined since Rel-15 - Aspect 2: SSB based Inter-frequency measurement without MG, including: - TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - CSSFinter for inter-frequency measurement without gap. - Aspect 3: Inter-RAT SSB measurement without MG, including: - CSSFinterRAT for inter-RAT measurement without gap if the UE indicates ‘nogap-noncsg’ via NeedForGapNCSG-InfoEUTRA for the inter-RAT measurement. - MG related features to be considered in aspect 1/2/3 including: - R16 Inter-frequency measurement without gap where SSB is completely contained in active BWP - R17 NCSG measurement with ‘nogap-noncsg’ - R18 NeedForGaps measurement with ‘no-gap-no-interruption’ or with “no-gap-with-interruption” |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the test cases to verify the Rel-19 CSSF enhancement solution 1 to consider Aspect 3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirement for Rel-19 CSSF enhancement solution 1 for UE supporting Rel-19 CSSF enhancement cannot be verified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (new) A.7.6.x, A.7.6.x.1, A.7.6.x.x.1, A.7.6.x.x.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change #1>

A.7.6.x	Inter-RAT Measurements

#### A.7.6.x.1	SA event triggered reporting test without gap under non-DRX for UE configured with [MeasuringoneCCperFR2band] in FR2 inter-band CA

##### A.7.6.x.x.1	Test purpose and Environment

The purpose of this test is to verify that if the UE indicates ‘nogap-noncsg’ via NeedForGapNCSG-InfoEUTRA for the inter-RAT measurement, the UE is configured by the network with [MeasuringoneCCperFR2band], the UE makes correct reporting of an event. This test will partly verify the SA inter-RAT cell search requirements in clause 9.4.8.3 and 9.4.8.4 based on the enhanced CSSFoutside,gap in clause 9.1.5.1 by measuring one serving carrier per FR2 band. Supported test configurations are shown in table A.7.6.x.x.1-1.

Table A.7.6.1.x.x-1: SA event triggered reporting tests without SSB index reading for FR2

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations. |  |

In this test, there are three cells: NR cell 1 as PCell in FR2 on RF channel 1, NR cell 2 as SCell in FR2 on RF channel 2, which is within the same band with RF channel 1, and E-UTRA cell 3 as neighbour cell in FR1 on RF channel 3. The SSB of cell 3 is completely within UE’s active BWP BW. The PRBs containing SSB from cell 1 and cell 3 should be different in frequency location within the cell bandwidth. The test parameters for the cell 1 and cell 2 are given in table A.7.6.x.x.1-2, A.7.6.x.x.1-3 and A.7.6.x.x.1-4 below. The test parameters for cell 3 are given in tables A.7.6.x.x.1-5, A.7.6.x.x.1-6 and A.7.6.x.x.1-7.

The cell specific test parameters for E-UTRA Cell 1 as PCell are defined in clause A.3.7.2.2.

In the measurement control information, measurement objects are configured for the frequency of the PCell and SCells respectively, and it is indicated to the UE that event-triggered reporting with Event A3 is used.

The test consists of two successive time periods, with time duration of T1, and T2 respectively. At the beginning of T1 the UE receives an RRC message by which the SCells (Cell 2 and Cell 3) becomes configured on NR. UE is also indicated to perform enhanced measurement by measuring one serving CC per band. During time duration T1, the UE shall have no timing information of Cell 2 and Cell 3.

Table A.7.6.x.x.1-2: General test parameters for intra-frequency event triggered reporting for SA with TDD PCell in FR2 without gap without DRX

| Parameter | Unit | Config | Value | Comment |
| --- | --- | --- | --- | --- |
| Active primary cell |  | 1, 2 | PCell (Cell 1) |  |
| Active secondary cell |  | 1, 2 | SCell (Cell 2) |  |
| Neighbour cell |  | 1, 2 | E-UTRA Cell 3 | Neighbour cell to be identified. |
| RF Channel Number |  | 1, 2 | 1: Cell 1 2: Cell 2 3: Cell 3 | NR Cell 1 is on RF channel number 1. NR Cell 2 is on channel number 2, which is within the same band with RF channel 1 in FR2. E-UTRA Cell 3 is on RF channel 3. Which is in FR 1. |
| SMTC configuration |  | 1, 2 | SMTC.1 |  |
| A3-Offset | dB | 1, 2 | -11 |  |
| CP length |  | 1, 2 | Normal |  |
| Hysteresis | dB | 1, 2 | 0 |  |
| Time To Trigger | s | 1, 2 | 0 |  |
| Filter coefficient |  | 1, 2 | 0 | L3 filtering is not used |
| DRX |  | 1, 2 | OFF |  |
| Time offset between serving and neighbour |  | 1, 2 | 3 s | Synchronous cells |
| T1 | s | 1, 2 | 5 |  |
| T2 | s | 1, 2 | 5 |  |

Table A.7.6.x.x.1-3: Cell specific test parameters for intra-frequency event triggered reporting for SA with TDD PCell in FR2 without gap without DRX

| Parameter | Unit | Config | Cell 1, Cell 2 |  | Cell3 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1, 2 | TDDConf.3.1 |  | TDDConf.3.1 |  |
| BWchannel | MHz | 1, 2 | 100: NPRB,c = 66 |  | 100: NPRB,c = 66 |  |
| Data PRBs allocated |  | 1 | 24 |  | 24 |  |
|  |  | 2 | 48 |  | 48 |  |
| Intial BWP configuration |  | 1, 2 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2 | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2 | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2 | SSB |  | SSB |  |
| PDSCH RMC configuration |  | 1 | SR.3.2 TDD |  | N/A |  |
|  |  | 2 | SR.3.3 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.3.1 TDD |  | N/A |  |
|  |  | 2 | CR.3.2 TDD |  | N/A |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.3.1 TDD |  | N/A |  |
|  |  | 2 | CCR.3.7 TDD |  | N/A |  |
| TRS configuration |  | 1, 2 | TRS.2.1 TDD |  | N/A |  |
| PDSCH/PDCCH TCI states |  | 1, 2 | TCI.State.2 |  | N/A |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 1, 2 | 120 |  | 120 |  |
| OCNG Patterns |  | 1, 2 | OP.5 |  | N/A |  |
| cellIndividualOffset | dB | 1~2 | N/A |  | 16 |  |
| SSB |  | 1 | SSB.1 FR2 |  | SSB.7 FR2 |  |
|  |  | 2 | SSB.2 FR2 |  | SSB.8 FR2 |  |
| Propagation Condition |  | 1, 2 | No external noise (Note 1) |  | No external noise (Note 1) |  |
| NOTE 1: The downlink connection between the System Simulator and the UE is without Additive White Gaussian Noise, and has no fading or multipath effects as specified in TS 38.521-2 B.0 [40]. |  |  |  |  |  |  |

Table A.7.6.x.x.1-4: NR OTA Cell specific test parameters for intra-frequency event triggered reporting for SA with TDD PCell in FR2 without gap without DRX

| Parameter | Unit | Config | Cell 1 |  | Cell 2/Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 |  | T2 |
| AoA setup |  | 1, 2 | Setup 3 defined in A.3.15.3 |  |  |  |  |
|  |  |  | AoA1 |  | AoA2 |  |  |
| Beam assumptionNote 4 |  | 1,2 | Rough |  | Rough |  |  |
| Es | dBm/SCS | 1 | -89 | -89 |  | -Infinity | -89 |
|  |  | 2 | -86 | -86 |  | -Infinity | -86 |
| BB Note 5 | dB | 1, 2 | -0.12 | -0.12 |  | -Infinity | -0.12 |
| SSB_RP | dBm/SCS | 1 | -89 | -89 | -Infinity |  | -89 |
|  |  | 2 | -86 | -86 | -Infinity |  | -86 |
|  | dBm/95.04 MHz | 1 | -64.41 | -64.41 | -Infinity |  | -64.41 |
|  |  | 2 | -61.41 | -61.41 | -Infinity |  | -61.41 |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Void NOTE 3: Es/Iot, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: Information about types of UE beam is given in clause B.2.1.3, and does not limit UE implementation or test system implementation NOTE 5: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1 dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |  |  |  |

##### A.7.6.x.x.2	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X ms from the beginning of time period T2, where X is

5120 for UE supporting power class 1, or

3200 for UE supporting other power class.

The UE is not required to report SSB time index. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90 %.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

<End of Change #1>