3GPP TSG-RAN WG4 Meeting #116-bis	R4-2514045

Prague, Czech Republic, 13rd – 17th October 2025

Source:	Nokia

Title:	Discussion on system parameter and UE RF aspects of 6 MHz channel bandwidth

Agenda item:	7.1.5.2

Document for:	Approval

Introduction

RAN#109 has agreed the WI on UE RF enhancements for NR FR1 Phase 5, where a new channel bandwidth 6 MHz is proposed for NR bands n5, n8, n12, n29, and n85 [1]. The objectives of the core part of this WI for the introduction of 6 MHz are quoted in the following.

6MHz channel bandwidth - Specify the generic core requirements to support 6 MHz NR FR 1 channel bandwidth [RAN 4] - Only include 15 kHz SCS - Specify the spectrum utilization - Specify UE RF requirements for UL and DL - Specify BS RF requirements (OTA and conductive) for UL and DL - Specify the band specific requirements to introduce 6 MHz for NR band n5, n8, n12, n85, n29 [RAN 4] - BS and UE Channel bandwidth per operating band - UE reference sensitivity - When needed - Additional Maximum Power Reduction (A-MPR) - Once RAN 4 has introduced the new channel bandwidth, RAN 2 will introduce the necessary signalling for this channel bandwidth [RAN 2] - Note This should be similar to the signalling for 7 MHz channel bandwidth - Once RAN 4 has introduced the new channel bandwidth, RAN 3 will introduce the necessary signalling for this channel bandwidth [RAN 3] - Note This should be similar to the signalling for 7 MHz channel bandwidth NOTE: Once this WI is completed, the 6 MHz bandwidth can be introduced also for other bands (not addressed in this WI) via the appropriate basket WI. The support of 6MHz bandwidth is release independent.

In this contribution, we discuss the system parameters and UE RF aspects of introducing a channel bandwidth of 6 MHz.

System parameters

Spectrum utilization

The new channel bandwidth of 6 MHz only includes 15 kHz SCS. For the existing 5 MHz and 7 MHz channel bandwidth, the maximum transmission bandwidth configuration is 25 and 35 RBs, respectively, as defined in Table 5.3.2-1 of TS 38.101-1. Both of the maximum transmission bandwidth configurations correspond to 90% spectrum utilization.

It is proposed to assume 30 RBs (with 15 kHz SCS only) for the maximum transmission bandwidth configuration of 6 MHz channel bandwidth.

Minimum guardband

The minimum guardband is defined in Table 5.3.3-1 of TS 38.101-1. As explained in TR 38.817-01, this is based on the formula (CBW-NRB*SCS*12-SCS)/2 for all the existing channel bandwidths, where CBW is channel bandwidth, NRB is the maximum transmission bandwidth configuration and SCS is the subcarrier spacing.

With this formula applied to NRB=30 for 6 MHz, the minimum guardband is calculated to 292.5 kHz.

The minimum guardband for 6 MHz is 292.5 kHz.

Channel arrangement

Channel spacing, channel raster (including the enhanced channel raster) and sync raster are specified in 5.4.1, 5.4.2 and 5.4.3 of TS 38.101-1. The synchronization raster for above 3 MHz channel bandwidth will support any channel raster locations for 6 MHz channel bandwidth.

Channel spacing, channel raster and sync raster are not affected by the introduction of 6 MHz channel bandwidth.

UE RF

Minimum output power

It is proposed to assume the same minimum output power for 6 MHz as 5 MHz and 7 MHz.

Table 6.3.1-1: Minimum output power

| Channel bandwidth | (MHz) | 3,5,6,7,10,15,20 | 25,30,35,40,45,50 | 60,70,80,90,100 |
| --- | --- | --- | --- | --- |
| REF_SCS | (kHz) | 15 |  | 30 |
| Minimum output power | (dBm) | -40 | -40+10log10 (BWChannel /20) | -40+10log10 (BWChannel /20) |
| Measurement bandwidth | (MHz) | MBW=REF_SCS*(12*NRB+1)/1000 |  |  |
| NOTE: The minimum output power value is rounded to the nearest number down to one decimal point. |  |  |  |  |

The minimum output power for 6 MHz shall be -40 dBm.

Transmit OFF power

It is proposed to assume the same transmit off power for 6 MHz as 5 MHz and 7 MHz.

Table 6.3.2-1: Transmit OFF power

| Channel bandwidth | (MHz) | 3,5,6,7,10,15,20,25,30,35,40,45,50 | 60,70,80,90,100 |
| --- | --- | --- | --- |
| REF_SCS | (kHz) | 15 | 30 |
| Transmit OFF power | (dBm) | -50 |  |
| Measurement bandwidth | (MHz) | MBW=REF_SCS*(12*NRB+1)/1000 |  |
| NOTE : “NRB” in the formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1. |  |  |  |

Transmit OFF power for 6 MHz shall be -50 dBm.

Occupied bandwidth

It is proposed to assume the same occupied channel bandwidth framework for 6 MHz as 5 MHz and 7 MHz.

Table 6.5.1-1: Occupied channel bandwidth

|  | NR channel bandwidth (MHz) |
| --- | --- |
|  | 3, 5, 6, 7, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| Occupied channel bandwidth (MHz) | Same as NR channel bandwidth |

Occupied bandwidth for 6 MHz channel bandwidth is the same as the channel bandwidth.

Spectrum emission mask

It is proposed to assume the same emission mask framework for 6 MHz as 7 MHz.

Table 6.5.2.2-1: General NR spectrum emission mask

| ΔfOOB (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) |  |  |  | Measurement bandwidth |
| --- | --- | --- | --- | --- | --- |
|  | 3 | 5 | 6, 7, 10, 15, 20, 25, 30, 35, 40, 45 | 50, 60, 70, 80, 90, 100 |  |
| ± 0-1 | -13 | -13 | -13 |  | 1 % of channel BW |
| ± 0-1 |  |  |  | -24 | 30 kHz |
| ± 1-5 | -10 | -10 | -10 |  | 1 MHz |
| ± 5-6 | -25 | -13 |  |  |  |
| ± 6-10 |  | -25 |  |  |  |
| ± 5-BWChannel |  |  | -13 |  |  |
| ± BWChannel-(BWChannel+5) |  |  | -25 |  |  |

Spectrum emission mask for 6 MHz shall be based on the same formula as for 7 MHz.

ACLR

NR ACLR and UTRA ACLR are specified in 6.5.2.4.1 and 6.5.2.4.2 of TS 38.101-1. For 6 MHz channel bandwidth, the same ACLR levels (in Table 6.5.2.4.1-2 and 6.5.2.4.2-1 of TS 38.101-1) shall be assumed to support the adjacent NR and UTRA channel coexistence scenario in the same way as other existing channel bandwidths. The relaxation of ACLR shall not be considered to relax the channel filter requirement for 6 MHz.

The measurement shall be based on the same formula as other channel bandwidth specified in Table 6.6.2.4.1-1.

Table 6.5.2.4.1-1: NR ACLR measurement bandwidth

| Channel bandwidth | (MHz) | 3,5,6,7,10,15,20,25,30,35,40,45,50 | 60,70,80,90,100 |
| --- | --- | --- | --- |
| REF_SCS | (kHz) | 15 | 30 |
| NR ACLR measurement bandwidth | (MHz) | MBW=REF_SCS*(12*NRB+1)/1000 |  |
| NOTE : “NRB” in the formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1. |  |  |  |

Table 6.5.2.4.1-2: NR ACLR requirement

|  | Power class 1 | Power class 1.5 | Power class 2 | Power class 3 |
| --- | --- | --- | --- | --- |
| NR ACLR | 37 dB | 31 dB | 31 dB | 30 dB |
| NOTE 1: Void |  |  |  |  |

The same NR and UTRA ACLR level for existing channel bandwidth shall be assumed for 6 MHz.

Spurious emissions

General spurious emissions are specified in 6.5.3.1 of TS 38.101-1. The spurious emissions limits shall follow Table 6.5.3.1-2. The formula for OOB boundary for 7 MHz channel bandwidth in Table 6.5.3.1-1 shall be the same as the existing channel bandwidth 5 MHz and wider channel bandwidth, i.e., FooB=BW_channel + 5 (MHz).

Table 6.5.3.1-1: Boundary between NR out of band and general spurious emission domain

| Channel bandwidth | OOB boundary FOOB (MHz) |
| --- | --- |
| 3 | 6 |
| 5, 6, 7, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | BWChannel + 5 |

Table 6.5.3.1-2: Requirement for general spurious emissions limits

| Frequency Range | Maximum Level | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
| 9 kHz ≤ f < 150 kHz | -36 dBm | 1 kHz |  |
| 150 kHz ≤ f < 30 MHz | -36 dBm | 10 kHz |  |
| 30 MHz ≤ f < 1000 MHz | -36 dBm | 100 kHz |  |
| 1 GHz ≤ f < 12.75 GHz | -30 dBm | 1 MHz | 4 |
|  | -25 dBm | 1 MHz | 3 |
| 12.75 GHz ≤ f < 5th harmonic of the upper frequency edge of the UL operating band in GHz | -30 dBm | 1 MHz | 1 |
| 12.75 GHz < f < 26 GHz | -30 dBm | 1 MHz | 2 |
| NOTE 1: Applies for Band for which the upper frequency edge of the UL Band is greater than 2.55 GHz and less than or equal to 5.2 GHz NOTE 2: Applies for Band that the upper frequency edge of the UL Band more than 5.2 GHz NOTE 3: Applies for Band n41, CA configurations including Band n41, and EN-DC configurations that include n41 specified in clause 5.2B of TS 38.101-3 [3] when NS_04 is signalled. NOTE 4: Does not apply for Band n41, CA configurations including Band n41, and EN-DC configurations that include n41 specified in subclause 5.2B of TS 38.101-3 [3] when NS_04 is signalled. |  |  |  |

Spurious emissions for 6 MHz channel bandwidth shall follow the formula for 5 MHz and wider channel bandwidths.

A-MPR

NS_06 for band n12 and n85 are specified to meet the spectrum emission mask requirement to comply with FCC rules in the US, where out of band emissions are limited by 43+log(P) dB. 30 kHz resolution bandwidth is applied immediately outside 100 kHz of the channel. 100 kHz or greater resolution bandwidth is applied otherwise.

For 6 MHz channel bandwidth the following mask is proposed to align with the proposed general mask above in section 3.4 so that -15 dBm/30 kHz up to 100 kHz, -13 dBm/100 kHz for 100 kHz to 1 MHz, -13 dBm/MHz for 1 MHz to 6 MHz and -25 dBm/MHz for 6 MHz to 11 MHz should be applied.

Table 6.5.2.3.4-1: Additional requirements for "NS_06" or "NS_07"

| ΔfOOB (MHz) |  | Channel bandwidth (MHz) / Spectrum emission limit (dBm) |  |  |  | Measurement bandwidth |
| --- | --- | --- | --- | --- | --- | --- |
|  | 3 | 5 | 6 | 10 | 15 |  |
| ± 0 – 0.1 | -13 | -15 | -15 | -18 | -20 | 30 kHz |
| ± 0.1 – 1 | -13 | -13 | -13 | -13 | -13 | 100 kHz |
| ± 1 – 5 | -13 | -13 | -13 | -13 | -13 | 1 MHz |
| ± 5 – 6 | -25 | -13 | -13 | -13 | -13 | 1 MHz |
| ± 6 – 10 |  | -25 | -25 |  |  |  |
| ± 10 – 11 |  |  |  | -25 |  |  |
| ± 11 – 15 |  |  |  |  |  |  |
| ± 15 – 20 |  |  |  |  | -25 |  |

NS_14 and NS_15 for band n5 are specified in case the protection of public safety narrow band systems are required.

NS_43 and NS_43U for band n8 are specified for the coexistence of NR band n8 with NR bands n18 and n5 (or E-UTRA bands 18 and 19) in Japanese regulations. Only 5, 10 or 15 MHz channel bandwidths are expected in Japan, thus A-MPR study for NS_43 and NS_43U is not required for band n8.

NS_06, NS_14, and NS_15 A-MPR for 6 MHz channel bandwidth shall be further evaluated for NR bands n5, n12, and n85.

NS_06 spectrum emission mask for 6 MHz is defined as -15 dBm/30 kHz up to 100 kHz, -13 dBm/100 kHz for 100 kHz to 1 MHz, -13 dBm/MHz for 1 MHz to 6 MHz and -25 dBm/MHz for 6 MHz to 11 MHz.

No A-MPR study for NS_43 and NS_43U is needed for band n8.

Furthermore, it is proposed to study whether any restriction to the channel location of 6 MHz is applied for A-MPR simulation for each band. Such information is useful to optimize the A-MPR values for actual deployment scenarios.

It is proposed to study whether any restriction to the channel location of 6 MHz is applied for A-MPR simulation for each band.

Reference sensitivity

The reference sensitivity for 6 MHz is proposed to be scaled from 5 MHz assuming UL allocation aligned with the configuration of 5 MHz in the following tables.

The reference sensitivity for 6 MHz is scaled from 5 MHz assuming UL allocation aligned with the configuration of 5 MHz.

Table 7.3.2-1a: Two antenna port reference sensitivity QPSK PREFSENS for FDD bands

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 3 MHz (dBm) | 5 MHz (dBm) | 6 MHz (dBm) | 7 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
| n5 | 15 | -100.2 | -98.0 | -97.2 | -96.5 | -94.8 | -93.0 | -86.8 | -84.8 |  |  |  |  |  |
|  | 30 |  |  |  |  | -95.1 | -93.1 | -88.6 | -84.9 |  |  |  |  |  |
| n8 | 15 |  | -97.0 | -96.2 |  | -93.8 | -91.4 | -85.8 | -83.6 | -81.3 | -78.4 |  |  |  |
|  | 30 |  |  |  |  | -94.1 | -91.7 | -87.2 | -84.7 | -81.4 | -78.5 |  |  |  |
| n12 | 15 | -99.2 | -97.0 | -96.2 |  | -93.8 | -84.0 |  |  |  |  |  |  |  |
|  | 30 |  |  |  |  | -94.1 | -84.1 |  |  |  |  |  |  |  |
| n85 | 15 | -99.2 | -97.0 | -96.2 |  | -93.8 | -84.0 |  |  |  |  |  |  |  |
|  | 30 |  |  |  |  | -94.1 | -84.1 |  |  |  |  |  |  |  |

Table 7.3.2-1b: Two antenna port reference sensitivity QPSK PREFSENS for TDD, SDL and FDD with variable duplex operation bands

| Operating band / SCS / Channel bandwidth / REFSENS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operating band | SCS kHz | Channel bandwidth (MHz) | REFSENS (dBm)8 | Duplex Mode |
| n297 | 15 | 5,6,10 | -97 + 10log10(NRB/25) | SDL |
|  | 30 | 10 | -94.1 + 10log10(NRB/24) |  |
| NOTE 7: For SDL bands, the reference sensitivity requirements shall be verified by inter-band CA combinations with SDL band, which are supported by UE. |  |  |  |  |

Table 7.3.2-1c Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE not supporting Tx Diversity

| Operating Band | 3 MHz (dB) | 5 MHz (dB) | 6 MHz (dB) | 7 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n5 | 0.5 | 0.5 | 0.5 | 0.6 | 0.7 | 0.8 | 1.5 | 2.6 | 2.8 |  |  |  |  |
| n8 |  | 0.5 | 0.5 |  | 0.7 | 0.8 | 2.3 | 2.8 | 3.2 | 3.1 |  |  |  |
| n85 | 0.6 | 0.6 | 0.6 |  | 1.0 | 2.6 |  |  |  |  |  |  |  |

Table 7.3.2-1d Reference Sensitivity Degradation from PC3 to PC2 for
FDD bands for UE supporting Tx Diversity

| Operating Band | 3 MHz (dB) | 5 MHz (dB) | 6 MHz (dB) | 7 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n5 | 1.1 | 1.1 | 1.1 | 1.1 | 1.2 | 1.4 | 4.5 | 6.1 | 6.5 |  |  |  |  |
| n8 |  | 1.3 | 1.3 |  | 1.4 | 2.1 | 5.8 | 6.1 | 6.5 | 7.0 |  |  |  |
| n85 | 1.2 | 1.2 | 1.2 |  | 1.4 | 6.4 |  |  |  |  |  |  |  |

Table 7.3.2-3: Uplink configuration for reference sensitivity

| Operating band / SCS (kHz) / Channel bandwidth (MHz) / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS | 3 | 5 | 6 | 7 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 | Duplex Mode |
| n5 | 15 | 15 | 25 | 251 | 251 | 251 | 201 | 201 | Note 5 |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  |  |  | 121 | 101 | 101 | Note 5 |  |  |  |  |  |  |  |  |  |  |  |
| n8 | 15 |  | 25 | 251 |  | 251 | 201 | 201 | Note 5 | Note 5 | Note 5 |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  |  |  | 121 | 101 | 101 | Note 5 | Note 5 | Note 5 |  |  |  |  |  |  |  |  |  |
| n12 | 15 | 15 | 201 | 201 |  | 201 | 201 |  |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  |  |  | 101 | 101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n85 | 15 | 15 | 201 | 201 |  | 201 | 201 |  |  |  |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  |  |  |  | 101 | 101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Maximum input level

It is proposed to assume the same maximum input level for 6 MHz as 5 MHz and 7 MHz.

Table 7.4-1: Maximum input level

| Rx Parameter | Units | Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 3, 5, 6, 7,10, 15, 20 | 25, 30, 35, 40, 45, 50 | 60, 70, 80, 90, 100 |
| Power in Transmission Bandwidth Configuration4 | dBm | -252 | -25 + 10log10(BWChannel /20)Note 2 | -202 |
|  |  | -273,5 | -27 + 10log10(BWChannel /20)Note 3,5 | -223,5 |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 and 7.3F.2-3 for shared spectrum channel access operation with PCMAX_L,f,c as defined in clause 6.2.4. NOTE 2: Reference measurement channel is A.3.2.3 or A.3.3.3 for 64 QAM. NOTE 3: Reference measurement channel is A.3.2.4 or A.3.3.4 for 256 QAM. NOTE 4: 10log10(x) is rounded to the nearest 0.5dB value. NOTE 5: Reference measurement channel is A.3.2.5 or A.3.3.5 for 1024 QAM. |  |  |  |  |

Maximum input level for 6 MHz channel bandwidth is the same as 5 MHz and 7 MHz.

ACS

NR ACS are specified in 7.5 of TS 38.101-1. For 6 MHz channel bandwidth, the same ACS levels (in Table 7.5-1, 7.5-3 and 7.5-4 of TS 38.101-1) shall be assumed to support the adjacent channel coexistence scenario in the same way as other existing channel bandwidths 5 MHz and 10 MHz. The relaxation of ACS shall not be considered to relax the channel filter requirement for 7 MHz.

Table 7.5-1: ACS for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| RX parameter | Units | Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 3, 5, 6, 7, 10 | 15 | 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| ACS | dB | 33 | 30 | 27 – 10log10(BWChannel /20) |
| NOTE1: ACS value is rounded to the next higher 0.5dB value |  |  |  |  |

Table 7.5-3: Test parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz, case 1

| RX parameter | Units | Channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5, 6, 7, 10 | 15 | 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| Power in transmission bandwidth configuration | dBm | REFSENS + 14 dB |  |  |  |
| Pinterferer4 | dBm |  | REFSENS + 45.5 dB | REFSENS + 42.5 dB | REFSENS + 39.5 – 10log10(BWChannel /20) |
| BWinterferer | MHz | 3 | 5 |  |  |
| Finterferer (offset) | MHz | 3 /- 3 | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 3: The interferer consists of the NR interferer RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. NOTE 4: 10log10(x) is rounded to the next higher 0.5dB value. |  |  |  |  |  |

Table 7.5-4: Test parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz, case 2

| RX parameter | Units | Channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5, 6, 7, 10 | 15 | 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| Power in transmission bandwidth configuration4 | dBm | -56.5 |  | -53.5 | -50.5 + 10log10(BWChannel /20) |
| Pinterferer | dBm | -25 |  |  |  |
| BWinterferer | MHz | 3 | 5 |  |  |
| Finterferer (offset) | MHz | 3 /- 3 | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |  |  |
| NOTE 1: The transmitter shall be set to 24 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 NOTE 4: 10log10(x) is rounded to the next higher 0.5dB value. |  |  |  |  |  |

For 6 MHz channel bandwidth, the same ACS level for 5 MHz and 7 MHz shall be assumed.

In-band blocking

It is proposed to assume the same in-band blocking level for 6 MHz as 5 MHz and 7 MHz.

Table 7.6.2-1: In-band blocking parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| RX parameter | Units | Channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5, 6, 7, 10 | 15 | 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| Power in transmission bandwidth configuration3 | dBm | REFSENS + 6 dB |  | REFSENS + 7 dB | REFSENS + (9 + 10log10(BWChannel /20)) dB |
| BWinterferer | MHz | 3 | 5 |  |  |
| FIoffset, case 1 | MHz | 4.5 | 7.5 |  |  |
| FIoffset, case 2 | MHz | 7.5 | 12.5 |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 and 15 kHz SCS. NOTE 3: 10log10(x) is rounded to the next higher 0.5dB value. |  |  |  |  |  |

It is proposed to assume the same in-band blocking level for 6 MHz as 5 MHz and 7 MHz.

Out-of-band blocking

It is proposed to assume the same out-of-band blocking level for 6 MHz as 5 MHz and 7 MHz.

Table 7.6.3-1: Out-of-band blocking parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| Channel bandwidth | Power in transmission bandwidth configuration [dBm] |
| --- | --- |
| 3 MHz | REFSENS + 6.0 dB |
| 5 MHz | REFSENS + 6.0 dB |
| 6 MHz | REFSENS + 6.0 dB |
| 7 MHz | REFSENS + 6.0 dB |
| 10 MHz | REFSENS + 6.0 dB |
| 15 MHz | REFSENS + 7.0 dB |
| 20 MHz | REFSENS + 9.0 dB |
| 25 MHz | REFSENS + 10.0 dB |
| 30 MHz | REFSENS + 11.0 dB |
| 35 MHz | REFSENS + 11.5 dB |
| 40 MHz | REFSENS + 12.0 dB |
| 45 MHz | REFSENS + 12.5 dB |
| 50 MHz | REFSENS + 13.0 dB |
| 60 MHz | REFSENS + 14.0 dB |
| 70 MHz | REFSENS + 14.5 dB |
| 80 MHz | REFSENS + 15.0 dB |
| 90 MHz | REFSENS + 15.5 dB |
| 100 MHz | REFSENS + 16.0 dB |
| NOTE: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. |  |

It is proposed to assume the same out-of-band blocking level for 6 MHz as 5 MHz and 7 MHz.

Narrow band blocking

It is proposed to scale the narrow band blocking level for 6MHz from 5 MHz and 7 MHz. Pw=14.5 dB for 6 MHz shall be assumed.

Table 7.6.4-1: Narrow Band Blocking

| NR band |  | Parameter | Unit | Channel Bandwidth (MHz) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 3 | 5 | 6 | 7 | 10 | 15 | 20 | 25, 30, 35, 40, 45, 50 | 60, 70, 80, 90, 100 |
| n1, n2, n3, n5, n7, n8, n12, n13, n14, n18, n20, n24, n25, n26, n28, n29, n30, n31, n34, n38, n39, n40, n41, n48, n50, n51, n53, n65, n66, n67, n68, n70, n71, n72, n74, n75, n76, n85, n87, n88, n100, n101, n106, n110 |  | Pw | dBm | PREFSENS + channel-bandwidth specific value below |  |  |  |  |  |  |  |  |
|  |  |  | dB | 18 | 16 | 15 | 14 | 13 | 14 | 16 | 16 | 16 |
|  |  | Puw (CW) | dBm | -55 |  |  |  |  |  |  |  |  |
|  |  | Fuw (offset SCS= 15 kHz) 4 | MHz | BWChannel2+0.2SCS+0.5+0.5SCS |  |  |  |  |  |  | BWChannel2+BWGB,ChannelSCS+0.5+0.5SCS | NA |
|  |  | Fuw (offset SCS= 30 kHz)4 | MHz | NA |  |  |  |  |  |  |  | BWChannel2+BWGB,ChannelSCS+0.5+0.5SCS |
|  | NOTE 1: The transmitter shall be set a 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4 NOTE 2: Reference measurement channel is specified in Annexes A.3.2 and A.3.3 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. NOTE 3: The PREFSENS power level is specified in Table 7.3.2-1a, Table 7.3.2-1b, Table 7.3.2-2 and Table 7.3.2-2a for two, four and eight antenna ports, respectively. NOTE 4: Fuw shall be rounded to half of SCS. NOTE 5: For SDL bands, requirements shall be applied only for CA band combination cases. |  |  |  |  |  |  |  |  |  |  |  |

It is proposed to scale the narrow band blocking level for 6 MHz from 5 MHz and 7 MHz. Pw=15 dB for 6 MHz shall be assumed.

Spurious response

It is proposed to assume the spurious response level for 6 MHz as 5 MHz and 7 MHz.

Table 7.7-1: Spurious response parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| RX parameter | Units | Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 3, 5, 6, 7, 10 | 15 | 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| Power in transmission bandwidth configuration2 | dBm | REFSENS + 6 dB | REFSENS + 7 dB | REFSENS + (9 + 10log10(BWChannel /20)) dB |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: 10log10(x) is rounded to the next higher 0.5dB value. |  |  |  |  |

It is proposed to assume the same spurious response level for 6 MHz as 5 MHz and 7 MHz.

Wide band intermodulation

It is proposed to assume the same wide band intermodulation level for 6 MHz as 5 MHz and 7 MHz.

Table 7.8.2-1: Wide band intermodulation parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| Rx parameter | Units | Channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5, 6, 7, 10 | 15 | 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| Pw in Transmission Bandwidth Configuration, per CC5 | dBm | REFSENS + 8 dB | REFSENS + 6 dB | REFSENS + 7 dB | REFSENS + (9 + 10log10(BWChannel /20)) dB |
| PInterferer 1 (CW) | dBm | -46 |  |  |  |
| PInterferer 2 (Modulated) | dBm | -46 |  |  |  |
| BWInterferer 2 | MHz | 3 | 5 |  |  |
| FInterferer 1 (Offset) | MHz | -BW/2 – 4.5 /+BW/2 + 4.5 | -BWChannel/2 – 7.5 / +BWChannel/2 + 7.5 |  |  |
| FInterferer 2 (Offset) | MHz | 2*FInterferer 1 |  |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: Reference measurement channel is specified in Annexes A.2.2, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1). NOTE 3: The modulated interferer consists of the Reference measurement channel specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 and 15 kHz SCS. NOTE 4: The Finterferer 1 (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the CW interferer and Finterferer 2 (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the modulated interferer. NOTE 5: 10log10(x) is rounded to the next higher 0.5dB value. |  |  |  |  |  |

It is proposed to assume the same wide band intermodulation level for 6 MHz as 5 MHz and 7 MHz.

Conclusion

The following Observations and Proposals were made:

Proposal 1: It is proposed to assume 30 RBs (with 15 kHz SCS only) for the maximum transmission bandwidth configuration of 6 MHz channel bandwidth.

Proposal 2: The minimum guardband for 6 MHz is 292.5 kHz.

Proposal 3: Channel spacing, channel raster and sync raster are not affected by the introduction of 6 MHz channel bandwidth.

Proposal 4: The minimum output power for 6 MHz shall be -40 dBm.

Proposal 5: Transmit OFF power for 6 MHz shall be -50 dBm.

Proposal 6: Occupied bandwidth for 6 MHz channel bandwidth is the same as the channel bandwidth.

Proposal 7: Spectrum emission mask for 6 MHz shall be based on the same formula as for 7 MHz.

Proposal 8: The same NR and UTRA ACLR level for existing channel bandwidth shall be assumed for 6 MHz.

Proposal 9: Spurious emissions for 6 MHz channel bandwidth shall follow the formula for 5 MHz and wider channel bandwidths.

Proposal 10: NS_06, NS_14, and NS_15 A-MPR for 6 MHz channel bandwidth shall be further evaluated for NR bands n5, n12, and n85.

Proposal 11: NS_06 spectrum emission mask for 6 MHz is defined as -15 dBm/30 kHz up to 100 kHz, -13 dBm/100 kHz for 100 kHz to 1 MHz, -13 dBm/MHz for 1 MHz to 6 MHz and -25 dBm/MHz for 6 MHz to 11 MHz.

Proposal 12: No A-MPR study for NS_43 and NS_43U is needed for band n8.

Proposal 13: It is proposed to study whether any restriction to the channel location of 6 MHz is applied for A-MPR simulation for each band.

Proposal 14: The reference sensitivity for 6 MHz is scaled from 5 MHz assuming UL allocation aligned with the configuration of 5 MHz.

Proposal 15: Maximum input level for 6 MHz channel bandwidth is the same as 5 MHz and 7 MHz.

Proposal 16: For 6 MHz channel bandwidth, the same ACS level for 5 MHz and 7 MHz shall be assumed.

Proposal 17: It is proposed to assume the same in-band blocking level for 6 MHz as 5 MHz and 7 MHz.

Proposal 18: It is proposed to assume the same out-of-band blocking level for 6 MHz as 5 MHz and 7 MHz.

Proposal 19: It is proposed to scale the narrow band blocking level for 6 MHz from 5 MHz and 7 MHz. Pw=15 dB for 6 MHz shall be assumed.

Proposal 20: It is proposed to assume the same spurious response level for 6 MHz as 5 MHz and 7 MHz.

Proposal 21: It is proposed to assume the same wide band intermodulation level for 6 MHz as 5 MHz and 7 MHz.

- References

- RP‑252952 New WID: UE RF enhancements for NR FR1 Phase 5, vivo, RAN#109