3GPP TSG RAN WG4 Meeting #116-bis	R4-2514076

Prague, Czech, 13 – 17 October, 2025

Agenda item:			6.3.2

Source:	vivo

Title:	On RRM performance requirements for LB CA via switching

Document for:	Discussion

# Introduction

In the last RAN4 meetings, RAN4 completed RRM core requirements for Rel-19 LB CA via switching. A semi-static switching pattern is configured to UE by RRC reconfiguration. UE will follow the switching pattern to switch between PCell and SDL SCell under two cases.

-	Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2

-	Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1

In this contribution, we provide our views on RRM performance requirements for Rel-19 LB CA via switching.

# Discussion

## 2.1 SCell operation

RRM requirements considering switching pattern are specified for SDL SCell operation, including SCell activation/deactivation, L3 measurements for activated/deactivated SCell, L1 measurements for SCell including BFD/CBD, L1-RSRP, L1-SINR.

The purpose of the test is to verify UE can perform L3/L1 measurements on SDL SCell and activate/deactivate the SCell correctly when switching pattern is configured, and corresponding requirements should be met.

For SCell activation, a test should be introduced to very FR1 inter-band SCell activation delay requirements where UE activates the SCell by following the configured switching pattern.

Proposal 1: Introduce a test to verify SCell activation delay requirements when switching pattern is configured.

For activated SDL SCell L3/L1 measurement, UE performs the SDL SCell measurement on the overlapped occasions of SMTC window/L1-RS (SSB or CSI-RS) and SDL SCell reception duration indicated by switching pattern. It is new UE measurement behaviour and requirements are different from legacy. Tests should be introduced to verify requirements for the L3/L1 measurements on SDL SCell. For L1 measurement, it would be enough to only verify the L1-RSRP measurements.

Proposal 2: Introduce a test to verify L3 measurements on activated SCell when switching pattern is configured.

Proposal 3: Introduce a test to verify L1-RSRP measurement on activated SCell when switching pattern is configured.

For deactivated SCell measurements, legacy delay requirements are reused. There is no need to define new tests. However, the interruption on PCell could be different from legacy one. It is suggested to define a test to verify interruption requirements for deactivated SCell measurements.

Proposal 4: Introduce a test to verify interruption requirements due to L3 measurements on deactivated SCell when switching pattern is configured.

## 2.2 PCell operation

RRM requirements considering switching pattern are specified for PCell operation, including RLM, BFD/CBD, L1-RSRP, L1-SINR, PL-RS, TCI state switching and intra-frequency measurement without gap.

The purpose of the test is to verify UE can perform L3/L1 measurements on FDD PCell and perform necessary procedures correctly when switching pattern is configured, and corresponding requirements should be met. On the other hand, RAN4 should avoid defining multiple tests to verify the same purpose.

Considering the test for SCell operation, following tests for FDD PCell operation are proposed.

Proposal 5: Introduce a test to verify L3 intra-frequency measurement without gap when switching pattern is configured.

Proposal 6: Introduce a test to verify TCI state switching when switching pattern is configured.

Proposal 7: Introduce a test to verify RLM measurement when switching pattern is configured.

# Summary

In this contribution, we provided views on RRM performance requirements for Rel-19 LB-CA via switching. Following proposals and observations are provided.

Proposal 1: Introduce a test to verify SCell activation delay requirements when switching pattern is configured.

Proposal 2: Introduce a test to verify L3 measurements on activated SCell when switching pattern is configured.

Proposal 3: Introduce a test to verify L1-RSRP measurement on activated SCell when switching pattern is configured.

Proposal 4: Introduce a test to verify interruption requirements due to L3 measurements on deactivated SCell when switching pattern is configured.

Proposal 5: Introduce a test to verify L3 intra-frequency measurement without gap when switching pattern is configured.

Proposal 6: Introduce a test to verify TCI state switching when switching pattern is configured.

Proposal 7: Introduce a test to verify RLM measurement when switching pattern is configured.

- References