3GPP TSG RAN WG4 Meeting #116-bis	R4-2514079

Prague, Czech, 13 – 17 October, 2025

Agenda item:			6.17.2.1

Source:	vivo

Title:	On performance requirements for LTM event triggered reporting

Document for:	Discussion

# Introduction

In the last RAN4 meetings, RAN4 completed RRM core requirements for Rel-19 LTM. The core requirements were specified for CSI-RS based L1 measurement, event triggered L1 measurement reporting and conditional LTM.

There were proposals on performance requirements for mobility enhancement on event triggered L1 measurement reporting, which were captured in the topic summary [1].

## Event-triggered L1 reporting #### Issue 4-1-1: test case list for event-triggered L1 reporting Candidate options: - Option 1: Nokia - FR1 and FR2 Intra-frequency SSB-based event-triggered L1 reporting (without measurement gaps) - FR1 and FR2 Inter-frequency SSB-based event-triggered L1 reporting (with and without measurement gaps) - FR1 and FR2 Intra-frequency CSI-RS-based event-triggered L1 reporting (without measurement gaps) - At least 1 test case for event-triggered periodic reporting. - Option 2: Apple - Event-triggered Reporting based on SSB based L1 RSRP measurement in FR1 - Event-triggered Reporting based on CSI-RS based L1 RSRP measurement in FR1 - Event-triggered Reporting based on SSB based L1 RSRP measurement in FR2 - Event-triggered Reporting based on CSI-RS based L1 RSRP measurement in FR2 - Option 3: CATT - The new test cases for event triggered L1 measurement reporting need to defined in the clauses defined for R18 LTM. - TC for LTM Intra-frequency event triggered SSB based L1 measurement reporting tests without gap under non-DRX - TC for LTM Inter-frequency event triggered SSB based L1 measurement reporting tests with gaps under non-DRX - TC for LTM Intra-frequency event triggered SSB based L1 measurement reporting tests without gap under non-DRX with SSB index reading - For event triggered CSI-RS based L1 measurement reporting, only new TC for LTM Intra-frequency event triggered reporting tests without gap under non-DRX should be defined. - Option 4: cover both SSB and CSI-RS based, in FR1 and FR2, repsectively (vivo) - Option 5: follow aspects should be considered (ZTE) - With gap &without gap, intra-f & inter-f - non-DRX & DRX - With SSB index reading & without SSB index reading - Other aspects are not precluded. - Option 6: CMCC - Both FR1 and FR2-1 are considered. - For SSB based event triggered L1 measurement reporting, intra-frequency measurement, inter-frequency measurement without gap, gap-based inter-frequency measurement need to be considered. - For CSI-RS based event triggered L1 measurement reporting, measurement without gap need to be considered. - Option 7: RAN4 to consider the list of test cases agreed in Release 18 to perform event-triggered L1-RSRP testing, thereby defining the same list of test cases for event-triggered L1-RSRP. Adopt the following list of test cases for Release 19 SSB-based event-triggered reporting. (E///) - Intra-f L1-RSRP measurement for LTM - Intra-f L1-RSRP measurement in FR1 - Intra-f L1-RSRP measurement in FR2 - 1 serving cell, 2 neighbor cells, none of neighbor cells’ TCI state activated - Intra-f L1-RSRP measurement in FR2 - 1 serving cell, 2 neighbor cells, one of neighbor cells’ TCI state activated - Inter-f L1-RSRP measurement with MG - Inter-f L1-RSRP measurement with MG in FR1 - Inter-f L1-RSRP measurement with MG in FR2 - Inter-f L1-RSRP measurement without gap for LTM - Inter-f L1-RSRP measurement without gap in FR1 - Inter-f L1-RSRP measurement without gap in FR2 - 2 neighbor cells, none of neighbor cells’ TCI state activated - Option 8: HW - LTM Intra-frequency event triggered reporting tests without gap (SSB based L1-RSRP measurement) - LTM Intra-frequency event triggered reporting tests without gap (CSI-RS based L1-RSRP measurement) - LTM Inter-frequency event triggered reporting tests with gap (SSB based L1-RSRP measurement) - LTM Inter-frequency event triggered reporting tests without gap (SSB based L1-RSRP measurement) - FFS on LTM Inter-frequency event triggered reporting tests for CSI-RS based L1 measurement #### Issue 4-1-2: test configuration for event-triggered L1 reporting Candidate options: - Option 1: On high level, the test cases for event-triggered L1 reporting can have two cells (serving and candidate), and the signal level of one of these cells is to be changed during the test so that the reporting condition becomes met. UE shall send the report within the specified delay after the condition becomes met. (Nokia) #### Issue 4-1-3: others Candidate options: - Option 1: it is proposed to define test cases for LTM event 2/3/4/5 (CMCC) - Option 2: At least Event LTM3 and Event LTM4 be included in the RRM testcase. (E///)

In this contribution, we further provide our views on performance requirements for event triggered L1 measurement reporting.

# Discussion

Event triggered L1 measurement reporting apply to both SSB-based L1 measurement and CSI-RS based L1 measurement. There are no new accuracy requirements needed regarding event triggered L1 measurement reporting.

Test cases

It is necessary to verify whether UE can meet event triggered reporting requirements. Since UE measurement behaviour is verified by other tests, there is no need to define many tests to verify all combination of cases/conditions. In our view, four test cases would be enough to cover SSB-based L1 measurement in FR1 and in FR2, respectively. For CSI-RS-based L1 measurement, two test cases should be enough.

For LTM, following events were introduced.

- Event LTM2: Beam of serving cell becomes worse than absolute threshold;

- Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

- Event LTM4: Beam of candidate cell becomes better than absolute threshold;

- Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

Event LTM3 and event LTM4 can be considered in the test.

Proposal 1: Define following test cases to verify whether UE can meet event triggered reporting requirements, covering SSB-based L1 measurement and CSI-RS based L1 measurement in FR1 and in FR2, respectively.

For SSB-based measurement,

Intra-frequency measurement without gap in FR1 for event LTM3

Inter-frequency measurement with gap in FR1 for event LTM4

Intra-frequency measurement without gap in FR2 for event LTM3

Inter-frequency measurement with gap in FR2 for event LTM4

For CSI-RS-based measurement,

Intra-frequency measurement without gap in FR1 for event LTM3

Intra-frequency measurement without gap in FR1 for event LTM3

- References

- R4-2509066	Topic summary for [116][223] NR_Mob_Ph4_Part1, Moderator (Apple)