3GPP TSG RAN4 Meeting #116bis					    	         R4-2514134

Prague, The Czech Republic, Oct 13-17, 2025

Agenda item:	7.7.3

Source:	Huawei, HiSilicon

Title:	Discussion on UAV co-existence evaluation

Document for:	Discussion

Background

The new WID for enhancement of NR RF and RRM requirements for uncrewed aerial vehicle (UAV) was approved at RAN#109 [1]. The objectives were agreed as below:

The core part of WI:

- Scenario:

  - BSs for UAV service are deployed on the ground by adjusting the tilt angle upwards. The coverage height is up to 600~1000m. The ISD for BSs for UAV service is about 2~4km.

  - A direct radio link between BS on the ground and UAV UE.

- Specify RF requirements for UAV UE to support higher UL output power.

  - Example bands:

    - TDD with same TDD pattern, including n41, n77, n78, n79,

    - FDD: n3

  - Consider UAV UE with omni-direction antenna or with antenna array

  - Consider UAV UE with up to 33dBm max total conductive output power

  - Analyse co-existence for UAV and considering the details relevant to UAV including antenna array model, and specify requirements if needed.

  - Specify UE RF requirements, e.g., maximum output power, configured transmission power, SEM.

- Specify RRM requirements, if needed

Note 1: No RAN1 impact are expected

This document provides our initial thinking on the co-existence evaluation aspect.

Discussion

During discussions in previous RAN4 and RAN meetings, the similarity between UAV and ATG was mentioned by companies. The co-existence was extensively studied for ATG and captured in TR 38.876. The output could be taken as the starting point for study of UAV.

Co-existence scenario

The co-existence simulation of ATG was done for 14 scenarios, including ATG DL <--> TN DL, ATG UL <--> TN UL for synchronized network, and ATG/TN UL --> TN/ATG DL for non-synchronized network. In the Rel-20 UAV WI, the improvements are mainly related to higher UE power class, therefore the study could focus on the scenarios with UAV UL as aggressor.

Proposal 1: The co-existence study focuses on the scenarios with UAV UL as aggressor, and prioritize the synchronized scenario.

Table 1: Scenarios for UAV coexistence study

|  |  | Aggressor |  | Victim |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. | Combination | deployment scenario UL/DL | CBW duplex mode | deployment scenario UL/DL | CBW duplex mode | Simulation frequency | Notes | Study Phase |
| 1 | TN with UAV | UAV UL | 100MHz TDD | TN rural UL | 100MHz TDD | 4 GHz |  | Phase 1 |
| 2 | TN with UAV | UAV UL | 100MHz TDD | TN rural DL | 100MHz TDD | 4 GHz |  | Phase 2 |
| 3 | TN with UAV | UAV UL | 20MHz FDD | TN rural UL | 20MHz FDD | 2 GHz |  | Phase 1 |
| 4 | TN with UAV | UAV UL | 20MHz FDD | TN rural DL | 20MHz TDD | 2 GHz | n1/n39 | Phase 2 |
| NOTE 1: ACLR/ACS requirement for UAV BS and UAV UE are derived based on the synchronized scenario in Phase 1. NOTE 2: the non-synchronized coexistence scenarios listed in phase 2 are mainly used to identify the isolation distance between UAV BS and the legacy TN BSs. And it’s agreed to reuse 45dB ACLR and 46dB ACS for both TN and UAV BS for non-synchronized scenarios to derive isolated distance. NOTE 3: For scenario 2 and 4 which is CLI between UAV UE and TN UE, it’s not expected to have further coexistence simulation evaluation in Phase 2 due to low interference level. |  |  |  |  |  |  |  |  |

Vertical coverage angel range

The UAV WI assumes the UAV BSs are deployed on the ground by adjusting the tilt angle upwards, which is similar as ATG. The vertical coverage is summarized in Table 2 for UAV and ATG.

Table 2: Vertical coverage of UAV and ATG

|  | UAV | ATG |
| --- | --- | --- |
| Coverage height | Up to 600~1000m | Between 3000m and 10000m |
| ISD for BSs for UAV | About 2~4km | 20~100km for non sub-array 50~100km for sub-arrays |

Below figure compares the vertical coverage angel range of UAV and ATG. It shows the vertical coverage range of UAV is inside the coverage range of ATG, therefore many assumptions of ATG could also apply to UAV. Even if the UAV flies at a low altitude around 100m, it’s still within the ATG vertical coverage.

Observation 1: The vertical coverage range of UAV is inside the coverage range of ATG when the altitude is >100m, therefore many assumptions of ATG could also apply to UAV.

If the UAV flies below 100m, it could exceed the evaluated ATG vertical coverage, and it’s not clear whether the ATG assumptions could still apply. For example, does the BS tilt angel need to be adjusted to serve this UE, or will the UE handover to a TN cell?

Proposal 2: RAN4 to clarify the UE/BS assumption when UAV UE is at an altitude <100m.

Antenna model

In ATG co-existence study, two options were evaluated for the ATG BS: non sub-array model, and sub-array model. For ATG UE, omini-directional antenna was assumed for 2GHz, and antenna array was assumed for 4GHz.

It’s needs to confirm whether the same assumption could be used for UAV BS and UE.

Proposal 3: RAN4 to confirm the antenna model assumption for UAV BS and UE, i.e. whether the same assumption for ATG could be reused:

- UAV BS: Non sub-array model, and sub-array model.

- UAV UE: Omini-directional antenna was assumed for 2GHz, and antenna array was assumed for 4GHz.

Propagation model

The evaluation of ATG was based on the suburban and rural macro environment considering the aircraft’s route.

The typical use cases for UAV include package delivery, monitoring of critical infrastructure, flying camera and surveillance. It’s likely the UAV could also be applicable in urban scenario. RAN4 needs to discuss and decide whether the existing propagation model for urban environment will be evaluated.

Proposal 4: RAN4 to decide whether to evaluate the existing propagation model in urban environment.

Conclusion

This document provide our view on the UAV co-existence evaluation. Following proposals and observations are for RAN4 discussion.

Proposal 1: The co-existence study focuses on the scenarios with UAV UL as aggressor, and prioritize the synchronized scenario.

Observation 1: The vertical coverage range of UAV is inside the coverage range of ATG when the altitude is >100m, therefore many assumptions of ATG could also apply to UAV.

Proposal 2: RAN4 to clarify the UE/BS assumption when UAV UE is at an altitude <100m.

Proposal 3: RAN4 to confirm the antenna model assumption for UAV BS and UE, i.e. whether the same assumption for ATG could be reused:

- UAV BS: Non sub-array model, and sub-array model.

- UAV UE: Omini-directional antenna was assumed for 2GHz, and antenna array was assumed for 4GHz.

Proposal 4: RAN4 to decide whether to evaluate the existing propagation model in urban environment.

References

[1] RP-252954, New WID on enhancement of NR RF and RRM requirements for uncrewed aerial vehicle (UAV), CMCC