3GPP TSG-RAN WG4 Meeting #116-bis	R4-2514136

Prague, CZ, 13th – 17th October 2025

Source:	Nokia

Title:	Discussion on RRM requirements for (e)RedCap UE enhancements

Agenda item:	7.4.3.2

Document for:	Discussion

Introduction

The work item description [1] for Phase-6 RRM from 3GPP TSG RAN Meeting #109 is as follows:

- Specify the following requirements for (e)RedCap UEs [RAN4] - Relaxed RLM/BFD RRM requirements - Use R17 RLM/BFD relaxation for legacy UE as a baseline - Support of less than 5 MHz channel bandwidth (15 PRB transmission bandwidth configuration and 12 PRB SSB) - RF requirements, considering example bands: n100, n106, n26, n28 and n85 - RRM requirements including relaxed RLM/BFD RRM requirements - Note 1: Use R18 3 MHz requirements for non-RedCap UE as a baseline - Note 2: Consider both 1Rx and 2Rx (e)RedCap UEs and HD-FDD

Discussion

The Rel-20 work item NR_RRM_Ph6 (RP-252955) includes enhancements for (e)RedCap UEs, specifically extending support for less than 5 MHz channel bandwidth to reduce complexity, cost, and power consumption for devices like wearables and IoT sensors. The feature targets 15 PRB transmission bandwidth configuration and 12 PRB SSB                                           applicable to 1Rx and 2Rx eRedCap UEs, including HD-FDD modes. This document discusses key aspects from TS 38.133 focusing on RRM implications in clauses 8 (RLM/BFD), 9 (measurement procedures), and 10 (measurement performance).

Relaxed RLM/BFD RRM requirements

Relaxed RLM/BFD requirements allow UEs to perform less frequent monitoring of downlink radio link quality under stable conditions which directly contributes to power savings for battery constrained devices like eRedCap UEs.

38.133 already defines relaxed RLM/BFD for non-RedCap devices in sections 8.1.2.4 (for SSB-based RLM) and section 8.1.3.4 (for CSI-RS based RLM). The applicability rules for when UE is allowed to perform relaxed RLM are defined in section 8.1.1.1 and based on UE supporting rlm-Relaxation-r17and configured with explicit signalling goodServingCellEvaluationRLM.

Hence, to enable RLM relaxation for RedCap UE, the applicability rules in 8.1.1.1 would need to be updated to include at least references to sections covering minimum requirement of SSB based and CSI-RS based radio link monitoring for UE fulfilling relaxed measurement criteria.

8.1.1.1 would need to be updated to include at least references to sections covering minimum requirement of SSB based and CSI-RS based radio link monitoring for UE fulfilling relaxed measurement criteria.

RAN4 would need to make new sections covering minimum requirement of SSB based and CSI-RS based radio link monitoring for UE fulfilling relaxed measurement criteria.

Define new sections covering minimum requirement of SSB based and CSI-RS based radio link monitoring for UE fulfilling relaxed measurement criteria.

We see that there RAN4 has defined a set of PDCCH transmission parameters for out-of-sync and in-sync evaluation for RedCap UE operating with 2Rx and 1Rx branches.

For a 2RX RedCap UE, we assume that the requirements defined in Rel-17 could be readily used. For 1RX RedCap devices we expect that the PDCCH transmission parameters for out-of-sync and in-sync evaluation can be used when not considering <5MHz.

Less than 5 MHz for 1Rx and 2Rx

The less than 5 MHz channel bandwidth support for eRedCap UEs is designed to allow RedCap operation in < 5MHz bands with Rel-18 baseline. For eRedCap, this involves 15 PRB for data transmission and 12 PRB for SSB, allowing the SSB to fit within the narrow BW without full 20 PRB SSB usage.

SSB Puncturing

Traditional 5MHz SSB configuration PBCH occupies bandwidth of 3.6 MHz, or 20 RBs. SSB puncturing is needed when the SSB transmission exceeds the channel BW, resulting that the UE PBCH part of the SSB is nulled / cannot be read by the UE receiver. In rel-18 work, the PBCH was punctured to the desired transmission bandwidth as follows:

Figure 1: 12 PRB SSB in Rel-18

In Rel-18 SSB was punctured to support 12 PRB SSB

The same principles apply for eRedCap operating in <5MHz.Bands supported by 12 PRB and 15 PRB

In R18 < 5MHz two different configurations were defined, 12 PRB configuration that targets FRMCS bands and 15 PRB configuration targeting other bands. The WID currently states that there is support for 15 PRB transmission bandwidth configuration and 12 PRB SSB. This indicates that the only configuration supported by RedCap bands is the 15 PRB configuration.

Introduce support for only 15 PRB transmission bandwidth configuration and 12 PRB SSB i.e., 12 PRB transmission BW configuration with 12 PRB SSB configuration is not supported for RedCap devices.

Use of Soft combining for 1Rx UEs

Soft combining refers to the UE’s ability to combine multiple instances of the same signal (e.g., SSB repetitions) at the symbol or bit level to improve detection reliability in low-SNR scenarios. In TS 38.133, soft combining is not assumed in Rel-15 or in Rel-18 < 5MHz baseline. This means that the number of samples allowed for measurements is relatively large. Based on previous simulations soft combining adds 3dB to performance gain.

Legacy requirements do not assume soft combining for Rel-15 UEs or Rel-18 <5MHz UEs

Soft combining introduces 3dB performance gain for UEs

The Rel-18 requirements for < 5MHz was developed based on the assumption of 2Rx UEs. Hence, it should be quite straightforward to reuse the 15 PRB transmission BW requirements for 2Rx RedCap devices.

2Rx Redcap requirements could use requirements developed in Rel-18 for < 5MHz directly due to similar assumptions.

For 1Rx UEs, we propose that RAN4 discusses whether it is possible to change the 1Rx RedCap UE assumption on soft-combining (to utilise it) and reuse the 2Rx < 5MHz requirements for 1Rx RedCap.

Discuss whether it is possible to use 2Rx requirements developed in Rel-18 for < 5MHz devices with 1Rx UE with the assumption that UE use soft combining.

In such case, only one set of requirements would be needed for Rel-20 < 5MHz RedCap UE covering both 1Rx and 2Rx RedCap.

Conclusion

In this paper the following Observations and Proposals were made:

Observation 1: 8.1.1.1 would need to be updated to include at least references to sections covering minimum requirement of SSB based and CSI-RS based radio link monitoring for UE fulfilling relaxed measurement criteria.

Proposal 1: Define new sections covering minimum requirement of SSB based and CSI-RS based radio link monitoring for UE fulfilling relaxed measurement criteria.

Observation 2: In Rel-18 SSB was punctured to support 12 PRB SSB

Proposal 2: Introduce support for only 15 PRB transmission bandwidth configuration and 12 PRB SSB i.e., 12 PRB transmission BW configuration with 12 PRB SSB configuration is not supported for RedCap devices.

Observation 3: Legacy requirements do not assume soft combining for Rel-15 UEs or Rel-18 <5MHz UEs

Observation 4: Soft combining introduces 3dB performance gain for UEs

Observation 5: 2Rx Redcap requirements could use requirements developed in Rel-18 for < 5MHz directly due to similar assumptions.

Proposal 3: Discuss whether it is possible to use 2Rx requirements developed in Rel-18 for < 5MHz devices with 1Rx UE with the assumption that UE use soft combining.

References

[1]   RP-252955, WID: NR Radio Resource Management (RRM) Phase 6, Apple, RAN Meeting #109, September 2025.