# 3GPP TSG-RAN WG4 Meeting #116bis                                 R4-2514145

Prague, CZ, 13rd – 17th, Oct. 2025

# Title: 	Discussion on UE and BS demodulation performance requirements for SBFD

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	6.13.4

# Document for:	Approval

# 1 Introduction

In RAN#102, a new R19 WI on Evolution of NR duplex operation: Sub-band full duplex (SBFD) was approved, and the revised WID at RAN#104 can be found in [1].

The objectives related to RAN4 core and performance parts are listed.

- Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4] - Information exchange among gNBs, including [RAN3] - Semi-static cell-specific SBFD time and frequency location configuration - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS - Strongest DL beam information - CLI-mitigation request - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain - PUSCH resource mapping, i.e., rate-matching around the muted REs - UCI resource determination in symbols with muted REs - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] (Reuse major principles) - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource - At least aperiodic reporting - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices - UCI bits generation - Priority rules for multiple CSI reporting - CLI measurement accuracy requirement - Specify applicable RRM core requirements for CLI handling mechanisms [RAN4] - Specify other RRM core requirements for SBFD operation, if identified [RAN4] The objectives are as follows [RAN4]: - Specify BS RF conformance requirements to support SBFD operation at gNB - Specify necessary RRM performance requirements to support SBFD operation - Specify necessary BS and/or UE demodulation performance requirements to support SBFD operation

In this contribution, we give some initial discussion on BS and/or UE demodulation performance requirements to support SBFD operation according to the last meeting agreements.

# 2 Discussion

BS demodulation requirements

Types of interference modeling for demodulation requirement with UL muting

- Agreement - Evaluate PUSCH performance with UL muting under inter-site gNB interference modelling - FFS on self-interference and intra-site inter sector interference

gNB self-interference modeling

For gNB self-interference, it is caused by the fact that when the gNB transmits and receives signals at the same time. The transmitted signal leaks to its own receiving end, seriously interfering with the reception of the uplink signal. Methods to suppress the self-interference include passive methods which rely on the antenna isolation between TX and RX antennas, active, active methods which utilize RF or digital processing, hybrid methods using a beamforming nulling/isolation, etc. During the study item and work item phases, the detailed evaluation in RF session was discussed and verified.

In general, the above suppression methodologies are basically based on the antenna, RFFE or spatial suppression. When the RF signal enters the baseband module, only residual power interference can be characterized in baseband. Although this residual interference is significantly reduced in intensity, it still has a critical impact on demodulation performance.

According to the conclusion of RF session, RAN4 agreed that 0.7dB receiver sensitivity desensitization could be achieved by self-interference cancellation capability for feasibility study.

Specifically, RAN1 considered a more detailed modeling approach, a ratio of self-interference (RSI) was introduced to represent the self-interference suppression capability of gNB for system level simulation, means of spatial isolation, subband frequency isolation, digital interference cancellation and beamforming nulling/isolation, etc.

Observation 1. 0.7dB receiver sensitivity desensitization is achieved by self-interference cancellation capability for feasibility study in RF session.

Proposal 1. Propose modelling as additive white Gaussian noise with 1dB receiver desensitization for gNB self-interference if RAN4 agrees to evaluate the self-interference.

inter-sector gNB-gNB interference

Regarding inter-sector gNB-gNB interference, it is caused by the channel between gNB of aggressor and victim are near field when two gNB are co-location. According to the previous feasibility study, the implementation of controlling the co-site inter sector co-channel inter-subband interference depends on spatial isolation, beam nulling/isolation capability, frequency isolation, and digital interference suppression, etc. From this point of view, co-site inter-sector gNB-gNB interference is more similar to gNB self-interference. Meanwhile, according to the conclusion of TR 38.858, 1dB receiver sensitivity desensitization could be achieved by co-site inter-sector co-channel inter-subband interference. However, another conclusion mentioned that 1dB desensitization can not able to achieve that because the receiver is saturated, and the RC processing is not feasible.

Observation 2. The implementation of controlling the co-site inter sector co-channel inter-subband interference depends on spatial isolation, beam nulling/isolation, frequency isolation and digital interference suppression.

Observation 3. Co-site inter-sector gNB-gNB interference is more similar to gNB self-interference.

Observation 4. 0.7 dB receiver sensitivity desensitization may or may not be achieved by co-site inter-sector co-channel inter-subband interference.

Proposal 2. Propose modelling as additive white Gaussian noise with 0.7dB receiver desensitization for gNB self-interference if RAN4 agrees to evaluate the intra-site inter sector interference.

how to model interference modeling for performance evaluation

- Agreement - Considering TDL+INR type of interference model for evaluation - Note: the RF assumption can be considered for interference modelling - The following candidate value for INR can be considered for evaluation as starting point - Option 1: based on link budget analysis Conditions; FR1. Interf1 > Interf2; INR1 <= 17.2dB, INR2 <= 7.2dB Trial simulation could try following sets: [INR1, INR2] = {[5, -5], [9, -1], [13, 3], [17, 7]} dB. Interf1 = Interf2; INR1 = INR2 <=11.2dB Trial simulation could try following sets: [INR1, INR2] = {[-1, -1], [3, 3], [7, 7], [11, 11]} dB. Note: The interference wave form could use PUSCH with 1+1 DM-RS and CP-OFDM to simplify simulation setup.. - Other options are not concluded - Interesting companies can provide the system level simulation to derive the INR value

Regarding how to model interference modeling, currently, considering the limited remain time for SBFD demod work, some companies provided a simple approach to derive the interference profile, link budget. Based on our understanding, we think that is more like a worst-case scenario assumption. Because in the gNB side, the coordination of gNBs or beam level coordination will be assumed for SBFD operation. From this point of view, the actual interference profiles will be less than the value of link budget. The motivation of system simulation level is to evaluate the interference profile based on the more comprehensive analysis and scheduling. However, the key issue is how to handle the beam coordination between gNBs. Currently, gNB-gNB co-channel interference also could be suppressed beam management, such as Tx beam nulling, etc. If RAN4 wants to evaluate the gNB-gNB interference, this beam coordination issue should be discussed. Different beam coordination means different interference level. As mentioned before, consider the limited time, which will be a challenge for the timeline. Therefore, we are fine to use this simple approach to derive the interference profiles.

Based on the recommend values, we selected the maximum and minimum values to evaluate the baseband performance. From our simulation results [3], muting symbol scheme can achieve a better performance compared to muting disable scheme, especially when using a higher interference level. the receiver’s ability to suppress interference is affected by the level of interference.

Observation 5. Based on our simulation results, the receiver’s ability to suppress interference is affected by the level of interference.

Proposal 3. If RAN4 agrees to use link budget approach to derive the interference profile, then the upper limit of interference reduction must be considered.

Muting symbol configurations

- Agreement - For performance evaluation - Option 1 - Option 2 - Companies are encouraged to provide the evaluation results based on following muting symbol patterns in RAN4#116bis meeting

Currently, according to our simulation results [3], the significant difference is that a gap of more than 1dB gap is observed between 1 symbol and two symbols muting schemes. Also, no difference is observed between 1 symbol and two symbols muting schemes in some test cases. From RAN4 minimum perspective, 1 symbol muting scheme achieves worse than 2 symbols scheme. Therefore, we propose to consider 1 symbol muting scheme when defining demodulation requirements.

Observation 6. According to our simulation results [3], the significant difference is that a gap of more than 1dB gap is observed between 1 symbol and two symbols muting schemes

Proposal 4. Propose to consider 1 symbol muting scheme when defining demodulation requirements.

# 3 Conclusion

In this contribution, we give some discussions on demodulation performance requirements of SBFD demodulation requirements, the conclusions are:

Observation 1. 0.7dB receiver sensitivity desensitization is achieved by self-interference cancellation capability for feasibility study in RF session.

Observation 2. The implementation of controlling the co-site inter sector co-channel inter-subband interference depends on spatial isolation, beam nulling/isolation, frequency isolation and digital interference suppression.

Observation 3. Co-site inter-sector gNB-gNB interference is more similar to gNB self-interference.

Observation 4. 0.7 dB receiver sensitivity desensitization may or may not be achieved by co-site inter-sector co-channel inter-subband interference.

Observation 5. Based on our simulation results, the receiver’s ability to suppress interference is affected by the level of interference.

Observation 6. According to our simulation results [3], the significant difference is that a gap of more than 1dB gap is observed between 1 symbol and two symbols muting schemes

Proposal 1. Propose modelling as additive white Gaussian noise with 1dB receiver desensitization for gNB self-interference if RAN4 agrees to evaluate the self-interference.

Proposal 2. Propose modelling as additive white Gaussian noise with 0.7dB receiver desensitization for gNB self-interference if RAN4 agrees to evaluate the intra-site inter sector interference.

Proposal 3. If RAN4 agrees to use link budget approach to derive the interference profile, then the upper limit of interference reduction must be considered.

Proposal 5. Propose to consider 1 symbol muting scheme when defining demodulation requirements.

# 4	Reference

- RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei

- R4-2512615, Way Forward for [116][328] NR_duplex_evo_demod, Samsung.

- R4-2514146, Simulation results on NR SBFD. ZTE Corporation, Sanechips