# 3GPP TSG-RAN WG4 Meeting #116bis                                 R4-2514147

# Prague, CZ, 13th – 17th October 2025

# Title: 	Discussion on demodulation performance requirements for A-IoT for BS side

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	6.14.5

# Document for:	Approval

# 1 Introduction

In n the RAN#106 meeting, the new WI related with solutions for Ambient IoT (Internet of Things) in NR was endorsed in [1] with the following objectives

RAN4 scope - Specify RF requirements for Ambient-IoT BS, device 1, and CW - RF requirements for Type 1-C Ambient-IoT BS - RF requirements for device 1 - RF requirements for CW - Specify RRM core requirements for device 1, if necessary - Study and develop OTA test methodology for A-IoT device 1 - Consider test methods specified in TR 38.870 as starting point. Take test system reuse, test system complexity and test time into account, when developing test methods suitable for Ambient IoT. - Develop the preliminary Measurement Uncertainty (MU) assessment for the test system - Use band n8 as an example band - Objective of Performance part WI - Specify the necessary conformance testing for Ambient-IoT BS - Specify the necessary RRM performance requirements for device 1 - Specify the necessary demodulation performance requirements for device 1 and Ambient-IoT BS

In the following section, we will provide the detailed discussions for A-IoT BS side demodulation requirements based on the last meeting conclusion [2].

# 2 Discussion

In the last meeting, RAN4 discussed the technical issues in detail, but still have many open issues [2]. Next, we would like to address in the following for A-IoT BS demodulation requirements.

Whether to define PDRCH demodulation requirement for Ambient-IoT

- Way forward - Option 1: Define PDRCH demodulation requirement. - Option 2: Not define PDRCH demodulation requirements

Regarding RDR, a physical PDRCH carries any higher-layer payload, the response transmitted from device to reader during the contention-based access procedure, and L1 D2R control information. Currently, two potential approaches for PDRCH is studied shown in the following Figures.

Figure 1. Single PDRCH generation at the device with line coding, for the case where FEC and repetition are present

Figure 2. Single PDRCH generation at the device without line coding, for the case where FEC and repetition are present.

According to the assumption of RAN1, A-IoT random access procedure is used for the Ambient IoT device of access the network for data transmission with fours steps, e.g., Message 0, Message 1, Message 2, and Message 3. The message 0 is used to indicate the deceive to access the network according to the paging. Message 1 is used to send the 16-bit random device ID to the reader. When reader detect s a device ID, it will respond with that device ID and provide scheduling information for Message 3, including modulation scheme, repetition, etc,. In this time, it means that the device have been completely access the network. And then the device will apply the data transmission based on the allocated the resources, coding scheme, repetition, and modulation scheme for message 3.

Based on the above procedure, message 3 carries the data transmission from the device to the reader, including basedband processing functions such as demodulation and decoding. If RAN4 wants to define the requirements for A-IoT reader, we think that the message 3 is a candidate options.

Observation 1. The message 2 carries the data transmission from the device to the reader, including baseband processing functions, such as demodulation and decoding in reader side.

Proposal 1. Propose to consider PDRCH demodulation requirements with message 3 reception.

D2R message structure for testing

- Way forward - Option 1: Short D2R transmission without midamble with 31 bits Preamble - Option 2: Long D2R transmission with midamble inserted with 31 bits Preamble and 31 bits Midamble - Other options are not precluded

In general, D2R physical and signals are defined:

-	Physical device-to-reader channel, PDRCH

-	D2R-amble signals consisting of the D2R preamble and D2R midamble

A midamble is supported for clock synchronization and channel estimation. Both the D2R preamble and midamble sequences use m-sequences. The m-sequence of length MPN =7 or 31.

Observation 2. A midamble is supported for clock synchronization and channel estimation.

Propose 2. Propose to consider long D2R transmission with midamble inserted with 31 bits Preamble and 31 bits Midamble.

Modulation

- Way forward - Option 1: Both BPSK and OOK modulation scheme - Option 2: OOK modulation

Currently, D2R transmission supports both BPSK and OOK modulation schemes, the specific method used depends on the device implementation. Thus, we think that both BPSK and OOK schemes also need to be consider for different devices.

Propose 3. Propose to consider both BPSK and OOK modulation schemes.

Waveform

- Way forward - Option 1: backscattering a carrier wave (CW) - Option 2: Unmodulated single tone (CW)

Regarding waveform, currently, only unmodulated single tone is consider in RAN1 for D2R. Therefore, we prefer to consider unmodulated single tone.

Propose 4. Propose to consider unmodulated single tone waveform.

Block level Repetition

- Way forward - Option 1: 1 - Option 2: 2

D2R supports block-level retransmission, with the number of retransmission attempts being either 1 or 2.

Propose 5. Propose to consider 2 block level repetitions.

SFO assumption

- Way forward - Option 1: 104 ppm~105 ppm - Other options

For the D2R link in A-IoT, the primary cause of SFO issues lies in the use of low-precision crystal oscillators in the end devices, which are employed to meet cost and power consumption requirements. This leads to a mismatch in sampling clock frequency between the end device and the router, resulting in symbol timing drift, inter-carrier interference, and phase rotation, which severely degrades system performance. Therefore, verifying and compensating for SFO through the use of midamble is crucial for baseband processing. In the RF session, an SFO of 10% was assumed.

Propose 6. Propose to consider10^4 ppm~10^5 ppm for D2R link evaluation.

# 3 Conclusion

In this contribution, we give some discussions on the MIMO layers related to 6Rx requirements, the conclusions are:

Observation 1. The message 2 carries the data transmission from the device to the reader, including baseband processing functions, such as demodulation and decoding in reader side.

Observation 2. A midamble is supported for clock synchronization and channel estimation.

Proposal 1. Propose to consider PDRCH demodulation requirements with message 3 reception.

Propose 2. Propose to consider long D2R transmission with midamble inserted with 31 bits Preamble and 31 bits Midamble.

Propose 3. Propose to consider both BPSK and OOK modulation schemes.

Propose 4. Propose to consider unmodulated single tone waveform.

Propose 5. Propose to consider 2 block level repetitions.

Propose 6. Propose to consider10^4 ppm~10^5 ppm for D2R link evaluation.

# 4 References

- RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR.

- R4-2512603, Way Forward for [116][335] A-IoT_demod, Huawei, CMCC.