3GPP TSG-RAN WG4 Meeting #116-bis	R4-2514170

Prague, Czech Republic, 13 – 17 October, 2025

Title: 	Discussion on RRM core requirements for Rel-19 NR NTN

Source: 	Huawei, HiSilicon

Agenda item:	4.25.3

Document for:	Approval

Introduction

RRM requirements for other (than RedCap) enhancements in NR NTN are discussed in RAN4#116, and the outcomes are captured in WF [1]. There are some remaining issues to be further discussed.

- SMTC enhancement

- RACH scheduling restriction

In this paper, we will provide our views on RRM requirements for other enhancements in NR NTN.

Discussion

SMTC enhancement

Applicability for FR2-NTN

In RAN2#131 the following is agreed related to applicability of SMTC enhancements.

We introduce a capability to support STMC enhancements to FR1 (FFS on FR2 depending on RAN4 progress)

The issue was also discussed in RAN4#116 but without conclusion. In our view, there is no need to limit the applicability of SMTC enhancements for FR2-NTN.

The mobility and measurement requirements for FR2-NTN are mostly defined for single satellite scenario, and they are same as those for FR1-NTN. One exception is the inter-satellite blind HO requirements. In RAN4#116 it is agreed that with SMTC enhancement, the SMTC in HO requirements corresponding to the physical cell ID of the target cell. As HO is concerned with a single target cell, the agreement can be applied directly to inter-satellite blind HO in FR2-NTN. So, there is not much efforts to define requirements for SMTC enhancements for FR2-NTN.

On the other hand, the SMTC enhancements are motivated by DL coverage enhancements. The scenario under consideration where different cells served by the same satellite may transmit SSB with different periodicities and offsets is applicable for both FR1-NTN and FR2-NTN.

Proposal 1: SMTC enhancements are supported for both FR1-NTN and FR2-NTN.

Support 7 SMTCs per carrier in IDLE/INACTIVE requirements

In RAN4#116 there is no conclusion on RRM impact to support 7 SMTCs per carrier in IDLE/INACTIVE. RAN4 cannot define applicable requirements unless we enhance the UE capability in number of SMTC (the max candidate value is extended from 4 to 7), or we define UE requirements on SMTC selection.

In RAN2#131 an LS is sent to RAN4 [2] with following contents.

- RAN2 reconfirms the understanding that if the NW configures more than 4 STMCs the UE in idle/inactive the UE selects up to 4 STMCs (or less, depending on the UE capability) based on location and only considers them for measurements.

From the yellow highlighted texts, we understand RAN2 does not intend to enhance the UE capability but it is made clear that UE selects SMTC based on location. In this sense, we support to define requirements based on UE selected SMTCs. A text proposal is given in Proposal 2.

Proposal 2: Add the following texts to cell reselection requirements:

For UE supporting Location based SMTC selection for RRC_IDLE/RRC_INACTIVE, if refLocList is provided, the requirements in this clause apply for the SMTCs selected by the UE based on the UE location and the reference location associated with SMTCs, subject to parallelSMTC-r17.

Another issue related to this enhancement is whether to define larger M1/M2/M3/M4 values when more than 4 SMTCs are configured.  Our view is that this is not needed. Even number of configured SMTCs can be >4, UE will not measure >4 SMTCs at any time based on RAN2 agreement.

Proposal 3: Do not define larger M1/M2/M3/M4 values in cell reselection requirements.

Support Two different SMTC periodicities (with different offsets)

One remaining issue in this enhancement is whether “or the longer SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing” is needed.

- For Trs in Handover requirement: - Trs is the SMTC corresponding to the physical cell ID of the target cell - Further discuss the proposal “or the longer SMTCconfigured in the measObjectNR having the same SSB frequency and subcarrier spacing.”

We do not think the use of longer SMTC configured for the MO is needed. It is possible that SMTC is not configured for the target cell in HO command. In this case, Trs in the requirements would refer to SMTC configured for the MO. However, when 2 SMTC periodicities are configured for the MO, NW needs to indicate UE which cells are to be measured with SMTC periodicity 1 and which with SMTC periodicity 2, otherwise UE may measure a cell with SMTC periodicity smaller than the SSB periodicity used by the cell. It means UE would know SMTC periodicity corresponding to the target cell from MO configuration.

Proposal 4: For HO requirements, Trs is defined as the SMTC corresponding to the physical cell ID of the target cell, and “longer SMTC configured for the MO” is not needed.

Another remaining issue in this enhancement is whether PCI list will be provided in RRC re-direction.

- For Trs in RRC Connection Release with Redirection requirement: - If physical cell ID is provided in RAN2 signalling design, Trs is the SMTC corresponding to the physical cell ID of the target cell - Otherwise, Trs is the maximum SMTC periodicity configured in the measObjectNR having the same SSB frequency and subcarrier spacing.

Based on the latest 38.331, the SMTC enhancement is not applicable to RRC re-direction, i.e. if SMTC is configured in RRCRelease message, only legacy smtc can be configured which is without PCI information.

However, smtc is optional in RRCRelease message, and in RAN4 requirements the SMTC configuration in MO will be used when it is not present. PCI information can be provided in SMTC configuration in MO, so the case where PCI is provided exists, and in this case Trs should be the SMTC corresponding to the PCI of the target cell.

Proposal 5: Confirm the RAN4#116 agreement on RRC re-direction.

There was no agreement in RAN4#116 regarding how to define Kmulti_SMTC. Our proposal for RAN4#116 is re-produced below, and we also analyse the proposal from [3].

In current requirements, Kmulti_SMTC is defined separately for the cases when SMTCs do not overlap with each other and when SMTCs partially overlap with each other.

Kmulti_SMTC is the scaling factor for measurement of multiple SMTCs or multiple satellites, and if SMTCs do not overlap with each other, - Kmulti_SMTC=1, if GEO satellites are measured on the carrier; - Kmulti_SMTC=NLEO,iNLEO,simul, if LEO satellites are measured on the carrier; if SMTCs partially overlap with each other, - Kmulti_SMTC=NSMTC,overlap, if only GEO satellites are measured on the carrier; - Kmulti_SMTC=i=1NSMTC,overlapNLEO,iNLEO,simul, if only LEO satellites are measured on the carrier; where - NLEO,i is the number of LEO satellites to be measured within i-th SMTC, - NLEO,simul is the number of LEO satellites that UE can measure in parallel within an SMTC, - NSMTC,overlap is the number of SMTCs that partially overlap with each other.

It is however not very clear when SMTCs are considered to be partially overlapping with each other. In Figure 1 we show examples of 2 cases where UE is configured with smtc1 and smtc2 in Case 1, and smtc1 and smtc4 in Case 2. It is noted that both cases are supported in R17 spec, and in our view both cases are considered as “SMTCs that partially overlap with each other”.

Figure 1: Example of existing cases where “SMTCs that partially overlap with each other”

RAN4 has clear definition for SMTC “colliding”.  In clause 9.1C.9.3, the definition of collision between occasions of multiple SMTCs is specified, and in clause 4.2C.2.3, it is specified how to determine whether 2 SMTCs are overlapping based on collision of their occasions.

Excerpt from clause 9.1C.9.3 Collision between occasions of multiple SMTCs may occur as specified in this clause. When UE is configured with more than one SMTC on a SAN carrier, two SMTC occasions outside measurement gap are considered colliding if at least one of the following conditions is met: - the two SMTC occasions are fully or partially overlapping in time domain, or - the magnitude of the distance between two SMTC occasions in time domain is less than or equals to 4 ms. The distance between two SMTC occasions is defined as the time difference between the ending point of the earlier SMTC occasion and the starting point of the later SMTC occasion. Excerpt from clause 4.2C.2.3 Note: for deriving Kmulti_SMTC for Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra, two SMTCs are considered as overlapping if they overlap in one or more occasions during a single Tdetect,NR_Intra, Tmeasure,NR_Intra or Tevaluate,NR_Intra.

In TN requirements, Case 1 does not require any scaling, but in NTN there is an additional consideration factor about different satellites, i.e. SMTC#1 and SMTC#2 can be associated with different satellites that UE cannot measure at a single occasion.

With above interpretation of existing requirements, we will discuss the new requirements based on the example in Figure 2, where UE is configured with 3 SMTCs with different periodicities and offsets.

Figure 2: Example of new cases where “SMTCs that partially overlap with each other”

In our view, Kmulti_SMTC should be defined SMTC specific as different SMTC may see different collision. SMTC#1 is colliding with 2 other SMTCs, but SMTC#2 or #3 is only colliding with 1 other SMTC. It is unreasonable to define NSMTC,overlap as 3 for every SMTC.

In addition, for one single SMTC the collision may be different at different occasions within a period. The 1st and 2nd occasions of SMTC#1 have collision while the 3rd and 4th occasions of SMTC#1 do not. It is means the calculated Kmulti_SMTC for SMTC#1 may be different at different occasions of SMTC#1 (note Kmulti_SMTC may also be different for the 1st and 2nd occasions as number of satellites to be measured in SMTC#2 and SMTC#3 may be different). To define the minimum requirements, we suggest to use the max Kmulti_SMTC among all occasions as the SMTC specific scaling factor.

Proposal 6: To support configuring two different SMTC periodicities with different offsets, RAN4 to update the definition of scaling factor Kmulti_SMTC for measurement requirements:

For each SMTC#i, Kmulti_SMTC,i = maxj(Kmulti_SMTC,i,j), where Kmulti_SMTC,i,j is calculated in the legacy way for the j-th occasion of SMTC#i.

In [3] it is proposed that

Proposal 1: To minimize ambiguity in the scaling factor definition for overlapping SMTCs with different periodicities under the proximity distance condition, RAN4 to update the NTN-specific scaling factor, K_multi_SMTC, as follows: - SMTCs associated with the same satellite: Treat as a single unified SMTC (i.e., do not double count). - SMTCs associated with different satellites: Prioritize the SMTC with the longer periodicity, i.e. exclude the colliding SMTC occasion from the scaling factor derivation in the colliding SMTC occasion.

Our view is that it may be difficult to differ the case “SMTCs associated with the same satellite” with that where “SMTCs associated with the different satellites” because each SMTC can be associated with a set of satellites rather than a single satellite. Also, when multiple SMTCs are associated with the same satellite it is not always the case that they can be treated as a single SMTC. UE may need to combine the occasions from 2 SMTCs to form a new SMTC occasion, and this is a new behavior and we prefer to not consider it for defining min requirements.

Another issue is the collision handling when multiple SMTCs are associated with different satellites. In [3] it is proposed to use priority based approach, while existing requirements are based on sharing based approach. Priority based approach can work but it also requires further discussion, e.g. what happens when there is no available occasion for the SMTC with smaller periodicity due to collision with multiple SMTCs with larger periodicities. At this stage of the WI we prefer not to introduce a new approach for collision handling.

RACH scheduling restriction

In RAN4#114-bis, the issue of RACH scheduling restriction was shortly discussed based on [3] but without agreement. In our view, the issue is valid and needs to be addressed by both RAN1 and RAN4.

- Although the objective is for DL coverage enhancement, the issue of insufficient satellite beams that can be simultaneously active is valid for both Tx and Rx. Therefore, beam hopping may also apply for UL.

- Offset of RACH occasion (RO) cannot be configured, but it is always in radio frame 0 or 1 according to Table 6.3.3.2-2 of 38211.

  - If 160ms RO periodicity is used, then only PRACH from UEs in a limited subset of satellite footprints can be received by the NW because in radio frame 0 or 1 the Rx beam is active only for a subset of satellite footprints.

  - If <160ms RO periodicity is used, then for a specific satellite footprint, not all ROs will be valid because the Rx beam is active only in a subset of ROs for this satellite footprint, and it is possible that UE selects an RO that will not be received by the NW as raised up in [3].

For above reasons, we see it meaningful to apply at least a time offset for valid RO determination so that UE in each satellite footprint will transmit PRACH on ROs that corresponds to active Rx beam for its satellite footprint. The detailed solution, including how the offset are determined and whether the offset also applies for Tx and Rx of other RACH related signals and channels, can be up to RAN1. Once RAN1 decides on the solution, RAN4 can define scheduling restriction requirements accordingly.

Proposal 7: Send LS to inform RAN1 about the need for time offset for PRACH transmission considering UL beam hopping.

A draft LS is provided in the Annex.

Conclusions

In this paper we provided our views on RRM requirements for other enhancements in NR NTN.

Proposal 1: SMTC enhancements are supported for both FR1-NTN and FR2-NTN.

Proposal 2: Add the following texts to cell reselection requirements:

For UE supporting Location based SMTC selection for RRC_IDLE/RRC_INACTIVE, if refLocList is provided, the requirements in this clause apply for the SMTCs selected by the UE based on the UE location and the reference location associated with SMTCs, subject to parallelSMTC-r17.

Proposal 3: Do not define larger M1/M2/M3/M4 values in cell reselection requirements.

Proposal 4: For HO requirements, Trs is defined as the SMTC corresponding to the physical cell ID of the target cell, and “longer SMTC configured for the MO” is not needed.

Proposal 5: Confirm the RAN4#116 agreement on RRC re-direction.

Proposal 6: To support configuring two different SMTC periodicities with different offsets, RAN4 to update the definition of scaling factor Kmulti_SMTC for measurement requirements:

For each SMTC#i, Kmulti_SMTC,i = maxj(Kmulti_SMTC,i,j), where Kmulti_SMTC,i,j is calculated in the legacy way for the j-th occasion of SMTC#i.

Proposal 7: Send LS to inform RAN1 about the need for time offset for PRACH transmission considering UL beam hopping.

Reference

- R4-2512337, WF on RRM requirements for NR_NTN_Ph3_Part2, Moderator (Qualcomm Incorporated)

- R2-2506298, LS on additional agreements on SMTC enhancements, RAN2

- R4-2511513, Downlink coverage enhancement for NTN, Qualcomm Incorporated

Annex: draft LS on scheduling restriction for RACH related signals and channels

3GPP TSG-RAN WG4 Meeting #115 R4-250xxxx Malta, MT, 19 – 23 May, 2025 Title: LS on scheduling restriction for RACH related signals and channels Response to: Release: Release 19 Work Item: NR_NTN_Ph3-Core Source: RAN4 To: RAN1 Cc: Contact Person: #### Name: Li Zhang - E-mail Address: zhangli164@huawei.com Attachments: - 1. Overall Description: RAN4 is defining requirements for DL coverage enhancement, and it is RAN4 understanding that the satellite beam hopping may also apply in UL. RAN4 is discussing scheduling restriction requirements for PRACH and other Tx and Rx of other RACH related signals and channels, and it is identified that at least a time offset for PRACH transmission is needed considering UL beam hopping. RAN4 kindly asks RAN1 to decide the details of the time offset for PRACH transmission considering UL beam hopping, and whether the time offset applies to other Tx and Rx of other RACH related signals and channels. 2. Actions: To RAN1: RAN4 kindly asks RAN1 to decide the details of the time offset for PRACH transmission considering UL beam hopping, and whether the time offset applies to other Tx and Rx of other RACH related signals and channels. 3. Date of Next TSG-RAN4 Meetings: RAN WG4 Meeting #116 August 25 – August 29, 2025 India (TBC), IN RAN WG4 Meeting #116-bis October 13 – October 17, 2025 Prague, CZ