3GPP TSG-RAN WG4 Meeting #116bis                                                           R4-2514210

Prague, Czech Republic, 13th – 17th October, 2025

Source:	Samsung

Title:	Discussion on remaining issues of OD-SSB SCell operation

Agenda item:			4.21.1

Document for:	Discussion

1	Introduction

In this meeting, we will continue giving some analyses on the remaining issue for OD-SSB based on WF [1] agreed in last meeting.

2	Remaining issues

2.1 Issue 1-4-4: OD-SSB based L3 reporting

Issue 1-4-4: OD-SSB based L3 reporting Agreement The discussion of the issue shall not impact the RAN4 core part completion of the WI.

Regarding this remaining issue involving RAN2 impact, we still have negative attitude on introducing OD-SSB based L3 reporting. The reasons are:

- There are two ways to report L3 measurement results: conditional and periodical. In the current spec., reporting mechanisms containing event triggered reporting and periodical reporting are widely used. In Rel-18 FR2 CA enhancement, the type of one-time report configuration (i.e., reportOnScellActivation-r18) was defined, to be specific, SCell activation MAC-CE can be used to trigger UE to report valid L3 measurement results during FR2 unknown SCell activation.

- No doubt that some benefits, for example, expedite SCell activation process based on timely measurement and report when OD-SSB is activated, can be obtained by introducing OD-SSB MAC CE triggered MR. However, new enhancements with RAN2 on the existing signalling (i.e., reportOnScellActivation-r18) or additional MAC-CE triggered solution analysis including: signalling design and definition, applicable scenarios (not only scenario #2A), ReportConfigNR definition linked to a measID, etc, maybe is necessary, and will be discussed thoroughly. We have concerns on the huge workload will be brought to RAN2 if RAN4 truly to introduce OD-SSB MAC CE triggered MR in the future.

- At the same time, from our understanding, the existing event triggered reporting, e.g., A1/A2, can also work in L3 measurement based on OD-SSB just with small TTT configuration (e.g., extreme condition: 0ms)

-- ASN1START -- TAG-TIMETOTRIGGER-START TimeToTrigger ::= ENUMERATED { ms0, ms40, ms64, ms80, ms100, ms128, ms160, ms256, ms320, ms480, ms512, ms640, ms1024, ms1280, ms2560, ms5120} -- TAG-TIMETOTRIGGER-STOP -- ASN1STOP

Proposal 1: Not to introduce OD-SSB MAC CE triggered MR in Rel-19 NES OD-SSB based L3 measurement

- RAN4 can to discuss the feasibility/applicability of the existing reporting mechanism, e.g., event triggered reporting A1/A2 with small TTT configuration in Rel-19 NES OD-SSB based measurement and report

3	Conclusion

In this contribution, we provided our viewpoints on remaining issues on OD-SSB based L3 reporting for NES OD-SSB. The following proposals are obtained:

Proposal 1: Not to introduce OD-SSB MAC CE triggered MR in Rel-19 NES OD-SSB based L3 measurement

- RAN4 can to discuss the feasibility/applicability of the existing reporting mechanism, e.g., event triggered reporting A1/A2 with small TTT configuration in Rel-19 NES OD-SSB based measurement and report

Reference

[1] R4-2512327, WF on RRM requirements for Netw_Energy_NR_enh_Part1 v1