3GPP TSG-RAN WG4 Meeting#116bis		R4-2514212

Prague, Czech, Oct 13th -17th, 2025

Agenda item:	7.1.4

Source: 	Spreadtrum, UNISOC

Title: 	Discussion on HPUE requirements for 4TX

Document for:	Discussion

Introduction

In RAN-P#109 meeting, a new WID [1] was approved. In this contribution, we want to provide some initial views on HPUE requirements.

The core part of WI: High power UE (HPUE) for NR single carrier operation - Specify the UE RF requirements for single carrier TDD to support 4Tx for CPE/FWA UE with 4x26dBm PA configurations - Co-existence study for ACLR, specify ACLR requirements if needed - Specify the UE RF requirements including maximum output power (MOP), MPR requirements, etc. - Supporting 4x4 UL MIMO and TxD - Specify the band specific requirements for exemplary bands - Example bands: n41, n77/n78, n79, n104 - Maximum output power is up to 32 dBm - RAN4 to determine whether to reuse PC1 or introduce new power class - PC1.5 with 2Tx for FDD bands for handheld and FWA UE - Specify the UE RF requirements including maximum output power (MOP), MPR requirements, etc. - Supporting 2x2 UL MIMO and TxD - Baseline PA configuration: 2 x 26 dBm - Specify the band specific requirements for exemplary bands - Example bands: n1, n25 - PC1.5 1Tx for TDD bands for FWA UE - Study the possibility of applying PC1.5 with 2Tx MPR/A-MPR requirements to PC1.5 with 1Tx, if feasible, specify the corresponding UE RF requirements, including maximum output power (MOP), MPR/A-MPR requirements, etc. - Baseline PA configuration: 1 x 29 dBm - Example bands: n41, n77/n78, n79 - This work is to start in 2026 Q4 - 2Tx for TDD bands for FWA UE - Study the possibility of applying PC1 with 4Tx A-MPR/MPR requirements to PC1 with 2Tx, if feasible, specify the corresponding UE RF requirements, including maximum output power (MOP), MPR/A-MPR requirements, etc. - Baseline PA configuration: 2 x 29 dBm - Example bands: n41, n77/n78, n79, n104 - Maximum output power is up to 32 dBm - RAN4 to determine whether to reuse PC1 or introduce new power class - This work is to start in 2026 Q4

Discussion

4Tx for CPE/FWA with 4*26dBm PA configurations

Maximum output power

According to the WID, the maximum output power is 32dBm, it is observed that to fully utilize the full power of 4*26dBm PA configurations. However, the maximum output power is 32dBm, whether to reuse PC1 or introduce new power class needs to be further discussed. If we reuse PC1, the MOP of PC1 is 31dBm, in order to achieve 32dBm, power boosting, adding upper limit of tolerance or other solutions needs to be defined. If we reuse PC1.x, the MOP of PC1.X can be 32dBm, Pcmax,c tolerance and power class related issues need to be address. From LTE to NR, all power classes keep the step of 3dB to increase and evolution except PC1, and with the entry into 6G evolution, maybe more new power classes will be introduced. In our understanding, new power class can be introduced to define the MOP of 32dBm.

Proposal 1: New power class (e.g., PC1bis) can be introduced to define the MOP of 32dBm.

Pcmax tolerance

For 4Tx of UL MIMO, Pcmax,c tolerance is defined in TS 38.101-1, the upper limit power is 29dBm, based on the MOP of 32dBm, Pcmax,c tolerance needs to be modified. It can be modified as follow.

Table 1: PCMAX,c tolerance in closed-loop spatial multiplexing scheme for 4Tx

| PCMAX,c (dBm) | Tolerance TLOW(PCMAX_L,c) (dB) | Tolerance THIGH(PCMAX_H,c) (dB) |
| --- | --- | --- |
| 24.5 ≤ PCMAX,c ≤ 32 | 3.0 | 2.0 |
| 23.5 ≤ PCMAX,c < 24.5 | 5.0 | 2.0 |
| 22.5 ≤ PCMAX,c < 23.5 | 5.0 | 3.0 |
| 21.5 ≤ PCMAX,c < 22.5 | 5.0 | 4.0 |
| 18 ≤ PCMAX,c < 21.5 | 5.0 |  |
| 13 ≤ PCMAX,c < 18 | 6.0 |  |
| -40 ≤ PCMAX,c < 13 | 7.0 |  |

Proposal 2: PCMAX,c tolerance in closed-loop spatial multiplexing scheme for 4Tx of 32dBm can be modified to Table 1.

Conclusions

In this paper, we provide our initial views on HPUE requirements, we have made the following proposals.

Proposal 1: New power class (e.g., PC1bis) can be introduced to define the MOP of 32dBm.

Proposal 2: PCMAX,c tolerance in closed-loop spatial multiplexing scheme for 4Tx of 32dBm can be modified to Table 1.

Table 1: PCMAX,c tolerance in closed-loop spatial multiplexing scheme for 4Tx

| PCMAX,c (dBm) | Tolerance TLOW(PCMAX_L,c) (dB) | Tolerance THIGH(PCMAX_H,c) (dB) |
| --- | --- | --- |
| 24.5 ≤ PCMAX,c ≤ 32 | 3.0 | 2.0 |
| 23.5 ≤ PCMAX,c < 24.5 | 5.0 | 2.0 |
| 22.5 ≤ PCMAX,c < 23.5 | 5.0 | 3.0 |
| 21.5 ≤ PCMAX,c < 22.5 | 5.0 | 4.0 |
| 18 ≤ PCMAX,c < 21.5 | 5.0 |  |
| 13 ≤ PCMAX,c < 18 | 6.0 |  |
| -40 ≤ PCMAX,c < 13 | 7.0 |  |

- Reference

- RP-252952, New WID: UE RF enhancements for NR FR1, Phase 5. Huawei. TSG-RAN WG 109 meeting