3GPP TSG-RAN WG4 Meeting #116-bis 	R4-2514246

Prague, CZ, October 13th – 17th, 2025

Agenda Item: 6.18.2

Source: MediaTek inc.

Title: Discussion on RRM performance requirements for XR

Document for: Discussion

# Discussion on performance part

In this contribution, we provide further discussion on the performance part issues captured in the WF [1].

## Discussion on Test case list

Issue 2-2-1: Scenarios for the definition of the test cases <Way forward> Discuss the proposals - Proposal 1: it is proposed to define test cases for NR-SA. - Proposal 2: it is proposed to define test cases for FR1 and FR2-1. - Other proposals not precluded. Issue 2-2-2: Accuracy requirements <Agreement> RAN4 confirms the agreement from RAN4 #113 in R4-2420103: - Reuse existing measurement accuracy requirements for UE under XR operation. Issue 2-2-3: Per UE and per FR gaps <Way forward> Discuss the following options - Option 1: Test cases are specified only for per-UE MGs - Option 2: Test cases are specified for both per-UE and per-FR MGs Issue 2-2-4: Type 1 and Type 2 gaps <Way forward> Discuss the following options - Option 1: Test cases are defined for both Type 1 and Type 2 gaps - Option 2: Test cases are defined only for Type 1 gaps Issue 2-2-5: DRX proposals <Way forward> Discuss the following proposals: - Proposal 1: Define the test cases for the cases without DRX and with DRX. - Proposal 2: Test cases without DRX should have higher priority than with DRX. - Test cases with DRX should cover at least inter-frequency measurements with Type 1 MG. - Inter-RAT with DRX and with Type 1 MG can be developed with lower (second) priority. - Focus on non-DRX test cases for Type 2 MGs. - Proposal 3: Each type of test case is specified for either DRX or non-DRX but not both. Issue 2-2-6: Test target <Way forward> - RRM test cases should verify - Proposal 1: accuracy requirements - Proposal 2: delay extension with gap cancellation - Proposal 3: Reception/transmission without scheduling restrictions during cancelled gap occasions - Proposal 4: The UE processing timeline of receiving the DCI-based MG skipping indication Issue 2-2-7: List of test cases <Way forward> - Define test cases for - Proposal 1: Intra-frequency measurements with gaps - Proposal 2: Inter-frequency measurements with gaps - Proposal 3: Inter-RAT E-UTRAN FDD measurements - Proposal 4: Inter-RAT E-UTRAN TDD measurements - Proposal 5: Inter-frequency L1-RSRP with MG - Proposal 6: L3 and L1 measurements without gaps

Issue 2-2-1: Scenarios for the definition of the test cases:

Given that the WI already cover SA and FR1 and FR2-1, then test cases can be defined accordingly.

- RAN4 RRM to define test cases for NR-SA, and FR1 and FR2-1.

Issue 2-2-3: Per UE and per FR gaps:

In legacy test cases configurations, majority of test cases for gaps are defined per-UE MGs. Therefore, our preference it to define test cases based on per-UE MGs.

- RAN4 RRM to define test cases based on per-UE MGs configuration only.

Issue 2-2-4: Type 1 and Type 2 gaps:

Type-1 MG is the baseline configuration for all MGs. Also, the main difference of Type-1 and Type-2 is that Type-2 is configured with priority and association to a specific MO, which is irrelevant to the gap cancellation.

- RAN4 RRM to define test cases for Type-1 gap only.

Issue 2-2-5: DRX proposals:

There is no direct link of MG cancellation and DRX, hence it is not necessary to define test cases for with/without DRX. Nevertheless, each type of test case is specified for either DRX or non-DRX but not both.

- RAN4 to define TCs that each type of test case is specified for either DRX or non-DRX but not both.

Issue 2-2-6: Test target:

RRM group already agreed that accuracy requirements are not impacted. In addition, it is RAN1 responsibility to assess DCI processing timeline. Besides, the DCI used for MG cancellation is based on existing legacy DCI with additional bit to cover MG cancellation, hence there is no need to define test case for UE processing timeline of receiving the DCI-based MG skipping indication. On the other hand, delay extension due to MG cancellation and Tx/Rx without scheduling restrictions during cancelled gap occasions can be tested with new TCs.

- RAN4 to not define TCs for accuracy requirements due to MG cancellation nor TCs for UE processing timeline of receiving the DCI-based MG skipping indication.

- RAN4 to define test cases for (i) delay extension with gap cancellation and (ii) reception/transmission without scheduling restrictions during cancelled gap occasions.

Issue 2-2-7: List of test cases:

RAN4 to define the test cases for inter-frequency measurement with gaps because this is the typical use case of MG.

- RAN4 to define TCs with working assumption that the gap is configured for inter-frequency measurement with gaps.

## Discussion on test case configuration

Issue 2-3-1: Type 2 test configurations:

Type-1 MG is the baseline configuration for all MGs. Also, the main difference of Type-1 and Type-2 is that Type-2 is configured with priority and association to a specific MO, which is irrelevant to the gap cancellation.

- RAN4 RRM to define test cases for Type-1 gap only and there is no need to consider Type-2 gap in defining test cases.

## Discussion on MG cancellation model

Issue 2-4-1: Cancelation model for XR test cases:

Issue 2-4-2: Cancelation ratio for Type 1 gaps:

Issue 2-4-3: Cancelation ratio for Type 2 gaps:

On the topic of which measurement gaps to be cancelled during test cases, RAN4 RRM should use a repetitive sequence where the test machine configures the UE with DCI containing MG cancellation according to this sequence.

- RAN4 RRM should use a repetitive sequence where the test machine configures the UE with DCI containing MG cancellation according to this sequence.

## Discussion on DCI aspects

Issue 2-5-1: Toffset between DCI and start of MG:

On the topic of 2-5-1: Toffset between DCI and start of MG, 5ms offset is sufficient for defining test cases, however, given that a UE capability is defined for 3ms then it is reasonable to consider 3ms in the test case. However, a single test case can include both 5ms and 3ms with UE capability used as a condition.

- RAN4 RRM define a single test case that include time offset of 5ms and 3ms with UE capability used as a condition.

# Summary

In this contribution, we have the following proposals:

Proposal 1: RAN4 RRM to define test cases for NR-SA, and FR1 and FR2-1.

Proposal 2: RAN4 RRM to define test cases based on per-UE MGs configuration only.

Proposal 3: RAN4 RRM to define test cases for Type-1 gap only.

Proposal 4: RAN4 to define TCs that each type of test case is specified for either DRX or non-DRX but not both.

Proposal 5: RAN4 to not define TCs for accuracy requirements due to MG cancellation nor TCs for UE processing timeline of receiving the DCI-based MG skipping indication.

Proposal 6: RAN4 to define test cases for (i) delay extension with gap cancellation and (ii) reception/transmission without scheduling restrictions during cancelled gap occasions.

Proposal 7: RAN4 to define TCs with working assumption that the gap is configured for inter-frequency measurement with gaps.

Proposal 8: RAN4 RRM to define test cases for Type-1 gap only and there is no need to consider Type-2 gap in defining test cases.

Proposal 9: RAN4 RRM should use a repetitive sequence where the test machine configures the UE with DCI containing MG cancellation according to this sequence.

Proposal 10: RAN4 RRM define a single test case that include time offset of 5ms and 3ms with UE capability used as a condition.

# References

- R4-2512144, “WF on RRM requirements for NR_XR_Ph3”, Nokia, Bengaluru, India, August, 2025.