3GPP TSG-RAN WG4 Meeting # 116bis	R4-2514249

# Prague, CZ, 13th - 17th, Oct 2026

Source: 	Samsung

Title: 	Discussion on Rel-20 HPUE

Agenda Item:	7.1.4

Document for:	Discussion

- Introduction

In RAN#109, the Rel-20 WI for UE RF enhancement phase 5 has been approved [1], with HPUE as one of the objectives with four items as following. Two out of four commence from 2026 Q4. We outline our views on the first 2 items in this paper.

- Discussion

## 2.1 Four Tx for CPE/FWA with MOP up to 32dBm

The first thing needs to settle down is the power class, i.e., define a new power class, or reuse PC1 but with different MOP upper limit tolerance, and in any case, the maximum output power is up to 32dBm.

It is advisable to avoid the introduction of new power class, as all requirements are built upon the power class, anything new would leads to significant spec impact, hence it is not preferable considering the time constrains of this work. Therefore, we propose to reuse PC1 with higher upper tolerance, i.e., 3dB.

Proposal 1: Reuse PC1 with MOP tolerance as +3/-3 dB.

One issue identified for the reuse of PC1 is the differention of the existing PC1. The existing PC1 was introduced for many bands, all of them is with Note 6 indicating that PC1 UE is not targeted for smartphone form factor which is also true for the new PC1. Among them, only one band, band 14 has an additional Note 7 saying that PC1 requirements for Band n14 are applicable for public safety scenario only. From this, it can be understood that there are cases PC1 was defined but not for public safety scenario.

Observation 1: PC1 is specified for n14 for public safety scenario only, but not for other bands.

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  |  |  | 26 | +2/-3 | 23 | ±2 |
| n2 |  |  |  |  | 26 | +2/-33 | 23 | ±23 |
| n3 |  |  |  |  | 26 | +2/-33 | 23 | ±23 |
| n5 |  |  |  |  | 26 | +2/-33 | 23 | ±2 |
| n7 | 316 | +2/-3 |  |  | 26 | +2/-33 | 23 | ±23 |
| n8 |  |  |  |  | 26 | +2/-33 | 23 | ±23 |
| n12 |  |  |  |  |  |  | 23 | ±23 |
| n13 |  |  |  |  | 26 | +2/-3 | 23 | ±2 |
| n14 | 316,7 | +2/-3 |  |  | 26 | +2/-3 | 23 | ±2 |
| n18 |  |  |  |  |  |  | 23 | ±2 |
| n20 |  |  |  |  |  |  | 23 | ±23 |
| n24 |  |  |  |  |  |  | 23 | +2/-33 |
| n25 | 316 | +2/-3 |  |  | 26 | +2/-33 | 23 | ±23 |
| n26 |  |  |  |  | 26 | +2/-33 | 23 | ±23 |
| n28 |  |  |  |  | 26 | +2/-33 | 23 | +2/-2.5 |
| n30 |  |  |  |  |  |  | 23 | ±2 |
| n31 | 316 | +2/-3 |  |  |  |  | 23 | ±2 |
| n34 |  |  | 295 | +2/-3 | 26 | +2/-3 | 23 | ±2 |
| n38 |  |  |  |  |  |  | 23 | ±2 |
| n39 |  |  | 295 | +2/-3 | 26 | +2/-3 | 23 | ±2 |
| n40 | 316 | +2/-3 | 295 | +2/-3 | 26 | +2/-3 | 23 | ±2 |
| n41 | 316 | +2/-3 | 295 | +2/-33 | 26 | +2/-33 | 23 | ±23 |
| NOTE 6: Generally, PC1 UE is not targeted for smartphone form factor. NOTE 7: The UE power class 1 requirements for Band n14 are applicable for public safety scenario only. |  |  |  |  |  |  |  |  |

Take n41 where PC1 is already supported as example, if PC1 with 4Tx is also introduced, there would be 2 kinds of PC1 both targeted for FWA/CPE but with different MOP tolerance. Distinguish between them would be necessary for both NW scheduling purpose and conformance test purpose. One alternative is proposed as following, by leveraging existing FSPC 4Tx-TxD capability (i.e., txDiversity4Tx-r18) which can also be used for single band.

| n41 | 316,x | +2/-3 | 295 | +2/-33 | 26 | +2/-33 | 23 | ±23 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NOTE 6: Generally, PC1 UE is not targeted for smartphone form factor. NOTE 7: The UE power class 1 requirements for Band n14 are applicable for public safety scenario only. NOTE X: Tolerance is modified to +3/-3 dB if UE indicates txDiversity4Tx-r18. |  |  |  |  |  |  |  |  |

Proposal 2: Leverage the existing capability 4Tx-TxD(i.e., txDiversity4Tx-r18) to indicate for the new PC1 with modified power class tolerance. Spec modification as following, take n41 as example.

| n41 | 316,x | +2/-3 | 295 | +2/-33 | 26 | +2/-33 | 23 | ±23 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NOTE 6: Generally, PC1 UE is not targeted for smartphone form factor. NOTE 7: The UE power class 1 requirements for Band n14 are applicable for public safety scenario only. NOTE X: Tolerance is modified to +3/-3 dB if UE indicates txDiversity4Tx-r18. |  |  |  |  |  |  |  |  |

For UL MIMO, the exiting maximum PC is PC1.5, PC1 has not been specified yet, therefore there should not be any ambiguity.

For MPR/A-MPR requirements, it is suggested to adopt 20dB to perform the evaluation. As 20dB is assumed for FWA/CPE for PC1.5 for both single CC and intra-band CA.

Proposal 3: Adopt 20dB antenna isolation to perform MPR/A-MPR evaluation.

## 2.1 PC1.5 with 2Tx for FDD bands

PC1.5 with 2Tx was already specified for TDD bands, for both UL MIMO and 2Tx-TxD, for both handheld UE and FWA. In our view, all existing general requirements specified for PC1.5 TDD originally are applicable for FDD as well.

Proposal 4: For PC1.5 with 2Tx, all existing general requirements defined based on TDD are applicable for FDD.

In terms of band specific requirements, A-MPR and RSD needs to be checked. For A-MPR, the following need to be defined. For RSD, the delta value from PC3(or PC2) to PC1.5 for UE supporting TxD needs to be specified.

| NR band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n1 | NS_01 | NS_100 | NS_05 | NS_05U | NS_48 | NS_49 |  | Reserved |
| n25 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  | Reserved |

Proposal 5: RAN4 to define A-MPR for n1 and n25 for PC1.5

-N1: NS_05/05U, NS_48, NS_49

-N25: NS_03/03U

Proposal 6: RAN4 to define Reference Sensitivity degradation for PC1.5 FDD bands for UE indicating TxD

- Conclusion

- 4Tx for CPE/FWA for TDD band

Proposal 1: Reuse PC1 with MOP tolerance as +3/-3 dB.

Observation 1: PC1 is specified for n14 for public safety scenario only, but not for other bands.

Proposal 2: Leverage the existing capability 4Tx-TxD(i.e., txDiversity4Tx-r18) to indicate for the new PC1 with modified power class tolerance. Spec modification as following, take n41 as example.

| n41 | 316,x | +2/-3 | 295 | +2/-33 | 26 | +2/-33 | 23 | ±23 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NOTE 6: Generally, PC1 UE is not targeted for smartphone form factor. NOTE 7: The UE power class 1 requirements for Band n14 are applicable for public safety scenario only. NOTE X: Tolerance is modified to +3/-3 dB if UE indicates txDiversity4Tx-r18. |  |  |  |  |  |  |  |  |

Proposal 3: Adopt 20dB antenna isolation to perform MPR/A-MPR evaluation.

- 2Tx PC1.5 for FWA/handheld for FDD band

Proposal 4: For PC1.5 with 2Tx, all existing general requirements defined based on TDD are applicable for FDD.

Proposal 5: RAN4 to define A-MPR for n1 and n25 for PC1.5.

-N1: NS_05/05U, NS_48, NS_49

-N25: NS_03/03U

Proposal 6: RAN4 to define Reference Sensitivity degradation for PC1.5 FDD bands for UE indicating TxD.

# 4. Reference

[1] RP-252942, New WID: UE RF enhancements for NR FR1, Phase 5, RAN#109