3GPP TSG-RAN WG4 Meeting#116bis                                R4-2514270

# Prague, CZ, 13th – 17th Oct, 2025

Agenda item:	6.8.3.1

Source: 	ZTE Corporation, Sanechips

Title: 	Further discussion on Coexistence spurious emission requirements

Document for:	Approval

Introduction

In the RAN#103 meeting, new WID for Rel-19 BS RF evolution has been approved and it’s expected to start the normative work from this April meeting. In the RAN#106 meeting, the following objective is added for the reply LS to ETSI TFES. In this contribution, we would like to share our understanding on the simulation assumptions for the further evaluation in RAN4 and initial calibration results captured in the attached excel sheet.

Transmitter co-existence spurious emission requirements - Evaluate co-existence transmitter spurious emission limits according to the request from ETSI/TFES - Improve transmitter co-existence spurious emission requirements if needed. - Note: start the work after March, 2025 and focus on bands in the upper region of FR1 with priority above 5.9 GHz.

Discussion

## 2.1. Simulation assumption for BS2BS coupling loss analysis

Details on urban macro network layout model for BS2BScoupling loss analysis are listed in Table 2.1-1 and 2.1-2.

Table 2.1-1: Single operator layout for urban macro

| Parameters |  | Values | Remark |
| --- | --- | --- | --- |
| Network layout |  | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |  |
| Inter-site distance |  | 500 m |  |
| BS antenna height |  | 25 m |  |
| UE location | Outdoor/indoor | Outdoor and indoor |  |
|  | Indoor UE ratio | 20% | ` |
|  | Low/high Penetration loss ratio | 50% low loss, 50% high loss |  |
|  | LOS/NLOS | LOS and NLOS |  |
|  | UE antenna height | Same as 3D-UMa in TR 36.873 |  |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS - UE distance (2D) |  | 35 m |  |
| Channel model |  | UMa |  |
| BS2 UE Shadowing correlation |  | Between cells: 1.0 Between sites: 0.5 |  |
| Pathloss model for BS2BS |  |  | Free space pathloss model |
| Pathloss model for BS2UE |  | Please use the UMa model in the TR 38.901 7.4.1 Uma channel model |  |

Table 2.1-2: Multi operators layout for urban macro

| Parameters | Values | Remark |
| --- | --- | --- |
| Multi operators layout | Un-coordinated operation (100% Grid Shift) |  |

| Coordinated Operation: each network with co-location of sites |  |
| --- | --- |

The antenna configurations for 2GHz and U6GHz WA BS are summarized in the following Table 2.1-3.

Table 2.1-3: Antenna configurations for 2GHz and U6GHz BS in Macro Urban scenario

| U6GHz BS antenna configuration |  | 2GHz BS antenna configuration |  |
| --- | --- | --- | --- |
| Parameter | Macro urban | Parameter | Macro urban |
| Am | 30 dB | Am | 30 dB |
| SLAv | 30 dB | SLAv | 30 dB |
| j3dB | 90 deg. | j3dB | 90 deg. |
| q3dB | 65 deg. | q3dB | 65 deg. |
| GE,max | 6.4 dBi | GE,max | 6.4 dBi |
| Msub | 3 | Msub | 3 |
| dv,sub | 0.7l m | dv,sub | 0.7l m |
| qsubtilt | 3 deg. | qsubtilt | 3 deg. |
| M | 8 | M | 4 |
| N | 16 | N | 8 |
| dh | 0.5l m | dh | 0.5l m |
| dv | 2.1l m | dv | 2.1l m |
| jescan | deg. | jescan | deg. |
| r | 1 | r | 1 |
| Ptx | 46 dBm | Ptx | 46 dBm |
| qmech | 6 deg. | qmech | 6 deg. |

The extended AAS antenna modelling is provided in the following Table 2.1-4.

Table 2.1-4: Extended AAS antenna modelling

| Description | Equation |
| --- | --- |
| Peak normalized element radiation pattern | Aθ,φ=−min−−min12φφ3dB2,Am−min12θ−90θ3dB2,SLAv ,Am |
| Peak gain normalized element radiation pattern | AEθ,φ=GE,max+Aθ,φ |
| Sub-array excitation | wm=1Msubexpj2πm−1dv,subλsinθsubtilt |
| Sub-array radiation pattern | Asubθ,φ=AEθ,φ+10log10m=1Msubwmvm2 , where vm=expj2πm−1dv,subλcosθ |
| Array excitation | wm,n=1MNexpj2πm−1dvλsinθetilt−n−1dℎλcosθetiltsinφescan |
| Composite array radiation pattern | AAθ,φ=Asubθ,φ+10log10m=1Mn=1Nwm,nvm,n2 , where vm,n=expj2πm−1dvλcosθ+n−1dℎλsinθsinφ |

For the calibration procedures for BS2BS coupling loss:

Step 1: Calibrate the pathloss between aggressive BSs and victim BSs without antenna gain;

Step 2: Calibrate the pathloss between aggressive BSs and victim BSs with aggressor’s element gain (including 3dB diversity gain)

Step 3: Calibrate the pathloss between aggressive BSs and victim BSs with Aggressor_Tx_element_gain+ Aggressor_Tx_sub_array gain+Aggressor_Tx_array gain;

Step 4: Calibrate the pathloss between aggressive BSs and victim BSs with Aggressor_Tx_element_gain+ Aggressor_Tx_sub_array gain+Aggressor_Tx_array gain+Victim_Rx_element_gain+Victim_Rx_sub_array gain+Victim_Rx_array_gain;

Proposal 1: propose to adopt the simulation assumptions in clause 2.1 for the BS2BS coupling loss calibration and to derive the requirement definition with the certain correlation level for transmitter spurious emission;

## 2.2. Simulation assumption for BS2UE coupling loss analysis

Details on Dense Urban network layout model for BS2UE coupling loss analysis are listed in Table 2.2-1 and 2.2-2.

Table 2.2-1: Single operator layout for dense urban

| Parameters |  | Values | Remark |
| --- | --- | --- | --- |
| Network layout |  | Fixed cluster circle within a macro cell. | note1 |
| Number of micro BSs per macro cell As |  | 3 | 3 cluster circles are in a macro cell. 1 cluster circle has 1 micro BS. As shown in Figure 2.2-1 |
| Radius of UE dropping within a micro cell |  | < 72.17 m | Calculated based ISD. Nokia CR number: Sqrt(3)/4*ISD/3 |
| BS antenna height |  | 10 m |  |
| UE location | Outdoor/indoor | Outdoor and indoor |  |
|  | Indoor UE ratio | 80 % |  |
|  | 50% low loss, 50% high loss | Low/high Penetration loss ratio |  |
|  | LOS/NLOS | LOS and NLOS |  |
|  | UE antenna height | Same as 3D-UMi in TR 36.873 |  |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS - UE distance (2D) |  | 3m |  |
| Channel model |  | UMi | TR 38.901 Umi channel model should be used |
| Shadowing correlation |  | Between Micro site: 0.5 |  |
| Note 1: Micro BS is randomly dropped on an edge of the cluster circle. All UEs communicate with micro BS, i.e. macro cell is only used for determining position of micro BS. As a layout of macro cell, hexagonal grid, 19 macro sites, 3 sectors per site model with wrap around with ISD =500 m is assumed. |  |  |  |

Figure 2.2-1: Network layout for dense urban

Table 2.2-2: Multi operators layout for dense urban

| Parameters | Values | Remark |
| --- | --- | --- |
| Multi operator layout | Cluster circle is coordinated | Note 1 |
| Minimum distance between micro BSs in different operator | 10 m |  |
| Note 1: Macro cell is collocated. Micro BS itself is randomly dropped. |  |  |

The antenna configurations for 2GHz and U6GHz WA BS are summarized in the following Table 2.2-3

Table 2.2-3: Antenna configurations for 2GHz and U6GHz BS in Dense Urban scenario

| U6GHz BS antenna configuration |  | 2GHz BS antenna configuration |  |
| --- | --- | --- | --- |
| Parameter | Micro BS | Parameter | Micro BS |
| Am | 30 dB | Am | 30 dB |
| SLAv | 30 dB | SLAv | 30 dB |
| j3dB | 90 deg. | j3dB | 90 deg. |
| q3dB | 65 deg. | q3dB | 65 deg. |
| GE,max | 6.4 dBi | GE,max | 6.4 dBi |
| Msub | N/A | Msub | N/A |
| dv,sub | N/A | dv,sub | N/A |
| qsubtilt | N/A | qsubtilt | N/A |
| M | 8 | M | 8 |
| N | 8 | N | 8 |
| dh | 0.5l m | dh | 0.5l m |
| dv | 0.7l m | dv | 0.7l m |
| r | 1 | r | 1 |
| Ptx | 37 dBm | Ptx | 37 dBm |
| qmech | N/A | qmech | N/A |

The AAS antenna modelling without sub-array configuration is provided in the following Table 2.2-4.

Table 2.2-4: Composite antenna pattern for AAS antenna modelling without sub-array configuration

| Parameter | Values |
| --- | --- |
| Composite Array radiation pattern in dB | For beam i: the super position vector is given by: the weighting is given by: |

For the calibration procedures for BSUE coupling loss:

Step 1: Calibrate the pathloss between aggressive BSs and victim UEs without antenna gain;

Step 2: Calibrate the pathloss between aggressive BSs and victim UEs with aggressor’s element gain (including 3dB diversity gain)

Step 3: Calibrate the pathloss between aggressive BSs and victim UEs with Aggressor_Tx_element_gain

+Aggressor_Tx_array gain;

Proposal 2: propose to adopt the simulation assumptions in clause 2.2 for the BS2UE calibration and to derive the requirement definition with the certain correlation level for transmitter spurious emission;

Conclusions

In this contribution, we shared some initial views on the applicability improvement of TRP in Annex in RAN4 and proposals are made as following:

Proposal 1: propose to adopt the simulation assumptions in clause 2.1 for the BS2BS coupling loss calibration and to derive the requirement definition with the certain correlation level for transmitter spurious emission;

Proposal 2: propose to adopt the simulation assumptions in clause 2.2 for the BS2UE coupling loss calibration and to derive the requirement definition with the certain correlation level for transmitter spurious emission;

References

- RP-240829,New WID: WI resulting from discussion on BS RF requirement evolution, Approved.

- R4-2419797	Way Forward for [113][303] NR_BS_RF_Part2_CLTA

- R4-2419827	Way Forward for [113][302] NR_BS_RF_Part1_E_EIRP
[4] R4-2419828	Way Forward for [113][304] NR_BS_RF_Part3_OTA_TRP

- R4-2504769	Way Forward for Tx spurious emissions co-existence limits re-evaluation, Huawei, approved.

- R4-2504298   Huawei, HiSilicon

- R4-2504267,   Ericsson