3GPP TSG-RAN WG4 Meeting #116bis	R4-2514307

Prague, Czech Republic, October 13th – October 17th, 2025

Source:	Nokia

Title:	On General Aspects of AI/ML for NR Air Interface Phase 1

Agenda item:	6.11.2

Document for:	Discussion

Introduction

In RAN4#116 meeting, RAN4 continued the discussion on the general aspects of AI/ML based on the objectives agreed in RAN4#107, WI ‎‎[1].

In this paper, we share our further views on the following topics, mainly for UE sided model cases:

- Life-Cycle Management related aspects

- Generalization related aspects

Discussion

Life-Cycle Management related aspects

The following agreement was reached in RAN4#116 meeting:

Agreement: - Do not define performance monitoring delay requirements for LCM purpose - If the monitoring delay is identificated as critical in the future, this issue can be revisited. No agreements on performance monitoring accuracy.

Agreement (RAN 1 #120b)

For UE-sided model monitoring Type 1 option 2, support the following combination for inference report type and monitoring report type:

| Monitoring report type Inference report type | P report | SP report | AP report |
| --- | --- | --- | --- |
| AP report | Not support | Not support | Support |  |
| SP report | Not support | Support | Support |  |
| P report | Support | Support | Support |  |

Agreement (RAN 1 #120bis)

- For UE-sided model monitoring Type 1 option 2, regarding the type of resource for the set for monitoring, support at least periodic CSI-RS, semi-persistent CSI-RS and SSB

Agreement (RAN1 #119)

At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), support to reuse CSI framework for the configuration for monitoring result report in L1 signaling:

- Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring

  - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration

    - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams

  - FFS on whether to support all the combination on time domain behavior of the reportConfigType for inference report and the reportConfigType for monitoring report

  - FFS on the timing related issues

  - UE measures the dedicated resource set(s) for monitoring.

RAN1 achieved following agreement for performance monitoring mechanism and metric:

RAN1#121 Agreement

For beam prediction accuracy report for monitoring, the report quantity RS-PAI is Np (0 ≤ Np≤ N)

- Where Np is the total count of accurate reference signal prediction instance(s) that meets the condition, among N latest transmission occasion(s) of monitoring resources that no later than CSI reference resource corresponding to the CSI report for monitoring

  - condition:

    - for the transmssion occasion of monitoring resources, it has a linked inference report

    - at least one of the nrofBestBeamforMonitoring-r19 identified CSI-RS resources, or SS/PBCH Block resources mapped to one of the nrofreportedpredictedrs-r19 reported P-CRI(s) or P-SSBRI(s), of the linked report of the CSI Reporting Setting for inference

  - if this condition is met, the transmission occasion is counted as an accurate reference signal prediction instance; otherwise, it is not counted as an accurate reference signal prediction instance.

- Where N = 1, 3, 7, 15 is configured in CSI-ReportConfig

- the size of CSI field associated with the RS-PAI is log2(N+1)

RAN1 has agreed on the definition of report quantity of beam prediction accuracy, RS-PAI, which can be defined as the total count of accurate reference signal prediction instance(s), Np, that meets the conditions and Np is calculated over N valid performance monitoring instances. The need to determine the N latest transmission occasion(s) of monitoring resources is considered.

- Accordingly, RAN4 will need to define new accuracy requirements for the performance monitoring metric for AI/ML BM, RS-PAI, corresponding to number of N latest transmission occasion(s) of monitoring resources. It is critical to verify the accuracy of this performance monitoring metric in RAN4, since many network sided decision regarding life-cycle management of AI/ML configuration depend on this metric. Furthermore, performance degradation can be caused if the accuracy of the performance monitoring metric is not guaranteed since the UE may continue using a badly performing AI/ML configuration.

It is clear from the RAN1 definition of the report quantity of the beam prediction accuracy report for monitoring that the performance monitoring metric is different from the beam prediction accuracy metrics considered in RAN4 as part of the performance requirements. Therefore, a separate accuracy requirement is needed in RAN4 to verify the accuracy of performance monitoring metric, RS-PAI.

RAN4 needs to define accuracy requirements for UE-assisted monitoring where the UE reports the beam prediction accuracy, RS-PAI, in BM-Case1 and BM-Case2. The accuracy of RS-PAI should be tested at least in static radio conditions.

For CSI prediction use case, following agreement was reached:

Issue 1-1: Performance monitoring Agreement: RAN4 will introduce requirements for Type 3 performance monitoring for CSI prediction FSS which requirements are to be introduced.

An open issue is whether it is feasible to define and test a reporting accuracy requirement for performance monitoring in AI/ML-based CSI prediction. A potential test framework can be specified in RAN4, where the UE calculates and reports SGCS values under static and controlled radio conditions. Since the environment remains unchanged, any variation in reported values indicates potential inaccuracies in the UE’s reporting mechanism.

By evaluating the consistency of SGCS reports across multiple monitoring instances, the test equipment (TE) can determine whether the UE reliably reports the performance monitoring metric. This approach avoids the need for ground-truth measurements or dynamic environmental changes and still provides a valid basis for defining accuracy requirements. Although this framework differs from conventional RAN4 test designs, it is suitable for verifying the stability of UE-reported metrics. Under static conditions, reported values should remain stable, and significant deviations may signal issues in the AI/ML-based prediction or reporting function.

Since RAN1 has already defined the performance monitoring mechanism and SGCS reporting structure, RAN4 must define corresponding test procedures to ensure the reliability of UE-reported metrics. Without such testing, performance monitoring reports used for AI/ML life-cycle management may be unreliable, potentially leading to degraded network performance. RAN4 can define an accuracy requirement using a statistical consistency-based approach. For example, it may require that in at least 90% of test iterations, reported SGCS values remain within a defined tolerance range under static conditions. This ensures the reported values are stable and suitable for performance monitoring.

RAN4 can define reporting accuracy requirements for AI/ML-based CSI prediction by using a test framework that monitors SGCS consistency under static radio conditions.

RAN4 should define reporting accuracy requirements for CSI prediction performance monitoring using a test framework that evaluates the stability of reported values in a fixed environment.

Generalization related aspects

The following Agreements were reached in RAN4#115:

Agreement: The high level and general guidelines for the generalization test are provided as follow. The exact decisions are to be made case by case. - Study and, if feasible, define requirements for each AI/ML functionality - Define one test per UE capability as a minimum - FFS >1 test per UE capability - Define a minimum set of test configurations (including NW sided conditions if any) as mandatory for testing of AI/ML-enabled Feature Potential areas to consider for generalization testing: - gNB array parameters - port layouts, array size, antenna virtualization - propagation conditions - Deployment scenarios - Carrier frequencies - Speeds - Indoor/outdoor - Bandwidths - SNR

Furthermore, on the issue of static and non-static scenarios for testing, following agreement was reached in RAN4#114bis meeting:

Agreement: - Static scenarios are assumed by default. Non-static scenarios are introduced on a use-case specific basis if needed for testing. - FFS whether a mixed datset can be created for testing generalization, and whether such a mixed dataset would be a static or non-static scenario

Generalization - Metric and test framework

The generalization capability of a functionality can impact the performance of the functionality, for example if the functionality is not generalized enough across a set of scenarios, this will result in frequent switch of the functionality resulting in the degradation of the performance.

Less generalized functionalities across a set of scenarios can result in frequent switching of functionality/configurations resulting in performance degradation.

To mitigate the problems caused by less generalized functionalities, it is important to ensure that the functionality is generalized enough. Ensuring the generalization aspects of functionality across different scenarios and conditions is not straightforward. At the outset we need the scenarios and conditions that needs to be identified and aligned. KPIs for generalization need to be defined. And then the tests need to be carried out on all the scenarios and based on the validation result it can be decided if the functionality is generalized enough. To carry out all these identified steps, it becomes imperative to have a test framework to test the generalization aspects of a functionality.

In order to address the testability issue of generalization aspects of AI/ML-based functionalities, we propose a new metric, referred to as the “Generalization Score”, to quantitatively indicate the generalization capabilities of an AI/ML functionality or model. This metric can be used to verify/check whether an AI/ML functionality (developed and validated for a specific baseline setup) generalizes well to other scenario(s) and/or configuration(s) of interest.

For example, if it is identified that there are 3 scenarios of interest. Any one scenario of these 3 scenarios can be the baseline setup (depending on functionality implementation) and the remaining 2 scenarios will then be used as other set ups to test the generalization aspects. An overall generalization score can be calculated as a function of individual scenario verdicts, e.g., as simple average or weighted average of the individual generalization verdict of each Scenario ‘j’ (i.e., Scenario ‘1’ to Scenario ‘N), where the verdict of Scenario ‘j’ = 1 if the functionality generalizes to Scenario ‘j’ else it is equal to 0.

Concept of Generalization Score

The underlying principle of the Generalization Score is to evaluate the performance of an AI/ML functionality across multiple test setups beyond the baseline setup. For each setup, a binary verdict is determined:

- Verdict_j = 1 if the performance requirements are fulfilled for scenario j

- Verdict_j = 0 otherwise

The Overall Generalization Score is then computed as a simple or weighted average of the verdicts across all test setups using:

Overall Generalization Score= (VerdictScenario1+…+VerdictScenarioN)N×100%

where N is the number of selected scenarios for generalization testing.

Now this score can be generally applied to the generalization capabilities of any AIML enabled functionality.

RAN4 to use the following metric, indicative of generalization capabilities of AI/ML functionality/configurations, to verify the generalization performance of the functionality/configuration in different scenarios:

Overall Generalization Score= (VerdictScenario1+…+VerdictScenarioN)N×100%

- Where N is the number of selected scenarios for generalization testing

  - The value of N to be discussed

- Verdict_j = 1 if the performance requirements are fulfilled for scenario j

- Verdict_j = 0 otherwise

Need for a Generalization Test Framework

To enable effective evaluation of the generalization aspects of an AIML enabled functionality, it is important to:

- Identify and align the relevant scenarios and configurations.

- Define KPIs for generalization.

- Establish a test framework to evaluate the functionality/model across all identified setups.

Such a framework, coupled with the proposed Generalization Score metric, will provide RAN4 with a standardized mechanism to evaluate and compare the generalization performance of AI/ML-based functionalities (e.g., BM, CSI Feedback) across multiple deployment conditions.

RAN4 should consider discussing on a generalization test framework that can help to evaluate the generalization capabilities of the AIML enabled functionalities.

Conclusion

In this contribution, we addressed the following topics mainly for UE sided model cases:

- Life-Cycle Management related aspects

- Generalization related aspects

Following Observations and Proposals were made in this paper:

Proposal 1: RAN4 needs to define accuracy requirements for UE-assisted monitoring where the UE reports the beam prediction accuracy, RS-PAI, in BM-Case1 and BM-Case2. The accuracy of RS-PAI should be tested at least in static radio conditions.

Observation 1: RAN4 can define reporting accuracy requirements for AI/ML-based CSI prediction by using a test framework that monitors SGCS consistency under static radio conditions.

Proposal 2: RAN4 should define reporting accuracy requirements for CSI prediction performance monitoring using a test framework that evaluates the stability of reported values in a fixed environment.

Observation 2: Less generalized functionalities across a set of scenarios can result in frequent switching of functionality/configurations resulting in performance degradation.

Proposal 3: RAN4 to use the following metric, indicative of generalization capabilities of AI/ML functionality/configurations, to verify the generalization performance of the functionality/configuration in different scenarios:

Overall Generalization Score= (VerdictScenario1+…+VerdictScenarioN)N×100%

- Where N is the number of selected scenarios for generalization testing

  - The value of N to be discussed

- Verdict_j = 1 if the performance requirements are fulfilled for scenario j

- Verdict_j = 0 otherwise

Proposal 4: RAN4 should consider discussing on a generalization test framework that can help to evaluate the generalization capabilities of the AIML enabled functionalities.

- References

[1] RP-250792, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, RAN#107, March 12-14, 2025, Incheon, South Korea.