3GPP TSG-RAN WG4 Meeting #116	R4-2514342

Bengaluru, India, August 25th – August 29th, 2025

Source:	Nokia

Title:	Open issues the measurement framework for the new IOT TDD Punctured frame

Agenda item:	4.27.4

Document for:	Discussion

Introduction

In the plenary meeting RAN #105, a new WI for Rel-19 was approved to introduce a TDD frame operation for NTN [1]. The work item is designed to provide extended support of 3GPP NB-IoT features leveraging existing satellite systems deployed in Non-Geostationary Satellite Orbit (NGSO). It also aims at enabling low-complexity payloads for satellite operations that are non-reliant on a diplexer.

To achieve the aforementioned goals, the WID was designed to introduce an NTN-compatible TDD frame, that while based on minimum necessary changes on the NB-IoT NTN FDD specification provides a new approach to TDD periodicity.  RAN4 has analysed the potential impacts and specified the requirements for the new frame structure. Currently for the core part, just a limited number of open issues were left for further assessment [2]. In this document we discuss these issues.

Discussion

RRC re-establishment (6.5A)

In this document we provide our views in the open issues presented in [2]. We will start by the open issue in Issue 3:

## Issue 3: RRC re-establishment (6.5A) Agreement (online - Wed): - Tsearch_NB1-NC,i in RRC re-establishment requirement is - If the target cell is known, then Tsearch_NB1-NC,i = 0 ms. If the target cell is unknown, Tsearch_NB1-NC,i = Ksatellite,i *2.52 sec. - FFS the definition of Ksatellite,i

The definition of Ksatellite in this work item has been revised depending on whether or not the UE has information about the measurement periods of the neighbor cells. If there are overlapping cells associated to different satellites to be measured in the same DL measurement opportunity, then the value of Ksatellite,I is equal to 2. Otherwise, it is equal to 1.

For re-establishment requirements, if the UE is provided with information about the DL periods of the neighbor cells, the value of Ksatellite is equal to 2 if there are overlapping DL opportunities from cells associated to different satellites, otherwise, Ksatellite equal to 1.

RRC Connection redirection to Non-Anchor Carrier (6.9A)

The following item was left for further discussion in RAN4:

To be further discussed and concluded in RAN4#116-bis: - Change ‘110 ms’ in the following bullets to an integer number of 90ms + DL-to-UL gap within the 90 ms TDD pattern, where the exact value is TBD upon completion of RAN1 spec. - TRRC_procedure_delay: It is the RRC procedure for processing the received message “CarrierConfigDedicated-NB”. It shall be less than 110 ms. - When the NPUSCH ACK transmission for the received RRC message takes longer than 110ms, the overall RRC connection redirection delay may be extended. - TBD for the values of 10 ms, 40 ms, and 8 ms in the following bullets, and whether to retain the DL-Bitmap-NB IE upon completion of RAN1 and RAN2 spec: - TDL-UL switch: It is the time between the end of the last subframe in the repetition period of NPDCCH received on the non-anchor carrier and the start of the first subframe in the repetition period of the corresponding NPUSCH transmitted on the non-anchor carrier. - Option 1: TDL-UL switch is 8 ms. - Option 2: TDL-UL switch is 50 ms (THALES, Iridium) TDL-UL switch: It is the time between the end of the last subframe in the repetition period of NPDCCH received on the nonanchor carrier and the start of the first subframe in the repetition period of the corresponding NPUSCH transmitted on the non-anchor carrier. TDL-UL switch is 8 ms.

There is a fundamental difference between the current frame structure with previous frame structures utilized for NB-IoT and NB-IoT NTN, therefore it is important that these distinctions are reflected in the requirements, such that the specification is harmony with potential implementations of the solutions.

For RRC Connection redicretion to Non-Anchor Carrier, we have the following requirement, about the maximum time delay:

Tconnection_redirect_non-anchor = TRRC_procedure_delay + Tperiod_DL_bitmap + TUL_grant + TDL-UL switch + TNPUSCH

Where each factor is defined as follows:

- TRRC_procedure_delay: is the RRC procedure for processing the received RRC message (CarrierConfigDedicated-NB). It shall be less than 110 ms.

- Tperiod_DL_bitmap: It is the periodicity of the downlink subframe configuration for downlink transmission on the non-anchor carrier. It is configured via IE DL-Bitmap-NB and can be 10 ms or 40 ms.

- TUL_grant: It is the time required to acquire uplink grant from the non-anchor carrier for transmitting NPUSCH on the non-anchor carrier. The value of TUL_grant depends on Tperiod_DL_bitmap and the number of repetitions of NPDCCH used in the non-anchor carrier.

- TDL-UL switch: It is the time between the end of the last subframe in the repetition period of NPDCCH received on the non-anchor carrier and the start of the first subframe in the repetition period of the corresponding NPUSCH transmitted on the non-anchor carrier. TDL-UL switch is 8 ms.

- TNPUSCH: It is the time required to transmit NPUSCH on the non-anchor carrier. The value of TNPUSCH depends on the number of repetitions of NPUSCH used in the non-anchor carrier.

Obs: RAN4 has agreed that for the Tperiod_DL_bitmap for the IoT NTN TDD frame structure in Rel-19 is equal to 0.

Upon receiving the reconfiguration message, the UE has up to 110 ms to decode and process it. Considering the relevant frame structure where there is a periodicity of 90 ms between DL opportunities, if the UE process this message in less than 90 ms, the UE is capable of monitoring for UL grant on the non-anchor carrier already in the next DL set of opportunities. If the UE utilizes the full 110 ms, it will take the second set of DL opportunities (180 ms after the current one).

We do not favor hard-coding one requirement (e.g. an integer number of 90ms + DL-to-UL gap) as the frame structure might be adapted in the future by other parameters but utilizing the same logical structure (consecutive D DL and U UL frames that repeat periodically at every N frames). But on the other hand, it is important to consider that there are “discrete” opportunities for the UE to acquire the UL grant. This can be introduced in the TUL_grant, as this parameter was firstly introduced exactly to define the opportunities the UE have to acquire the DL information.

Therefore we propose:

For Connection redirection to non-anchor carrier, when the TDD frame structure is in use, the value of TUL_grant shall be described as: “It is the time required to acquire uplink grant from the non-anchor carrier for transmitting NPUSCH on the non-anchor carrier. The value of TUL_grant depends on the interval between the UE processing the RRC message and the next DL opportunity in the non-anchor carrier and the number of repetitions of NPDCCH used in the non-anchor carrier.”

If the above is accepted, there is only the need to indicate that upon receiving the UL grant the UE needs to wait until the next UL opportunity in the non-anchor carrier to transmit the NPUSCH:

For Connection redirection to non-anchor carrier, when the TDD frame structure is in use, the value of TDL-UL switch shall be described as: “It is the time between the end of the last subframe in the repetition period of NPDCCH received on the non-anchor carrier and the start of the first subframe in the repetition period of the corresponding NPUSCH transmitted on the non-anchor carrier. TDL-UL switch is equal to the DL-to-UL gap.”

Based on the discussion above, redefining these two parameters, TNPUSCH will not need to be redefined.

Conclusion

In this paper we discussed the TDD punctured pattern for IoT NTN UEs. The following Observations and Proposals were made:

Proposal 1: For re-establishment requirements, if the UE is provided with information about the DL periods of the neighbor cells, the value of Ksatellite is equal to 2 if there are overlapping DL opportunities from cells associated to different satellites, otherwise, Ksatellite equal to 1.

Proposal 2: For Connection redirection to non-anchor carrier, when the TDD frame structure is in use, the value of TUL_grant shall be described as: “It is the time required to acquire uplink grant from the non-anchor carrier for transmitting NPUSCH on the non-anchor carrier. The value of TUL_grant depends on the interval between the UE processing the RRC message and the next DL opportunity in the non-anchor carrier and the number of repetitions of NPDCCH used in the non-anchor carrier.”

Proposal 3: For Connection redirection to non-anchor carrier, when the TDD frame structure is in use, the value of TDL-UL switch shall be described as: “It is the time between the end of the last subframe in the repetition period of NPDCCH received on the non-anchor carrier and the start of the first subframe in the repetition period of the corresponding NPUSCH transmitted on the non-anchor carrier. TDL-UL switch is equal to the DL-to-UL gap.”

- References

- RP-242415, “New WID on introduction of IoT-NTN TDD mode”, September 2024

- R4-2512149,  WF on RRM requirements for IoT_NTN_TDD,   Moderator (Qualcomm Incorporated),  Bengaluru, India, August 25th – 29th, 2025