3GPP TSG-RAN4 Meeting #116-bis	R4-2514392

Prague, CZ, Oct. 13 – 17, 2025

Source:	Nokia

Title:	General AC test method and XR OTA

Agenda item:	4.13.1

Document for:	Discussion

Introduction

Based on the agreement in the WF from RAN4 #116 meeting [1], in this paper, we’d like to present our views on the related issues for discussion of new OTA test methods brought up in the 3GPP meetings. We also would like to present our views on the LS from CTIA related to test procedure deviations in [2], and the LS from RAN5 related to test tolerance values for Release 19 NR FR1 TRP and TRS enhancements in [3].

# WF in [1]

# Topic #1: General AC test method and XR OTA

### Sub-topic 1-1 General aspect of OTA test method

Issue 1-1-2: Adding NFTF OTA TRP test method in TR 38.870

Agreements:

  - This alternative method is not in Rel-19 scope. But can be further discussed in RAN4 with a full package of test setup, test procedure, MU assessment.

### Sub-topic 1-2 Test method and Performance metric for XR devices

Issue 1-2-2: If TRP/TRS is kept as baseline, whether a new additional XR performance metric can be considered (as optional)?

Agreements:

  - For XR, keep current TRP/TRS metric as baseline and add NHPRP/NHPRS into TR. RAN4 is not intended to preclude NHPRP/NHPRS as long as it is technically justified.

  - FFS future requirements, single metric will be used for future XR OTA requirements.

LS From CTIA [2]

The CTIA Certification OTA Working Group would like to update relevant industry organizations on the NB-IoT NTN OTA test development.

Discussion

The CTIA Certification OTA Working Group recently finalized the inclusion of NB-IoT NTN OTA testing for NTN bands 255 and 256 as informative in Version 9 of the CTIA OTA Test Plan.

CTIA Certification has approved TRP and TIS OTA test cases for NB-IoT NTN using the 3GPP TS 36.521-4 test procedures for maximum output power and reference sensitivity as the baseline for TRP and TIS, respectively. To address excessive test time concerns, especially the sensitivity search for TIS, the approved OTA test procedures deviate from current 3GPP TS 36.521-4 test procedures as follows:

- For both TRP and TIS, implementation of a co-located configuration was selected where the UE location is the same as the satellite location, and the kOffset is set to 0ms.

- For TRP and TIS, P0(Nominal PUSCH) may be adjusted higher in order to ensure that the DUT transmits at maximum TX power

- For TIS, the uplink is control only, i.e. no user plane PUSCH data shall be configured.

The CTIA Certification OTA Working Group elected to include NB-IoT NTN OTA testing as only informative in Version 9 of the CTIA Certification OTA Test Plan to allow time for system provider validation of the deviations listed above. The CTIA Certification OTA Working Group expects to transition the NB-IoT NTN OTA testing to normative in Version 10 of the CTIA Certification OTA Test Plan pending successful authorization of test equipment; additionally, CTIA Certification plans to incorporate NR NTN OTA test cases in Version 10. Version 9 of the CTIA Certification OTA Test Plan is planned for release by the end of this year. Version 10 of the CTIA Certification OTA Test Plan is planned for release in the second half of 2026.

The CTIA Certification OTA Working Group kindly requests that RAN4 and RAN5 consider incorporation of the deviations listed above when considering the development of corresponding NB-IoT NTN OTA test cases.

The CTIA Certification OTA Working Group will provide further updates as the validation progresses and as the updates for Version 10 of the CTIA Certification OTA Test Plan are finalized.

LS from RAN5 [3]

# Overall description

Following the finalization of the RAN4 Rel-19 NR FR1 TRP TRS core and performance parts as part of the TRP_TRS_MIMO_OTA_Ph3 Work item, RAN5 will introduce a new Rel-19 OTA conformance work item by RAN5#109 in November 2025, to deliver conformance test cases for Rel-19 NR FR1 TRP and TRS requirements. It is general process that RAN5 will have primary responsibility for Measurement Uncertainty (MU) finalization for Rel-19 FR1 TRP TRS for which a preliminary/placeholder MU table has been added in Annex B of TR 38.870 and TS 38.161 for Rel-19 specific updates. Further optimizing and finalizing MU values and then determination of Test Tolerance (TT) for the defined test requirements will occur in RAN5 and incorporated in TS 38.561.

With regards to the derivation of TT for OTA testing, there has been precedence of RAN4 kindly providing recommendations on TT along with core requirements, methodology and MU for 5G NR FR1.

1) In Rel-17, RAN5 request in [8] was responded by RAN4 by providing the test tolerance recommendations in [9] to be utilized as a package together with Rel-17 core requirements.

2) In Rel-18, RAN5 request in [10] was responded by RAN4 by providing the test tolerance recommendations in [11] to be utilized as a package together with Rel-18 core requirements.

Based on the process being followed in RAN4 to derive NR FR1 TRP TRS requirements built on performance campaign test results, RAN5 understands that there is a deep relation between radiated performance requirement definition in RAN4, derived from results of a measurement campaign, and TT as indicated by the precedence in 1) and 2) above.

Therefore, RAN5 respectfully requests RAN4 to provide recommendations on test tolerance values for Release 19 NR FR1 TRP and TRS enhancements along with a thorough description of the core requirements definition process so implications from MU and TT can be clarified and considered during the finalization of Rel-19 FR1 TRP TRS TT in the RAN5 conformance test specification.

# 2	Actions

To RAN4

ACTION: 	 RAN5 respectfully asks RAN4 to take the above information, including the indicated RAN5 conformance work item timeline, into account and provide RAN5 with recommendations on test tolerance values for Release 19 NR FR1 TRP and TRS along with a thorough description of the core requirements definition process so that implications from MU and TT can be clarified and considered during the definition of Release 19 FR1 TRP TRS TT in RAN5 conformance test specification.

Discussion

Adding NFTF OTA TRP test method in TR 38.870

The TRP/TRS test requirements and test methods based on the anechoic chamber (AC) have long been defined in 3GPP standards. This approach has formed the foundation for enabling implementation and ensuring broad acceptance of the standard in the market.

Since the introduction of 5G, 3GPP standard documents have grown significantly with many companies contributing. As a result, the 3GPP specifications have expanded rapidly, reflecting the wide variety of technologies and implementation options introduced. This growth has also led to increased complexity in the 5G system, as frequently noted by UE vendors, network vendors, and operators. At the same time, the market continues to depend heavily on robust support for the fundamental functionalities. For this reason, “lean” 6G has become a central theme in discussions of future standardization.

That said, 3GPP must continue to evolve in step with advances in telecommunications. Studying new OTA methods, such as the Reverberation Chamber (RC) method and Near-Field to Far-Field Transform (NFTF), is important to ensure technical progress. The results of such studies can be captured in 3GPP TRs to broaden our collective understanding.

However, when it comes to defining the official standard OTA test method, we need to proceed with caution. The principle of “lean” 6G should guide our decisions: the standard should remain streamlined and avoid unnecessary inclusiveness. In practice, this means selecting a single primary method as the standard OTA approach, with only one carefully chosen alternative to address minority market requirements.

It is valuable to study new methods, such as RC and NFTF, in TRs based on which 3GPP continues to evolve in step with advances in telecommunications

Keep a “lean” 6G in mind; we should keep the standard streamlined when choosing the standard OTA test methods.

The official standard should be limited to one primary OTA method and a single alternative to address minority needs.

We often see both online and offline discussions struggling with how to handle new technologies and new OTA test methods when they are brought up by companies to the 3GPP meetings. When new technologies or new OTA test methods are proposed in 3GPP, they should be seen not as a source of confusion but as opportunities for necessary evolution, with a clear and standardized procedure guiding their study. By documenting pros and cons, comparisons to baseline methods, and typical use cases in TPs, we can provide guidance for selecting which methods may ultimately become standards while also recording the evolution steps of OTA testing in 3GPP.

New OTA test method proposals brought by companies to 3GPP should be regarded as opportunities for advancing and evolving the standard. There should be no struggle in handling them.

With TPs documenting pros and cons, baseline comparisons, and use cases to support clear decisions on future standards while recording the evolution of OTA testing.

Standardize the procedure of studying the new OTA test methods proposed in the 3GPP meetings.

Standardize the TP structure to capture pros and cons, baseline comparisons, and use cases of new methods, so that the confusion and struggles often faced in 3GPP meetings when new methods are proposed can be transformed into a clear and transparent decision-making process.

NFTF OTA TRP test method can be introduced in TR 38.870 following the standardized new method study procedure.

New additional XR performance metric NHPRP/NHPRS

Based on the above observations and proposals, we suggest that the new XR OTA methods proposed in the recent 3GPP meeting follow a standardized study procedure and be documented in TPs using the agreed structure.

Since this method is still new and lacks supporting performance data and baseline comparisons, we suggest waiting for such results while recording the study of NHPRP/NHPRS under the standardized procedure in the TPs, which will serve as a reference for future standardization decisions in 3GPP.

Since NHPRP/NHPRS method is still new and lacks supporting data and baseline comparisons, we suggest waiting for such results while recording the study of NHPRP/NHPRS under the standardized procedure in the TPs, which will serve as a reference for future standardization decisions in OTA WI standards.

Reply LS from CTIA

Thanks to CTIA for informing 3GPP about the NTN test procedure deviation to address excessive test time concerns, especially the sensitivity search for TIS. The test procedure deviation for TRP and TIS OTA test cases of NB-IoT NTN compared with the 3GPP TS 36.521-4 test procedures for maximum output power and reference sensitivity are as follows:

- For both TRP and TIS, implementation of a co-located configuration was selected where the UE location is the same as the satellite location, and the kOffset is set to 0ms.

- For TRP and TIS, P0(Nominal PUSCH) may be adjusted higher in order to ensure that the DUT transmits at maximum TX power

- For TIS, the uplink is control only, i.e. no user plane PUSCH data shall be configured.

These deviations will be carefully considered and can be studied within 3GPP to support the development of NB-IoT NTN OTA test cases. We expect the test data to highlight the differences between the 3GPP and CTIA test procedures. Such differences can be documented in 3GPP TPs following the standardized study procedure for new OTA methods, providing a clear reference to facilitate final decision-making in 3GPP standardization.

Integrate the above points when drafting the reply to the LS from CTIA.

Record and document the study procedures and results of OTA test procedure deviations in TPs using a standardized study procedure, which facilitates final decision-making in 3GPP standardization.

Reply LS from RAN5

Thanks to RAN5 for sending LS to request the detailed test tolerance values for Release 19 NR FR1 TRP and TRS, along with a comprehensive description of the core requirements definition process. RAN4 will investigate how to determine the TT values in alignment with the core requirements definition process. This investigation may take some time to complete.

In the LS, RAN5 also mentioned that:

With regards to the derivation of TT for OTA testing, there has been precedence of RAN4 kindly providing recommendations on TT along with core requirements, methodology and MU for 5G NR FR1.

1) In Rel-17, RAN5 request in [8] was responded by RAN4 by providing the test tolerance recommendations in [9] to be utilized as a package together with Rel-17 core requirements.

2) In Rel-18, RAN5 request in [10] was responded by RAN4 by providing the test tolerance recommendations in [11] to be utilized as a package together with Rel-18 core requirements

It appears that an implicit procedure for close cooperation between RAN4 and RAN5 has already existed in the past. However, RAN5 still needs to send an LS to RAN4 to formally trigger this procedure. If this procedure could be standardized, RAN4 and RAN5 could potentially collaborate more efficiently. Additionally, it would clarify the final results/report of the OTA WI in RAN4. Specifically, the recommendations on TT, along with core requirements, methodology, and MU from RAN4 to RAN5, could be included directly in the RAN4 OTA WI final report without requiring a separate LS from RAN5.

An implicit procedure for close cooperation between RAN4 and RAN5 has already existed in the past. However, RAN5 still needs to send an LS to RAN4 to formally trigger this procedure.

Standardize the procedure for how RAN4 and RAN5 could potentially collaborate more efficiently. The recommendations on TT, along with core requirements, methodology, and MU from RAN4 to RAN5 could be included directly in the RAN4 OTA WI final report.

Conclusion

The following observations and proposals were made:

- Keep a “lean” 6G in mind; we should keep the standard streamlined when choosing the standard OTA test methods.

- The official standard should be limited to one primary OTA method and a single alternative to address minority needs.

New OTA test method proposals brought by companies to 3GPP should be regarded as opportunities for advancing and evolving the standard. There should be no struggle in handling them.

With TPs documenting pros and cons, baseline comparisons, and use cases to support clear decisions on future standards while recording the evolution of OTA testing.

- Standardize the procedure of studying the new OTA test methods proposed in the 3GPP meetings.

- Standardize the TP structure to capture pros and cons, baseline comparisons, and use cases of new methods, so that the confusion and struggles often faced in 3GPP meetings when new methods are proposed can be transformed into a clear and transparent decision-making process.

- NFTF OTA TRP test method can be introduced in TR 38.870 following the standardized new method study procedure.

- Since NHPRP/NHPRS method is still new and lacks supporting data and baseline comparisons, we suggest waiting for such results while recording the study of NHPRP/NHPRS under the standardized procedure in the TPs, which will serve as a reference for future standardization decisions in OTA WI standards.

- Integrate the above points when drafting the reply to the LS from CTIA.

- Record and document the study procedures and results of OTA test procedure deviations in TPs using a standardized study procedure, which facilitates final decision-making in 3GPP standardization.

An implicit procedure for close cooperation between RAN4 and RAN5 has already existed in the past. However, RAN5 still needs to send an LS to RAN4 to formally trigger this procedure.

- Standardize the procedure for how RAN4 and RAN5 could potentially collaborate more efficiently. The recommendations on TT, along with core requirements, methodology, and MU from RAN4 to RAN5 could be included directly in the RAN4 OTA WI final report.

- References

- R4-2512693, Way Forward for [116][337] TRP_TRS_MIMO_OTA_Ph3

- R4-2511714, LS on NB-IoT NTN OTA Testing

- R5-255348, Rel-19 NR TRP TRS test tolerance values with description (LS from RAN5)