3GPP TSG RAN WG4 Meeting #116-bis	R4-2514396

Prague, CZ 13th – 17th Oct. 2025

Agenda Item:	7.4.3.2

Source: 	Ericsson

Title:	Discussion on Rel-20 RRM Enhancements Phase 6- (e)RedCap UE enhancement

Document for:	Discussion

# Introduction

In September's Plenary, NR RRM Phase 6 got approved with the following objective:

### 4.1 Objective of SI or Core part WI or Testing part WI - Specify the following requirements for (e)RedCap UEs [RAN4] - Relaxed RLM/BFD RRM requirements - Use R17 RLM/BFD relaxation for legacy UE as a baseline - Support of less than 5 MHz channel bandwidth (15 PRB transmission bandwidth configuration and 12 PRB SSB) - RF requirements, considering example bands: n100, n106, n26, n28 and n85 - RRM requirements including relaxed RLM/BFD RRM requirements - Note 1: Use R18 3 MHz requirements for non-RedCap UE as a baseline - Note 2: Consider both 1Rx and 2Rx (e)RedCap UEs and HD-FDD - For intra-band co-located contiguous CA, specify necessary RRM requirements to support data scheduling on SCell prior to UE transmitting valid CSI report (to be started in Q4 2026) [RAN4]

According to the Objectives, RAN4 should extend support of less than 5 MHz CBW for RedCap. Also, relaxed RLM/BFD requirements should be extended to (e)RedCap UEs. This is the first meeting of the WI. This contribution looks at the RedCap RRM requirements that may be impacted due to the introduction of Less than 5 MHz CBW and relaxed RLM/BFD.

# RedCap Less than 5 MHz

NR Less than 5 MHz is specified in Rel-18. According to the WID Rel-18, 3 MHz requirements for non-RedCap UE should be baseline for RedCap UE supporting less than 5 MHz. If we look at Rel-18 NR Less than 5 MHz requirements, the support of reduced CBW impacted the following high-level categories:

- RRC_CONNECTED state Mobility requirements

- Radio Link Monitoring Performance

- Link Recovery procedure

- Measurement requirements performance

In this section, we analyse the impact of less than 5 MHz on RedCap RRM requirements.

## General

For Redcap UE with 2Rx (FD-FDD, TDD, and HD-FDD), introducing support for less than 5 MHz does not impact Handover, RLM performance, link recovery performance, or intra/inter frequency measurement. Hence, RAN4 should reuse Rel-18 less than 5 MHz requirements for RedCap Less than 5 MHz for UE with 2 Rx and HD-FDD UE.

For 2Rx RedCap for all three duplex modes (FD-FDD, TDD, and HD-FDD), RAN4 should reuse the following requirements from NR less than 5 MHz in Rel-18 as baseline.

- Handover

- RLM/ BFD

- Intra-frequency/Inter-frequency measurement

Furthermore, RedCap supports both FR1 and FR2. Support for NR less than 5 MHz is limited to FR1 and is specified for FDD spectrums.

Rel-18 NR less than 5 MHz only supports FR1 and is dedicated to FDD spectrums.

Hence, only the NR FR1 cell cases will be subject to impact on RedCap Less than 5 MHz.

RAN4 only needs to define the requirements for NR FR1 for RedCap less than 5 MHz.

## Mobility requirements

From the Rel-18 less than 5 MHz work item, we know that reduced BW has an impact on RRC_CONNECTED state mobility. To be precise, in the handover requirements, cell search has a performance impact that depends on channel bandwidth (CBW). The SSB index reading and MIB readings require 3 extra samples for NR less than 5 MHz, which are captured in clause 6.1.1.2.2 in TS 38.133. As Rel-18 is the baseline for RedCap, with less than 5 MHz requirements, we can capture the same impact for 2 RX RedCap.

However, RedCap also supports UE with 1Rx. For 1 Rx RedCap UE, HO requirements shall be further investigated to assess the impact on SSB and MIB reading performance at 1 Rx RedCap with less than 5 MHz CBW.

For 1 Rx RedCap less than 5 MHz, T∆ should be further investigated based on the simulation outcome.

## RLM/ BFD Performance

For NR less than 5 MHz, RAN4 specified that RLM out-of-sync (OOS) is declared when the link quality falls below Qout, corresponding to a hypothetical PDCCH BLER of 10%, using PDCCH parameters of AL=4 CCEs at 12 PRB or AL=8 CCEs at 15 PRB with 4 dB boosting (Table 8.1.2.1-3 of TS 38.133). Additionally, in-sync (IS) is declared when the link quality is above Qin, corresponding to a BLER of 2%, with AL = 2 CCEs at 12 PRB or AL = 4 CCEs at 15 PRB, as defined in Table 8.1.2.1-4 of TS 38.133. In both cases, link quality is evaluated for SSB only according to the network configuration. For BFD, the threshold Qout_LR_SSB also corresponds to a hypothetical PDCCH BLER of 10% with the same transmission parameters as for OOS, i.e., AL=4 CCEs at 12 PRB and AL=8 CCEs at 15 PRB, as specified in Rel-18 Table 8.5.2.1-2 of TS 38.133. We believe these requirements can be directly reused for RedCap UEs with 2Rx to support RedCap Less than 5 MHz.

Rel-18 NR less than 5 MHz requirements for RLM and BFD can be reused for 2Rx RedCap less than 5 MHz to ensure equivalent coverage and reliability.

For RedCap UEs, the current method of ensuring coverage equivalence between 1Rx and 2Rx relies on evaluating over a wider bandwidth (48 vs. 24 PRBs) and using higher aggregation levels (16/8 vs. 8/4), as specified in Table 8.1B.2.1-1

for OOS and Table 8.1B.2.1-2 for IS. However, a 1Rx RedCap UE operating on less than 5 MHz CBW may not rely on the legacy AL scaling approach to achieve the same link robustness as a 2 Rx RedCap UE, creating a risk of reduced RLM coverage and performance imbalance. Therefore, RAN4 should first agree on the coverage equivalence between 1Rx and 2Rx RedCap UEs on less than 5 MHz as a baseline principle.

RAN4 should agree on coverage equivalence between 1Rx and 2Rx RedCap UEs on less than 5 MHz CBW.

## Measurement Performance

In Rel-18, the impact of reducing bandwidth on the SSB index reading delay is captured for both intra- and inter-frequency measurements. The time period for time index detection for 12 PRB SSB is captured in Table 9.2.6.2-12 and Table 9.3.9.1-5 for intra- and inter-frequency measurements, respectively. We think that these specified requirements can be reused for 2 Rx RedCap UEs. However, the SSB index reading and MIB reading impact should be studied for 1 Rx RedCap less than 5 MHz.

RAN4 to analyze the SSB index reading and MIB reading impact for 1 Rx RedCap with less than 5 MHz based on simulation outcome.

# RedCap Relaxed RLM/BFD

In this section, we discuss RLM/BFD relaxation support for RedCap. RLM/ BFD Relaxation is first introduced in Rel-17. Hence, it is agreed in the WID to use R17 RLM/BFD relaxation for legacy UE as a baseline in [1].

## Scenarios and Criteria for Relaxed RLM/BFD

In Rel-17, RLM/BFD Relaxation was specified for the following four cases.

- SSB based RLM/BFD measurement relaxation in FR1

- CSI-RS based RLM/BFD measurement relaxation in FR1

- SSB based RLM/BFD measurement relaxation in FR2

- CSI-RS based RLM/BFD measurement relaxation in FR2

For RedCap with RLM/BFD Relaxation, we think the same cases should be supported and specified in Rel-20.

RedCap RLM/BFD relaxation should support SSB and CSI-RS based RLM/BFD relaxation for both FR1 and FR2.

Furthermore, low mobility criteria and good serving cell quality were specified as relaxed measurement criteria for NR. We believe both criteria are applicable for RedCap RLM/BFD relaxation.

NR RLM/BFD relaxed measurement criteria can be reused for RedCap RLM/BFD relaxation.

## RLM/BFD Relaxation Requirements for RedCap UE

In Rel 17, the RLM relaxation criteria for NR are defined in Clause 8.1.1.1 in TS 38.133, and the BFD relaxation criteria are described in Clause 8.5.1.1 in TS 38.133. The relaxation parameter K factor was introduced to scale the RLM/BFD evaluation windows in stable conditions, thereby achieving energy efficiency while maintaining minimum safety bounds to preserve mobility reliability. As RedCap UE, with 2 RX capabilities, is similar to NR UE Capabilities, we think the requirements specified for NR Relaxed RLM and BFD can be reused for RedCap RLM and BFD relaxation.

NR RLM and BFD relaxation requirements should be reused for 2 RX RedCap RLM/BFD relaxation.

However, for 1 RX RedCap UE, as shown in Table 1, the cycle length was doubled compared to the 2 RX RedCap. In Rel-17, the RLM relaxation requirements were captured as depicted in Table 2. Hence, we need to study the new relaxation factors further and adjust the K factors for RedCap UE with 1 Rx. A similar study should also be conducted for BFD.

Table 1: Evaluation period TEvaluate_out_SSB,RedCap and TEvaluate_in_SSB,RedCap for FR1 for 1Rx RedCap UE

| Configuration | TEvaluate_out_SSB,RedCap (ms) | TEvaluate_in_SSB,RedCap (ms) |
| --- | --- | --- |
| no DRX | Max(400, Ceil(20 P) TSSB) | Max(100, Ceil(5 P) TSSB) |
| DRX cycle≤320 ms | Max(400, Ceil(30 P) Max(TDRX,TSSB)) | Max(100, Ceil(7.5 P) Max(TDRX,TSSB)) |
| DRX cycle>320 ms | Ceil(20 P) TDRX | Ceil(5 P) TDRX |
| NOTE: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length. |  |  |

Table 2: Evaluation period TEvaluate_out_SSB_Relax for FR1

| Configuration | TEvaluate_out_SSB_Relax (ms) |
| --- | --- |
| Max(TDRX,TSSB) ≤80 ms | Max(200 K3 NOTE3, Ceil(15 K1NOTE2 P) Max(TDRX,TSSB)) |
| 80 ms < Max(TDRX,TSSB) ≤160 ms | Ceil(15 P) Max(TDRX,TSSB) |
| NOTE 1: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length and no longer than 80 ms. NOTE 2: K1 = 4 for Max(TDRX,TSSB) ≤40 ms and K1 = 2 for 40 ms<Max(TDRX,TSSB) ≤80 ms. NOTE 3: K3 = K1, if K1 ≤ 2; otherwise K3 = 1. |  |

RLM/BFD relaxation requirements for 1 RX RedCap UE need further study.

## Other Issues

For the rest of the requirements, we need to study whether NR RLM/BFD relaxation requirements, like signaling indications, entry and exit criteria, and applicability rules, can be reused for RedCap RLM/BFD relaxation especially considering the safety-critical deployments cases for FRMCS in less than 5 MHz and low cost stationary sensor device cases for 1 RXRedCap UE.

RAN4 to study whether NR RLM/BFD relaxation requirements, like signaling indications, entry and exit criteria, and applicability rules, can be directly reused for RedCap RLM/BFD relaxation.

# Summary and Conclusion

In this contribution, we have analyzed RAN4 RRM aspects for RedCap Less than 5 MHz and made the following observations and proposals.

Observation 1	Rel-18 NR less than 5 MHz only supports FR1 and is dedicated to FDD spectrums.

Observation 2	Rel-18 NR less than 5 MHz requirements for RLM and BFD can be reused for 2Rx RedCap less than 5 MHz to ensure equivalent coverage and reliability.

Observation 3	RedCap RLM/BFD relaxation should support SSB and CSI-RS based RLM/BFD relaxation for both FR1 and FR2.

Observation 4	NR RLM/BFD relaxed measurement criteria can be reused for RedCap RLM/BFD relaxation.

Based on the discussion in the previous sections, we propose the following:

Proposal 1	For 2Rx RedCap for all three duplex modes (FD-FDD, TDD, and HD-FDD), RAN4 should reuse the following requirements from NR less than 5 MHz in Rel-18 as baseline.

	Handover

	RLM/ BFD

	Intra-frequency/Inter-frequency measurement

Proposal 2	RAN4 only needs to define the requirements for NR FR1 for RedCap less than 5 MHz.

Proposal 3	For 1 Rx RedCap less than 5 MHz, T∆ should be further investigated based on the simulation outcome.

Proposal 4	RAN4 should agree on coverage equivalence between 1Rx and 2Rx RedCap UEs on less than 5 MHz CBW.

Proposal 5	RAN4 to analyze the SSB index reading and MIB reading impact for 1 Rx RedCap with less than 5 MHz based on simulation outcome.

Proposal 6	NR RLM and BFD relaxation requirements should be reused for 2 RX RedCap RLM/BFD relaxation.

Proposal 7	RLM/BFD relaxation requirements for 1 RX RedCap UE need further study.

Proposal 8	RAN4 to study whether NR RLM/BFD relaxation requirements, like signaling indications, entry and exit criteria, and applicability rules, can be directly reused for RedCap RLM/BFD relaxation.

# References

- RP-252955, WID: NR Radio Resource Management (RRM) Phase 6, Apple

- R4-2410395, BigCR for NR_FR1_lessthan_5MHz_BW, Nokia