3GPP TSG-RAN WG4 Meeting # 116-bis	R4-2514399

Prague City, CZ, Oct 13 – Oct 17, 2025

Agenda Item: 6.12.2.1

Source: MediaTek Inc.

Title: Discussion on the performance part for R19 MIMO for UE-initiated event beam management

Document for: Discussion

# Introduction

In this paper we discuss the performance part for R19 MIMO phase 5, specifically for part 1, which is related to UE-initiated event beam management enhancement. RAN4 had some open issues in WF [1] in the last meeting without conclusion. We provide our view on the performance requirements for these issues.

# Discussion

In the last meeting, RAN4 discussed the test cases for UE-initiated/event-driven beam management as captured in the WF [1]:

#### Issue 2-1-1: Test scope <Way forward > QC; Apple; Samsung; CMCC; Huawei; ZTE; Nokia; Ericsson; MediaTek. High level: Define TCs to verify UEIBM; Y; Y; Y; Y; Y; Y; Y; Y; Y. Whether to cover Event-2/Event-1/Event-7?; Event-2: Y Event-1/Event-7: FFS; Event-2: Y Event-1/Event-7: No; Event-2: Y Event-1: limited TC Event-7: No; All; Event-2 FFS on others; All; Event-2; Event-2&7; Event-2. Whether to cover Mode A and Mode B?; Either A or B; Both; Both; Both; Mode A&B as Test configurations. Can be skip depends on UE capability; Mode A&B as Test configurations. Can be skip depends on UE capability. Event-trigger for Intra-cell BM for SSB; Y; N; Y; Y; Y; Y; Y; Y; Y. Event-trigger for Intra-cell BM for CSI-RS; Y; Y; Y; Y; Y; Y; Y; Y; Y. Event-trigger for Inter-cell BM for SSB; Y; Y; Y; Y; FFS; Y; Y; ?; Y. Event-trigger for Inter-cell BM for CSI-RS; N; N; N; Y; Y; N; N. Whether to cover no time window and eventInstanceCount-r19 > 1?; N; N; Y; Y; Y; Y; Y. Frequency Range – FR2?; Y. No FR1; Y. FR2; Y. FR2. Beam switching delay for TCI states switching; Y Both known and unknown TCI; N; FFS on core req.; FFS on core req.; No MAC CE-based TCI state switching FFS on DCI-based TCI state switching. L1 measurement reporting delay; Y. CMI; N.

To test UE-initiated/event-driven beam management enhancement, we prefer to focus on event-2 report triggering since event-2 is the basic feature, which include both current beam and new beam. Also, this can be tested for

- Event-trigger for Intra-cell BM for SSB

- Event-trigger for Intra-cell BM for CSI-RS

- Event-trigger for Inter-cell BM for SSB

Proposal 1: RAN4 to prioritize event-2 for testing each of

- Event-trigger for Intra-cell BM for SSB

- Event-trigger for Intra-cell BM for CSI-RS

- Event-trigger for Inter-cell BM for SSB

For UE beam reporting based on mode A or mode B, it is subject to UE capability defined in RAN1. The difference between these modes lies in the uplink resource allocation: Mode A utilizes DCI for beam reporting, whereas Mode B relies on pre-configured uplink resources. Given the minimal differences between Mode A and Mode B, we recommend selecting one of these modes as the test case.

Proposal 2: RAN4 to define test case for either one of mode A or mode B for event 2.

# Summary

In this paper, the discussion of R19 MIMO is provided. The following observations and proposals are concluded:

Proposal 1: RAN4 to prioritize event-2 for testing each of

- Event-trigger for Intra-cell BM for SSB

- Event-trigger for Intra-cell BM for CSI-RS

- Event-trigger for Inter-cell BM for SSB

Proposal 2: RAN4 to define test case for either one of mode A or mode B for event 2.

# Reference

- R4-2512130, WF on RRM requirements for NR_MIMO_Ph5_Part1, Samsung