# 3GPP TSG-RAN WG4 Meeting #116bis	R4-2514468

# Prague, Czech Republic, Oct. 13-17, 2025

Title: 	Scenarios and general consideration on co-location requirements

Agenda Item:	7.2.3

Source: 	Huawei, HiSilicon

Document for:	Discussion

# 1	Introduction

The work on co-location requirements and conformance methods is almost concluded in Rel-19 however developments in AAS in higher frequencies has identified some areas where the co-location requirement may be amended in future releases.

In this paper we discuss the potential changes in the co-location scenarios which may them impact the conformance methods.

# 2	Discussion

## 2.1	Core scenarios

The current OTA co-location core requirements are based on what is considered the worst-case co-location scenario in that 2 antennas are placed 10 cm from each other edge-to-edge. The Co-Location Reference Antenna (CLRA) is a single column antenna with the same vertical radiating dimension as the antenna under test.

The CLRA is defined based on the worst case coupling whilst the Co-Location Test Antenna (CLTA) is defined as a more practical implementation of the CLRA. If the CLTA is a different frequency then the same vertical dimension requirement can be replaced with the same vertical beam width.

The issue of mis-matched frequencies and hence antennas of greatly different dimensions is one of the reasons the current co-location scenarios were questioned.

The critical parts of the scenario which may potentially not be suitable for advanced AAS BS and high FR1 frequencies are:

- Worst case being 10 cm edge-to-edge,

- Assumption that co-located antenna is single column antenna,

- Same vertical dimension / vertical beam width.

In addition, the effect of the DUT having multiple columns has been discussed. While the OTA nature of the test means that the DUT antenna is included as part of the test, the ability of a multi-column DUT to beam steer horizontally has been discussed and could potentially affect the scenario or the conformance test method.

### 2.1.1	Co-located antenna location

Co-located antenna location has been discussed at length and the evidence presented has mainly been observational i.e., examples of installations and how different antennas are co-located, and if 10 cm edge-to-edge condition is still a reasonable worst case.

Single column antennas placed on top of each other have much lower coupling than those placed side by side. The width of the antennas is much smaller than the height and as such the coupled area is much smaller. In addition, antenna elements tend to be designed to radiate horizontally (like a donut) and coupling in the vertical direction is lower.

Whilst multi-column antennas are much wider than single column and hence when placed on top of each other coupling will be higher than for a single column, the predominant antenna dimensions are still taller than they are wide.

Measurements of antenna coupling in the Rel-19 WI [1] showed clearly that side-by-side coupling is still the worst case.

Observation 1: Side by side coupling for the co-location scenario is still worst case.

The next issue to be considered is whether the side-by-side scenario is still to be used. Once again, observation of site installations shows a wide range of scenarios particularly when antennas sizes are very different (due to different operating bands) they are less likely to be placed close to each other. This is likely a matter of practicality, however it is still possible to find examples of BS antennas of all types being placed next to each other side-by-side. Whilst 10 cm side-by-side is an extreme worst case, it remains a reasonable practical limitation.

Observation 2: Side-by-side antenna coupling remains a reasonable practical limitation.

There seems to be no strong case to change the 10 cm edge-to-edge definition of the OTA co-location scenario.

Observation 3: the existing 10 cm edge-to-edge definition of the OTA co-location scenario may not need modifying.

### 2.1.2	Single column antenna assumption

Previous analysis has shown that a single column co-located antenna is the worst-case for the OTA co-location requirements.

- If the co-located system is a victim, then the closest column suffers the highest level of interference which reduces over the more distant columns. As such, the average SNR effect on the victim is higher for multi-column antennas.

- If the co-located system is an aggressor, then the full power of the system is subject to the highest coupling when the co-located system is a single column.

Observation 4: The single column co-located antenna is worst case.

As such, if the co-located system is multi-column, then potentially the requirements will be different, generally the coupling will be lower so the requirements on the system being tested will be lower.

If the impact of the multi-column co-located antenna are to be incorporated in the core requirement, then the OTA co-location scenario needs to be modified so that the co-located system has a minimum number of columns defined.

Observation 5: If the effects of multi-column CLRA are to be used, a minimum number of columns must be attributed to the CLRA.

For advanced AAS BS, particularly at high frequencies, this may be a reasonable restriction particularly for Wide Area systems. In order to overcome increased pathloss, high gain AAS BS are required and they will by definition have high gain and multiple columns. In simulations for 2 GHz assumption, we have used 8 column antennas, and for 6.4 GHz we use have used 16 column antennas. Whilst these may not represent useful minimums, they indicate that multi-column antennas are the norm.

Observation 6: For high frequencies, multi-column antennas are likely to be the norm.

For high FR1 it is a possibility that the scenarios could be modified so that the CLRA is a multi-column antenna. This possibility could be further investigated.

Proposal 1: Further investigate the CLRA being defined with a minimum number of columns for high FR1 frequencies.

### 2.1.3	CLRA height definition

As antenna becomes more mismatched in frequency, so their physical dimensions become more mismatched. The goal of the CLRA definition is to identify the worst-case couple for a co-located antenna. As with the number of columns, the height of the co-located antenna depends on its use case. Higher frequency antennas tend to have more elements in order to increase their gain to overcome the higher pathloss, so matching antennas height makes some sense, however at some point the height mismatch becomes unrealistic. The current CLTA definition has a solution for this, where the vertical beam width can be matched instead.

One issue with this is that the CLTA definition is somewhat dependent on the DUT, this has some pros and cons. It is common that we make assumptions about co-located systems based on the system under test. For IMD interference power for example is assumed to be the same as the system under test, the assumption being that co-located systems will be performing similar functions (Wide Area for example). At the same time modifying it and having different CLTA’s for different DUTs can make testing impractical.

Observation 7: The CLRA height definition is currently dependent on the DUT.

As with the number of columns it may be a better approach to define a certain size or beam-width CLRA for each operating band, this would also help to standardise the CLTA’s.

Proposal 2: Investigate standardising the height of the CLRA for each operating band.

## 2.2	Conformance

There are effects on conformance testing due to potential changes to the core requirement, when looking for alternatives to the CLTA to identify more standardised test antenna to be potentially used.

### 2.2.1	Effect of core requirement changes

One of the outcomes of the Rel-19 WI [1] was using near field analysis to understand the effect of using multi-column antennas and the effect of beams steering when those antennas were used.

Once multi-column antennas and beam steering are considered, the coupling is no longer a fixed parameter. As soon as the coupling is affected by either DUT beam steering or CLRA beam steering then the coupling becomes a statistical parameter.

The requirements may then set by a realistic distribution of aggressor and victim beam steering directions. Preliminary simulations were discussed in the Rel-19 WI [1]. This seems premature, as it is necessary to first agree changes to the core CLRA definition (multi-column) before this even becomes necessary. However once that is done investigating the coupling by statistical simulation seems reasonable.

Once a statistical assumption for coupling is agreed, then it becomes necessary to derive a test scenario that will effectively test that scenario. The existing conformance methodology is to test the requirement as described. As the requirement is a static worst case, this is possible.

However, if the statistical analysis identifies the coupling for the required performance is higher than the current static assumption, there is number of ways this can be used.

For example:

Required emissions (at CLTA output) are -117 dBm

If statistical coupling is 55 dB

If existing CLTA has coupling of 35 dB at 10 cm and

Either: identify separation where coupling is 55 dB and test for -117 dBm

Or: test at 10 cm for a level of -97 dBm

Clearly as testing very low power levels with the current co-location measurements testing higher power levels is potentially preferable.

The same applies to interference injection requirements (TX IMD and Blocking) where it would potentially be possible to use lower test powers with higher coupling.

As near field analysis becomes more trustworthy, it may also be possible to use this to derive alternative test methods to find other alternative test antennas for OTA co-locations testing.

# 3	Conclusions

In this paper we have discussed potential changes to the core co-location scenarios based on changes due to advancement of AAS BS technology and higher FR1 operating bands. We have looked at each of the key CLRA assumptions and if they should be modified based on those changes.

Observation 1: Side by side coupling for the co-location scenario is still worst case.

Observation 2: Side-by-side antenna coupling remains a reasonable practical limitation.

Observation 3: the existing 10 cm edge-to-edge definition of the OTA co-location scenario may not need modifying.

Observation 4: The single column co-located antenna is worst case.

Observation 5: If the effects of multi-column CLRA are to be used, a minimum number of columns must be attributed to the CLRA.

Observation 6: For high frequencies, multi-column antennas are likely to be the norm.

Proposal 1: Further investigate the CLRA being defined with a minimum number of columns for high FR1 frequencies.

Observation 7: The CLRA height definition is currently dependent on the DUT.

Proposal 2: Investigate standardising the height of the CLRA for each operating band.

Further we considered the effect of these changes on conformance testing requirements. The introduction of a minimum number of columns for the CLRA introduces the necessity to incorporate beam steering into the requirement and opens up the possibility of setting the requirement by a statistical simulation.

Once a statistical coupling is identified this potentially allows for modification of the static test cases to make measurement easier.

# References

[1]	RP-252578, NR base station (BS) RF requirement evolution for FR1/FR2 and testing, NR_BS_RF_req_evo, WID, Rel-19