3GPP TSG-RAN WG4 Meeting #116bis                                                           R4-2514514
Prague, Czech Republic, 13th – 17th October, 2025

Agenda item:			8.1

Source:	Moderator (Samsung)

Title:	Topic Summary for [116bis][107] 6G AI

Document for:	Decision

# Introduction

In RAN#108, RAN plenary approved a new RAN working group level SID on 6G Radio [RP-251881], which has been further revised into [RP-252912] in RAN#109. Specifically, AI/ML is the indispensable enabler for 6G RAN, for which at least AI/ML framework and new AI/ML use cases are highlighted as key aspects to be studied, with objectives provided as follows [RP-252912]:

- AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4] - Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…) NOTE: lead WG depends on the use case. - AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including - LCM procedures [RAN2, RAN1, RAN3, RAN4] - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1] Note: NW for AI is assumed to be covered by new services 6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

# Pre-meeting FL Summary

## Topic#1: General scope and timeline

##### Issue 1-1: RAN4 6G AI/ML study scope

[Background Summary] It is proposed by some companies that the 6G AI/ML study scope needs to be clarfied, based on what is provided in SID.

[Relevent Proposals in Companies’ Tdocs]

| Company | Tdoc No. | Proposal |
| --- | --- | --- |
| Samsung | R4-2513049 | Proposal 1: In 6G AI/ML study, the detailed RAN4 scope shall include: a) Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… i. RAN4-driven use cases: RAN4 shall identify, select and study only those AI/ML use cases that fall within RAN4 core responsibility. ii. Other WG-driven use cases: RAN4 shall assist other working groups in these use cases by studying RAN4 impacts with RAN4 analysis and evaluation relevant to implementation and performance. b) AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including i. LCM procedures: RAN4 shall study relevant RAN4 requirements for LCM procedures. ii. RAN4 specific issues to guarantee AI/ML functionalities is implementable and testable, including post-deployment, AI/ML model complexity and power consumption, UE and network AI/ML capability, etc. |
| Qualcomm | R4-2513042 | Proposal 1: The RAN4 Rel-20 study item on AI/ML should not duplicate the efforts of RAN1/2/3. RAN4 should ensure a clear split between RAN4 driven AI/ML work and RAN1/2/3 driven work. Observation 1: In 5G, AI/ML related air interface topics, covering wide range of topics like CSF, beam management, OTA and positioning, got discussed in one session. Observation 2: Unlike RRM, demod and RF, AI/ML based training and inference are just tools that are used to analyze and solve different core issues. Proposal 3: AI/ML should not be discussed in a dedicated standalone session within 6G. Instead, AI/ML topics should be integrated into individual sessions where relevant experts can participate and contribute. |
| NTT DoCoMo | R4-2513293 | Proposal 1: When introducing these features in 6G, it is necessary for RAN4 to develop test specifications in close collaboration with RAN1 and RAN2. |
| ZTE Corporation, Sanechips | R4-2513286 | Proposal 1: The following high-level principles should be considered to raise 6GR AI/ML use cases: - Significant performance gains compared to non-AI-based solution - Compelling trade-off between performance and complexity/overhead - Strong robustness to ensure the stability - Testability to easily verify the performance gain - Energy-efficient AI to facilitate less advanced/power consumption limited device |

[FL Recommended Discussion Point]

FL: The following is summarized for the general RAN4 study scope on AI/ML, and the key principle. Discussion on the following bullets firstly. For Qualcomm’s proposal 3, it is related to RAN4 chair’s meeting arrangement, and this thread shall focus on technical discussion.

- In 6G AI/ML study, the detailed RAN4 scope shall include

  - Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc…

    - RAN4-driven use cases: RAN4 shall identify, select and study only those AI/ML use cases that fall within RAN4 core responsibility.

    - Other WG-driven use cases: RAN4 shall assist other working groups in these use cases by studying RAN4 impacts with RAN4 analysis and evaluation relevant to implementation and performance.

  - AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including

    - LCM procedures: RAN4 shall study relevant RAN4 requirements for LCM procedures.

    - RAN4 specific issues to guarantee AI/ML functionalities is implementable and testable, including post-deployment, AI/ML model complexity and power consumption, UE and network AI/ML capability, AI/ML model generalization, etc.

  - Other principle:

    - RAN4 should ensure a clear split between RAN4 driven AI/ML work and RAN1/2/3 driven work, while maintain close collaboration with other WGs.

[View Collection on above Discussion Point]

Note: Companies’ view can be provided on the above discussion point recommended by FL. Comments from companies are not mandatory but greatly encouraged to facilitate the discussion.

| Company | Comments |
| --- | --- |

##### Issue 1-2: RAN4 6G AI/ML study timeline

[Relevent Proposals in Companies’ Tdocs]

| Company | Tdoc No. | Proposal |
| --- | --- | --- |
| Samsung | R4-2513049 | Proposal 2: Suggested RAN4 timeline for Rel-20 6G AI/ML study Stage 1: Q4'25 and Q1'26  To clarify study scope for RAN4 AI/ML framework  To select the top-prioritized AI/ML use cases, with the detailed evaluation plan concluded for Stage-2 Stage 2: Q2’26 to Q2’27  To analyse and conclude RAN4 AI/ML framework (including RAN4 aspects for LCM procedures and other RAN4 specific issues)  To study comprehensively the use case(s) selected in Stage 1  To study other WG-driven use cases with RAN4 analysis and evaluation |

[FL Recommended Discussion Point]

FL: Discussion on the following timeline planning firstly.

- RAN4 timeline for Rel-20 6G AI/ML study

  - Stage 1: Q4'25 and Q1'26

    - To clarify study scope for RAN4 AI/ML framework

    - To select the top-prioritized AI/ML use cases, with the detailed evaluation plan concluded for Stage-2

  - Stage 2: Q2’26 to Q2’27

    - To analyse and conclude RAN4 AI/ML framework (including RAN4 aspects for LCM procedures and other RAN4 specific issues)

    - To study comprehensively the use case(s) selected in Stage 1

    - To study other WG-driven use cases with RAN4 analysis and evaluation

[View Collection on above Discussion Point]

Note: Companies’ view can be provided on the above discussion point recommended by FL. Comments from companies are not mandatory but greatly encouraged to facilitate the discussion.

| Company | Comments |
| --- | --- |

## Topic#2: RAN4 AI/ML framework

##### Issue 2-1: RAN4 related study for LCM procedure

[Relevent Proposals in Companies’ Tdocs]

| Company | Tdoc No. | Proposal |
| --- | --- | --- |
| Samsung | R4-2513049 | Proposal 3: The same and unified LCM framework shall be targeted for RAN4-driven use cases and use cases driven by other working groups. Proposal 4: For RAN4 study of LCM procedure for AI/ML framework, take 5G NR AI/ML framework as a starting point for RAN4-driven use cases, by at least considering - Terminologies in TR 38.843 - UE-side and NW-side data collection if needed - Applicability report - Associated ID to indicate additional conditions that may not be explicitly configured - Performance monitoring - Dedicated AI/ML processing unit (APU) and timeline - Testing and Performance Requirements |
| Ericsson | R4-2513060 | Observation 29: LCM procedures like model training, model retraining, and model update are critical to ensure reliability of the AI/ML model performance. Observation 30: LCM procedures like model training, model retraining, and model update impose complexity to testing procedures. Proposal 23: RAN4 to study testing aspects related to performance validation of the UE based AI/ML model that is updated after deployment and the model that is trained after deployment. |
| OPPO | R4-2513318 | Observation 1: For 6G, RAN4 should take a comprehensive approach to LCM testing to fully support AI use cases throughout their lifecycle and enable native AI in 6G. Proposal 2: For 6G, More LCM features, e.g., data collection, model selection, training, update, and UE capability, need further study in RAN4 AI, including testability issues, core requirements, performance requirements. |

[FL Recommended Discussion Point]

- For RAN4 requirements for LCM procedure in 6G

  - The same and unified LCM framework shall be targeted for RAN4-driven use cases and use cases driven by other working groups.

  - For RAN4 study of LCM procedure for AI/ML framework, take 5G NR AI/ML framework as a starting point for RAN4-driven use cases, by at least considering

    - Terminologies in TR 38.843

    - UE-side and NW-side data collection if needed

    - Applicability report

    - Associated ID to indicate additional conditions that may not be explicitly configured

    - Performance monitoring

    - Dedicated AI/ML processing unit (APU) and timeline

    - RAN4 requirement including core and performance requirements

    - Testing aspects

[View Collection on above Discussion Point]

Note: Companies’ view can be provided on the above discussion point recommended by FL. Comments from companies are not mandatory but greatly encouraged to facilitate the discussion.

| Company | Comments |
| --- | --- |

##### Issue 2-2: Post-deployment enhancement

[Background Summary] During Rel-18/19 5G-A AI/ML discussion, there are two options identified for post-deployment model testing, i.e., Option 1 (Post-deployment pre-activation functionality/configuration update testing) and Option 2 (Post-deployment post-activation functionality testing based on performance monitoring). Between two options, the Option 2 is prioritized while it should be further investigated/established that reliable and testable monitoring procedures can be established for all use cases.

[Relevent Proposals in Companies’ Tdocs]

| Company | Tdoc No. | Proposal |
| --- | --- | --- |
| Nokia | R4-2513072 | Proposal 1: RAN4 to further discuss and consider studying post-deployment testing and validation enhancements in 6G in collaboration with RAN2 from early phases of the 6GR SI. Proposal 2: RAN4 should consider, as part of the post-deployment testing and validation study, solutions for enhanced UE-side AI/ML functionality behaviour/performance reporting. |
| Ericsson | R4-2513060 | Observation 29: LCM procedures like model training, model retraining, and model update are critical to ensure reliability of the AI/ML model performance. Observation 30: LCM procedures like model training, model retraining, and model update impose complexity to testing procedures. Proposal 23: RAN4 to study testing aspects related to performance validation of the UE based AI/ML model that is updated after deployment and the model that is trained after deployment. |

[FL Recommended Discussion Point]

FL: Based on the proposals and discussion history in 5G-A, it is suggested to discuss on whether or not the following bullets can be agreed as WF.

- For post-deployment enhancement, RAN4 shall further study post-deployment testing and validation enhancements in 6G study:

  - The study can be conducted in collaboration with RAN2 from early phases of the 6GR SI.

  - Study directions:

    - Solutions for enhanced UE-side AI/ML functionality behaviour/performance reporting

    - Other directions are not precluded

[View Collection on above Discussion Point]

Note: Companies’ view can be provided on the above discussion point recommended by FL. Comments from companies are not mandatory but greatly encouraged to facilitate the discussion.

| Company | Comments |
| --- | --- |

##### Issue 2-3: Model standardization and deployment

[Relevent Proposals in Companies’ Tdocs]

| Company | Tdoc No. | Proposal |
| --- | --- | --- |
| OPPO | R4-2513318 | Proposal 3: For 6G, further study is needed on model standardization and deployment, RAN4 needs to consider following two aspects: - Effective Model Design, to ensure the reference models developed by RAN4 are practical and feasible for field implementation. - Constraints on Model Deployment, e.g., to study and define what makes a model "complex" or "simple” and understand the power consumption differences between different models. |

[FL Recommended Discussion Point]

FL: Based on the proposal, discuss on the two aspects as below firstly. On the other hand, the proponent is encouraged to provide more clarification how these two aspects will impact final RAN4 6G requirements for AI/ML use cases.

- For model standardization and deployment, following two aspects can be further studied:

  - Effective Model Design, to ensure the reference models developed by RAN4 are practical and feasible for field implementation.

  - Constraints on Model Deployment, e.g., to study and define what makes a model "complex" or "simple” and understand the power consumption differences between different models.

[View Collection on above Discussion Point]

Note: Companies’ view can be provided on the above discussion point recommended by FL. Comments from companies are not mandatory but greatly encouraged to facilitate the discussion.

| Company | Comments |
| --- | --- |

##### Issue 2-4: Model generalization

[Relevent Proposals in Companies’ Tdocs]

| Company | Tdoc No. | Proposal |
| --- | --- | --- |
| Qualcomm | R4-2513042 | Observation 11: “Generalization” is an important aspect of AI/ML, i.e., one needs to see if the designed AI/ML model is “general” enough to handle different test environments. Observation 12: An AI/ML model’s ability to generalize can be truly tested in transient scenarios, where channel parameters (e.g., Doppler in AI/ML based CSI prediction and temporal beam prediction) change during the tests. Proposal 6: RAN4 should study if and how to change channel parameters during a performance test to check the generalization aspects of AI/ML models. |
| Ericsson | R4-2513060 | Observation 20: For a given use case, adequate performance across scenarios/conditions (i.e., generalization) cannot be guaranteed. Observation 21: Applicability reporting framework, as introduced/discussed in 5G-A SI/WI for AI/ML based features risk inducing additional reporting delay when UE is configured to report the inferred measurement in scenarios where the UE deems its AI/ML model as inapplicable. Observation 22: Applicability reporting framework largely depends on the UE capability to accurately determine when its model becomes inapplicable and how quickly it can fallback to non-AI/ML based operation. Observation 23: Testing UE based AI/ML feature in multitude of test scenarios makes testing a cumbersome procedure for UE and TE vendors. Proposal 19: RAN4 to study generalization of UE based AI/ML functionality for 6G and identify its impact on RAN4 specification. |

[FL Recommended Discussion Point]

FL: How to guarantee the model generalization has been discussed in 5G-A AI/ML, while there is no solid conclusion reached. Based on the above proposals, RAN4 could further study the generalization of UE based AI/ML functionality for 6G and identify its impact on RAN4 specification.

- For model generalization, RAN4 shall study generalization of UE-based AI/ML functionality for 6G and identify its impact on RAN4 specification.

  - Study if and how to change channel parameters during a performance test to check the generalization aspects of AI/ML models

[View Collection on above Discussion Point]

Note: Companies’ view can be provided on the above discussion point recommended by FL. Comments from companies are not mandatory but greatly encouraged to facilitate the discussion.

| Company | Comments |
| --- | --- |

##### Issue 2-5: Test method for AI models

[Relevent Proposals in Companies’ Tdocs]

| Company | Tdoc No. | Proposal |
| --- | --- | --- |
| OPPO | R4-2513318 | Observation 2: The over-simplified OTA assumptions being used may offer little practical guidance for field deployment. Proposal 4: For 6G, RAN4 could explore whether/how to decouple the signal acquisition testing (e.g., for model inputs) from model performance evaluation (e.g., for model performance) for AI use cases. Proposal 5: Signal acquisition could continue to be tested via cable or OTA methods, e.g., evaluating BM Set B measurement accuracy. Proposal 6: Model performance could be assessed using standardized datasets defined by RAN4 (RAN4 specified datasets), enabling a virtual testing environment. |
| Ericsson | R4-2513060 | Observation 31: Performance of AI/ML based feature observed in one scenario may not necessarily hold true in another scenario. This behavior of AI/ML based feature makes testing more complex and time consuming requiring a careful consideration from the beginning of the 6G study item. Observation 32: 6G is expected to also operate in new range of frequencies from 7 GHz to 15 GHz. Testing in this new range of frequencies may have their own challenges mandating a need to study testability of AI/ML based feature from the beginning of the 6G study item. Observation 33: Test conditions need to be closely aligned with real-world parameters to verify the performance of AI/ML based feature where the model is trained with real world data. |
| China Unicom | R4-2513298 | Proposal: To investigate the existing testing framework, and make necessary updates to accommodate the testing of AI/ML related features, making sure the testing is sufficient and reliable. |

[FL Recommended Discussion Point]

FL: In the 6G RAN4 topic of “OTA and testability”, it is expected to include “testability methodology framework and key assumptions, including both AI and non-AI features”. However, based on Rel-19 experience for AI-BM, the use case related testing issues are per-use-case specific, and it is not harmful to discussion these proposals here. Therefore, suggest to discuss the following bullets (reorganized based on OPPO, Ericsson and China Unicom’s proposals) firstly. Also note the generalization related testing issues has been summarized in Issue 2-4.

- For test method for AI models, study necessary update to existing test framework, with the following detailed aspects to be further studied:

  - Whether/how to decouple the signal acquisition testing (e.g., for model inputs) from model performance evaluation (e.g., for model performance) for AI use cases.

    - Signal acquisition could continue to be tested via cable or OTA methods, e.g., evaluating BM Set B measurement accuracy.

    - Model performance could be assessed using standardized datasets defined by RAN4 (RAN4 specified datasets), enabling a virtual testing environment.

  - FFS test conditions to be closely aligned with real-world parameters to verify the performance of AI/ML based feature where the model is trained with real world data.

  - FFS the following aspects’ impact on AI/ML feature:

    - New range of frequencies from 7 – 15GHz

[View Collection on above Discussion Point]

Note: Companies’ view can be provided on the above discussion point recommended by FL. Comments from companies are not mandatory but greatly encouraged to facilitate the discussion.

| Company | Comments |
| --- | --- |

##### Issue 2-6: AI/ML model complexity and UE category

[Relevent Proposals in Companies’ Tdocs]

| Company | Tdoc No. | Proposal |
| --- | --- | --- |
| Samsung | R4-2513049 | Proposal 5: For RAN4 specific issues in AI/ML framework, the following issue can be further studied: - The AI/ML model complexity - UE and network AI/ML capability and capability-level categorization - Power consumption model for AI/ML inference and other AI/ML relevant procedures. |
| OPPO | R4-2513318 | Observation 3: For computing resources, in current terminal devices, most computing resources are located outside the modem. Computing resources inside the modem, especially dedicated processors like GPUs or NPUs, are very limited. Observation 4: For wireless AI use cases, available computing resources may fall into: - AI computing resource on device, but outside modem - Non-AI computing resource on device, but outside modem - AI computing resource inside modem - Non-AI computing resource inside modem Observation 5: UE with different capabilities may support different reference models. If RAN4 defines reference models without taking into account the diverse AI capabilities of UEs, it could lead to AI features being available only to high-end devices. Proposal 7: For 6G, RAN4 could study/identify these factors and estimate whether and how to introduce AI categories in RAN4 to serve different UE and different AI features. Following aspects could be considered first: - Computing Resource - Energy Consumption - Memory Tiers - Types of Computing Resources - Location of Computing Resources - Supported Reference Models Proposal 8: The feasibility of power consumption testing could be studied from 6G day1 in RAN4 AI. - Whether/how to use flops/ops to reflect the AI power consumption. - Further study other impacts that may affect the AI power consumption (if any, e.g., due to different AI computing resource/location, different deployment assumptions). |

[FL Recommended Discussion Point]

FL: In two companies’ contributions, AI/ML model complexity and relevant UE capabilities are discussed as the issues for AI/ML to be implemented. It is suggested to discuss the following bullets to be agreed as WF.

- For AI/ML model complexity and AI/ML relevant capability/category in 6G, RAN4 further discuss on:

  - The AI/ML model complexity

  - UE and network AI/ML capability and capability-level categorization, by considering:

    - Computing Resource

    - Energy Consumption

    - Memory Tiers

    - Types of Computing Resources

    - Location of Computing Resources

    - Supported Reference Models

  - Power consumption model for AI/ML:

    - Whether/how to use flops/ops to reflect the AI power consumption.

    - Further study other impacts that may affect the AI power consumption (if any, e.g., due to different AI computing resource/location, different deployment assumptions).

[View Collection on above Discussion Point]

Note: Companies’ view can be provided on the above discussion point recommended by FL. Comments from companies are not mandatory but greatly encouraged to facilitate the discussion.

| Company | Comments |
| --- | --- |

##### Issue 2-7: Interoperability for one-sided and two-sided model

[Relevent Proposals in Companies’ Tdocs]

| Company | Tdoc No. | Proposal |
| --- | --- | --- |
| Ericsson | R4-2513060 | Observation 24: Type of AI/ML model to be deployed for AI/ML based feature can be up to UE implementation. Observation 25: For the same feature different flavors of AI/ML model may be deployed by different UE vendors. Proposal 20: RAN4 to study performance monitoring aspects related to UE based AI/ML functionality and identify its potential impact on RAN4 specification. Observation 26: In 6G, for the same functionality, network needs to support a mix of UEs that can use AI/ML model to infer measurements and those which do not support AI/ML for measurement inferencing. Observation 27: If the AI/ML model performance varies between different UEs and their models, then the network needs to know which UE it is talking to, when/where it can configure AI/ML functionality to this UE and the performance that can be expected from this UE. Needing to have such knowledge for each of the UEs will compromise interoperability and increase the complexity of deploying the AI/ML based functionality in 6G. Proposal 21: Interoperability for one-sided model deployment refers to ensuring consistency between the performance of AI/ML capable and non-AI/ML capable UEs for the same feature. Proposal 22: RAN4 to study how to ensure consistent behaviour between UEs supporting AI/ML and UEs not supporting AI/ML for the same functionality. Observation 28: Interoperability for two-side model is being addressed for CSI compression in 5G-A Rel. 20 WI. Learnings from this WI and the approaches identified to handle interoperability issues in two-side AI/ML model deployment can potentially be used to address interoperability issues in AI/ML based 6G features that require two-side model deployment. |

[FL Recommended Discussion Point]

FL: Interoperability for two-sided and one-sided models are one of the key focused topics in 5G-A study and work item. It should be noted that the Rel-20 5G-A WI will specify the interoperability (by considering Direction C and Direction A with both Option 3a-1 and Option 4-1) and it is expected that the conclusion can be followed in 6G study.

- Interoperability for one-sided model, refers to ensuring consistency between the performance of AI/ML capable and non-AI/ML capable UEs for the same feature

  - RAN4 to study how to ensure consistent behavior between UEs supporting AI/ML and UEs not supporting AI/ML for the same functionality.

- Interoperability for two-sided model,

  - FFS the specified mechanism for CSI compression in Rel-20 5G-A WI can be followed in 6G study.

[View Collection on above Discussion Point]

Note: Companies’ view can be provided on the above discussion point recommended by FL. Comments from companies are not mandatory but greatly encouraged to facilitate the discussion.

| Company | Comments |
| --- | --- |

##### Issue 2-8: RAN4 requirement for other WGs’ use cases

[Relevent Proposals in Companies’ Tdocs]

| Company | Tdoc No. | Proposal |
| --- | --- | --- |
| Ericsson | R4-2513060 | Observation 34: To ensure AI/ML based feature identified by RAN1 is supported within the framework defined by RAN2, RAN4 role will be to define the RRM requirements for those use cases. Observation 35: Depending on the progress made by RAN1 and RAN2, RAN4 may have to start discussions to identify the impact of AI/ML use cases driven by other working groups on RAN4 specification earlier than planned timeline which is RAN4 meetings after Q1 2026. Observation 36: Unlike in 5G-A SI and WI, RAN4 should involve from the beginning of evaluation work where RAN4, based on its expertise, should support other working groups by providing inputs in terms of realistic impairment models that needs to be considered when feasibility of AI/ML based feature is being studied. |

[FL Recommended Discussion Point]

- For RAN1/2-driven use cases:

  - RAN4 role will be to define the RRM requirements for those use cases

  - Depending on the progress made by RAN1 and RAN2, RAN4 may have to start discussions to identify the impact of AI/ML use cases driven by other working groups on RAN4 specification earlier than planned timeline which is RAN4 meetings after Q1 2026.

  - RAN4 should involve from the beginning of evaluation work where RAN4, based on its expertise, should support other working groups by providing inputs in terms of realistic impairment models that needs to be considered when feasibility of AI/ML based feature is being studied.

[View Collection on above Discussion Point]

Note: Companies’ view can be provided on the above discussion point recommended by FL. Comments from companies are not mandatory but greatly encouraged to facilitate the discussion.

| Company | Comments |
| --- | --- |

## Topic#3: RAN4-driven AI/ML use case (General Issues)

##### Issue 3-1: Principle for AI/ML use cases selection

[Relevent Proposals in Companies’ Tdocs]

| Company | Tdoc No. | Proposal |
| --- | --- | --- |
| Samsung | R4-2513049 | Proposal 6: A use case can be selected as RAN4-driven AI/ML, if at least one of the following conditions are met: (1) The use case is to solve key pain point which is better understood in RAN4. (2) The use case can be classified as AI-receiver with no or limited RAN1/2 impacts, and major specification impacts are related to RAN4. Note: Collaboration/interaction between UE and NW shall be minimized in RAN4-driven use case. Proposal 7: Coordinated discussions with other working groups are required to avoid overlapping scope between different working groups. |
| MediaTek | R4-2513079 | Proposal 1: Prioritize use cases with system gain, e.g., the gain is straightforward or showed by evaluation results. Proposal 2: Prioritize use cases with clear spec impact and clear differentiation to non-AI based performance. |
| Huawei | R4-2513310 | Proposal 1: Principles for identifying RAN4-led AI/ML uses case are outlined below. - Prioritize use cases focusing on common issues or pain points identified within RAN4 that could be solved by AI. - Performance gains of AI-based solutions over non-AI counterparts can be verified with minimal reliance on fundamental design inputs from other 3GPP working groups. - Strive for an appropriate trade-off between performance and complexity during use case identification. |
| Qualcomm | R4-2513042 | Proposal 1: The RAN4 Rel-20 study item on AI/ML should not duplicate the efforts of RAN1/2/3. RAN4 should ensure a clear split between RAN4 driven AI/ML work and RAN1/2/3 driven work. Proposal 2: RAN4 should study the potential benefits of AI/ML across different scenarios and consider defining RAN4 requirements only if the performance benefits of AI/ML are proven and justified. |
| ZTE Corporation, Sanechips | R4-2513286 | Proposal 1: The following high-level principles should be considered to raise 6GR AI/ML use cases: - Significant performance gains compared to non-AI-based solution - Compelling trade-off between performance and complexity/overhead - Strong robustness to ensure the stability - Testability to easily verify the performance gain - Energy-efficient AI to facilitate less advanced/power consumption limited device |

[FL Recommended Discussion Point]

- Principles for AI/ML RAN4-driven use cases selection

  - RAN4 shall not duplicate the efforts of RAN1/2/3.

    - RAN4 should ensure a clear split between RAN4 driven AI/ML work and RAN1/2/3 driven work.

    - Coordinated discussions with RAN1/2/3 are required to avoid overlapping scope.

  - Typical RAN4-driven use case shall

    - solve key pain point which is better understood in RAN4, e.g.,

    - have no or limited RAN1/2 impacts, and major specification impacts are related to RAN4.

      - collaboration/interaction between UE and NW shall be minimized.

  - Prioritize:

    - use cases with system gain over non-AI counterpart

    - use cases with clear spec impact and clear differentiation to non-AI based performance.

    - use cases with trade-off between performance and complexity/overhead

  - Note: 5G use cases are not guaranteed to have requirements introduced to 6G.

[View Collection on above Discussion Point]

Note: Companies’ view can be provided on the above discussion point recommended by FL. Comments from companies are not mandatory but greatly encouraged to facilitate the discussion.

| Company | Comments |
| --- | --- |

##### Issue 3-2: Aspects of RAN4-driven use case to be studied

[Relevent Proposals in Companies’ Tdocs]

| Company | Tdoc No. | Proposal |
| --- | --- | --- |
| Samsung | R4-2513049 | Proposal 8: At least the following RAN1 agreed aspects to be studied and reported for each proposed (sub-)use case should be considered for RAN4 study for AI/ML use cases identification/categorization: • Definition of each (sub-)use case, including at least AI/ML model input/output • The evaluation assumption, methodology, KPIs, benchmark, and preliminary simulation results • Assumption on training types, e.g., o offline training, online training/finetuning o Label construction (if applicable), including whether/how to obtain label data for model training • Assumption on model location for inference, e.g., UE-sided model, NW-sided model, and two-sided model • Collaboration/interaction between UE and NW, e.g., o no collaboration/interaction o UE/Network collaboration targeting at separate or joint ML operation • High level potential specification impact Proposal 9: Additionally, RAN4 study for AI/ML use case identification/categorization shall consider implementation feasibility, including computational complexity, power consumption etc. |
| MediaTek | R4-2513079 | Proposal 3: 5G use cases are not guaranteed to have requirements introduced to 6G. Still need to check the gain, feasibility, complexity/cost under 6G assumptions. |
| OPPO | R4-2513318 | Proposal 1: For RAN4-driven use cases, reuse the guidance that agreed in RAN1 #122 meeting as starting point, and add “Assumption on testability”. |

[FL Recommended Discussion Point]

- Aspects of RAN4-driven use case to be studied

  - By following RAN1 agreement, for each (sub-)use case proposed, proponent companies are encouraged to study and report the following:

    - Definition of each (sub-)use case, including at least AI model input/output

    - The evaluation assumption, methodology, KPIs, benchmark, and preliminary simulation results

    - Assumption on training types, e.g.,

      - offline training, online training, finetuning

      - Label construction (if applicable), including whether/how to obtain label data for model training

    - Assumption on model location for inference, e.g., UE-sided model, NW-sided model, and two-sided model

    - Collaboration/interaction between UE and NW, e.g.,

      - no collaboration/interaction

      - UE/Network collaboration targeting at separate or joint ML operation

    - High level potential specification impact

  - Additionally, the following RAN4 aspect(s) are encouraged to study and report:

    - Feasibility, complexity/cost under 6G assumptions

    - Assumption on testability

[View Collection on above Discussion Point]

Note: Companies’ view can be provided on the above discussion point recommended by FL. Comments from companies are not mandatory but greatly encouraged to facilitate the discussion.

| Company | Comments |
| --- | --- |

## Topic#4: AI/ML use case for RF issues (Per-use case discussion)

### Use Case#1: AI-nonlinearity compensation

##### Issue 4-1a: Use case definition and supporting level from companies

[FL Recommended Discussion Point]

| Sub-use case | Model location and training | Views |
| --- | --- | --- |
| NW-sided DPoD (Digital Post-Distortion) and receiver | NW-sided model (online training, or offline training to be updated in a certain time period) | Companies show interests, with numerical results (3) Samsung, Ericsson, vivo, OPPO Companies show interests, without numerical results (5) MediaTek, CMCC, CATT, ZTE |
| UE-sided DPD (Digital Pre-Distortion) | UE-sided model | Companies show interests, with numerical results (1) vivo, OPPO Companies show interests, without numerical results (3) Xiaomi, ZTE Companies show no interests for standardized solution MediaTek (not standardization solution, but allow UE to use), |
| UE-sided DPoD (Digital Post-Distortion) and receiver | UE-sided model | Companies show interests, without numerical results (1) ZTE |
| NW-sided DPD (Digital Pre-Distortion) | NW-sided model | Companies show interests, without numerical results (1) ZTE |
| Study system level AI application for overall RF performance optimization (filter parameters and PA parameters optimization) at NW side and UE side | UE-sided model and/or NW-sided model | Companies show interests, without numerical results (1) ZTE |

[View Collection on above Discussion Point]

Note: Companies’ view can be provided on the above discussion point recommended by FL. Comments from companies are not mandatory but greatly encouraged to facilitate the discussion.

| Company | Comments |
| --- | --- |

##### Issue 4-1b: Leading group for DPD/DPoD

[FL Recommended Discussion Point]

- Leading WG for AI-DPD/DPoD:

  - RAN4 shall be the leading Working Group.

[View Collection on above Discussion Point]

Note: Companies’ view can be provided on the above discussion point recommended by FL. Comments from companies are not mandatory but greatly encouraged to facilitate the discussion.

| Company | Comments |
| --- | --- |

##### Issue 4-1c: Evaluation for AI-DPoD

[FL Recommended Discussion Point]

- Non-linearity model(s) of transmission signals:

  - FFS in 6G UE RF session, which should be common for BS compensation with AI and Non-AI solutions (Samsung).

  - FFS RF front-end variation from multiple UEs in the market into account (Ericsson).

  - RAN4 to clarify the extent to which 6G UEs may support digital pre-distortion (Ericsson).

- Possible sub-use cases and consider possible down-selection: (OPPO)

  - Option 2a: NW side NC by AI demod

  - Option 2b: NW side NC by a separate AI NC module +Non-AI demod

  - Option 2c: NW side NC by a separate AI NC module + AI demod

  - Option 2x: other solutions

- Evaluation Procedures and some detailed discussion points (Samsung, Ericsson)

  - Step-1: Selection of Non-linearity model(s) of transmission signals

  - Step-2: Evaluation methodology by applying non-linearity model(s) of transmission signals to baseband evaluation with the following procedure

    - Detailed assumptions for evaluation (including data collection, training and inference) are mentioned in R4-2513060 (Ericsson) and R4-2513049 (Samsung), including

      - FFS the extent to which UE EVM requirements can be adjusted when AI-DPoD compensates for PA and other RF components non-linearity at the base station.

      - FFS suitable AI/ML model, type of training.

      - FFS different data collection methods for AI/ML model training, focusing on UE operation in the compressed (non-linear) PA region: e.g., simulation-based datasets (with realistic Tx front-end and PA models) and measurement-based datasets (from multiple UEs).

      - FFS how to define a reference AI/ML model for evaluating AI/ML DPoD performance and ensuring comparability across different implementations

      - Focusing on higher modulation orders focusing on high-SNR conditions

  - Step-3: For a given relaxed EVM value (e.g., 6% for a certain modulation order), conduct TX MPR simulation.

    - The potential MPR reduction enabled by AI-DPoD is evaluated.

      - The requirements for other gating factors that impact MPR including ACLR, SEM and SE remain unchanged

    - The impact on UL coverage and UE transmit energy efficiency is evaluated.

- Other design principle to be considered:

  - UE may adjust EVM and reduce MPR only under explicit network control; otherwise, existing RF requirements apply (Ericsson).

[View Collection on above Discussion Point]

Note: Companies’ view can be provided on the above discussion point recommended by FL. Comments from companies are not mandatory but greatly encouraged to facilitate the discussion.

| Company | Comments |
| --- | --- |

##### Issue 4-1d: Evaluation for AI-DPD

[FL Recommended Discussion Point]

- Whether or not AI-DPD shall be included in 6G RAN4 AI study:

  - Option 1: shall be included.

  - Option 2: Not included, but UE is still allowed to use AI based DPD even not specifying the feature (MediaTek)

- The following aspects are mentioned for AI-DPD (Xiaomi):

  - Model Precision

  - Real-time adaptation

  - Online calibration

[View Collection on above Discussion Point]

Note: Companies’ view can be provided on the above discussion point recommended by FL. Comments from companies are not mandatory but greatly encouraged to facilitate the discussion.

| Company | Comments |
| --- | --- |

## Topic#5: AI/ML use case for Demod (Per-use case discussion)

### Use Case#2: AI-based channel estimation

##### Issue 5-1a: Use case definition and supporting level from companies

[FL Recommended Discussion Point]

| Sub-use case | Model location | Views |
| --- | --- | --- |
| AI-based channel estimation with DMRS reduction | NW-sided model for PUSCH UE-sided model for PDSCH | Companies show interests, with numerical results (1) Samsung (NW-sided) Companies show interests, without numerical results (2) Ericsson (both UE and NW-sided), Nokia (UE-sided) |
| AI-based channel estimation with CSI-RS reduction | UE-sided model | Companies show interests, without numerical results (2) Ericsson, Huawei |

[View Collection on above Discussion Point]

Note: Companies’ view can be provided on the above discussion point recommended by FL. Comments from companies are not mandatory but greatly encouraged to facilitate the discussion.

| Company | Comments |
| --- | --- |

##### Issue 5-1b: RAN4 evaluation assumption and procedures

[FL Recommended Discussion Point]

- Similar use cases are discussed in RAN1

  - Leading group for this use case:

    - FL: If new DMRS design involved, clarify whether it should be treated in RAN1 firstly.

  - How/when RAN4 shall be involvement, if it is RAN1-led use case:

    - As soon as RAN1 agrees to consider the use case of CSI-RS overhead reduction, RAN4 should consider starting the study on the impacts on RAN4. This should be the case particularly for UE sided model scenario (Nokia).

    - As soon as RAN1 agrees to consider the use case of DMRS design with AI receiver, RAN4 should consider starting the study on the impacts on RAN4 (Nokia).

[View Collection on above Discussion Point]

Note: Companies’ view can be provided on the above discussion point recommended by FL. Comments from companies are not mandatory but greatly encouraged to facilitate the discussion.

| Company | Comments |
| --- | --- |

### Use Case#3: AI-SRS-assisted channel reconstruction

##### Issue 5-2a: Use case definition and supporting level from companies

[FL Recommended Discussion Point]

| Sub-use case | Model location | Views |
| --- | --- | --- |
| AI-SRS-assisted channel reconstruction for (a) SRS IL imbalance, (b) SRS aging | Case 1: NW-side model-based Case 2: Two-sided model-based approach | Companies show interests, with numerical results (2) Huawei, Samsung (Case-1 and Case-2) Companies show interests, without numerical results (1) OPPO |

##### Issue 5-2b: RAN4 evaluation assumption and procedures

[FL Recommended Discussion Point]

- Leading group for this use case:

  - RAN1 or RAN4?

- RAN4 study AI-SRS-assisted channel reconstruction, including the following possible sub-use cases:

  - AI-SRS-assisted channel reconstruction from poor SRS, including SRS IL imbalance and SRS aging (NW-sided model)

  - AI-SRS-assisted channel reconstruction based on SRS and spatially sub-sampled CSI-RS measurement (NW-sided model or two-sided model)

- Proponents can further trigger discussion on detailed evaluation methodology and assumptions

[View Collection on above Discussion Point]

Note: Companies’ view can be provided on the above discussion point recommended by FL. Comments from companies are not mandatory but greatly encouraged to facilitate the discussion.

| Company | Comments |
| --- | --- |

### Use Case#4: AI-enabled PRACH receiver

##### Issue 5-3a: Use case definition and supporting level from companies

[FL Recommended Discussion Point]

| Sub-use case | Model location | Views |
| --- | --- | --- |
| AI-enabled PRACH receiver | NW-side model | Companies show interests, with numerical results (1) Samsung |

[View Collection on above Discussion Point]

Note: Companies’ view can be provided on the above discussion point recommended by FL. Comments from companies are not mandatory but greatly encouraged to facilitate the discussion.

| Company | Comments |
| --- | --- |

##### Issue 5-3b: RAN4 evaluation assumption and procedures

[FL Recommended Discussion Point]

- RAN4 study AI/ML-enable PRACH receiver for 6GR, including both single-user and multi-user collision scenario based on existing PRACH preamble formats as starting point, including LCM procedure, data collection, performance monitoring, and model interference pending on the assumption of AI/ML receiver.

[View Collection on above Discussion Point]

Note: Companies’ view can be provided on the above discussion point recommended by FL. Comments from companies are not mandatory but greatly encouraged to facilitate the discussion.

| Company | Comments |
| --- | --- |

### Other AI-enabled demodulation issues

##### Issue 5-4: RAN4 evaluation assumption and procedures

[FL Recommended Discussion Point]

The group can discuss on the following bullets firstly

- Other possible use case:

  - AI for high modulation order (OPPO)

- General views on AI-demod:

  - to deprioritize the study of AI/ML usage in Demod unless advanced receivers emerge (CATT)

- Other issues:

  - RAN4 can consider the model driven approach for study of AI receiver for 6G (ZTE)

  - RAN4 to study following aspects when applying CDL models for demodulation requirements definition for AI-based receiver (Ericsson)

    - Model variants depending on scenarios covered by AI feature

    - Model variants depending on time, frequency and spatial drifting of the channel and UE capabilities

    - Balance between channel model richness and test complexity.

    - How to avoid test signals being used for AI model training.

[View Collection on above Discussion Point]

Note: Companies’ view can be provided on the above discussion point recommended by FL. Comments from companies are not mandatory but greatly encouraged to facilitate the discussion.

| Company | Comments |
| --- | --- |

## Topic#6: AI/ML use case for RRM (Per-use case discussion)

### Use Case#5: AI-based measurement prediction

##### Issue 6-1a: Use case definition and supporting level from companies

| Index | use cases | Model location (Training) | Proposed by companies |
| --- | --- | --- | --- |
|  | L1 beam measurement: - Time domain (intra-cell) - Time and spatial domain (intra-cell) - Freq. and spatial domain (inter-cell) - Spatial domain (inter-cell) - Time and spatial domain (inter-cell) | NW and UE sided model (Offline training) | OPPO (a, b, c), Nokia ((d, e), as RAN1-led study), Apple (c, d, e) |
|  | L3 beam measurement: - Time domain (intra-cell) - Time and spatial domain (intra-cell) - Freq. and spatial domain (inter-cell) - Time and freq. domain (inter-cell) - Freq. domain prediction for non-collocated cells (inter-cell) - Freq. domain prediction for N carriers single cell | NW and UE sided model (Offline training) | OPPO (a, b, c, e), Apple (c,d, e), CMCC (e), ZTE (e), Samsung (e, f), Nokia (c, d, e) |
|  | L3 cell measurement: - Time domain (intra-cell) - Freq. domain (inter-cell) - Freq. domain prediction for non-collocated cells (inter-cell) | NW and UE sided model (Offline training) | OPPO (a, b, c) Apple (c), Nokia (c) |
|  | Multi-Domain L3 level Prediction for Measurement-Gap Reduction | UE sided model (Offline training) | Apple, vivo, Samsung |
|  | AI-assisted measurement scheduling | UE-sided model | MediaTek |
|  | Dynamic Adaptation of Measurement Procedure (may or may not transparent to NW) | UE sided model | Qualcomm, Apple |
|  | Spatial-Domain Joint Tx/Rx Beam-Level L3/L1 Prediction | UE sided model (Offline training) | Apple (L1 for scheduling restriction and delay reductions), vivo |
|  | Reduced RX Beam Sweeping Factor in FR2 (L1/L3?) | UE sided model | Qualcomm, Apple, MediaTek, CMCC, Xiaomi |
|  | AI/ML based RLM/BFD/CBD, LTM by measurement prediction | UE-sided model | OPPO, vivo (AI for RLM) |
|  | AI/ML based SCell related enhancement by measurement prediction | UE-sided model | OPPO |
|  | AI-based prediction for enhanced s-measure mechanism | UE-sided model | vivo |
|  | AI/ML based AGC through temporal/frequency domain prediction | UE-sided model | ZTE |
|  | AI/ML based DL sync through temporal/frequency domain prediction | UE-sided model | ZTE |
|  | AI abstraction of the wireless environment to facilitate data scheduling and network power saving | NW-sided model | CATT |

Based on the above proposed use cases, further view collection on the popularity can be done firstly, and it is expected that only few use cases to be studied.

[View Collection on above Discussion Point]

Note: Companies’ view can be provided on the above discussion point recommended by FL. Comments from companies are not mandatory but greatly encouraged to facilitate the discussion.

| Company | Comments |
| --- | --- |

##### Issue 6-1b: Unified AI-RRM measurement framework

[FL Recommended Discussion Point]

- General principle for measurement prediction:

  - Unified RRM L1/L3 measurement framework (OPPO, Xiaomi)

  - Unified RRM L1/L3 measurement framework shall include (Xiaomi):

    - Investigating and characterizing potential gains in measurement latency, UE power consumption, and network spectral efficiency by targeting the reduction of TDM-based measurement conflicts and exhaustive beam sweeps.

    - Defining the framework for new UE capabilities that would enable predictive RRM measurements.

    - Establishing the principles for new or modified RRM performance requirements, including accuracy, reliability, and testability aspects for a UE implementing such capabilities.

    - The study will explicitly not standardize specific AI/ML algorithms, models, or training methodologies, preserving implementation freedom and focusing solely on the RAN4-centric aspects of performance and interoperability.

[View Collection on above Discussion Point]

Note: Companies’ view can be provided on the above discussion point recommended by FL. Comments from companies are not mandatory but greatly encouraged to facilitate the discussion.

| Company | Comments |
| --- | --- |

# First Round Discussion

Note: Placeholder for 1st round discussion

# Second Round Discussion

Note: Placeholder for 2nd round discussion

# Post-meeting running WF/summary

Note: Per requested by RAN4 chair, this section is intended for “maintaining the post-meeting running WF” for the issues discussed in previous RAN4 meeting

To be updated after RAN4#116bis.

# Existing RAN4 Agreement

Note: Per requested by RAN4 chair, this section is intended for including all key agreements and decisions in the past meetings and the key relevant agreements from other WG.

To be updated after RAN4#116bis.

# For information

## Contact information

| Company | Delegate(s) | Email address |
| --- | --- | --- |
| Feature Lead | He Wang (Jackson) | h0809.wang@samsung.com |
| Samsung | Yunchuan Yang Yanze Fu Yuanyuan Zhang (Tina) | yc0301.yang@samsung.com yanze.fu@samsung.com tina55.zhang@samsung.com |

## Reference for RAN4#116bis (Oct. 2025, Prague)

R4-2513031		Discussion on AI/ML-Based 6G RRM Use Cases				Apple

R4-2513042		On RAN4-driven AI/ML aspects of 6GR						Qualcomm Technologies Ireland

R4-2513049		Discussion on AI framework and use cases for 6GR			Samsung

R4-2513060		RAN4 study on 6G AI/ML										Ericsson AB.

R4-2513072		6G Considerations on AI										Nokia

R4-2513079		Views on 6G AI/ML												MediaTek Inc.

R4-2513125		Discussion on AI/ML for 6GR study							CMCC

R4-2513229		Discussion on RAN4 driven AI use cases						CATT

R4-2513283		Overview for 6GR RAN4 centric AI use cases 				Xiaomi

R4-2513286		Views on AI study in 6G										ZTE Corporation, Sanechips

R4-2513293		6G RAN4 Study on AI											NTT DOCOMO, INC.

R4-2513298		Views on 6G AI study RAN4 aspects							China Unicom

R4-2513310		Consideration on RAN4 AI study for 6G						Huawei, HiSilicon

R4-2513318		RAN4 driven AI based use cases								OPPO

R4-2513324		Initial discussion on 6G AI for RAN4 driven cases			vivo

## Template used for FL

[Background Summary]

[Relevent Proposals in Companies’ Tdocs]

| Company | Tdoc No. | Proposal |
| --- | --- | --- |

[FL Recommended Discussion Point]

- AAA

  - BBB

  - BBB

    - CCC

      - DDD

        - DDD

    - CCC

[View Collection on above Discussion Point]

Note: Companies’ view can be provided on the above discussion point recommended by FL. Comments from companies are not mandatory but greatly encouraged to facilitate the discussion.

| Company | Comments |
| --- | --- |