3GPP TSG-RAN WG4 Meeting #116bis									                    R4-2513507

Prague, Czech Republic, Oct. 13-17, 2025

Agenda item:			6.2.1

Source:	Moderator (Huawei, HiSilicon)

Title:	Topic summary for [116bis][201] NonCol_intraB_ENDC_NR_CA_Ph2_RRM

Document for:	Information

Introduction

This email thread discusses the RRM core and performance requirements of WI on R19 Support of intra-band non-collocated EN-DC/NR-CA deployment Phase2: new receiver type(s).

Recommended Topics to be treated online:

Issue 2-1: Whether to introduce new sperate RRM test case for R19 non-collocated FR1 intra-band NRCA

Issue 2-2: Test case for interruptions during measurements on deactivated NR SCC in FR1

Topic #1:	RRM Core requirements

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2513480 | Huawei, HiSilicon | Draft CR |
| R4-2513614 | Apple | Draft CR |
| R4-2513615 | Apple | Draft CR |
| R4-2513902 | ZTE Corporation, Sanechips | Draft CR |

Open issues summary

Sub-topic 1: RRM core requirements

Issue 1: CR handling

Background

Based on the approved LS R4-2505161 and agreements in RAN2#130 meeting, the BS signallings are summarized as below.

BS signalling: - When maxMIMO-layers > 4, Type 1(collocated) requirements apply. No new BS signaling is needed - When maxMIMO-layers=2, UE follow Rel-18 specification. No new BS signaling is needed - If Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is present, UE is Type 1. - If Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is absent, UE is Type 2. - When maxMIMO-layers=4, new BS signalling(s) is needed for NR-CA and EN-DC, - For BS signaling, introduce Rel-19 nonCollocatedTypeNR-CA/MRDC-v19xy with two code points {Type 1, Type 4} into CellGroupConfig, applicable when MaxMIMO-layers is configured as 4. - If Rel-19 nonCollocatedTypeNR-CA/MRDC- v19xy is absent, Rel-19 UE(s) apply Rel-18 principle accordingly.

- Recommended WF

The CRs listed above are about corrections, and no specific issue needs to be handled online. Encourage companies directly discuss the CRs offline.

Topic #2:	RRM Performance requirements

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2513481 | Huawei, HiSilicon | Proposal 1: No new RRM test case is needed for R19 non-collocated FR1 intra-band NR-CA. Proposal 2: The test case for interruptions during measurements on deactivated NR SCC in FR1 in A.6.5.2.1.2 is to be revised to cover type 4 UE. |
| R4-2513616 | Apple | Proposal 1: No need to define MRTD/MTTD test cases for CA type 4 UE. Observation 1: For interruptions during measurement on deactivated NR SCC, the test setting in A.6.5.2.1 can be reused. Test requirements need to be updated to cover CA type 4 UE in intra-band non-collocated CA case. Proposal 2: it is proposed to update the test requirements in A.6.5.2.1 to cover testing for CA type 4 UE. Proposal 3: No need to introduce separate test case for SCell activation delay for CA type 4 UE. Observation 2: No need to consider BFD/CBD test as existing test case already tested it from functionality perspective and there is no limitation on CA combinations. Proposal 4: No need to introduce test cases for Scheduling availability for CA type 4 UE. Proposal 5: Follow similar approach for testing of EN-DC type 4 UE as for CA type 4 UE. Proposal 6: For interruptions during measurement on deactivated NR SCC, the test setting in A.6.5.2.1 can be reused. Test requirements need to be updated to cover CA type 4 UE in intra-band non-collocated CA case. Below provided how to modify the requirement description. |
| R4-2513617 | Apple | Draft CR |
| R4-2513892 | ZTE Corporation, Sanechips | Observation 1: The enhancements made to the core part in Rel-19 are all based on the requirements of Rel-18 Type 2 UE, except for the introduction of new signaling to indicate UE capabilities and UE types. Proposal 1: Similar to Rel-18, no new RRM test case is needed for non-collocated FR1 intra-band EN-DC/NR-CA in Rel-19. Proposal 2: RAN4 should consider whether any updates are needed to existing test cases to cover type 4 UE. |
| R4-2513988 | Ericsson | Observation 1: The big CR summarizes the RRM core requirement changes. Observation 2: Use RRM core changes from the WI to analyze development of performance requirements. The affected core requirement areas are: - Timing: MTTD, MRTD - Scheduling availability for CA - Interruption - SCell Activation and deactivation Delay - Link Recovery Procedures Observation 3: The release 18 WI only had a demodulation performance part, so there are no explicit RRM performance requirements tests introduced in release-18. Proposal 1: Develop tests for cases where there is a complete lack of tests for ENDC and CA configurations, from release-15 and onwards. These tests can contain several sets of test parameters covering ENDC and CA in general, as well as non-collocated intra band CA. - Timing: MTTD, MRTD - Scheduling availability for CA - Interruptions at SCell addition/release. |

Open issues summary

Sub-topic 2: RRM performance requirements

Issue 2-1: Whether to introduce new sperate RRM test case for R19 non-collocated FR1 intra-band NRCA

Background For Information

In R18 intra-band non-collocated WI, which kind of test cases needs to be defined has made the following agreements: No new test case is introduced and RAN4 modify the existing test for interruptions during measurements on deactivated NR SCC in FR1 (A.6.5.2.1).

R4-2317286 Issue 2-1: whether to introduce new RRM test case for non-collocated FR1 intra-band NRCA - Agreement: - No new RRM test case is needed for non-collocated FR1 intra-band NR-CA. - FFS whether to modify the exiting interruption test case by considering the new RRC signaling for UE capable of type 2 requirements.

Agreements in RAN4#110 Chairman notes Issue 2-1: Impacts on existing test for interruptions during measurements on deactivated NR SCC in FR1 (A.6.5.2.1) Agreement: - The test requirements for interruptions during measurements on deactivated NR SCC in FR1 in A.6.5.2.1.2 need to be revised to cover type 2 UE. Further discuss on the wording based on the CR.

- Proposals

  - Option 1(Huawei, Apple, ZTE):

Option 1a (Huawei, ZTE): No new sperate RRM test case is needed for R19 non-collocated FR1 intra-band NR-CA.

Option 1b (Apple):

    - No need to define MRTD/MTTD test cases for CA type 4 UE

    - No need to introduce separate test case for SCell activation delay for CA type 4 UE.

    - No need to introduce test cases for Scheduling availability for CA type 4 UE.

    - For interruptions during measurement on deactivated NR SCC, the test setting in A.6.5.2.1 can be reused. Test requirements need to be updated to cover CA type 4 UE in intra-band non-collocated CA case.

  - Option 2(Ericsson): Develop tests for cases where there is a complete lack of tests for ENDC and CA configurations, from release-15 and onwards:

    - Timing: MTTD, MRTD

    - Scheduling availability for CA

    - Interruptions at SCell addition/release.

- Recommended WF

Further discussion

Issue 2-2: Test case for interruptions during measurements on deactivated NR SCC in FR1

- Proposals

  - Option 1 (Huawei, Apple, ZTE): The test case for interruptions during measurements on deactivated NR SCC in FR1 in A.6.5.2.1.2 is to be revised to cover type 4 UE.

- Recommended WF

To discuss whether option 1 is agreeable.