3GPP TSG-RAN WG4 Meeting # 116bis											R4-2513512

Prague, Czech Republic, 13th – 17th October, 2025

Agenda item:			6.12.1

Source:	Moderator (Samsung)

Title:	Topic summary for [116bis][206] NR_MIMO_Ph5_RRM_Part1

Document for:	Information

Introduction

This topic summary covers the contributions submitted under the following AI for part of Rel-19 RRM for NR MIMO Phase 5:

- 4.18	NR MIMO Phase 5

- 4.18.2 RRM core requirements

- 6.12	NR MIMO Phase 5

- 6.12.2 RRM performance requirements

  - 6.12.2.1 Enhancement for UE-initiated/event-driven beam management

Recommended issues to online discussion:

2-1: To finalize the test case list for UEIBM

2-2

1-2

1-3

Topic #1: RRM core requirements maintenance

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2513457 | Xiaomi | Proposal 1: Update the specification to remove the bracket around "UL TCI state(s)" and make the requirement explicitly include both DL and UL TCI states as valid sources for timing reference: - if “PRACH association indicator” in DCI format 1_0 is 1, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL or UL TCI state(s) of the reference cell associated with the second TAG. - For inter-cell multi-TRP, for PUCCH/PUSCH/SRS: For each TAG, the reference point for PUCCH/PUSCH/SRS, is the first detected path (in time) of one of the corresponding downlink reference signal(s) of the reference cell associated with one of the DLorJointTCIState or UL TCI states having the same TAG as the uplink signal. |
| R4-2514050 | Nokia | Proposal 1: If the reporting condition becomes met after restarting the timer for the beam to be reported, the reporting delay is the time from when the event condition is met for the first time till the PUCCH transmission, which also includes windows during which the reporting condition was not fulfilled. Proposal 2: RAN4 to clarify in the specification that the current beam and the new beam shall follow the same value of M. |

Open issues summary

Sub-topic 1-1

- Issue 1-1: Timing requirements for two TAGs clarification

- Proposals

  - Option 1: (Xiaomi)

    - Update the specification to remove the bracket around "UL TCI state(s)" and make the requirement explicitly include both DL and UL TCI states as valid sources for timing reference

- Recommended WF

  - Close the issue. The bracket has been removed in last RAN#116 meeting and spec v19.2.0

- Issue 1-2: Further update reporting delay requirements for the case if  eventDetectionTimeWindowLength-r19 is configured?

- Proposals

  - Option 1: (Nokia)

    - If the reporting condition becomes met after restarting the timer for the beam to be reported, the reporting delay is the time from when the event condition is met for the first time till the PUCCH transmission, which also includes windows during which the reporting condition was not fulfilled.

- Recommended WF

  - TBA

- Issue 1-3: Whether to add restriction for current beam and new beams for “timeRestrictionForChannelMeasurement”?

- Proposals

  - Option 1: (Nokia)

    - RAN4 to clarify in the specification that the current beam and the new beam shall follow the same value of M.

- Recommended WF

  - TBA

Sub-topic 1-2: Draft CRs

| T-doc number | Company |  |
| --- | --- | --- |
| R4-2513700 | vivo | DraftCR on core requirements maintenance for Rel-19 MIMO UEIEBM |
| R4-2513701 | vivo | DraftCR on core requirements maintenance for Rel-19 MIMO asymmetric TRP |
| R4-2514051 | Nokia | DraftCR for UEIBM reporting delay |
| R4-2514259 | Nokia | Draft CR MTTD requirements for UL only TRP |
| R4-2514260 | Nokia | Draft CR on transmit timing requirements for asymmetric DL sTRP/UL mTRP deployment |

Topic #2: RRM performance requirements for UE-initiated/event-driven beam management

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2513471 | Huawei, HiSilicon | Proposal 1: For UEIBR, define TC as follows: - SSB-based UEIBR for Event-2. - CSI-RS based UEIBR for Event-2. Note 1: At least define TC when timeRestrictionForChannelMeasurement is configured. Note 2: At least define TC for intra-cell. Proposal 2: RAN4 to discuss whether to define TC for other Events, i.e. Event 1 and Event 7. Proposal 3: RAN4 not to define the TC for the case that no time window and eventInstanceCount-r19 > 1. Proposal 4: RAN4 to discuss whether to define TC for inter-cell UEIBR. Proposal 5: RAN4 not to define the TC for Beam switching delay for TCI states switching. Proposal 6: RAN4 defines TC for both Mode A and Mode B. The UE only need to pass one of them. Proposal 7: RAN4 to define the test cases of UEIEBM in FR2. |
| R4-2513551 | Qualcomm Technologies Ireland | Proposal 1: RAN4 should define test cases for event-2. Whether to define test cases for event-7 and event-1 can be further discussed. Proposal 2: RAN4 should not define test cases for CSI-RS based inter-cell beam management. Proposal 3: No performance requirements for TCI state switching should be defined. Proposal 4: No performance requirements should be defined for eventInstanceCout-r19 > 1. Performance requirements should only be defined for the case when no time window is defined. |
| R4-2513600 | Apple | Observation#1: Introducing test case for Event 2 is sufficient and doesn’t test anything different with test cases for Event 1 or Event 7. Proposal 1: Introduce test case for Event 2 alone. Observation#2: For Event 7 at a minimum 4 simultaneous AoAs are required for test set-up to support it. Proposal 2: Do not introduce test case for Event 7 as it is not feasible to be tested. Observation#3: Given the ending point of first PUCCH transmission, the requirements are the same for Mode A and Mode B. Proposal 3: Introduce test case for either Mode-A (preferred) or Mode-B. Proposal 4: Introduce test cases for (1) CSI-RS based event trigger and (2) inter-cell SSB based event trigger. Proposal 5: Do not introduce test cases for time window-based evaluation with eventInstanceCount-r19 > 1. Proposal 6: Introduce test cases in FR2 alone. Proposal 7: Do not introduce test cases for TCI state switching/ beam switching delay for UE-initiated/event-triggered beam management. |
| R4-2513642 | CMCC | Proposal 1: for UE-initiated/event-driven beam reporting, it is proposed to define test cases for: - SSB based intra-cell UEIBM, - SSB based inter-cell UEIBM, - CSI-RS based intra-cell UEIBM (CSI-RS is QCLed to the SSB of serving cell), - CSI-RS based inter-cell UEIBM (CSI-RS is QCLed to the SSB of additionPCI cell) Proposal 2: it is proposed to define test cases to cover Event-2, Event-1 and Event-7. Proposal 3: it is proposed to define test cases for both mode A and mode B. Proposal 4: it is proposed to define test cases for both no time window based and with time window based. Proposal 5: it is proposed to define test cases for L1 measurement reporting delay, since Tfirst UL channel is newly introduced as part of L1 measurement reporting delay. |
| R4-2513702 | vivo | Observation 1: Only define test cases for Event 2 is sufficient to verify the measurement performance of the feature. Proposal 1: Only define test cases for Event 2. Proposal 2: Define test cases for CSI-RS based Event 2 for intra-cell. Proposal 3: Define test case for SSB-based Event 2 for both intra-cell and inter-cell. If UE can the test case for inter-cell, the test case for intra-cell can be skipped. Proposal 4: Do not introduced test case for window-based Event 2 reporting. |
| R4-2513796 | Samsung | Observation 1: The testability of Event-7 has issue by using existing chamber for more AoAs. Proposal 1: RAN4 to add new test cases to cover: - sTRP serving cell SSB based L1-RSRP - sTRP serving cell CSI-RS based L1-RSRP - sTRP additional PCI cell SSB based L1-RSRP Prioritized for the tests are for event-2. We are open to choose limited TCs for event-1. No TCs are added for event-7. TCs are defined to cover both Mode A and Mode B. We support to define such TC for the case of eventInstanceCount-r19 > 1. Specify the test cases for FR2 test. Proposal 2: Test case list for UEIBM: TC1: event-2 sTRP serving cell SSB based L1-RSRP + FR2, mode A TC2: event-2 sTRP serving cell CSI-RS based L1-RSRP + FR2, mode B TC3: event-2 sTRP additional PCI cell SSB based L1-RSRP + FR2, mode A TC4: event-1 sTRP serving cell CSI-RS based L1-RSRP + FR2, mode B TC5: event-2 sTRP serving cell SSB based L1-RSRP + FR2, mode B, eventInstanceCount-r19 = 1 |
| R4-2513883 | ZTE Corporation, Sanechips | Proposal 1: To cover all the applicable cases in UEIBM, the following cases should be verified: - Intra-cell SSB based L1-RSRP UEIBM in FR2; - Inter-cell SSB based L1-RSRP UEIBM in FR2; - Intra/inter-cell CSI-RS based L1-RSRP UEIBM in FR2. Proposal 2: Since separate requirements for Case 1(no time window) and Case 2(with time window) are specified, both of them should be verified. Proposal 3: Both Mode A and Mode B should be verified. Proposal 4: RAN4 approved to define requirements for Event-2, Event-1 and Event-7. So all of these event types should be verified. |
| R4-2514054 | Nokia | Proposal 1: RAN4 to at least define a test case for Event-2. Proposal 2: If UE only supports Mode A, UE to be tested with Mode A. Proposal 3: If UE supports both Mode A and Mode B, UE to be tested with both modes. Proposal 4: Include reporting Mode A and Mode B as test configurations under each UEIBM test. No need to duplicate test cases. Proposal 5: RAN4 shall define test cases for Event-trigger for intra-cell and inter-cell BM for CSI-RS and SSB based reporting. Proposal 6: RAN4 to define test cases when eventInstanceCount-r19>1 for SSB and CSI-RS based reporting. Proposal 7: Since no new TCI state switch delay requirements have been introduced for UEIBM, no need to define a new test case. Proposal 8: RAN4 to not define a test case for the condition met indicator (CMI). |
| R4-2514299 | Ericsson | Proposal 1: RAN4 to define the test cases for Event-2 and Event-7 Proposal 2: Define test cases with time window-based and non-time window-based configurations. Proposal 3: For the SSB based event triggered reporting, RAN4 to define the test cases by considering the Scheme-2 (RS for the current beam is the SSB which is QCLed with the QCL RS in the indicated TCI state). Proposal 4: For CSI-RS based test case design, RAN4 to define the test cases by considering the Scheme-1 (RS for the current beam is the QCL RS in the indicated TCI state). Proposal 5: RAN4 to specify the periodic first PUCCH resource in the test configuration (at least the periodicityAndOffset). Proposal 6: No MAC CE-based TCI state switching test case is needed with event-driven reporting. Proposal 7: Consider CMI as part of the test reporting configuration. |
| R4-2514399 | MediaTek inc. | Proposal 1: RAN4 to prioritize event-2 for testing each of - Event-trigger for Intra-cell BM for SSB - Event-trigger for Intra-cell BM for CSI-RS - Event-trigger for Inter-cell BM for SSB Proposal 2: RAN4 to define test case for either one of mode A or mode B for event 2. |

Open issues summary

Sub-topic 2-1: Performance requirements for UE-initiated/event-driven beam management

- Issue 2-1: Test scope

|  | Huawei | QC | Apple | CMCC | vivo | Samsung | ZTE | Nokia | Ericsson | MTK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Whether to cover Event-2/Event-1/Event-7? | Event-2 FFS on others | Event-2 | Event-2 Only | Event1/2/7 | Event-2 | Event-2 FFS on one for event-1 No TC for Event-7 | Event-2/1/7 | Event-2 | Event-2&7 | Event-2 |
| Whether to cover Mode A and Mode B? | Both. UE only need to pass one of them. |  | Either A or B | Both |  | Both | both | Depends on UE capability. Capture A and B in each TC. by applicability |  | Either A or B |
| Event-trigger for Intra-cell BM for SSB | Y | Y | N | Y | Y UE pass one of Intra/Inter | Y | Y | Y |  | Y |
| Event-trigger for Intra-cell BM for CSI-RS | Y | Y | Y | Y | Y | Y | Y | Y |  | Y |
| Event-trigger for Inter-cell BM for SSB | FFS | Y | Y | Y | Y UE pass one of Intra/Inter | Y | Y | Y |  | Y |
| Event-trigger for Inter-cell BM for CSI-RS | FFS | N | N | Y |  | N | Y | Y |  | N |
| Whether to cover no time window and eventInstanceCount-r19 > 1? | N | N | N | Y | N | Y | Y | Y | Y |  |
| Frequency Range – FR2? | FR2 |  | FR2 |  |  | FR2 | FR2 |  |  |  |
| Beam switching delay for TCI states switching | N | N | N |  |  | N |  | N | N |  |

- Recommended WF

  - According to majority view, check whether it is agreeable:

    - RAN4 will not define new test cases for TCI states switching for UEIBM.

    - RAN4 will define all test cases for UEIBM for FR2.

  - For Event-2/Event-1/Event-7

    - All companies agreed Event-2.

    - For Evnet-7

      - HW: FFS

      - QC, Apple, vivo, Samsung, Nokia, MTK: No TC

      - CMCC, ZTE, Ericsson: add new TCs

    - For Event-1

      - HW: FFS

      - QC, Apple, vivo, Nokia, Ericsson, MTK: No TC

      - CMCC, ZTE: add new TCs

      - Samsung: add only one TC to verify.

  - For Mode A and Mode B, to decide one of below

    - Add either mode-A or Mode-B in separate cases.

    - Capture mode A and Mode B in test configuration of each TC, depends on UE capability, UE is only expected to pass one of them.

  - For window-based with eventInstanceCount-r19 > 1

    - QC, Apple, vivo: No TC

    - CMCC, Samsung, ZTE, Nokia, Ericsson, HW(where the time window has been configured): add TC

According to the views, at least these TCs are expected to be specified:

| TC1 (All support) | Event-2, FR2, Inter-cell SSB based, no window time based |
| --- | --- |
| TC2 (All support) | Event-2, FR2, Intra-cell CSI-RS based, no window time based |
| TC3 (TBA) | Event-2, FR2, Intra-cell SSB based, no window time based |
| TC4 (TBA) | Event-2, FR2, Inter-cell CSI-RS based, no window time based |
| Other TCs | Need further discussion on above issues. |

Moderator: mode-A is basic UE capability (59-1-1), mode-B (59-1-2) is optional (Mode-A is Prerequisite feature of Mode-B)

Please check R4-2513004

| Features | Index | Feature group | Components | Prerequisite feature groups |
| --- | --- | --- | --- | --- |
| 59. NR_MIMO_Ph5 | 59-1-1 | UE-initiated/event-driven beam management for Event-2 based measurement and report for Mode A | 1. Support of UE-initiated/event-driven beam report based on one event instance 2. Support of Event-2 based measurement and report 3. Support of Mode A UE-initiated/event-driven beam report 4. Maximum number of the configured RS(s) for new beam in the RS resource set 5. Support of current beam measurement by using QCL RS in the indicated TCI state and the corresponding QCL SSB for Scheme-1 and Scheme-2, respectively 6. Support the first PUCCH and second PUSCH from the same PUCCH group |  |
| 59. NR_MIMO_Ph5 | 59-1-2 | UE-initiated/event-driven beam management Mode B | 1. Support of Mode B UE-initiated/event-driven beam report 2. Supported minimum value of X symbols between the last symbol of sending first PUCCH and the first available symbol of transmission occasion of second PUSCH | 59-1-1 |

- Issue 2-2: Whether to add requirement to verify the performance of CMI?

- Proposals

  - Option 1: (Nokia)

    - No

  - Option 2: (Ericsson)

    - Consider CMI as part of the test reporting configuration.

- Recommended WF

  - TBA. This also depends on whether to add test case for window-based in Issue 2-1.