# 3GPP TSG-RAN WG4 Meeting #116bis	R4-2513828

# Prague, Czech Republic, Oct. 13-17, 2025

Source:			Ericsson

Title:			NTN Ku bands – VSAT RF and PFD limits

Agenda item:		4.2.7.2

Document for:		Approval

# Introduction

In last RAN4#116 meeting, RAN4 finalized the core aspects of the NTN Ku-bands WI ([1]) and agreed on a way forward ([5]) to capture the latest agreements made in that meeting.

Still, due to the lack of time, RAN4 didn’t agree on how to capture the regulatory PFD limits in TS 38.101-5. This contribution addresses this open issue.

# Discussion

## PFD limits

### Context

In last RAN4#116, how to capture the regulatory PFD limits was left for further discussion ([5]), leaving open if those limits should be captured in an informative or normative way in TS 38.101-5:

As we already explained in our previous contributions ([2] and [6]), if PFD limits for SAN might not be relevant for 3GPP as they are depending on SAN deployment, but the situation is not the same for the VSAT as the PFD limits are usually not depending on the type of deployment.

Also, the PFD limits being part of the Regulation, they are anyway mandatory to support for any VSAT operating in the corresponding NTN Ku-bands.

### About “% time” relaxation in requirement

In last RAN4#116 meeting, companies were concerned to capture the regulatory PFD limits when regulation states that this limit might be exceeded for a very short percentage of time. For example, ECC Decision 18(05), Annex 1 states the following:

If it’s true that RAN4 usually don’t specify RF requirements which have limits that might be exceeded for a short while, it should also be noted that such requirements have already been introduced in NTN specification TS 38.101-5, e.g. for the Off-axis EIRP limits (Table 9.2.2.3.2-1), the following note is added:

Observation 1: RAN4 already specified NTN limits that could be exceeded for a short amount of time.

Specifying a limit that could be exceeded for a short amount of time is then not a new approach in NTN context and would justify capturing similar regulatory PFD limit as well.

### About requirement based on simultaneous users transmitting

Another concern that was raised during that RAN4#116 meeting is that the PFD limit would apply for all VSATs operating in the mentioned frequency range and so it might be difficult to capture it in RAN4 specification. But, again, such type of requirements has already been captured in TS 38.101-5. For example, looking at Table 9.2.2.3.2-1, the limit is specified depending on the number of terminals transmitting simultaneously:

Observation 2: RAN4 already specified NTN limits depending on the number of terminals simultaneously transmitting. This could also be done for PFD limits if needed.

Nevertheless, similar to what was done in EN 302 186 ([3]), to simplify the requirements, we would suggest to not do so.

### About NCF implication

Yet another concern raised in RAN4#116 meeting was, as stated in ECC Decision(18)05, that ESIM might be under controlled of a NCF (Network Control Facility) to ensure compliance with the specified PFD limits:

This is valid comment but, as stated in this ECC Decision(18)05, it’s not mandatory that the monitoring is done by a NCF, this could also be done locally by the ESIM. Moreover, even if done by the NCF, that doesn’t mean the requirement could just be ignored and not tested. Looking at EN 302 186 ([3]), even if this Harmonized Standard clearly states that the NCF is out of its scope, it still specifies PFD requirements and associated testing procedure as shown below.

Observation 3: Even if the NCF is out of scope of EN 301 186, this Harmonized Standard specifies PFD limits and associated test procedures.

### Proposals

Based on these clarifications and observations, we make the following proposal:

Proposal 1: Capture the regulatory PFD limits as normative requirements. If not, TS 38.101-5 will not be complete.

Proposal 2: Capture the regulatory PFD limits for NTN VSAT via NS signalling, as suggested in our companion draft CR([4]).

We are of course open to discuss any other proposal to capture the regulatory PFD limits as normative requirement.

# Conclusion

In this contribution, we analyzed the remaining issue related to PFD requirements for NTN VSAT. We made the following observations and proposals:

Observation 1: RAN4 already specified NTN limits that could be exceeded for a short amount of time.

Observation 2: RAN4 already specified NTN limits depending on the number of terminals simultaneously transmitting. This could also be done for PFD limits if needed.

Observation 3: Even if the NCF is out of scope of EN 301 186, this Harmonized Standard specifies PFD limits and associated test procedures.

Proposal 1: Capture the regulatory PFD limits as normative requirements. If not, TS 38.101-5 will not be complete.

Proposal 2: Capture the regulatory PFD limits for NTN VSAT via NS signalling, as suggested in our companion draft CR([4]).

# References

- RP-250799, Introduction of Ku bands for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat, Airbus, Sharp, Mitre, JSAT

- R4-2506144, NTN Ku-bands – VSAT, Ericsson

- EN 302 186, Satellite mobile Aircraft Earth Stations (AESs) operating in the 11/12/14 GHz frequency bands; Harmonised Standard for access to radio spectrum.

- R4-2513829, Draft CR to TS 38.101-5 – Regional PFD requirements, Ericsson

- R4-2512696, Way Forward for [116][314] NR_NTN_Ku_Band_UE_SAN_RF, CHTTL

- R4-2510503, NTN Ku-band - NTN VSAT PFD, Ericsson