3GPP TSG-RAN WG4 Meeting #116bis															R4-2514433

Prague, Czech Republic, Oct. 13-17, 2025

Agenda item:			6.23

Source:	Moderator (MediaTek inc.)

Title:	Topic summary for [116bis][310] Rel-19 RRM performance_Part2

Document for:	Information

# Introduction

This document provides topic summary for NTN RRM performance part 2, including the following topics:

- Topic#1: RRM performance requirements for NR NTN Ku band (AI 5.10.2)

- Topic#2: NTN testing for NGSO (AI 6.6.3.2)

- Topic#3: RRM performance requirements for IoT NTN phase 3 (AI 6.20.3)

- Topic#4: RRM performance requirements for IoT-NTN TDD mode (AI 6.21.2)

Recommended issues for online discussion:

- Issue 1-1-1, 1-2-1, 1-2-2, 1-2-3, 1-2-4

- Issue 2-1-1, 2-2-1, 2-3-1

- Issue 3-1-1, 3-1-2, 3-1-3

- Issue 4-1-1, 4-1-2, 4-2-2, 4-2-3

# Topic# 1: NR NTN Ku band (AI 5.10.2)

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2513635 | MediaTek inc. | Proposal 1: introduce the following new test cases for Ku‑band VSAT - Satellite-switching for both FR1-NTN and FR2-NTN - Intra-frequency inter-satellite handover for FR1-NTN Proposal 2: Add a table clarifying test applicability for FR1‑NTN VSATs, as shown below. Requirement; Applicable test cases for VSAT with FR1-NTN numerology. Re-selection; A.14.1.1, A.14.1.5. Handover; A.14.2.1.1. Satellite switching; TBA. UE transmit timing; A.14.3.1.1. Timing Advance; A.14.3.2.1. Radio Link Monitoring; A.14.4.1.1, A.14.4.1.2. Measurement Procedure; A.14.5.1.1, A.14.5.3.1. Measurement performance; A.14.6.1.1. |
| R4-2514064 | Ericsson | Proposal 1: In performance part, add notes in the corresponding FR2-NTN test cases to clarify that they are applicable to NTN Ku band UEs, i.e., both mobile VSAT UEs operating with FR1-NTN numerology and VSAT UEs operating with FR2-NTN numerology. Proposal 2: Design a test case for satellite switching with re-synchronization for NTN Ku band UEs and FR2-NTN UEs. |
| R4-2514095 | ZTECorporation,Sanechips | Proposal 1: The following measurement accuracy requirements shall be considered: - Intra-frequency RSRP accuracy requirements - Inter-frequency RSRP accuracy requirements - Intra-frequency RSRQ accuracy requirements - Inter-frequency RSRQ accuracy requirements - Intra-frequency SINR accuracy requirements - Inter-frequency SINR accuracy requirements - L1-RSRP accuracy requirements Observation 1: The requirements defined for FR2-NTN apply for VSAT working in FR1-NTN including accuracy requirements. Proposal 2: The legacy measurement accuracy requirements for Ka band NTN shall be reused for Ku band FR1-NTN and FR2-NTN with different numerologies agreed in core part.. Proposal 3: For how to capture the measurement accuracy requirements, RAN4 shall add FR1 numerology in FR2-NTN accuracy requirements table. For where to capture the measurement accuracy requirements, RAN4 shall use the existing measurement accuracy requirements for FR2-NTN instead of introducing new clause. Proposal 4: For test principle and setups, the testing related to satellite access defined in A.3.36 with FR2 can be a reference. Proposal 5: For test configuration, Option 1: If add different SCS in one test configuration table, if mobile VSAT UE supports both GSO and NGSO, it is at least to pass two test config with FR1 numerology and FR numerology, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases. Option 2: If use different test configuration tables, if mobile VSAT UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases. |
| R4-2514161 | Huawei, HiSilicon | Proposal 1: For Ku bands in FR2-NTN, the existing test cases defined for FR2-NTN (in R18 for Ka band) are re-used, except the following - Measurement accuracy test cases are updated to include the new bands - New test cases are defined for satellite switch with re-sync - UL timing test cases are updated to include test configuration for mobile VSAT served by NGSO Proposal 2: For Ku bands in FR1-NTN, RAN4 to discuss whether it is feasible to re-use existing test cases for FR1-NTN corresponding to those defined for FR2-NTN, except the following - Test cases for blind HO and satellite switch with re-sync, for which the test requirements are updated to add requirements for VSAT in FR1-NTN. Proposal 3: RAN4 to discuss the test applicability for VSAT supporting Ku bands in both FR1-NTN and FR2-NTN, if such VSAT exists. |
| R4-2514348 | Nokia | Proposal 1: Introduce in all available test cases for VSAT UEs a new configuration that corresponds to the reference channels used for FR1-NTN numerology. Observation 1: If the mobility aspect of a VSAT UE is relevant enough to be distinguished via VSAT Type whether a UE is mobile or fixed, it is important that they are tested according to the type reported. Proposal 2: For the transmit timing test case, RAN4 to discuss how to emulate the mobility of the mobile VSAT UE. Proposal 3: Introduce NGSO configuration for transmit timing test case of a mobile VSAT. Proposal 4: Introduce new test cases for hard and soft satellite switching for VSAT UEs. |

## Open issues summary

Before f2f meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions..

### Sub-topic 1-1: Measurement Accuracy

Open issues and candidate options before meeting: N/A

#### Issue 1-1-1: Measurement accuracy with new band

Proposals from companies:

- Proposal 1 (ZTE): The following measurement accuracy requirements shall be considered:

  - Intra-frequency RSRP accuracy requirements

  - Inter-frequency RSRP accuracy requirements

  - Intra-frequency RSRQ accuracy requirements

  - Inter-frequency RSRQ accuracy requirements

  - Intra-frequency SINR accuracy requirements

  - Inter-frequency SINR accuracy requirements

  - L1-RSRP accuracy requirements

- Proposal 2 (ZTE):  The legacy measurement accuracy requirements for Ka band NTN shall be reused for Ku band FR1-NTN and FR2-NTN with different numerologies agreed in core part.

- Proposal 3 (ZTE):

  - For how to capture the measurement accuracy requirements, RAN4 shall add FR1 numerology in FR2-NTN accuracy requirements table.

  - For where to capture the measurement accuracy requirements, RAN4 shall use the existing measurement accuracy requirements for FR2-NTN instead of introducing new clause.

- Proposal 4 (Huawei):

  - For Ku bands in FR2-NTN, the existing test cases defined for FR2-NTN (in R18 for Ka band) are re-used, except the following

    - Measurement accuracy test cases are updated to include the new bands

Recommended WF:

- The legacy measurement accuracy requirements for Ka band NTN (FR2-NTN) shall be reused for Ku band FR1-NTN and FR2-NTN.

- Introduce FR1 numerology in FR2-NTN accuracy requirements table.

- The following measurement accuracy requirements shall be considered:

  - Intra-frequency RSRP accuracy requirements

  - Inter-frequency RSRP accuracy requirements

  - Intra-frequency RSRQ accuracy requirements

  - Inter-frequency RSRQ accuracy requirements

  - Intra-frequency SINR accuracy requirements

  - Inter-frequency SINR accuracy requirements

  - L1-RSRP accuracy requirements

- Measurement accuracy requirements and test cases are updated to include the new bands

### Sub-topic 1-2: Test cases

Open issues and candidate options before meeting: N/A

#### Issue 1-2-1: test cases with new requirement

Proposals from companies:

- Proposal 1 (MTK): introduce the following new test cases for Ku‑band VSAT

  - Satellite-switching for both FR1-NTN and FR2-NTN

  - Intra-frequency inter-satellite handover for FR1-NTN

- Proposal 2 (Ericsson): Design a test case for satellite switching with re-synchronization for NTN Ku band UEs and FR2-NTN UEs.

- Proposal 3 (Huawei):

  - For Ku bands in FR2-NTN, new test cases are defined for satellite switch with re-sync

  - Test cases for blind HO and satellite switch with re-sync, for which the test requirements are updated to add requirements for VSAT in FR1-NTN.

- Proposal 4 (Nokia): Introduce new test cases for hard and soft satellite switching for VSAT UEs.

Recommended WF

- For FR2 NTN, introduce new test cases for hard and soft satellite switching for VSAT UEs

- For FR1 NTN, for test cases of blind HO and satellite switch with re-sync, add new test requirements for VSAT in FR1-NTN.

#### Issue 1-2-2: UE transmit timing test for NGSO VSAT

Proposals from companies:

- Proposal 1 (Huawei): UL timing test cases are updated to include test configuration for mobile VSAT served by NGSO

- Proposal 2 (Nokia): Introduce NGSO configuration for transmit timing test case of a mobile VSAT.

Recommended WF

- Introduce NGSO configuration for transmit timing test case of a mobile VSAT.

#### Issue 1-2-3: Test cases for Ku bands in FR2-NTN

Proposals from companies:

- Proposal 1 (Ericsson): In performance part, add notes in the corresponding FR2-NTN test cases to clarify that they are applicable to NTN Ku band UEs, i.e., both mobile VSAT UEs operating with FR1-NTN numerology and VSAT UEs operating with FR2-NTN numerology.

- Proposal 2 (Huawei): For Ku bands in FR2-NTN, the existing test cases defined for FR2-NTN (in R18 for Ka band) are re-used, except the following

  - Measurement accuracy test cases are updated to include the new bands

  - New test cases are defined for satellite switch with re-sync

  - UL timing test cases are updated to include test configuration for mobile VSAT served by NGSO

- Proposal 3 (Nokia): For the transmit timing test case, RAN4 to discuss how to emulate the mobility of the mobile VSAT UE.

Recommended WF

- For Ku bands in FR2-NTN, the existing test cases defined for FR2-NTN (in R18 for Ka band) are re-used, unless otherwise specified.

- Further clarify test applicability.

  - Option 1: add notes in the each existing corresponding FR2-NTN test cases

  - Option 2: use a separate clause for clarification

- Further discuss how to emulate the mobility of the mobile VSAT UE.

#### Issue 1-2-4: Test cases for Ku bands in FR1-NTN

Proposals from companies:

- Proposal 1 (MTK): Add a table clarifying test applicability for FR1-NTN VSATs, by reusing exiting test cases defined for SAN FR1

- Proposal 2 (Huawei): For Ku bands in FR1-NTN, RAN4 to discuss whether it is feasible to re-use existing test cases for FR1-NTN corresponding to those defined for FR2-NTN, except the following

  - Test cases for blind HO and satellite switch with re-sync, for which the test requirements are updated to add requirements for VSAT in FR1-NTN.

- Proposal 3 (Nokia): Introduce in all available test cases for VSAT UEs a new configuration that corresponds to the reference channels used for FR1-NTN numerology.

Recommended WF:

Further discuss the following options

- Option 1: reuse exiting test cases defined for SAN FR1

  - Add a table clarifying test applicability for the corresponding test cases for VSAT UEs.

- Option 2: reuse existing test cases defined for FR2-NTN, unless otherwise specified

  - Introduce a new configuration that corresponds to the reference channels used for FR1-NTN numerology.

#### Issue 1-2-5: Test applicability and principle

Proposals from companies:

- Proposal 1 (ZTE): For test principle and setups, the testing related to satellite access defined in A.3.36 with FR2 can be a reference.

- Proposal 2 (ZTE): For test configuration,

  - Option 1: If add different SCS in one test configuration table, if mobile VSAT UE supports both GSO and NGSO, it is at least to pass two test config with FR1 numerology and FR numerology, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases.

  - Option 2: If use different test configuration tables, if mobile VSAT UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases.

- Proposal 3 (Huawei): RAN4 to discuss the test applicability for VSAT supporting Ku bands in both FR1-NTN and FR2-NTN, if such VSAT exists.

Recommended WF:

- the testing related to satellite access defined in A.3.36 with FR2 can be a reference.

- if mobile VSAT UE supports both GSO and NGSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases.

- RAN4 to discuss the test applicability for VSAT supporting Ku bands in both FR1-NTN and FR2-NTN, if such VSAT exists.

  - Option 1: it is at least to pass two test configurations: one with FR1 numerology and one with FR2 numerology.

  - Other options are not precluded.

# Topic# 2: NTN testing for NGSO (AI 6.6.3.2)

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2513191 | CATT | Proposal 1: The new NGSO channel model in Rel-19 shall be applied to the NTN and IoT-NTN uplink timing test cases defined in the current TS38.133 and TS36.133 according to the WID Proposal 2: For the timing advance test, FFS on whether addition margin should be added in the test requirements. Proposal 3: NTA,adjUE is calculated by the TE in the test, and clarification like option1b for assumption of TE calculation is ok for us. // Option 1b (Huawei): NTA,adjUE in test requirements is calculated based on the generated UL channel with time varying Doppler and delay shifts |
| R4-2513442 | Xiaomi | Observation 1: The NTN UL timing was tested under the legacy AWGN channel model. Observation 2: The over loading because of the other NTN testing cases beside the UL timing related needs to be considered. Observation 3: The other testing case beyond NGSO UL timing test applied the new TE channel model was not explicitly in the WID scope. Proposal 1: The new test configuration with NTN NGSO TE channel model shall be introduced for NTN NGSO UL timing tests ONLY. |
| R4-2513801 | Samsung | Proposal 1: The applicability of NGSO channel for NTN tests is for UE RRM performance test of uplink timing for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands. Proposal 2: First, NGSO channel model should be added in these TCs in the table. The corresponding configuration of NGSO channel should be added. - During the tests, UE position is needed to be added and set by AT command at the beginning of T1. In successive time periods of different T, UE position should keep unchanged. The TE adds the NGSO channel model by itself. Proposal 3: For NR-NTN - For transmit timing test, no impacts for the test procedure and test requirements. The NTA,adjUE in the test requirements is calculated by UE itself. - For the timing advance test, no impacts for the test requirements. Proposal 4: For five TCs for IoT-NTN, use the same principle as NR-NTN. |
| R4-2514158 | Huawei, HiSilicon | Proposal 1: The TE-emulated channel model with varying Doppler and delay shifts is applied to UL transmit timing test cases only and R19 UE only. Proposal 2: Apply the following adaptation to the test cases for NR NTN and IoT NTN for applying the TE-emulated channel model with varying Doppler and delay shifts - Change the “Propagation condition” from “AWGN” to “AWGN with time varying Doppler and delay shifts” - Clarify that NTA,adjUE in test requirements is calculated based on the generated UL channel with time varying Doppler and delay shifts |
| R4-2514321 | Qualcomm Incorporated | Applicability of the new channel model for the existing test cases Proposal 1: The satellite motion-based time varying channel model is used in ALL NTN “RRM” test cases for both IoT and NR. Applicability of the new channel model for pre-Rel-19 NTN UEs Proposal 2: The satellite motion-based time varying channel model can be retroactively applied to pre-Rel-19 test cases subject to DUT declaration. Applicability of pre-Rel-19 static NTN channel model Proposal 3: RAN4 to invalidate pre-Rel-19 test cases involving UE mobility (Location/Distance based handover and measurement report) if the satellite channel model in the test is based on Rel-17 static NTN channel model. The invalidated test cases can be applied if the Rel-19 satellite motion-based time varying NTN channel model can accommodate the UE mobility accordingly. |
| R4-2514337 | Nokia | Proposal 1: Apply the channel model to all test cases in RRM. Proposal 2: The satellite motion-based time varying channel model can be retroactively applied to pre-Rel-19 test cases subject to DUT declaration Proposal 3: N_"TA,adj" ^"UE" in test requirements is calculated based on the generated UL channel with time varying Doppler and delay shifts Proposal 4: Do not add new additional margins in the test case. Test tolerance is left for RAN5. |

## Open issues summary

### Sub-topic 2-1: New channel model in FR1-NTN for R19 NGSO uplink timing test

Background: RP-241281, Revised WID

NTN testing for NGSO - Specify the TE-emulated channel model with varying Doppler and delay shifts for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands - Match the satellite motion trajectory based on the ephemeris for NGSO (Non-Geostationary Orbit) scenarios - Inform RAN5 to assist specifying RF frequency error tests, if needed - Checking if the existing frequency error requirement, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE RRM performance test of uplink timing for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Checking if the existing RRM uplink timing requirements, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE demodulation performance requirement(s) of PDSCH for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Minimize the number of demodulation performance requirements - Specify the applicability of the tests under the TE-emulated channel model

The relevant UL timing tests of NTN is provided for reference:

For NR NTN FR1 tests in 36.133

## A.14.3.1 UE transmit timing for Satellite Access A.14.3.1.1 NR UE Transmit Timing Test for FR1 ## A.14.3.2 Timing advance for satellite access ## A.14.3.2.1 SA FR1 timing advance adjustment accuracy

Iot-NTN tests in 36.133

## A.13.4 Timing and signalling characteristics for satellite access ### A.13.4.1 UE transmit timing for satellite access A.13.4.1.1 E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under normal coverage for Satellite Access #### A.13.4.1.2 E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under enhanced coverage for Satellite Access #### A.13.4.1.3 E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under enhanced coverage with segment transmission in NGSO for Satellite Access ### A.13.4.2 UE timing advance for satellite access A.13.4.2.1 HD-FDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Normal Coverage for Satellite Access A.13.4.2.2 HD-FDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Enhance Coverage for Satellite Access

#### Issue 2-1-1: R19 UL timing tests and test requirement impact

Proposals from companies:

R4-2513191, CATT

- Proposal 1: The new NGSO channel model in Rel-19 shall be applied to the NTN and IoT-NTN uplink timing test cases defined in the current TS38.133 and TS36.133 according to the WID

- Proposal 2: For the timing advance test, FFS on whether addition margin should be added in the test requirements.

- Proposal 3: NTA,adjUE is calculated by the TE in the test, and clarification like option1b for assumption of  TE calculation is ok for us.

R4-2513442, Xiaomi

- Proposal 1: The new test configuration with NTN NGSO TE channel model shall be introduced for NTN NGSO UL timing tests ONLY.

R4-2513801, Samsung

- Proposal 2: First, NGSO channel model should be added in these TCs in the table. The corresponding configuration of NGSO channel should be added.

  - During the tests, UE position is needed to be added and set by AT command at the beginning of T1. In successive time periods of different T, UE position should keep unchanged. The TE adds the NGSO channel model by itself.

- Proposal 3: For NR-NTN

  - For transmit timing test, no impacts for the test procedure and test requirements.

    - The NTA,adjUE in the test requirements is calculated by UE itself.

  - For the timing advance test, no impacts for the test requirements.

- Proposal 4: For five TCs for IoT-NTN, use the same principle as NR-NTN.

R4-2514158, Huawei, HiSilicon

- Proposal 1: The TE-emulated channel model with varying Doppler and delay shifts is applied to UL transmit timing test cases only and R19 UE only.

- Proposal 2: Apply the following adaptation to the test cases for NR NTN and IoT NTN for applying the TE-emulated channel model with varying Doppler and delay shifts

  - Change the “Propagation condition” from “AWGN” to “AWGN with time varying Doppler and delay shifts”

  - Clarify that NTA,adjUE in test requirements is calculated based on the generated UL channel with time varying Doppler and delay shifts

R4-2514337, Nokia

- Proposal 3: NTA,adjUE in test requirements is calculated based on the generated UL channel with time varying Doppler and delay shifts

- Proposal 4: Do not add new additional margins in the test case. Test tolerance is left for RAN5.

Recommended WF:

- Apply the new FR1-NTN channel model for R19 UL transmit timing test cases

  - No impacts for the test procedure or test requirements; apply the following adaptation

    - Change the “Propagation condition” from “AWGN” to “AWGN with time varying Doppler and delay shifts”

    - Clarify that NTA,adjUE in test requirements is calculated based on the generated UL channel with time varying Doppler and delay shifts.

    - During the tests, UE position is needed to be added and set by AT command at the beginning of T1. In successive time periods of different T, UE position should keep unchanged. The TE adds the NGSO channel model by itself.

  - Do not add new additional margins in the test case. Test tolerance is left for RAN5

- FFS applicability of the timing advance test and its additional margin

### Sub-topic 2-2: New channel model for R19 tests other than UL timing

#### Issue 2-2-1: Extend the applicability of the new channel model for the existing test cases

Proposals from companies:

R4-2513191, CATT

- Proposal 1: The new NGSO channel model in Rel-19 shall be applied to the NTN and IoT-NTN uplink timing test cases defined in the current TS38.133 and TS36.133 according to the WID

R4-2513442, Xiaomi

- Proposal 1: The new test configuration with NTN NGSO TE channel model shall be introduced for NTN NGSO UL timing tests ONLY.

R4-2513801, Samsung

- Proposal 1: The applicability of NGSO channel for NTN tests is for UE RRM performance test of uplink timing for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands.

R4-2514158, Huawei, HiSilicon

- Proposal 1: The TE-emulated channel model with varying Doppler and delay shifts is applied to UL transmit timing test cases only and R19 UE only.

R4-2514321, Qualcomm Incorporated

- Proposal 1: The satellite motion-based time varying channel model is used in ALL NTN “RRM” test cases for both IoT and NR.

R4-2514337, Nokia

- Proposal 1: Apply the channel model to all test cases in RRM.

Recommended WF:

Please discuss the following options:

- Option 1: not apply to all NTN FR1 RRM tests

- Option 2: apply to all NTN FR1 RRM tests

### Sub-topic 2-3: New channel model for pre-Rel-19 NTN UEs

#### Issue 2-3-1: Applicability of the new channel model for pre-Rel-19 NTN UEs

Proposals from companies:

R4-2514321, Qualcomm Incorporated

- Proposal 2: The satellite motion-based time varying channel model can be retroactively applied to pre-Rel-19 test cases subject to DUT declaration.

R4-2514337, Nokia

- Proposal 2: The satellite motion-based time varying channel model can be retroactively applied to pre-Rel-19 test cases subject to DUT declaration

Recommended WF:

Please discuss the following options:

- Option 1: not apply to pre-Rel-19 test cases

- Option 2: apply to pre-Rel-19 test cases subject to DUT declaration

#### Issue 2-3-2: Applicability of pre-Rel-19 static NTN channel model

Proposals from companies:

R4-2514321, Qualcomm Incorporated

- Proposal 3: RAN4 to invalidate pre-Rel-19 test cases involving UE mobility (Location/Distance based handover and measurement report) if the satellite channel model in the test is based on Rel-17 static NTN channel model. The invalidated test cases can be applied if the Rel-19 satellite motion-based time varying NTN channel model can accommodate the UE mobility accordingly.

Recommended WF:

- Discuss the proposal

# Topic# 3: Testing for CB-Msg3-EDT (AI 6.20.3)

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2513638 | MediaTek inc. | Observation 1: In order to trigger CB-Msg3, conditions for initiating CB-Msg3 EDT in NTN should be met, as in TS 36.331, 5.3.3.1b. Proposal 1: The CB‑Msg3‑EDT configuration, as defined in CB‑Msg3‑ConfigSIB‑NB‑r19 should be provided, where cb-Msg3-MinRSRP-Threshold-NB-r19 can be {39, 24} ({-101, -116} dBm), and let RAN5 define the parameter details. |
| R4-2514339 | Nokia | Proposal 1: The RAN4 test case for CB-Msg3 shall test the following functionalities: a. Reception of an incorrect message over CB-RNTI b. Reception of a correct message of CB-RNTI c. No CB-Msg3 response within CB-Ms3ResponseTimer d. The calculated UE Tx power at every CB-Msg3 retransmission according to the respective formula e. The right amount of transmission of replicas. Proposal 2: RAN4 to include CQI reporting on the test cases for CB-MSG3. Proposal 3: The HARQ feedback upon reception of Msg4 shall be configured in the configuration of gNB parameters in the test case. Proposal 4: RAN4 shall discuss how to enforce the CB-MSG3 transmission for the test case. |

## Open issues summary

### Sub-topic 3-1: test cases

Background: WF in RAN4#116 (R4-2512148)

- Define only TC1.

| Random Access (A.13.3.2) | A.13.3.2.X1 Contention Based Random Access Test for UE category NB1 UEs in Satellite Access - Standalone mode in normal coverage for CB-msg3-EDT | TC1 |
| --- | --- | --- |

#### Issue 3-1-1: Test purpose

Proposals from companies:

- Proposal 1 (Nokia): The RAN4 test case for CB-Msg3 shall test the following functionalities:

a.	Reception of an incorrect message over CB-RNTI

b.	Reception of a correct message of CB-RNTI

c.	No CB-Msg3 response within CB-Ms3ResponseTimer

d.	The calculated UE Tx power at every CB-Msg3 retransmission according to the respective formula

e.	The right amount of transmission of replicas.

- Proposal 2 (Nokia): RAN4 to include CQI reporting on the test cases for CB-MSG3.

Recommended WF: Discuss whether Proposals 1 and 2 are acceptable

#### Issue 3-1-2: Test configuration for CB-Msg3-EDT

Proposals from companies:

- Proposal 1 (MTK): The CB-Msg3-EDT configuration, as defined in CB-Msg3-ConfigSIB-NB-r19 should be provided, where cb-Msg3-MinRSRP-Threshold-NB-r19 can be {39, 24} ({-101, -116} dBm), and let RAN5 define the parameter details.

Recommended WF:

- The CB-Msg3-EDT configuration, as defined in CB-Msg3-ConfigSIB-NB-r19 should be provided.

- The legacy RSRP threshold used in the random access test can be reused for cb-Msg3-MinRSRP-Threshold-NB-r19.

#### Issue 3-1-3: Test configuration for HARQ

Proposals from companies:

- Proposal 1 (Nokia): The HARQ feedback upon reception of Msg4 shall be configured in the configuration of gNB parameters in the test case.

Recommended WF: Discuss whether Proposals 1 is acceptable

#### Issue 3-1-4: Ensure CB-MSG3 transmission

Proposals from companies:

- Proposal 1 (Nokia): RAN4 shall discuss how to enforce the CB-MSG3 transmission for the test case

Recommended WF: Further discuss how to enforce the CB-MSG3 transmission for the test case

# Topic# 4: IoT-NTN TDD mode (AI 6.21.2)

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2513639 | MediaTek inc. | Proposal 1: Work split for the RRM tests cases in the provided list. Test case; Core requirement; Description; Reference. Re-selection; 4.6B; Intra-frequency case, in normal coverage; A.13.1.1. Inter-frequency case, in normal coverage; A.13.1.4. RRC Re-establishment; 6.5B; Intra-frequency RRC Re-establishment, under normal coverage; A.13.3.1. Random Access; 6.6B; Contention Based Random Access, in normal coverage; A.13.3.2.1. UE transmit timing; 7.20B; UE Transmit Timing Accuracy, in normal coverage; A.13.4.1.1. Timing Advance; 7.22B; UE Transmit Timing Advance Adjustment Accuracy, in normal coverage; A.13.4.2.1. Radio Link Monitoring; 7.23B; out-of-sync / in-sync test, with/without DRX, 4 tests in total; A.13.4.3.1/4/5/7. Measurements; 8.14B; Intra-frequency neighbour cell measurement, in normal coverage; A.13.5.1. Inter-frequency neighbour cell measurement, in normal coverage; A.13.5.2. Downlink channel quality reporting accuracy; TBA; Downlink channel quality reporting accuracy, in normal coverage; A.13.6.2.1. Downlink channel quality reporting accuracy on non-anchor carrier, in normal coverage; A.13.6.2.4. Proposal 2: Only NGSO configuration is applicable. Proposal 3: Define measurement accuracy for IoT NTN TDD, including NRSRP, NRSRQ, and downlink channel quality reporting accuracy. |
| R4-2513826 | Iridium, CCL, THALES | - Proposal #1: Band groups for NB-IoT for satellite access in Table 3.5.1A-1 shall be updated to include NTN TDD band 249 as follows: Moderator: table to be added in the below proposal - Proposal #2: Update Annex B.3.25A to include IoT NTN TDD band 249 and if required with different minimum NRSRP value as follows: Moderator: table to be added in the below proposal - Proposal #3: RAN4 to consider introducing a new section, 9.1.22X, focused on measurement accuracy for UE Category NB-1 in satellite access scenarios. This section will be based on the existing 9.1.22A, with updates to reflect specific requirements for IoT NTN in TDD mode. - Proposal #4: For measurement accuracy requirements in IoT NTN TDD mode, at least [1] DL subframe per 9 radio frame of measured cell is available at the UE for NRSRP measurement assuming measured cell is identified cell. - Proposal #5: RAN4 to consider introducing intra frequency absolute NRSRP accuracy requirement for IoT NTN TDD as follows: Moderator: table to be added in the below proposal - Proposal #6: For measurement accuracy requirements in IoT NTN TDD mode, at least [1] DL subframe per 9 radio frame of measured cell is available at the UE for NRSRQ measurement assuming measured cell is identified cell. - Proposal #7: RAN4 to consider introducing intra frequency absolute NRSRQ accuracy requirement for IoT NTN TDD as follows: Moderator: table to be added in the below proposal - Proposal #8: RAN4 to consider adding inter frequency absolute NRSRP accuracy requirement for IoT NTN TDD as follows: Moderator: table to be added in the below proposal - Proposal #9: RAN4 to consider inter frequency inter frequency absolute NRSRPQ accuracy requirement for IoT NTN TDD as follows: Moderator: table to be added in the below proposal - Proposal #10: The existing NRSRP measurement report mapping can be re-used for IoT NTN TDD mode without modification. 9.1.22A.5 - Proposal #11: The existing NRSRQ measurement report mapping can be re-used for IoT NTN TDD mode without modification. 9.1.22A.6 - Proposal #12: The existing msg3-based measurement report mapping can be re-used for IoT NTN TDD mode without modification 9.1.22A.7 - Proposal #13: RAN4 to consider introducing downlink channel quality reporting accuracy requirement for IoT NTN TDD as follows: Moderator: table to be added in the below proposal - Proposal #14: UE power class 6 should be excluded from the power headroom performance requirements for IoT NTN TDD mode. - Proposal #15: The power headroom performance requirements specified for UE category NB1 for satellite access in section 9.1.23A shall also apply to IoT NTN TDD mode. - Proposal #16: RAN4 to consider updating NPDSCH reference channel for UE category NB1 operating in IoT NTN TDD mode based on Table A.3.1.5.3.1 as follows: Moderator: table to be added in the below proposal - Proposal #17: The existing NPDCCH reference channel parameters specified for NB-IoT NTN FDD for standalone mode can also be applicable for IoT NTN TDD mode. - Proposal #18: RAN4 to consider updating OCNG parameters for UE category NB1 operating in IoT NTN TDD mode based on Table A.3.2.3.3.1 as follows: Moderator: table to be added in the below proposal - Proposal #19: RAN4 to consider updating NPRACH reference configurations for IoT NTN TDD mode based on Table A.3.18-1 as follows: Moderator: table to be added in the below proposal - Proposal #20: Following list of test cases from the existing NB-IoT NTN FDD mode are applicable for IoT NTN TDD: - A.13.1.1.1 HD – FDD and IoT NTN TDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage - A.13.1.1.3 HD – FDD and IoT NTN TDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage with UE specific DRX - A.13.1.1.4 HD – FDD and IoT NTN TDD Inter frequency case for UE Category NB1 Standalone mode in normal coverage with UE specific DRX - A.13.3.1.1 HD-FDD and IoT NTN TDD Intra-frequency RRC Re-establishment for UE category NB1 in Standalone mode under normal coverage - A.13.3.2.1 Contention Based Random Access Test for UE category NB1 UEs in Satellite Access - Standalone mode in normal coverage - A.13.4.1.1 E-UTRAN HD-FDD and IoT NTN TDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under normal coverage for Satellite Access - A.13.4.2.1 HD-FDD and IoT NTN TDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Normal Coverage for Satellite Access - A.13.4.3.1 HD-FDD and IoT NTN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 Standalone mode in normal coverage - A.13.4.3.4 HD-FDD and IoT NTN TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 Standalone mode in Normal Coverage - A.13.4.3.5 HD-FDD and IoT NTN TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage - A.13.4.3.7 HD-FDD and IoT NTN TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage - A.13.5.1 HD-FDD and IoT NTN TDD Intra-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage for Satellite Access - A.13.5.2 HD-FDD and IoT NTN TDD Inter-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage for Satellite Access - A.13.5.3 HD-FDD and IoT NTN TDD Intra-frequency location-based neighbour cell measurement for UE category NB1 in standalone mode under normal coverage for Satellite Access - A.13.6.2.1 E-UTRAN HD-FDD and IoT NTN TDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under normal coverage - A.13.6.2.3 E-UTRAN HD-FDD and IoT NTN TDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under normal coverage - A.13.6.2.5 E-UTRAN HD-FDD and IoT NTN TDD Downlink channel quality reporting accuracy in RRC_CONNECTED for UE Category NB1 Standalone mode under normal coverage - Proposal #21: The highlighted NPRACH procedure test configuration parameters from Table A.13.3.2.1.1-2 should be updated for IoT NTN TDD mode. Companies are encouraged to propose suitable values. Moderator: table to be added in the below proposal - Proposal #22: The highlighted timing advance accuracy test configuration parameters from Table A.13.4.2.1.1-2 should be updated for IoT NTN TDD mode. Companies are encouraged to propose suitable values. Moderator: table to be added in the below proposal - Proposal #23: The highlighted radio link monitoring test for out-of-sync in DRX test configuration parameters from Table A.13.4.3.1.1-2 should be updated for IoT NTN TDD mode. Companies are encouraged to propose suitable values. Moderator: table to be added in the below proposal |
| R4-2514182 | Huawei, HiSilicon | Proposal 1: RAN4 to define accuracy requirements for NRSRP/Q and channel quality report for IoT NTN TDD by re-using the existing requirements for NB-IoT NTN, except the side condition. Proposal 2: RAN4 to define PHR requirements for IoT NTN TDD by re-using the existing requirements for NB-IoT NTN. Proposal 3: RAN4 to define the following TCs for IoT NTN TDD. - Intra-frequency cell reselection - Inter-frequency cell reselection - Re-establishment - Random access - Transmit timing and TA accuracy - RLM OOS and RLM IS, both without DRX and with DRX - Intra-frequency neighbor cell measurement, with radio condition trigger and with location trigger - Inter-frequency neighbor cell measurement with radio condition trigger - Channel quality reporting accuracy, in both Msg3 and connected mode Proposal 4: RAN4 to discuss the Qin and Qout for RLM test cases based on Rmax =2. |
| R4-2514344 | Nokia | Proposal 1: RAN4 shall focus on defining new reference channels for NPDSCH, NPDCCH, NPRACH and any other applicable channel in Annex A.3 of TS 36.133. Proposal 2: For existing NB-IoT NTN test cases, create a new test configuration with the new reference channel models and reuse the test description. The test requirements when the new configuration is used might be adapted to reflect the agreements of the core requirements. Proposal 3: RAN4 to agree which tests needs to be tested for each configuration when the UE supports both TDD and FDD frame structures, to avoid the duplication of test effort. |

## Open issues summary

### Sub-topic 4-1: RRM performance requirements

Open issues and candidate options before meeting: N/A

#### Issue 4-1-1: Introduction of new band group

Proposals from companies:

R4-2513826	Iridium, CCL, THALES

- Proposal #1: Band groups for NB-IoT for satellite access in Table 3.5.1A-1 shall be updated to include NTN TDD band 249 as follows:

- Table 3.5.1A-1: Band groups for NB-IoT for satellite access

| Group | E-UTRA FDD |  |
| --- | --- | --- |
|  | Band group notation | Operating bands |
| A | NFDD_SAB_A |  |
| B | NFDD_SAB_B |  |
| C | NFDD_SAB_C |  |
| D | NFDD_SAB_D |  |
| E | NFDD_SAB_E |  |
| F | NFDD_SAB_F |  |
| G | NFDD_SAB_G | 253, 254, 255, 256 |
| H | NFDD_SAB_H |  |
| I | NFDD_SAB_I |  |
| J | NFDD_SAB_J |  |
| K | NFDD_SAB_K |  |
| L | NFDD_SAB_L |  |
| M | NFDD_SAB_M |  |
| N | NFDD_SAB_N |  |
| X | NTDD_SAB_N | 249 |

- Proposal #2: Update Annex B.3.25A to include IoT NTN TDD band 249 and if required with different minimum NRSRP value as follows:

- Table B.3.25A-1: NB-IoT intra-frequency absolute NRSRP and NRSRQ Accuracy Requirements

| Parameter | E-UTRA/NR operating band groups Note 1 | Minimum NRSRP |
| --- | --- | --- |
|  |  | dBm/15kHz |
| Conditions | NFDD_SAB _G | -139.8 |
|  | NTDD_SAB_X | FFS |
| NOTE 1: E-UTRA/NR operating band groups are as defined in Section 3.5. |  |  |

Recommended WF: agree on the above proposals

#### Issue 4-1-2: Measurement accuracy – General

Proposals from companies:

R4-2513639	MediaTek inc.

- Proposal 3: Define measurement accuracy for IoT NTN TDD, including NRSRP, NRSRQ, and downlink channel quality reporting accuracy.

R4-2514182	Huawei, HiSilicon

- Proposal 1: RAN4 to define accuracy requirements for NRSRP/Q and channel quality report for IoT NTN TDD by re-using the existing requirements for NB-IoT NTN, except the side condition.

R4-2513826	Iridium, CCL, THALES

- Proposal #3: RAN4 to consider introducing a new section, 9.1.22X, focused on measurement accuracy for UE Category NB-1 in satellite access scenarios. This section will be based on the existing 9.1.22A, with updates to reflect specific requirements for IoT NTN in TDD mode.

- Proposal #4: For measurement accuracy requirements in IoT NTN TDD mode, at least [1] DL subframe per 9 radio frame of measured cell is available at the UE for NRSRP measurement assuming measured cell is identified cell.

- Proposal #6: For measurement accuracy requirements in IoT NTN TDD mode, at least [1] DL subframe per 9 radio frame of measured cell is available at the UE for NRSRQ measurement assuming measured cell is identified cell.

Recommended WF:

- RAN4 to define accuracy requirements for NRSRP, NRSRQ and downlink channel quality report.

- RAN4 to consider adding section 9.1.22X (based on the existing 9.1.22A) with updated side conditions and band groups.

- measurement accuracy requirements apply, provided at least [1] DL subframe per 9 radio frame of measured cell is available at the UE for NRSRP/NRSRQ measurements assuming measured cell is identified cell.

#### Issue 4-1-3: Measurement accuracy – Detail

Proposals from companies:

R4-2513826	Iridium, CCL, THALES

- Proposal #5: RAN4 to consider introducing intra frequency absolute NRSRP accuracy requirement for IoT NTN TDD as follows:

Table 9.1.22X.1-1: NRSRP Intra frequency absolute accuracy for UE Category NB1

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  | E-UTRA/NR operating band groups Note 2 | Minimum Io |  | Maximum Io |
| dB | dB | dB |  | dBm/15kHz | dBm/BWChannel | dBm/BWChannel |
| 6 | 9 | -1 dB | NTDD_SAB_X | -122.9 | N/A | -70 |
| 8 | 11 | -1 dB | NTDD_SAB_X | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: E-UTRA/NR operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |

- Proposal #7: RAN4 to consider introducing intra frequency absolute NRSRQ accuracy requirement for IoT NTN TDD as follows:

Table 9.1.22X.2-1: NRSRQ Intra frequency absolute accuracy for UE Category NB1

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |
|  |  |  | E-UTRA/NR operating band groups Note 3 | Minimum Io | Maximum Io |
| dB | dB | dB |  | dBm/15kHz | dBm/BWChannel |
| 5.2 | 8.2 | -1 dB-3 dB | NTDD_SAB_X | -122.9 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: E-UTRA/NR operating band groups are as defined in Section 3.5. |  |  |  |  |  |

- Proposal #8: RAN4 to consider adding inter frequency absolute NRSRP accuracy requirement for IoT NTN TDD as follows:

Table 9.1.22X.3-1: NRSRP Inter frequency absolute accuracy for UE Category NB1

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  | E-UTRA/NR operating band groups Note 2 | Minimum Io |  | Maximum Io |
| dB | dB | dB |  | dBm/15kHz | dBm/BWChannel | dBm/BWChannel |
| 6 | 9 | -1 dB -6 dB | NTDD_SAB_X | -122.9 | N/A | -70 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: E-UTRA/NR operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |

- Proposal #9: RAN4 to consider inter frequency inter frequency absolute NRSRPQ accuracy requirement for IoT NTN TDD as follows:

Table 9.1.22X.4-1: NRSRQ Inter frequency absolute accuracy for UE Category NB1

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |
|  |  |  | E-UTRA/NR operating band groups Note 3 | Minimum Io | Maximum Io |
| dB | dB | dB |  | dBm/15kHz | dBm/BWChannel |
| 5.2 | 8.2 | -1 dB-3 dB | NTDD_SAB_X | -122.9 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: E-UTRA/NR operating band groups are as defined in Section 3.5. |  |  |  |  |  |

- Proposal #13: RAN4 to consider introducing downlink channel quality reporting accuracy requirement for IoT NTN TDD as follows:

Table 9.1.22X.8-1: Downlink channel quality reporting accuracy for UE Category NB1

| NPDCCH Repetition | Pm-Dsg (%) | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Ês/Iot | Io NOTE 1 range |  |  |  |
|  |  |  | E-UTRA/NR operating band groups NOTE 2 | Minimum Io |  | Maximum Io |
|  |  | dB |  | dBm/15kHz | dBm/BWChannel | dBm/BWChannel |
| R NOTE 1 | ≤1 | -1 dB-6 dB | NTDD_SAB_X | -122.9 | N/A | -70 |
| R/4 NOTE 1 | >1 | -1 dB -6 dB | NTDD_SAB_X | -122.9 | N/A | -70 |
| NOTE 1: R is the reported NPDCCH repetition level that UE has reported in CQI-NPDCCH-NB or CQI-NPDCCH-Short-NB. NOTE 2: Io is assumed to have constant EPRE across the bandwidth. NOTE 3: E-UTRA/NR operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |

Recommended WF: Agreed to the above proposed changes.

#### Issue 4-1-4: Measurement report mapping

Proposals from companies:

R4-2513826	Iridium, CCL, THALES

- Proposal #10: The existing NRSRP measurement report mapping can be re-used for IoT NTN TDD mode without modification.

- Proposal #11: The existing NRSRQ measurement report mapping can be re-used for IoT NTN TDD mode without modification.

- Proposal #12: The existing msg3-based measurement report mapping can be re-used for IoT NTN TDD mode without modification

Recommended WF:

- The existing NRSRP, NRSRQ, and msg3-based measurement report mapping can be re-used for IoT NTN TDD mode without modification.

#### Issue 4-1-5: PHR requirement

Proposals from companies:

R4-2514182	Huawei, HiSilicon

- Proposal 2: RAN4 to define PHR requirements for IoT NTN TDD by re-using the existing requirements for NB-IoT NTN.

R4-2513826	Iridium, CCL, THALES

- Proposal #14: UE power class 6 should be excluded from the power headroom performance requirements for IoT NTN TDD mode.

- Proposal #15: The power headroom performance requirements specified for UE category NB1 for satellite access in section 9.1.23A shall also apply to IoT NTN TDD mode.

Recommended WF:

- UE power class 6 should be excluded from the power headroom performance requirements for IoT NTN TDD mode.

- The power headroom performance requirements specified for UE category NB1 for satellite access in section 9.1.23A shall also apply to IoT NTN TDD mode.

### Sub-topic 4-2: Test case

#### Issue 4-2-1: Updated Reference Channel

Proposals from companies:

R4-2514344	Nokia

- Proposal 1: RAN4 shall focus on defining new reference channels for NPDSCH, NPDCCH, NPRACH and any other applicable channel in Annex A.3 of TS 36.133.

R4-2513826	Iridium, CCL, THALES

- Proposal #16: RAN4 to consider updating NPDSCH reference channel for UE category NB1 operating in IoT NTN TDD mode based on Table A.3.1.5.3.1 as follows:

- Table X.3.1.5.3-1: NPDSCH Reference Channel for UE category NB1 for standalone operation

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| NPDSCH Reference channel | - | R.18 HD-FDD | R.19 HD-FDD |
| Channel bandwidth | KHz | 200 | 200 |
| Number of transmitter antennas | - | 1 | 2 |
| Allocated subframes per Radio Frame | subframes | Note 1 | Note 1 |
| Modulation |  | QPSK | QPSK |
| Target coding rate |  | 1/3 | 1/3 |
| Information Bit Payload |  |  |  |
| For Sub-Frame 1, 2, 3, 6, 7, 8 | Bits | 72 Note 2 | 72 Note 2 |
| For Sub-Frame 4, 9 | Bits | Note 3 | Note 3 |
| For Sub-Frame 0, 5 | Bits | 0 | 0 |
| Number of Code Blocks per Sub-Frame |  |  |  |
| For Sub-Frame 1, 2, 3, 6, 7, 8 |  | 1 Note 4 | 1 Note 4 |
| For Sub-Frame 4, 9 |  | Note 5 | Note 5 |
| For Sub-Frame 0, 5 |  | 0 | 0 |
| Maximum number of repetitions | - | Note 6 | Note 6 |
| Cell ID | - | Note 7 | Note 7 |
| Note 1: Shall depend upon the NPDSCH scheduling Note 2: Only apply for subframes scheduled with NPDSCH. Note 3: 72 for subframes scheduled with NPDSCH when mod 2 ≠ 0. Otherwise 0. Note 4: only apply for subframes scheduled with NPDSCH.. Note 5: 1 for subframes scheduled with NPDSCH when mod 2 ≠ 0. Otherwise 0. Note 6: Maximum number of repetitions shall depend upon the test configuration. Note 7: Cell ID shall depend upon the test configuration. |  |  |  |

- Proposal #17: The existing NPDCCH reference channel parameters specified for NB-IoT NTN FDD for standalone mode can also be applicable for IoT NTN TDD mode.

- Proposal #18: RAN4 to consider updating OCNG parameters for UE category NB1 operating in IoT NTN TDD mode based on Table A.3.2.3.3.1 as follows:

Table X.3.2.3.3-1: NOP.3 FDD: OCNG FDD Pattern 3

| Allocation | Relative power level [dB] |  |  | NPDCCH and NPDSCH Data |
| --- | --- | --- | --- | --- |
|  | Subframe |  |  |  |
|  | 0, 4 | 5, 9 | 1-3, 6-8 |  |
| 0 | 0 (Note 2) | 0 (Note 2) | 0 | Note 1 |
| Note 1: This physical resource block is assigned to a virtual UE for transmission of NPDCCH or NPDSCH; the data transmitted over the NOCNG NPDCCH or NPDSCH shall be uncorrelated QPSK modulated pseudo random data. The parameter is used to scale the power of NPDCCH and NPDSCH. If two transmit antennas with NBRS are used in the test, the NPDCCH and NPDSCH parts of NOCNG shall be transmitted to the virtual users by both transmit antennas with NBRS and according to the antenna transmission mode 2. The parameter applies to each antenna port separately, so the transmit power of the NPDCCH and NPDSCH parts of NOCNG is equal between both the transmit antennas with NBRS used in the test. The antenna transmission modes are specified in clause 7.1 in TS 36.213. Note 2: Value of is applicable to PRBs not used for transmission of NPSS, NSSS and NPBCH in anchor cell (= 0). Note 3: SI transmissions, and NPDCCH and NPDSCH for the UE under test, have precedence over NOCNG. |  |  |  |  |

- Proposal #19: RAN4 to consider updating NPRACH reference configurations for IoT NTN TDD mode based on Table A.3.18-1 as follows:

- Table X.3.18-1: NPRACH.R-1: HD-FDD Reference NPRACH Configuration

| Field | Value |  |  | Comment |  |
| --- | --- | --- | --- | --- | --- |
| Parameters not per NPRACH coverage level |  |  |  |  |  |
| rsrp-ThresholdsPrach | {rsrp1, rsrp2} |  |  | The values of NPRACH RSRP thresholds for will be set according the requirement of individual test cases |  |
| nprach-CP-Length | us66dot7 |  |  | NPRACH format 0 |  |
| Parameters per NPRACH coverage Level |  |  |  |  |  |
| CE Level | Level 0 | Level 1 | Level 2 |  | Valid values as defined in TS 36.331 [2] |  |
| nprach-Periodicity | ms640ms40 | ms640ms40 | ms640ms40 |  | {ms40, ms80, ms160, ms240, ms320, ms640, ms1280, ms2560} |  |
| nprach-StartTime | ms8 | ms8 | ms8 |  | {ms8, ms16, ms32, ms64, ms128, ms256, ms512, ms1024} |  |
| nprach-SubcarrierOffset | n0 | n0 | n0 |  | {n0, n12, n24, n36, n2, n18, n34} |  |
| nprach-NumSubcarriers | n12 | n12 | n12 |  | {n12, n24, n36, n48} |  |
| nprach-SubcarrierMSG3-RangeStart | {one} | {one} | {one} |  | {zero, oneThird, twoThird, one} |  |
| maxNumPreambleAttemptCE | n3 | n5 | n7 |  | {n3, n4, n5, n6, n7, n8, n10} |  |
| numRepetitionsPerPreambleAttempt | n1 | n8 | n16n32 |  | {n1, n2, n4, n8, n16, n32, n64, n128} |  |
| npdcch-NumRepetitions-RA | r1 | r8 | r16r32 |  | {r1, r2, r4, r8, r16, r32, r64, r128, r256, r512, r1024, r2048} |  |
| npdcch-StartSF-CSS-RA | v64v8 | v32v2 | V64v2 |  | {v1dot5, v2, v4, v8, v16, v32, v48, v64} |  |
| npdcch-Offset-RA | zero | zero | zero |  | {zero, oneEighth, oneFourth, threeEighth} |  |
| nprach-NumCBRA-StartSubcarriers | n8 | n8 | n8 |  | {n8, n10, n11, n12, n20, n22, n23, n24, n32, n34, n35, n36, n40, n44, n48} |  |
| Note 1: See Clause 6.7.3.2 in TS 36.331 [2] for further information on the parameters in this table. |  |  |  |  |  |

Recommended WF:

- For UE category NB1 operating in IoT NTN TDD mode,

  - Update the NPDSCH reference channel based on Table A.3.1.5.3.1 as follows:

Table X.3.1.5.3-1: NPDSCH Reference Channel for UE category NB1 for standalone operation

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| NPDSCH Reference channel | - | R.18 HD-FDD | R.19 HD-FDD |
| Channel bandwidth | KHz | 200 | 200 |
| Number of transmitter antennas | - | 1 | 2 |
| Allocated subframes per Radio Frame | subframes | Note 1 | Note 1 |
| Modulation |  | QPSK | QPSK |
| Target coding rate |  | 1/3 | 1/3 |
| Information Bit Payload |  |  |  |
| For Sub-Frame 1, 2, 3, 6, 7, 8 | Bits | 72 Note 2 | 72 Note 2 |
| For Sub-Frame 4, 9 | Bits | Note 3 | Note 3 |
| For Sub-Frame 0, 5 | Bits | 0 | 0 |
| Number of Code Blocks per Sub-Frame |  |  |  |
| For Sub-Frame 1, 2, 3, 6, 7, 8 |  | 1 Note 4 | 1 Note 4 |
| For Sub-Frame 4, 9 |  | Note 5 | Note 5 |
| For Sub-Frame 0, 5 |  | 0 | 0 |
| Maximum number of repetitions | - | Note 6 | Note 6 |
| Cell ID | - | Note 7 | Note 7 |
| Note 1: Shall depend upon the NPDSCH scheduling Note 2: Only apply for subframes scheduled with NPDSCH. Note 3: 72 for subframes scheduled with NPDSCH when mod 2 ≠ 0. Otherwise 0. Note 4: only apply for subframes scheduled with NPDSCH.. Note 5: 1 for subframes scheduled with NPDSCH when mod 2 ≠ 0. Otherwise 0. Note 6: Maximum number of repetitions shall depend upon the test configuration. Note 7: Cell ID shall depend upon the test configuration. |  |  |  |

  - The existing NPDCCH reference channel parameters specified for NB-IoT NTN FDD for standalone mode can also be applicable for IoT NTN TDD mode

  - Update the OCNG parameters based on Table A.3.2.3.3.1 as follows:

Table X.3.2.3.3-1: NOP.3 FDD: OCNG FDD Pattern 3

| Allocation | Relative power level [dB] |  |  | NPDCCH and NPDSCH Data |
| --- | --- | --- | --- | --- |
|  | Subframe |  |  |  |
|  | 0, 4 | 5, 9 | 1-3, 6-8 |  |
| 0 | 0 (Note 2) | 0 (Note 2) | 0 | Note 1 |
| Note 1: This physical resource block is assigned to a virtual UE for transmission of NPDCCH or NPDSCH; the data transmitted over the NOCNG NPDCCH or NPDSCH shall be uncorrelated QPSK modulated pseudo random data. The parameter is used to scale the power of NPDCCH and NPDSCH. If two transmit antennas with NBRS are used in the test, the NPDCCH and NPDSCH parts of NOCNG shall be transmitted to the virtual users by both transmit antennas with NBRS and according to the antenna transmission mode 2. The parameter applies to each antenna port separately, so the transmit power of the NPDCCH and NPDSCH parts of NOCNG is equal between both the transmit antennas with NBRS used in the test. The antenna transmission modes are specified in clause 7.1 in TS 36.213. Note 2: Value of is applicable to PRBs not used for transmission of NPSS, NSSS and NPBCH in anchor cell (= 0). Note 3: SI transmissions, and NPDCCH and NPDSCH for the UE under test, have precedence over NOCNG. |  |  |  |  |

  - Update the NPRACH reference configurations based on Table A.3.18-1 as follows:

Table X.3.18-1: NPRACH.R-1: HD-FDD Reference NPRACH Configuration

| Field | Value |  |  | Comment |  |
| --- | --- | --- | --- | --- | --- |
| Parameters not per NPRACH coverage level |  |  |  |  |  |
| rsrp-ThresholdsPrach | {rsrp1, rsrp2} |  |  | The values of NPRACH RSRP thresholds for will be set according the requirement of individual test cases |  |
| nprach-CP-Length | us66dot7 |  |  | NPRACH format 0 |  |
| Parameters per NPRACH coverage Level |  |  |  |  |  |
| CE Level | Level 0 | Level 1 | Level 2 |  | Valid values as defined in TS 36.331 [2] |  |
| nprach-Periodicity | ms640ms40 | ms640ms40 | ms640ms40 |  | {ms40, ms80, ms160, ms240, ms320, ms640, ms1280, ms2560} |  |
| nprach-StartTime | ms8 | ms8 | ms8 |  | {ms8, ms16, ms32, ms64, ms128, ms256, ms512, ms1024} |  |
| nprach-SubcarrierOffset | n0 | n0 | n0 |  | {n0, n12, n24, n36, n2, n18, n34} |  |
| nprach-NumSubcarriers | n12 | n12 | n12 |  | {n12, n24, n36, n48} |  |
| nprach-SubcarrierMSG3-RangeStart | {one} | {one} | {one} |  | {zero, oneThird, twoThird, one} |  |
| maxNumPreambleAttemptCE | n3 | n5 | n7 |  | {n3, n4, n5, n6, n7, n8, n10} |  |
| numRepetitionsPerPreambleAttempt | n1 | n8 | n16n32 |  | {n1, n2, n4, n8, n16, n32, n64, n128} |  |
| npdcch-NumRepetitions-RA | r1 | r8 | r16r32 |  | {r1, r2, r4, r8, r16, r32, r64, r128, r256, r512, r1024, r2048} |  |
| npdcch-StartSF-CSS-RA | v64v8 | v32v2 | V64v2 |  | {v1dot5, v2, v4, v8, v16, v32, v48, v64} |  |
| npdcch-Offset-RA | zero | zero | zero |  | {zero, oneEighth, oneFourth, threeEighth} |  |
| nprach-NumCBRA-StartSubcarriers | n8 | n8 | n8 |  | {n8, n10, n11, n12, n20, n22, n23, n24, n32, n34, n35, n36, n40, n44, n48} |  |
| Note 1: See Clause 6.7.3.2 in TS 36.331 [2] for further information on the parameters in this table. |  |  |  |  |  |

#### Issue 4-2-2: Test case list

Proposals from companies:

R4-2513639	MediaTek inc.

- Proposal 1: Work split for the RRM tests cases in the provided list.

| Test case | Core requirement | Description | Reference |
| --- | --- | --- | --- |
| Re-selection | 4.6B | Intra-frequency case, in normal coverage | A.13.1.1 |
|  |  | Inter-frequency case, in normal coverage | A.13.1.4 |
| RRC Re-establishment | 6.5B | Intra-frequency RRC Re-establishment, under normal coverage | A.13.3.1 |
| Random Access | 6.6B | Contention Based Random Access, in normal coverage | A.13.3.2.1 |
| UE transmit timing | 7.20B | UE Transmit Timing Accuracy, in normal coverage | A.13.4.1.1 |
| Timing Advance | 7.22B | UE Transmit Timing Advance Adjustment Accuracy, in normal coverage | A.13.4.2.1 |
| Radio Link Monitoring | 7.23B | out-of-sync / in-sync test, with/without DRX, 4 tests in total | A.13.4.3.1/4/5/7 |
| Measurements | 8.14B | Intra-frequency neighbour cell measurement, in normal coverage | A.13.5.1 |
|  |  | Inter-frequency neighbour cell measurement, in normal coverage | A.13.5.2 |
| Downlink channel quality reporting accuracy | TBA | Downlink channel quality reporting accuracy, in normal coverage | A.13.6.2.1 |
|  |  | Downlink channel quality reporting accuracy on non-anchor carrier, in normal coverage | A.13.6.2.4 |

R4-2513826	Iridium, CCL, THALES

- Proposal #20: Following list of test cases from the existing NB-IoT NTN FDD mode are applicable for IoT NTN TDD:

- A.13.1.1.1 HD – FDD and IoT NTN TDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage

- A.13.1.1.3 HD – FDD and IoT NTN TDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage with UE specific DRX

- A.13.1.1.4 HD – FDD and IoT NTN TDD Inter frequency case for UE Category NB1 Standalone mode in normal coverage with UE specific DRX

- A.13.3.1.1 HD-FDD and IoT NTN TDD Intra-frequency RRC Re-establishment for UE category NB1 in Standalone mode under normal coverage

- A.13.3.2.1 Contention Based Random Access Test for UE category NB1 UEs in Satellite Access - Standalone mode in normal coverage

- A.13.4.1.1 E-UTRAN HD-FDD and IoT NTN TDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under normal coverage for Satellite Access

- A.13.4.2.1 HD-FDD and IoT NTN TDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Normal Coverage for Satellite Access

- A.13.4.3.1 HD-FDD and IoT NTN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 Standalone mode in normal coverage

- A.13.4.3.4 HD-FDD and IoT NTN TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 Standalone mode in Normal Coverage

- A.13.4.3.5 HD-FDD and IoT NTN TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage

- A.13.4.3.7 HD-FDD and IoT NTN TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage

- A.13.5.1 HD-FDD and IoT NTN TDD Intra-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage for Satellite Access

- A.13.5.2 HD-FDD and IoT NTN TDD Inter-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage for Satellite Access

- A.13.5.3 HD-FDD and IoT NTN TDD Intra-frequency location-based neighbour cell measurement for UE category NB1 in standalone mode under normal coverage for Satellite Access

- A.13.6.2.1 E-UTRAN HD-FDD and IoT NTN TDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under normal coverage

- A.13.6.2.3 E-UTRAN HD-FDD and IoT NTN TDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under normal coverage

- A.13.6.2.5 E-UTRAN HD-FDD and IoT NTN TDD Downlink channel quality reporting accuracy in RRC_CONNECTED for UE Category NB1 Standalone mode under normal coverage

R4-2514182	Huawei, HiSilicon

- Proposal 3: RAN4 to define the following TCs for IoT NTN TDD.

- Intra-frequency cell reselection

- Inter-frequency cell reselection

- Re-establishment

- Random access

- Transmit timing and TA accuracy

- RLM OOS and RLM IS, both without DRX and with DRX

- Intra-frequency neighbor cell measurement, with radio condition trigger and with location trigger

- Inter-frequency neighbor cell measurement with radio condition trigger

- Channel quality reporting accuracy, in both Msg3 and connected mode

Recommended WF:

- Following list of test cases from the existing NB-IoT NTN FDD mode are applicable for IoT NTN TDD:

| Test case | Description | Volunteer |
| --- | --- | --- |
| Re-selection | A.13.1.1.1 HD – FDD and IoT NTN TDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage A.13.1.1.3 HD – FDD and IoT NTN TDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage with UE specific DRX |  |
|  | A.13.1.1.4 HD – FDD and IoT NTN TDD Inter frequency case for UE Category NB1 Standalone mode in normal coverage with UE specific DRX |  |
| RRC Re-establishment | A.13.3.1.1 HD-FDD and IoT NTN TDD Intra-frequency RRC Re-establishment for UE category NB1 in Standalone mode under normal coverage |  |
| Random Access | A.13.3.2.1 Contention Based Random Access Test for UE category NB1 UEs in Satellite Access - Standalone mode in normal coverage |  |
| UE transmit timing | A.13.4.1.1 E-UTRAN HD-FDD and IoT NTN TDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under normal coverage for Satellite Access |  |
| Timing Advance | A.13.4.2.1 HD-FDD and IoT NTN TDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Normal Coverage for Satellite Access |  |
| Radio Link Monitoring | - A.13.4.3.1 HD-FDD and IoT NTN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 Standalone mode in normal coverage - A.13.4.3.4 HD-FDD and IoT NTN TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 Standalone mode in Normal Coverage - A.13.4.3.5 HD-FDD and IoT NTN TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage - A.13.4.3.7 HD-FDD and IoT NTN TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage |  |
| Measurements | A.13.5.1 HD-FDD and IoT NTN TDD Intra-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage for Satellite Access |  |
|  | A.13.5.2 HD-FDD and IoT NTN TDD Inter-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage for Satellite Access |  |
|  | - A.13.5.3 HD-FDD and IoT NTN TDD Intra-frequency location-based neighbour cell measurement for UE category NB1 in standalone mode under normal coverage for Satellite Access |  |
| Downlink channel quality reporting accuracy | A.13.6.2.1 E-UTRAN HD-FDD and IoT NTN TDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under normal coverage A.13.6.2.3 E-UTRAN HD-FDD and IoT NTN TDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under normal coverage A.13.6.2.5 E-UTRAN HD-FDD and IoT NTN TDD Downlink channel quality reporting accuracy in RRC_CONNECTED for UE Category NB1 Standalone mode under normal coverage |  |

#### Issue 4-2-3: Test configuration - General

Proposals from companies:

R4-2513639	MediaTek inc.

- Proposal 2: Only NGSO configuration is applicable.

R4-2514344	Nokia

- Proposal 2: For existing NB-IoT NTN test cases, create a new test configuration with the new reference channel models and reuse the test description. The test requirements when the new configuration is used might be adapted to reflect the agreements of the core requirements.

Recommended WF:

- Only NGSO configuration is applicable.

- For existing NB-IoT NTN test cases, create a new test configuration with the new reference channel models and reuse the test description. The test requirements when the new configuration is used might be adapted to reflect the agreements of the core requirements.

#### Issue 4-2-4: Principle of testing TDD and FDD frame structure

Proposals from companies:

R4-2514344	Nokia

- requirements.

- Proposal 3: RAN4 to agree which tests needs to be tested for each configuration when the UE supports both TDD and FDD frame structures, to avoid the duplication of test effort.

Recommended WF:

- Further discuss the principle of testing TDD and FDD frame structure.

  - Option 1: A UE capable of both TDD and FDD, the UE is required to pass either TDD or FDD per the DUT’s declaration.

  - Option 2: define a test case list to be tested for each configuration for the UE that supports both TDD and FDD frame structures

  - Other options are not precluded.

#### Issue 4-2-5: Test configuration for random access test

Proposals from companies:

R4-2513826	Iridium, CCL, THALES

- Proposal #21: The highlighted NPRACH procedure test configuration parameters from Table A.13.3.2.1.1-2 should be updated for IoT NTN TDD mode. Companies are encouraged to propose suitable values.

Table X.13.3.2.1.1-3: NPRACH-Configuration parameters for HD-FDD contention based random access test for UE category NB1 Standalone mode in Normal Coverage

| Field | Value |  |  |  | Comment |  |
| --- | --- | --- | --- | --- | --- | --- |
| Parameters not per NPRACH resource |  |  |  |  |  |  |
| RSRP-ThresholdsNPRACH-InfoList | {39, 24} |  |  |  | Corresponding to {-101, -116} dBm as defined in Table 9.1.4-1 |  |
| nprach-CP-Length | us66dot7 |  |  |  |  |  |
| nrs-Power | -5 dBm/15 kHz |  |  |  | As defined in clause 6.7.3 in TS 36.331. |  |
| Backoff Parameter Index | 1 |  |  |  | As defined in table 7.2-2 in TS 36.321 |  |
| Configured UE transmitted power () | 23 dBm for power class 3, 20 dBm for power class 5 |  |  |  | As defined in clause 6.2B.4 in TS 36.102 |  |
| powerRampingStep | dB2 |  |  |  |  |  |
| preambleInitialReceivedTargetPower | dBm-112 |  |  |  |  |  |
| preambleTransMax-CE | n6 |  |  |  |  |  |
| Parameters per NPRACH Resource |  |  |  |  |  |  |
| NPRACH Resource | Level 0 | Level 1 | Level 2 |  |  |
| nprach-Periodicity | ms40 | ms240 | ms1280 |  |  |
| nprach-StartTime | ms8 | ms64 | ms512 |  |  |
| nprach-SubcarrierOffset | n0 | n0 | n0 |  |  |
| nprach-NumSubcarriers | n12 | n12 | n12 |  |  |
| nprach-SubcarrierMSG3-RangeStart | zero | zero | zero |  |  |
| maxNumPreambleAttemptCE | n3 | n6 | n10 |  |  |
| numRepetitionsPerPreambleAttempt | n2 | n8 | n64 |  |  |
| npdcch-NumRepetitions-RA | r4 | r16 | r128 |  |  |
| npdcch-StartSF-CSS-RA | v1dot5 | v1dot5 | v1dot5 |  |  |
| npdcch-Offset-RA | zero | zero | Zero |  |  |
| nprach-NumCBRA-StartSubcarriers | n8 | n8 | n8 |  |  |
| ra-ResponseWindowSize (per NPRACH Resource) | pp2 | pp2 | pp2 |  |  |
| mac-ContentionResolutionTimer (per NPRACH Resource) | pp8 | pp8 | pp8 |  |  |
| Note 1: See Clause 6.7.3 in TS 36.331 for further information on the parameters in this table. |  |  |  |  |  |  |

Recommended WF:

- Update the NPRACH procedure test configuration parameters, based on Table A.13.3.2.1.1-2.

- FFS the suitable values.

#### Issue 4-2-6: Test configuration for timing test

Proposals from companies:

R4-2513826	Iridium, CCL, THALES

- Proposal #22: The highlighted timing advance accuracy test configuration parameters from Table A.13.4.2.1.1-2 should be updated for IoT NTN TDD mode. Companies are encouraged to propose suitable values.

Table X.13.4.2.1.1-2: General Test Parameters for E-UTRAN Timing Advance Accuracy Test for UE Category NB1 in Standalone Mode under Normal Coverage for Satellite Access

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NB-IoT operational mode |  |  | Standalone |  |
| CP Length |  |  | Normal |  |
| Satellite information | Config 1 |  | SSC.1 |  |
|  | Config 2 |  | SSC.2 |  |
| Timing Advance Command (TA) value during T1 |  |  | 31 | NTA = 0 for the purpose of establishing a reference value from which the timing advance adjustment accuracy can be measured during T2 |
| Timing Advance Command (TA) value during T2 |  |  | 39 | NTA = 128 |
| Number of repetitons | NPDCCH |  | 128 | Smaller repetition values for NTN TDD |
|  | NPDSCH |  | 128 |  |
|  | NPUSCH |  | 32 |  |
| DRX |  |  | OFF |  |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |

Recommended WF:

- Update the timing advance accuracy test configuration parameters, based on Table A.13.4.2.1.1-2.

- FFS the suitable values.

#### Issue 4-2-7: Test configuration for RLM test

Background: WF in R4#116 core requirement

- Rmax

  - For the RLM requirement definition, the applicable Rmax should be limited to [2]. Additionally, the ‘Maximum NPDCCH Repetition Level’ should be modified from Rmax/4 in the existing NB-NTN requirements to Rmax/2.

Proposals from companies:

R4-2513826	Iridium, CCL, THALES

- Proposal #23: The highlighted radio link monitoring test for out-of-sync in DRX test configuration parameters from Table A.13.4.3.1.1-2 should be updated for IoT NTN TDD mode. Companies are encouraged to propose suitable values.

- Table A.13.4.3.1.1-2: General test parameters for HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 Standalone mode in normal coverage

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | nCell 1 |  |
| CP length |  |  | Normal |  |
| Deployment Mode |  |  | Standalone |  |
| Satellite information | Config 1 |  | SSC.1 | GSO |
|  | Config 2 |  | SSC.2 | NGSO |
| NPDCCH transmission parameters Rmax |  |  | 8 | Other NPDCCH parameters are defined in “ out-of-sync” column in Table 7.23A.2-1 |
| DRX cycle |  | ms | 256 | See Table A.13.4.3.1.1-4 |
| Layer 3 filtering Note 2,3 |  |  | Enabled | Counters: N310 = 1 N311 = 1 |
| T310 timer Note 2,3 |  | ms | 0 | T310 is disabled |
| T311 timer Note 2,3 |  | ms | 1000 | T311 is enabled |
| T1 |  | s | 5.12 |  |
| dT |  | S | 0.8 |  |
| T2 |  | s | 10.24 |  |
| dT |  | S | 0.7 |  |
| T3 |  | s | 5.12 |  |
| dT |  | S | 1.4 |  |
| T4 |  | s | 5.12 |  |
| Note 1: NPDCCH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel. Note 2: N310, N311, T310 and T311 are defined in TS 36.331. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. |  |  |  |  |

R4-2514182	Huawei, HiSilicon

- Proposal 4: RAN4 to discuss the Qin and Qout for RLM test cases based on Rmax =2.

Recommended WF:

- RAN4 to discuss the Qin and Qout for RLM test cases based on Rmax =2.