# 3GPP TSG-RAN WG4 Meeting #116bis	R4-2514477

# Prague, Czech Republic, Oct. 13-17, 2025

Title: 	Further discussion on open issues related to the implementation of framework simplification for co-location/co-existence requirements

Agenda Item:	11.1

Source: 	Huawei, HiSilicon

Document for:	Discussion

# 1	Introduction

During previous meeting, Way-Forward for RAN Task on specification improvements was agreed in R4-2512581.

In this contribution, we provide further discussion on open issues related to the implementation of framework simplification for co-location/co-existence requirements.

# 2	Discussion

Based on the baseline content as captured in the WF in R4-2512581, below we present corrections (in Track Changes) which are required to be implemented in in all Draft CRs which are part of this RAN task.

-----------------------------------------------------------------------------------------------------------------------

Transmitter co-existence spurious emission requirement in 37.104, clause 6.6.1.3.1:

| System type to co-exist with (NOTE 8) | Frequency range for co-existence requirement (MHz) (NOTE 9) | Basic limits (dBm) | Measurement bandwidth (MHz) | Notes |
| --- | --- | --- | --- | --- |
| GSM850 or CDMA850 | 869 - 894 | -57 | 0,1 | NOTE 1, NOTE 3 |
|  | 824 - 849 | -61 |  |  |
| GSM900 | 921 - 960 | -57 |  |  |
|  | 876 - 915 | -61 |  |  |
| DCS1800 | 1805 - 1880 | -47 |  |  |
|  | 1710 - 1785 | -61 |  |  |
| PCS1900 | 1930 - 1990 | -47 |  |  |
|  | 1850 - 1910 | -61 |  |  |
| Other 3GPP system type | Frequency range of downlink operating band of the BS to co-exist with | -52 | 1 | NOTE 1, NOTE 3 |
|  | Frequency range of uplink operating band of the BS to co-exist with | -49 |  | NOTE 1, NOTE 3, NOTE 5, NOTE 6, NOTE 7 |

NOTE 1:	As defined in the scope for spurious emissions in this subclause, except for the cases where the noted requirements apply to a BS operating in Band 25, Band 27, Band 28 or Band 29, the co-existence requirements in Table 6.6.1.3.1-1 do not apply for the 10 MHz frequency range immediately outside the downlink operating band (see Tables 4.5-1 and 4.5-2). Emission limits for this excluded frequency range may be covered by local or regional requirements.

NOTE 2:	Table 6.6.1.3.1-1 assumes that two operating bands, where the frequency ranges in Table 4.5-1 or Table 4.5-2 would be overlapping, are not deployed in the same geographical area. For such a case of operation with overlapping frequency arrangements in the same geographical area, special co-existence requirements may apply that are not covered by the 3GPP specifications.

NOTE 3:	For the protection of DCS1800, UTRA Band III, E-UTRA Band 3 or NR Band n3 in China, the frequency ranges of the downlink and uplink protection requirements are 1805 – 1850 MHz and 1710 – 1755 MHz respectively.

NOTE 4:	TDD base stations deployed in the same geographical area, that are synchronized and use the same or adjacent operating bands can transmit without additional co-existence requirements. For unsynchronized base stations (except in Band 46), special co-existence requirements may apply that are not covered by the 3GPP specifications.

NOTE 5:	For Band 28 BS, specific solutions may be required to fulfil the spurious emissions limits for BS for co-existence with Band 27 UL operating band, where requirement applies 4 MHz above the Band n28 downlink operating band.

NOTE 6:	For Band 29 BS, specific solutions may be required to fulfil the spurious emissions limits for BS for co-existence with UTRA Band XII or E-UTRA Band 12 or NR Band n12 UL operating band or E-UTRA Band 17 UL operating band or E-UTRA Band 85 UL operating band, where requirement applies 1 MHz below the Band 29 downlink operating band.

NOTE 7:	For Band n67 BS, specific solutions may be required to fulfil the spurious emissions limits for NR BS co-existence with E-UTRA Band 28 or NR Band n28 UL operating band or NR Band n83 UL operating band, where requirement applies for 703 MHz to 736 MHz.

NOTE 8:	Does not apply for co-existence with standalone downlink bands (SDO) defined in TS 36.104 [x], table 5.5-1.

NOTE 9:	Frequency range of UTRA, E-UTRA and NR operating bands, as described in TS 37.104 [x], annex X.

Transmitter co-location spurious emission requirement in TS 37.104, clause 6.6.1.4.1:

| Frequency range of uplink operating band of the co-located BS (NOTE 5) | System type to co-locate with (NOTE 4) | Basic limits (dBm/100kHz) |  |  | Notes |
| --- | --- | --- | --- | --- | --- |
|  |  | WA BS | MR BS | LA BS |  |
| 824 - 849 MHz | GSM850 or CDMA850 | -98 | -91 | -70 | NOTE 1 |
| 876 - 915 MHz | GSM900 | -98 | -91 | -70 |  |
| 1710 - 1785 MHz | DCS1800 | -98 | -91 | -80 |  |
| 1850 - 1910 MHz | PCS1900 | -98 | -91 | -80 |  |
| UL frequency range of n51, n91, n93 | NR | N/A | N/A | -88 |  |
| UL frequency range of n53 | NR | N/A | -91 | -88 |  |
| UL frequency range of n96, n102 | NR | N/A | -90 | -87 |  |
| UL frequency range of n104 | NR | -95 | -90 | -87 |  |
| Frequency range of uplink operating band of the co-located BS | Other | -96 | -91 | -88 |  |

NOTE 1:	As defined in the scope for spurious emissions in this subclause, the co-location requirements in Table 6.6.1.4.1-1 do not apply for the ΔfOBUE frequency range immediately outside the BS transmit frequency range of a downlink operating band (see Tables 4.5-1 and 4.5-2). The current state-of-the-art technology does not allow a single generic solution for co-location with other system on adjacent frequencies for 30 dB BS-BS minimum coupling loss. However, there are certain site-engineering solutions that can be used. These techniques are addressed in TR 25.942 [7].

NOTE 2:	Table 6.6.1.4.1-1 assumes that two operating bands, where the corresponding BS transmit and receive frequency ranges in Table 4.5-1 or Table 4.5-2 would be overlapping, are not deployed in the same geographical area. For such a case of operation with overlapping frequency arrangements in the same geographical area, special co-location requirements may apply that are not covered by the 3GPP specifications.

NOTE 3:	Co-located TDD base stations that are synchronized and using the same or adjacent operating band can transmit without special co-locations requirements. For unsynchronized base stations, special co-location requirements may apply that are not covered by the 3GPP specifications.

NOTE 4:	Does not apply for co-location with V2X and SDO operating bands, as defined in TS 36.104 [x], table 5.5-1.

NOTE 5:	Frequency range of UTRA, E-UTRA and NR operating bands, as described in TS 37.104 [x], annex X.

Co-location receiver blocking requirement in TS 37.104, clause 7.5.2:

Table 7.5.2-1: Blocking requirement for co-location with BS in other frequency bands

| Frequency range of interfering signal | Wanted signal mean power (dBm) | Interfering signal mean power for WA BS (dBm) | Interfering signal mean power for MR BS (dBm) | Interfering signal mean power for LA BS (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| Frequency range of co-located downlink operating band | PREFSENS + x dB (Note 1) | +16 | +8 | -6 | CW carrier |
| NOTE 1: PREFSENS depends on the RAT, the BS class and the channel bandwidth, see subclause 7.2. "x" is equal to 3 in case of GSM/EDGE wanted signal and equal to 6 in case of NR or UTRA or E-UTRA or NB-IoT wanted signals. NOTE 2: Except for a BS operating in Band 13, these requirements do not apply when the interfering signal falls within any of the supported uplink operating band or in the ΔfOOB immediately outside any of the supported uplink operating band. For a BS operating in band 13 the requirements do not apply when the interfering signal falls within the frequency range 768-797 MHz. NOTE 3: Some combinations of bands may not be possible to co-site based on the requirements above. The current state-of-the-art technology does not allow a single generic solution for co-location of UTRA TDD or E-UTRA TDD or NR TDD with E-UTRA FDD or NR FDD on adjacent frequencies for 30dB BS-BS minimum coupling loss. However, there are certain site-engineering solutions that can be used. These techniques are addressed in TR 25.942 [7]. NOTE 4: In China, the blocking requirement for co-location with DCS1800 and Band III BS is only applicable in the frequency range 1805-1850MHz. NOTE 5: For a BS operating in band 11, 21, 74, the requirement for co-location with Band 32 applies for interfering signal within the frequency range 1475.9-1495.9 MHz. NOTE 6: Co-located TDD base stations that are synchronized and using the same or adjacent operating band can receive without special co-location requirements. For unsynchronized base stations, special co-location requirements may apply that are not covered by the 3GPP specifications. NOTE 7: For NB-IoT, up to 24 exceptions are allowed for spurious response frequencies in each wanted signal frequency when measured using a 1MHz step size. For these exceptions the above throughput requirement shall be met when the blocking signal is set to a level of -40 dBm for 15 kHz subcarrier spacing and -46 dBm for 3.75 kHz subcarrier spacing. In addition, each group of exceptions shall not exceed three contiguous measurements using a 1MHz step size. |  |  |  |  |  |

# 3	Conclusions

Based on the above discussion, the following proposals were formulated:

Proposal 1: Modifications presented in Track Changes shall be reflected in all Draft CRs which are part of this RAN task.

During previous discussions, TS 36.143 (	Evolved Universal Terrestrial Radio Access (E-UTRA); FDD repeater conformance testing) was requested to be included in this task. This was motivated by related ETSI MSG TFES work on the requested update of the E-UTRA Repeater Harmonized Standard (ETSI EN 301 908-15) to release 17. Its mature draft was already provided in TFES. It shall be noted, that there are no future updates planned for that Harmonized Standard. Therefore, RAN4 work on TS 36.143 updates is also questionable. Therefore, it is proposed to remove TS 36.143 from this RAN task, due to lack of interest.

Proposal 2: Remove TS 36.143 from this RAN task and keep legacy tables.