# 3GPP TSG-RAN WG4 Meeting #116-bis	R4-2514808

Prague, Czech Republic, Oct. 13-17, 2025

Agenda item:			6.15.1

Source:	Ericsson

Title:	WF on Netw_Energy_NR_enh_RRM_Part1

Document for:	Approval

# Topic #1: On-demand SSB(OD-SSB) performance part (AI 6.15.2.1)

Sub-topic 1-1: Test scope

Issue 1-1-1: General – Case 1 and Case 2

Agreement:

- The test cases should include deactivation SCell measurement, SCell activation procedure and the L1/L3 measurement for activated SCell.

  - Case #1: scenario #2, scenario #2A.

  - Case #2: scenario #2, scenario #2A and scenario #3B.

Issue 1-1-2: General – OD-SSB parameter update (scenario #3B)

Close the issue.

Issue 1-1-3: General – Different OD-SSB and AO-SSB relation in Case 2

Agreement:

RAN4 to define TCs for Alt Time-C1. Meanwhile,

- Option 1: Not define TC for Alt Time-C2

- Option 2: Select one test for Alt Time-C2. For UE supporting Alt Time-C2, the corresponding test case for Alt Time-C1 can be skipped.

Further discuss in one test case or separate test cases.

Issue 1-1-4: Scenario – Scell activation scope

Agreement:

RAN4 to only define the test cases for single Scell activation, Direct SCell activation.

Issue 1-1-5: Scell activation scope – known/unknown

Close the issue.

Issue 1-1-6: T2 for deactivated SCell measurement

Agreement:

Define one test case in case 1 to verify the UE measurement behaviour when the OD-SSB is reactivated relevant to T2.

Issue 1-2-1: Test case list

Agreement

- Interruption Test Case

The interruption test for OD-SSB activation/deactivation and during measurement on deactivated NR SCC are merged to SCell activation and deactivated SCell measurement test cases.

| No. | Test category | Test scenario | Test configuration | Test purpose | Company |
| --- | --- | --- | --- | --- | --- |
| TC1 | SCell Activation and deactivation of unknown SCell in DRX based on OD-SSB | SCell activation Interruption at OD-SSB activation/deactivation | OD-SSB periodicity 10ms | OD-SSB Case 1 OD-SSB scenario 2a Unknown SCell activation FR1 and FR2 | Nokia |
| TC2 | SCell Activation and deactivation of unknown SCell in DRX based on OD-SSB | SCell activation Interruption at OD-SSB activation/deactivation | OD-SSB periodicity 10ms | OD-SSB Case 1 OD-SSB scenario 2a Unknown SCell activation FR2 | Huawei |
| TC3 | SCell Activation and deactivation of unknown SCell in DRX based on OD-SSB | SCell activation Interruption at OD-SSB activation/deactivation | OD-SSB periodicity 10ms AO-SSB periodicity 20ms | OD-SSB Case 2 Alt Time-C1 - OD-SSB(5ms) is overlapping with AO-SSB(20ms) OD-SSB scenario 2a Unknown SCell activation FR1 | Ericsson |
| TC4 | SCell Activation and deactivation of known SCell in non-DRX based on OD-SSB | SCell activation Interruption at OD-SSB activation/deactivation | OD-SSB periodicity 20ms AO-SSB periodicity 20ms | OD-SSB Case 2 Alt Time-C1 - The union of OD-SSB and AO-SSB meets the legacy SSB periodicity OD-SSB scenario 2a known SCell activation FR1 | Samsung |
| TC5 | SCell Activation and deactivation of known SCell in non-DRX based on OD-SSB | SCell activation Interruption at OD-SSB activation/deactivation | OD-SSB periodicity 20ms AO-SSB periodicity 20ms | OD-SSB Case 2 Alt Time-C1 - The union of OD-SSB and AO-SSB meets the legacy SSB periodicity OD-SSB scenario 2a known SCell activation FR2 | Xiaomi |
| TC6 | Direct SCell activation at SCell addition based on OD-SSB | Direct SCell activation | OD-SSB periodicity 20ms AO-SSB periodicity 80ms | OD-SSB Case 2 Alt Time-C1 OD-SSB scenario 2a FR1 | CMCC |
| TC7 | Intra-frequency measurement(deactivated SCell) based on OD-SSB | Deactivated SCell measurement; Interruption during deactivated SCell measurement | OD-SSB periodicity 10ms | OD-SSB Case 1 FMW; UE behaviour relevant to T2 FR1 | OPPO |
| TC8 | Intra-frequency measurement(deactivated SCell) based on OD-SSB | Deactivated SCell measurement; Interruption during deactivated SCell measurement | OD-SSB periodicity 20ms AO-SSB periodicity 20ms | OD-SSB Case 2 FMW FR2 | vivo |
| TC9 | Intra-frequency measurement- SA event triggered reporting tests without gap for SCell under non DRX based on OD-SSB | L3-RSRP measurement | OD-SSB periodicity 10ms | OD-SSB scenario 3B; OD-SMTC and SMTC FR1 | LG |
| TC10 | Intra-frequency measurement- SA event triggered reporting tests without gap under non DRX based on OD-SSB | L3-RSRP measurement | OD-SSB periodicity 20ms AO-SSB periodicity 20ms | OD-SSB scenario 3B; OD-SMTC and SMTC FR2 FFS FR1 | CATT |
| TC11 | SSB based L1-RSRP measurement when DRX is not used based on OD-SSB | L1-RSRP measurement | OD-SSB periodicity 20ms -> 10ms | OD-SSB scenario 3B; Scheduling restriction to AO-SSB occasions; L1-RSRP based on EMP FR1 | ZTE |

FFS the following test cases.

- SCell activation

| No. | Test category | Test scenario | Test configuration | Test purpose |
| --- | --- | --- | --- | --- |
| FFS | SCell Activation and deactivation of known SCell in non-DRX based on OD-SSB | SCell activation Interruption at OD-SSB activation/deactivation | OD-SSB periodicity 10ms | OD-SSB Case 1 OD-SSB scenario 2 and 2a Known SCell activation FR1 |

- Direct SCell Activation

| No. | Test category | Test scenario | Test configuration | Test purpose |
| --- | --- | --- | --- | --- |
| FFS | Direct SCell activation at SCell addition based on OD-SSB | Direct SCell activation | OD-SSB periodicity 10ms | OD-SSB Case 1 OD-SSB scenario 2a FR1 |

- Deactivated SCell L3 measurement

| No. | Test category | Test scenario | Test configuration | Test purpose |
| --- | --- | --- | --- | --- |
| FFS | Intra-frequency measurement(deactivated SCell) based on OD-SSB | Deactivated SCell measurement; Interruption during deactivated SCell measurement | OD-SSB periodicity 20ms AO-SSB periodicity 20ms | OD-SSB Case 2 FMW FFS FR1 |

Issue 1-3-1: General principle

Agreement:

RAN4 to apply the following principles for OD-SSB test

- Both FR1 and FR2

- Test non-DRX mode, and select one test case with DRX configuration for deactivated SCell measurement.

- The test without SSB time index detection

- Follow the legacy test methodology to use L3 event trigger reporting and L1-RSRP reporting

- Only NCD-SSB for OD-SSB test case

# Topic #2: Maitenence (AI 4.21.1)

Issue 2-1-3: OD-SIB1 agreement clarification

Agreement

- Keep the current requirements of Tdetect,NR, Tmeasure,NR and Tevaluate,NR, if the target cell is a NES Cell supporting OD-SIB1

Issue 2-1-4: Terminology

Agreement

- RAN4 to clarify the definition of first SSB and OD-SSB in TS38.133 Terminology based on RAN2 signalling.

Issue 2-1-6: The number of od-ssb-nrofBurst

Agreement

- There is no action from RAN4 for this issue. Close the issue.

Way forward:

Open issues captured in Topic summary R4-2513514 will continue to be discussed in maintenance stage.

- Issue 2-1-5 for interruption due to OD-SSB is included.

  - The relevant specification will be updated if needed.